| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications for 16m direct handover (CR for IEEE Std 802.16-2009) |  |
| Date Submitted | 2011-05-10 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Seho Kim Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | A change request according to IEEE 802.16-06/046 (IEEE Std 802.16-2009) |  |
| Abstract | Changes are provided to clarify the handover operation from 16e BS to 16m only ABS, or vice versa in 802.16-2009 specification. |  |
| Purpose | Approve and add to Maintenance TG database |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications for 16m direct handover (CR for IEEE Std 802.16-2009)

Hyunjeong Kang, Yeongmoon Son, Seho Kim

Samsung Electronics

- Problem statement

Considering the mixed deployment of 16e BS and 16m only ABS, the handover from 16e BS to 16m only ABS, i.e., direct handover is required. Direct handover from 16e BS to 16m only ABS, or vice versa is supported in IEEE P802.16m standard, but there need clarifications to implement the direct handover.

There is no information of 16m only ABS in MOB_NBR-ADV message, so AMSs in 16e BS don’t know the presence of neighbor 16m only ABS. If an AMS wants to handover to neighbor 16m only ABS, the AMS needs to measure the signal strength of neighbor 16m only ABS by itself, which is called as “blind scanning” in IEEE P802.16m/D12. It takes long time to obtain target ABS information by blind scanning so that handover performance cannot be guaranteed in direct handover from 16e BS to 16m only ABS.

In other case where an AMS performs handover from 16m only ABS to 16e BS, the AMS obtains partial information of neighbor 16e BS through an AAI-NBR-ADV message in 16m only ABS. However the partial system information does not reduce handover interruption time because the AMS has to wait until it receives DCD or UCD message in the 16e BS.

Hence, following items should be implemented in the specification to support direct HO from 16e BS to 16m only ABS or from 16m only ABS to 16e BS;

- Full information (DL channel information + S-SFH SP1/SP2/SP3) of 802.16m ABS is provided in 802.16e BS by using a new message (AAI_NBR-INFO message). 16e BS broadcasts AAI_NBR-INFO of 16m only ABS periodically. When AAI_NBR-INFO is fragmented, the same fragmentation method for MOB_NBR-ADV is applied.

- 16m AMS scans 16m only ABS through coordinated scanning (MOB_SCN-REQ/RSP/REP). Add new TLVs for N_Recommended_16m ABS_Index to present 16m ABS in AAI_NBR-INFO and relevant information for scanning.

- 16m AMS decides its handover to 16m only ABS through controlled handover (MOB_MSHO-REQ/MOB_BSHO-REQ/MOB_BSHO-RSP/MOB_HO-IND) or uncontrolled handover. Add new TLVs for CCC (configuration change count) for AAI_NBR-INFO, Neighbor_BS_Index and Preamble index to present 16m ABS.

- During network reentry with a target 16e BS, HO process optimization TLV in a RNG-RSP message is used to indicate that SBC/REG transactions are required and PKM transaction/IP refresh are omissible.

- ARQ parameters in IEEE P802.16m standard are different from those of 802.16e. If ARQ operation has been applied for service flow(s) of an AMS in 16m ABS, then an update of ARQ parameters are necessary after performing handover from 16m only ABS to 16e BS. ARQ parameters are added in DSC-REQ/DSC-RSP messages to update ARQ parameters through a BS initiated DSC transaction.

Section 2 presents the required spec changes in IEEE Std. 802.16-2009 where remedy #1 to #15 are to support a handover from 16e BS to 16m only ABS, and remedy #16 to #17 is to support a handover from 16m only ABS to 16e BS.

- Text changes in IEEE Std. 802.16-2009

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Adopt the following new section 6.3.2.3.60 in IEEE 802.16-2009:]

Add new subclause 6.3.2.3.60 after section 6.3.2.3.59:

6.3.2.3.60 AAI_NBR-INFO

BSs supporting mobile functionality in association with neighboring ABSs shall transmit an AAI_NBR-INFO management message at a periodic interval (AAI_NBR-INFO interval; see Table 554) to identify the network and define the characteristics of neighbor ABSs. For the compression of neighbor BSIDs using this message in MOB_SCN-REQ, MOB_SCN-RSP, MOB_SCN-REP, MOB_BSHO-REQ and MOB_MSHO-REQ messages, the BS shall keep a mapping-table between neighbor ABS BSID and neighbor ABS indexes transmitted through AAI_NBR-INFO message, for each Configuration Change Count. Using the mapping-tables, BSs can derive 48-bit neighbor BSID from neighbor ABS index included.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_NBR-INFO_Message_format() { |  |  |
| Management Message Type = 110 | 8 |  |
| Configuration Change Count | 8 | Incremented each time the information for the associated neighbor ABS has changed. |
| Fragmentation Index | 4 | Indicates the current fragmentation index. |
| Total Fragmentation | 4 | Indicates the total number of fragmentations. |
| N_NEIGHBORS | 8 |  |
| For (j = 0 ; j < N_NEIGHBORS; j++) { | — | — |
| Length | 8 | Length of message information including all fields within the FOR loop |
| BSID | 48 | Neighbor ABS ID |
| SA-PREAMBLE index | 10 | SA-Preamble index of the neighbor ABS |
| Center frequency | 32 | Center frequency (in unit of Hz) of the neighbor ABS |
| Bandwidth | 3 | BW size of the neighbor ABS 0b000: 5 MHz 0b001: 7 MHz 0b010: 8.75 MHz 0b011: 10 MHz 0b100: 20 MHz 0b101~ 0b111: reserved |
| CP time | 2 | Indicates the CP time of the neighbor ABS 0b00: 1/8 0b01: 1/16 0b10: 1/4 0b11: reserved |
| reserved | 3 | Reserved: set to zero |
| S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS that is same as S-SFH change count in the ABS’s P-SFH IE |
| If (j ==0) { |  | 1st neighbor ABS |
| S-SFH SP1 IE | variable | See Table 833 (excluding MSB of superframe number and LSBs of 48 bit ABS MAC ID) in IEEE P80216m/D12 (for each parameter, byte alignment is required) |
| S-SFH SP2 IE | variable | See Table 834 (excluding MSB of 48 bit ABS MAC ID) in IEEE P80216m/D12 (for each parameter, byte alignment is required) |
| S-SFH SP3 IE | variable | See Table 835 in IEEE P80216m/D12 (for each parameter, byte alignment is required) |
| }else { |  |  |
| Delta information | variable | Indicates the delta encoding, between 1st neighbor ABS and current neighbor ABS. Delta information contains SFH SP1, SP2, and SP3 attributes as defined in Table 833, Table 834, and table 835 in IEEE P80216m/D12, respectively (for each parameter, byte alignment is required). Each delta information attribute is optional. |
| } |  |  |
| } |  |  |
| padding | variable |  |
| } |  |  |

[Remedy 2:]

[Adopt the following change in Table 554 in section 10.1 (IEEE802.16-2009):]

Change Table 554 as indicated:

| System | Name | Time Reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| BS | MOB_NBR-ADV Interval | Nominal time between transmission of MOB_NBR-ADV messages. | — | — | 30 s |
| BS | AAI_NBR-INFO Interval | Nominal time between transmission of AAI_NBR-INFO messages | — | — | 30 s |
| BS, MS | ASC-AGINGTIMER | Nominal time for aging of MS associations | 0.1 s | — | 10 s |

[Remedy 3:]

[Adopt the following changes in section 6.3.2.3.43 in IEEE802.16-2009:]

Change the text below Table 146 as indicated:

The MOB_SCN-REQ message may include the following parameters encoded as TLVs:

Sleep Mode Reactivation Information (see 11.20.2)

Recommended start frame (see 11.20)

For requesting to scan neighbor 16m only ABS, the MOB_SCN-REQ message shall include the following parameter encoded as TLVs:

Recommended Neighbor ABS Information for Scanning (see 11.20.3)

The MOB_SCN-REQ message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Remedy 4:]

[Adopt the following changes in section 11.20 in IEEE802.16-2009:]

Add new subclause 11.20.3 as follows:

11.20.3 Recommended Neighbor ABS Information for Scanning

The Recommended Neighbor ABS information for Scanning field indicates the Neighbor ABS index for scanning of 16m only neighbor ABS. Neighbor_ABS_Index indicates ABS index corresponding to position of ABS in AAI_NBR-INFO message.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 3 | variable; (16 bits + N_Recommended_ABS_Index × 8 bits) | Bits 0–7: Configuration change count for AAI_NBR-INFO Bits 8–15: N_Recommended_ABS_Index Subsequent (N_Recommended_ABS_Index × 8) bits: for (i = 0, i < N_Recommended_ABS_Index, i++) { 8-bits Neighbor_ABS_Index } | MOB_SCN-REQ MOB_SCN-RSP |

[Remedy 5:]

[Adopt the following changes in section 6.3.2.3.44 in IEEE802.16-2009:]

Change the text below Table 147 as indicated:

Report metric

Bitmap indicator of trigger metrics that the serving BS requests the MS to report. The serving BS shall indicate only the trigger metrics agreed during SBC-REQ/RSP negotiation. Metrics such as Relative delay and BS RTD shall not be applied for scanning report of 16m neighbor ABS. Each bit indicates whether reports will be initiated by trigger based on the corresponding metric:

Bit 0: BS CINR mean

Bit 1: BS RSSI mean

Bit 2: Relative delay

Bit 3: BS RTD; this metric shall be only measured on serving BS/anchor BS

Bits 4–7: Reserved; shall be set to zero

The MOB_SCN-RSP message may include the following parameters encoded as TLVs:

Sleep Mode Reactivation Information (see 11.20.2)

For scanning neighbor 16m only ABS, the MOB_SCN-RSP message shall include the following parameter encoded as TLVs:

Recommended Neighbor ABS Information for Scanning (see 11.20.3)

The MOB_SCN-RSP message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Remedy 6:]

[Adopt the following changes in section 6.3.2.3.45 in IEEE802.16-2009:]

Change the text below Table 148 as indicated:

TLV tuples specified in 11.19 may be included into MOB_SCN-REP message. Information provided by N-th TLV of this type is related to N-th BS listed in the message.

For measurement report of neighbor 16m only ABS, the MOB_SCN-REP message shall include the following parameter encoded as TLVs:

Recommended Neighbor ABS Information for SCN-REP (see 11.19.1)

The MOB_SCN-REP message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Remedy 7:]

[Adopt the following changes in section 11.19 in IEEE802.16-2009:]

Add new subclause 11.19.1 as follows:

11.19.1 Recommended Neighbor ABS Information for SCN-REP

The Recommended Neighbor ABS information for SCN-REP field indicates the Neighbor ABS index and measurement information of 16m only neighbor ABS such as CINR mean or RSSI mean. Neighbor_ABS_Index indicates ABS index corresponding to position of ABS in AAI_NBR-INFO message.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 2 | variable | Bits 0–7: Configuration change count for AAI_NBR-INFO Bits 8–15: N_Recommended_ABS_Index Subsequent bits: for (i = 0, i < N_Recommended_ABS_Index, i++) { 8-bits Neighbor_ABS_Index If(Report metric[Bit 0] == 1) 8-bits BS CINR mean If(Report metric[Bit 1] == 1) 8-bits BS RSSI mean } | MOB_SCN-REP |

[Remedy 8:]

[Adopt the following changes in section 6.3.2.3.47 in IEEE802.16-2009:]

Change the text below Table 150 as indicated:

For Mode = 0b00, the MOB_BSHO-REQ may contain the following TLV:

Additional Action Time (see 11.15.3)

Indicates a specific action time for each Neighbor BS listed in this message. The action times included in this TLV shall be ordered according to the Neighbor BS listed in this message.

For initiating a handover to 16m only ABS, the MOB_BSHO-REQ shall contain the following TLV:

Recommended Neighbor ABS Information (see 11.15.4)

The MOB_BSHO-REQ message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Remedy 9:]

[Adopt the following changes in section 11.15 in IEEE802.16-2009:]

Add new subclause 11.15.4 as follows:

11.15.4 Recommended Neighbor ABS Information

The Recommended Neighbor ABS information field indicates the Neighbor ABS index and SA-Preamble of 16m only neighbor ABS. Neighbor_ABS_Index indicates ABS index corresponding to position of ABS in AAI_NBR-INFO message.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 4 | variable; 16 bits + N_Recommended_ABS × 24 bits) | Bits 0–7: Configuration change count for AAI_NBR-INFO Bits 8–15: N_Recommended_ABS_Index Subsequent (N_Recommended_ABS_Index × 24) bits: for (i = 0, i < N_Recommended_ABS_Index, i++) { 8-bits Neighbor_ABS_Index 10-bits SA-PREAMBLE index 6-bits reserved } | MOB_BSHO-REQ, MOB_MSHO-REQ, |

[Remedy 10:]

[Adopt the following changes in section 6.3.2.3.48 in IEEE802.16-2009:]

Change the text below Table 151 as indicated:

When the MS supports FBSS/MDHO and has a nonempty diversity set, the MS shall include the following parameters for each active BS. When the MS does not support FBSS/MDHO or has an empty active, the MS shall include the following parameters for the current serving BS.

Temp BSID

When the MS support FBSS/MDHO and has a nonempty diversity set, Temp BSID is the diversity set member ID. When the MS does not support FBSS/MDHO or has an empty diversity set, Temp BSID shall be set to 0.

BS CINR mean

The BS CINR Mean parameter indicates the CINR in dB measured at the MS on the DL signal of a particular BS. The value shall be interpreted as a signed byte with the resolution of 0.5 dB. The measurement shall be performed on the frame preamble and averaged over the measurement period.

For initiating a handover to 16m only ABS, the MOB_MSHO-REQ shall include the following parameter encoded as TLVs:

Recommended Neighbor ABS Information (see 11.15.4)

The MOB_MSHO-REQ message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Remedy 11:]

[Adopt the following changes in section 6.3.2.3.49 in IEEE802.16-2009:]

Change the text below Table 152 as indicated:

For Mode = 0b00, the MOB_BSHO-RSP may contain the following TLV:

Additional Action Time (see 11.15.3)

Indicates a specific action time for each Neighbor BS listed in this message. The action times included in this TLV shall be ordered according to the Neighbor BS listed in this message.

For a handover to 16m only ABS, the MOB_BSHO-RSP shall include the following parameters encoded as TLVs:

Neighbor ABS Information (see 11.15.5)

Pre-assigned STID (see 11.15.6)

The MOB_BSHO-RSP message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Remedy 12:]

[Adopt the following changes in section 11.15 in IEEE802.16-2009:]

Add new subclause 11.15.5 as follows:

11.15.5 Neighbor ABS Information

The Neighbor ABS information field indicates the Neighbor ABS index, SA-Preamble, Reentry process optimization of 16m only neighbor ABS.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 5 | variable; 8 bits+ N_Recommended_ABS × 64 bits) | Bits 0–7: Configuration change count for AAI_NBR-INFO Bits 8–15: N_Recommended_ABS_Index Subsequent (N_Recommended_ABS_Index × 64) bits: for (i = 0, i < N_Recommended_ABS_Index, i++) { 48-bits BS_ID 10-bits SA-PREAMBLE index 5-bits Reentry process optimization 1-bits reserved } | MOB_BSHO-RSP |

5-bit Reentry process optimization is provided as part of this message is indicative only. For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted.

Bit 0: Omit AAI-SBC-REQ/RSP MAC control messages during reentry processing

Bit 1: Omit PKM Authentication phase

Bit 2: Omit AAI-REG-REQ/RSP message during reentry processing

Bit 3: Omit higher layer protocol triggering for IP address refresh during reentry processing

Bit 4: If Bit 4 = 1, Full service and operational state transfer or sharing between serving BS and target ABS (All static and dynamic context, e.g., ARQ windows, state machines)

[Remedy 13:]

[Adopt the following changes in section 11.15 in IEEE802.16-2009:]

Add new subclause 11.15.6 as follows:

11.15.6 Pre-assigned STID

The BS includes the Pre-assigned STID TLV in a MOB_BSHO-RSP message for direct HO from 16e BS to 16m only ABS. The BS shall set bits 0-11 to the Pre-assigned STID.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 6 | 2 | Bits 0–11: Pre-assigned STID Bits 12–15: reserved | MOB_BSHO-RSP |

[Remedy 14:]

[Adopt the following changes in section 6.3.2.3.50 in IEEE802.16-2009:]

Change Table 153 as indicated:

Table 153—MOB_HO-IND message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MOB_HO-IND_Message_format() { |  |  |
| Management Message Type = 59 | 8 |  |
| Reserved | 6 |  |
| Mode | 4 | 0b00: HO 0b01: MDHO/FBSS: Anchor BS update 0b10: MDHO/FBSS: Diversity set update 0b11: Direct HO from 16e BS to 16m ABS |
| if (Mode == 0b00) { | — | — |
| … |  |  |

Change the text below Table 153 as indicated:

The MOB_HO-IND message may include the following parameter encoded as a TLV tuple if HO_IND-type is set to 0b10:

Alternate_Target_BS (see 11.15.2)

If the MS sends the MOB_HO-IND with HO_IND_type set to 0b10 (HO reject), the MS may send this TLV to indicate its preferred handoff target BS.

If Mode is set to 0b11, the MOB_HO-IND message shall include the following parameter encoded as TLVs:

Target ABS (see 11.15.7)

The MOB_HO-IND message shall include the following parameters encoded as TLV tuples:

HMAC/CMAC Tuple (see 11.1.2)

The HMAC/CMAC Tuple shall be the last attribute in the message.

[Remedy 15:]

[Adopt the following changes in section 11.15 in IEEE802.16-2009:]

Add new subclause 11.15.7 as follows:

11.15.7 Target ABS

The AMS may include the Target_ABS TLV in a MOB_HO-IND message in which the Mode is set to 0b11 (Direct HO from 16e BS to 16m ABS, see 6.3.2.3.50) in order to indicate its preferred HO target. If the AMS includes this TLV, the AMS shall set bits 57-10 to the BS_ID of the proposed HO target ABS, and shall set bits 9-0 to the SA-Preamble of the target ABS.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| 7 | 8 | Bits 63–58: reserved Bits 57–10: BS_ID Bits 9–0: SA-PREAMBLE index | MOB_HO-IND |

[Remedy 16:]

[Adopt the following changes in section 6.3.21.2.10 in IEEE802.16-2009:]

Add texts at the end of subclause 6.3.21.2.10 as follows:

— Optimized HO during a handover from 16m only ABS to 16e BS

Security context and IP address are shared between the target 16m only ABS and the serving 16e BS.

HO process optimization TLV settings:

— HO process optimization bit 0 = 0

— HO process optimization bit 1 = 1

— HO process optimization bit 2 = 1

— HO process optimization bit 6 = 0

— HO process optimization bit 7 = 0

— HO process optimization bit 8 = 1

— HO process optimization bit 10 = 1

— HO process optimization bit 13 = 0

— All other bits, except reserved bits = Don’t care (i.e., not dependant on optimization case)

[Remedy 17:]

[Adopt the following changes in section 11.13.17 in IEEE802.16-2009:]

Add texts in subclause 11.13.17 as follows:

11.13.17 ARQ TLVs for ARQ-enabled connections

ARQ TLVs in DSC-REQ/DSC-RSP messages shall be included only for an AMS that has completed network reentry with a 16e BS after a handover from a 16m only ABS. In this case, ARQ parameters for multiple service flows may be updated in a single DSC procedure using “Group parameter Create/Change TLV”.

Change tables in 11.13.17 as indicated:

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].18 1.18 | 1 | 0 = ARQ Not Requested/Accepted 1 = ARQ Requested/Accepted | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].19 1.19 | 2 | ＞0 and  (ARQ_BSN_MODULUS/2) | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].20 1.20 | 2 | TRANSMITTER_DELAY 0-6553500s (100s granularity) | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |
| [145/146].21 1.21 | 2 | RECEIVER_DELAY 0-6553500s (100s granularity) | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].22 1.22 | 2 | 0 = Infinite 1-6553500s (100s granularity) | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].23 1.23 | 2 | 0 = Infinite 1-6553500s (100s granularity) | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].24 1.24 | 1 | 0 – Order of delivery is not preserved 1 – Order of delivery is preserved | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].25 1.25 | 2 | 0 = Infinite 0 – 6553500s (100s granularity) | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].26 1.26 | 1 | For DSA-REQ, DSC-REQ and REG-REQ: Bit 0–3: encoding for proposed minimum block size (M) Bit 4–7: encoding for proposed maximum block size (N) where: The minimum block size is equal to 2^(M+4) and the maximum block size is equal to 2^(N+4), M≤6, N≤6, and M≤Ν For DSA-RSP, DSC-RSP and REG-RSP: Bit 0–3: encoding for selected block size (P) Bit 4–7: set to 0 where: The selected block size is equal to 2^(P+4), P≤6 and M≤P≤N | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [145/146].27 1.27 | 1 | 0 – 255 | DSA-REQ, DSA-RSP DSC-REQ, DSC-RSP REG-REQ, REG-RSP |

-------------------------------  Text End  ---------------------------------------------------