| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |  |
| --- | --- | --- | --- |
| Title | Text changes related to A-MAP relevance and HARQ timing (IEEE Std 802.16m) |  |  |
| Date Submitted | 2011-05-10 |  |  |
| Source(s) | Jisoo Park, Sookjin Lee, Yongseouk Choi, Younok Park ETRI | Voice: +82-42-860-5748 jsp@etri.re.kr |  |
| Re: | Call for Change Requests regarding IEEE Std 802.16 (Volume: IEEE Std 802.16m) |  |  |
| Abstract | The contribution proposes the text changes related to A-MAP relevance and HARQ timing |  |  |
| Purpose | To be discussed and adopted by Maintenance Task Group for IEEE Std 802.16m |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |  |

Text changes related to A-MAP relevance and HARQ timing (16.2.14.2.2)

Jisoo Park, Sookjin Lee, Yongseouk Choi, Younok Park

ETRI

- Introduction

This contribution proposes the text changes for the cleanup text and clarification related to the A-MAP relevance and HARQ timing in [1].

- References

[1] IEEE Std 802.16m (from IEEE P802.16m/D12), “Amendment to IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, 2011-03-31

- Proposed Text Changes

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

----------------------------------------------- Start of Proposed Text Changes -----------------------------------------------

[Remedy#1: Adopt the following modification text on Table 786, in page 463, section 16.2.14.2.2.1.1]

| Table 786 - FDD DL HARQ timing |  |  |
| --- | --- | --- |
| Content | AAI Subframe Index | Frame Index |
| DL Assignment A-MAP IE Tx |  |  |
| DL HARQ Subpacket Tx |  |  |
| UL HARQ feedback Tx |  | Where z=0, ceil(F2)−NTTI)≥TDL_Rx_Processing z=0, if((ceil(F2)−NTTI)≥TDL_Rx_Processing) |

[Remedy#2: Adopt the following modification text on Table 787, in page 465, section 16.2.14.2.2.1.2]

| Table 787 - FDD UL HARQ timing |  |  |
| --- | --- | --- |
| Content | AAI Subframe Index | Frame Index |
| UL Assignment A-MAP IE Tx |  |  |
| UL HARQ Subpacket Tx | m=ceil(l+F2) mod F | Where v=0, (ceil(F2)−1)≥TUL_Tx_Processing v=0, if((ceil(F2)−1)≥TUL_Tx_Processing) |
| DL HARQ feedback Tx |  | Where w=0, floor(F2)−NTTI≥TUL_Rx_Processing w=0, if((floor(F2)−NTTI)≥TUL_Rx_Processing) |
| UL HARQ Subpacket ReTx |  | Where v=0, (ceil(F2)−1)≥TUL_Tx_Processing v=0, if((ceil(F2)−1)≥TUL_Tx_Processing) |

[Remedy#3: Adopt the following modification text in line 6 on page 466, section 16.2.14.2.2.1.2]

p k is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the HARQ acknowledgement is sent

k p is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where a retransmission of the HARQ subpacket begins

[Remedy#4: Adopt the following modification text on Table 788, in page 467, section 16.2.14.2.2.2.1]

| Table 763 - TDD DL HARQ timing |  |  |
| --- | --- | --- |
| Content | AAI Subframe Index | Frame Index |
| DL Assignment A-MAP IE Tx |  |  |
| DL HARQ Subpacket Tx |  |  |
| UL HARQ feedback Tx | For D>U where | where z=0, if(D−m−NTTI+n)≥TDL_Rx_Processing z=0, if((D−m−NTTI+n)≥TDL_Rx_Processing) |
|  | For D≤U where |  |

[Remedy #5: Adopt the following modification text in line 1 on page 469 through line 24 on page 285, section 16.2.14.2.2.2.2]

| Table 789 - TDD UL HARQ timing |  |  |
| --- | --- | --- |
| Content | AAI Subframe Index | Frame Index |
| UL Assignment A-MAP Tx IE in DL |  |  |
| UL HARQ Subpacket Tx | For default TTI and D≥U Where | Where v=0, D−l−1+m≥TUL_Tx_Processing v=0, if((D−l−1+m)≥TUL_Tx_Processing) |
|  | For default TTI and D<U Where |  |
|  | For long TTI m=0 |  |
| DL HARQ feedback Tx |  | Where w=0, U−m−NTTI+l≥TUL_Rx_Processing w=0, if((U−m−NTTI+l)≥TUL_Rx_Processing) |
| UL HARQ Subpacket ReTx |  | Where v=0, D−l−1+m≥TUL_Tx_Processing v=0, if((D−l−1+m)≥TUL_Tx_Processing) |

Where:

D is the number of downlink subframes as defined by the frame configuration table

U is the number of uplink subframes as defined by the frame configuration table

l is the reference to the DL subframe, starting from 0 for the first downlink subframe and numbering up to

D-1, where the A-MAP is transmitted or the HARQ acknowledgement is sent

m is the reference to the DL UL subframe, starting from 0 for the first downlink subframe and numbering up to U-1, where HARQ subpacket begins its transmission

i is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the A-MAP is transmitted

j is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the HARQ subpacket begins

p k is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where the HARQ acknowledgement is sent

k p is the reference for the frame, starting from 0 for the first frame of a superframe and numbering up to 3, where a retransmission of the HARQ subpacket begins

NTTI is the number of AAI subframes which a HARQ subpacket spans; i.e., 1 for the default TTI and D U for long TTI in TDD DL UL

[Remedy #6: Adopt the following modification text in line 30 on page 471 section 16.2.14.2.2.2.3]

The same equations and rule in Table 788 for the TDD DL and Table 789 for the TDD UL shall be applied for deciding HARQ timing with l, m, n, D, and U, except that l', m', n', D', and U' shall be used to set z, v, and w, as follows:

D’ D' is the number of downlink subframes as defined by the frame configuration table

U’ U' is the number of uplink subframes as defined by the frame configuration table

D is the DL length of the subframe provision as defined by thse frame configuration table

U is the UL length of the subframe provision as defined by the frame configuration table

For TDD DL HARQ timing,

z=0,  if((D'−m'−NTTI+n')≥TDL_Rx_Processing)                                     (172)

Where:

m is the reference to the DL subframe dedicated to the Advanced Air Interface operation in frame, starting from 0 for the first downlink subframe and numbering up to D-1, where HARQ subpacket begins its transmission

n is the reference for the UL subframe, starting from 0 for the first uplink subframe and numbering up to U-1, where the HARQ acknowledgement is sent

m’ m' is the reference to the DL subframe of TDD frame, starting from 0 for the first downlink subframe and numbering up to D’ D' -1, where HARQ subpacket begins its transmission

n’ n' is the reference for the UL subframe of TDD frame, starting from 0 for the first uplink subframe and numbering up to U’ U' -1, where the HARQ acknowledgement is sent

NTTI is the number of AAI subframes which a HARQ subpacket spans. Note that long TTI is not allowed for DL HARQ subpacket transmission, and NTTI is 1 for the default TTI in TDD DL

For TDD UL HARQ timing,

v=0,  if((D'−l'−1+m')≥TUL_Tx_Processing)                                     (173)

w=0,  if((U'−m'−NTTI+l')≥TUL_Rx_Processing)                                 (174)

Where:

l is the reference to the DL subframe dedicated to the Advanced Air Interface operation in frame, starting from 0 for the first downlink subframe and numbering up to D-1, where the A-MAP is transmitted

m is the reference to the UL subframe dedicated to the Advanced Air Interface operation in frame, starting from 0 for the first downlink subframe and numbering up to U-1, where HARQ subpacket begins its transmission

l’ l' is the reference to the DL subframe of TDD frame, starting from 0 for the first downlink subframe and numbering up to D’ D' -1, where the A-MAP is transmitted

m’ m' is the reference to the UL subframe of TDD frame, starting from 0 for the first downlink subframe and numbering up to U’ U' -1, where HARQ subpacket begins its transmission

NTTI is the number of AAI subframes which a HARQ subpacket spans; i.e., 1 for the default TTI and U for long TTI in TDD UL

Note that long TTI is not allowed for DL HARQ subpacket transmission.                     (175)

------------------------------------------------ Proposed texts end --------------------------------------------------------