| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text changes for TTI and A-MAP Relevance in UL assignments (IEEE Std 802.16m) |  |
| Date Submitted | 2011-05-17 |  |
| Source(s) | Jisoo Park, Sookjin Lee, Yongseouk Choi, Younok Park ETRI Yeongmoon Son Samsung Electronics | jsp@etri.re.kr +82-42-860-5748 ym1004.son@samsung.com +82-31-279-5845 |
| Re: | Call for Change Requests regarding IEEE Std 802.16 (Volume: IEEE Std 802.16m) |  |
| Abstract | The contribution proposes the text changes related to TTI and A-MAP Relevance in UL assignments |  |
| Purpose | To be discussed and adopted by Maintenance Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text changes for TTI and A-MAP Relevance in UL assignments

Jisoo Park, Sookjin Lee, Yongseouk Choi, Younok Park

ETRI

Yeongmoon Son, Hyunjeong Kang

Samsung Electronics

- Introduction

This contribution proposes the text changes for the clarification related to some UL assignments. UL data allocations can selectively occupy among some AAI subframes according to A-MAP relevance and HARQ timing defined in 16.2.14.2.2. So, text changes as ‘TTI and Relevance’ in [1] had proposed supporting default TTI using some UL assignments for supporting delay-sensitive services such as VoIP and adopted in [2], but those ‘Description/Notes’ may cause any confusion because they represent TTI type and A-MAP relevance in the case that number of DL AAI subframes is less than number of UL AAI subframes in TDD UL assignment as well as TTI type of UL assignment for both modes of TDD and FDD.

Then, this contribution provides cleanup sentences related to ‘TTI and Relevance’ for UL basic assignments A-MAP IE and UL persistent allocation A-MAP IE.

- References

[1] C80216m-10_0831r2, Hyunkyu Yu, Ji-Yun Seol, Seho Kim, “Proposed Text for Default TTI Support (Section 16.3.6.5).”

[2] IEEE Std 802.16m (from IEEE P802.16m/D12), “Amendment to IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface,” 2011-03-31

- Proposed Text Changes

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

----------------------------------------------- Start of Proposed Text Changes -----------------------------------------------

[Remedy#1: Adopt the following modification text on Table 847 in page 672, section 16.3.6.5.2.4.2, as]

Table 847 – UL Basic Assignment A-MAP IE*

| Syntax | Size (bit) | Description/ notes |
| --- | --- | --- |
| UL_Basic_Assignment_A-MAP_IE( ) { |  |  |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| … | … | … |
| if(PMI Indicator == 0b0){ | - | PMI not signaled |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI spanning 4 UL AAI subframes for FDD or all UL AAI subframes for TDD 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP 0b10 and 0b11 are only applicable if the number of DL AAI subframes is less than the number of UL AAI subframes in TDD mode. |
| if(CSM == 0b0){ | - | SU-MIMO (Non-adaptive/Adaptive precoding without PMI) |
| … | … | … |

[Remedy#2: Adopt the following modification text on Table 855 in page 699, section 16.3.6.5.2.4.9, as]

Table 855 – UL Persistent Allocation A-MAP IE*

| Syntax | Size (bit) | Description/ notes |
| --- | --- | --- |
| UL Persistent A-MAP IE () { |  |  |
| … | … | … |
| If (Allocation period == 0b00) { |  |  |
| Resource index | 11 | … |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI spanning 4 UL AAI subframes for FDD or all UL AAI subframes for TDD 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP 0b10 and 0b11 are only applicable if the number of DL AAI subframes is less than the number of UL AAI subframes in TDD mode. |
| HFA | 6 | … |
| Reserved | 15 | Reserved bits |
| … | … | … |
| Resource Index | 11 | … |
| TTI and Relevance | 2 | Indicates the TTI type and the location of UL subframe relevant to this A-MAP. 0b00: long TTI spanning 4 UL AAI subframes for FDD or all UL AAI subframes for TDD 0b01: default TTI, the first UL subframe relevant to this A-MAP 0b10: default TTI, the second UL subframe relevant to this A-MAP 0b11: default TTI, the third UL subframe relevant to this A-MAP 0b10 and 0b11 are only applicable if the number of DL AAI subframes is less than the number of UL AAI subframes in TDD mode. |
| HFA | 3 | HARQ Feedback Allocation |
| N_ACID | 2 | Number of ACIDs for implicit cycling of HARQ channel identifier. 0b00: 2 0b01: 3 0b10: 4 0b11: 8 |
| … | … | … |

------------------------------------------------ Proposed texts end --------------------------------------------------------