| roject | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comment for provision of DCD/UCD during Legacy HO |  |
| Date Submitted | 2011-5-16 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Jin Sam Kwak LG Electronics | E-mail: inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Reply comment on contribution C802.16maint-11/0002 |  |
| Abstract | The contribution proposes text modification on proposed Direct HO from 16e to 16m networks |  |
| Purpose | To be discussed and adopted by Maintenance Task Group for IEEE Std 802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comment for provision of DCD/UCD during Legacy HO

Inuk Jung, Kiseon Ryu, Jin Sam Kwak
LG Electronics

# Introduction

Including system information of legacy systems, in the AAI-NBR-ADV message, results in duplicative information broadcasting. This may incur overhead in case when the frequency of performing HO to a legacy BS is low. Hence, it may be more efficient to provided the system information of legacy systems upon request.

This contribution proposes to include DCD/UCD change count information in the AAI-HO-REQ message to support handover to the legacy BS.

# Proposed Text

Adopt the following Remedies.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy #1: Update Table 688 in pp. 126, line 58 in D10 as follows:]

16.2.3.11 AAI-HO-REQ

…

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| for(i=0; i < N_R1_BS_Full; i++) { |  | N_R1_BS_Full is the number of neighboring R1 BSs to be considered for handover, which is 48-bit BSID. Range: 0 ~ 256 |  |
| Neighbor BSID | 48 | The 48 bits neighbor BSID |  |
| Configuration Change Count | 8 | Incremented each time the information for the associated neighbor BS has changed. | May be included if it is available at the MS. |
| DCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current DCD configuration change count. | May be included if it is available at the MS. |
| UCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current UCD configuration change count. | May be included if it is available at the MS. |
| ABS CINR mean | 8 | CINR mean of the new BS | May be included if it is available at the MS. |
| ABS RSSI mean | 6 | RSSI mean of the new BS | May be included if it is available at the MS. |
| } |  |  |  |
| … | … | … | … |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------

[Remedy #2: Update Table 689 in pp. 128, line 1 in D10 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

16.2.3.12 AAI-HO-CMD

…

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … | … | … | … |
| S-SFH application indicator | 1 | Indicate if the SFH delta information at the T-ABS is applied at the Action Time 0b0: not applied 0b1: applied | Shall be included when SFH delta information is included |
| Legacy Configuration Change Count | 8 | This represents the configuration change count of the associated neighbor BS. | Shall be included if target BS is legacy BS |
| Legacy DCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current DCD configuration change count. | Shall be included if target BS is legacy BS |
| Legacy UCD Configuration Change Count | 4 | This represents the 4 LSBs of the neighbor BS current UCD configuration change count. | Shall be included if target BS is legacy BS |
| Legacy DCD_settings | Variable | The DCD_settings is a compound encapsulates information from the neighbor BS’ DCD message that may be transmitted in the advertised BS downlink channel, which conveys only DCD values that are different from the target BS corresponding values. The size depends on the difference of DCD information with the target BS | Shall be included if available |
| Legacy UCD_settings | Variable | The UCD_settings is a compound encapsulates information from the neighbor BS’ UCD message that may be transmitted in the advertised BS downlink channel, which conveys only UCD values that are different from the target BS corresponding values. The size depends on the difference of UCD information with the target BS | Shall be included if available |
| }//end of for(i=0; i < N_Target_BS; i++) |  |  |  |
| } |  |  |  |
| … | … | … | … |

------------------------------------------------------ End of Proposed Text ------------------------------------------------------

[Remedy #3: Update ASN.1 code based on the updated message table in pp. 1041, line 52 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

maxR1BSFull INTEGER ::= 256

AAI-HO-REQ ::=		SEQUENCE {

carrierPreassignmentIndicator BOOLEAN OPTIONAL,

newABSIndexList NewABSIndexList OPTIONAL,

newABSFullList NewABSFullList OPTIONAL,

r1BSFullList R1BSFullList OPTIONAL

}

R1BSFullList ::= SEQUENCE (SIZE (1..maxNewABSFull)) OF R1BSFullInfo

R1BSFullInfo ::= SEQUENCE {

nbrR1BSID BSID,

configurationChangeCount		INTEGER(0..255) OPTIONAL,

dcdConfigurationChangeCount	INTEGER(0..15)	OPTIONAL,

ucdConfigurationChangeCount	INTEGER(0..15)	OPTIONAL,

cinrMean CINRMean OPTIONAL,

rssiMean RSSIMean OPTIONAL

}

}

------------------------------------------------------ End of Proposed Text ------------------------------------------------------

[Remedy #4: Update ASN.1 code based on the updated message table in pp. 1041, line 52 as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

TargetBSInfo ::= SEQUENCE {

...

sfhDeltaInfo SEQUENCE {

sfhSubpacket1 OptSFHSubpacket1 OPTIONAL,

sfhSubpacket2 OptSFHSubpacket2 OPTIONAL,

sfhSubpacket3 OptSFHSubpacket3 OPTIONAL

} OPTIONAL

},

legacyConfigurationChangeCount	INTEGER(0..255) OPTIONAL,

legacyDCDConfigurationChangeCount	INTEGER(0..15) OPTIONAL,

legacyUCDConfigurationChangeCount	INTEGER(0..15) OPTIONAL,

legacyDCDDelta LegacyDCDDelta OPTIONAL,

legacyUCDDelta LegacyUCDDelta OPTIONAL,

}

LegacyDCDSettings ::= SEQUENCE {

ccc	INTEGER (0..255),

dlBurstProfile	SEQUENCE (SIZE (1..15)) OF LegacyDLBurstProfile OPTIONAL,

bsEIRP	INTEGER (0..65535) OPTIONAL,

ttg	INTEGER (0..65535) OPTIONAL,

rtg	INTEGER (0..255) OPTIONAL,

eirxPIR,max	INTEGER (0..65535) OPTIONAL,

bsRestartCount	INTEGER (0..255) OPTIONAL

}

LegacyUCDSettings ::= SEQUENCE {

ccc	INTEGER (0..255),

ulBurstProfile	SEQUENCE (SIZE (1..15)) OF LegacyULBurstProfile OPTIONAL,

cbReservationTimeout	INTEGER (0..255) OPTIONAL,

rngBackoffStart	INTEGER (0..255) OPTIONAL,

rngBackoffEnd	INTEGER (0..255) OPTIONAL,

reqBackoffStart	INTEGER (0..255) OPTIONAL,

reqBackoffEnd	INTEGER (0..255) OPTIONAL,

hoRngStart	INTEGER (0..255) OPTIONAL,

hoRngEnd	INTEGER (0..255) OPTIONAL,

periodicRngStart	INTEGER (0..255) OPTIONAL,

periodicRngEnd	INTEGER (0..255) OPTIONAL,

initialRngCodes	INTEGER (0..255) OPTIONAL,

periodicRngCodes	INTEGER (0..255) OPTIONAL,

bwReqCodes	INTEGER (0..255) OPTIONAL,

hoRngCodes	INTEGER (0..255) OPTIONAL,

startRngCodesGroup	INTEGER (0..255) OPTIONAL,

permBase	INTEGER (0..255) OPTIONAL,

harqACKdelayforDL	INTEGER (0..255) OPTIONAL,

Size of CQICH_ID field	INTEGER (0..255) OPTIONAL,

normC2Noverride2	BIT STRING (SIZE (64)) OPTIONAL,

initialRngInterval	INTEGER (0..255) OPTIONAL,

txPowerReport	BIT STRING (SIZE (24)) OPTIONAL,

relPowOffsetULHARQ	INTEGER (0..255) OPTIONAL,

ulInitialTXtiming	INTEGER (0..255) OPTIONAL,

fastFeedbackRegion	BIT STRING (SIZE (40)) OPTIONAL,

harqAckRegion	BIT STRING (SIZE (32)) OPTIONAL,

rngRegion	BIT STRING (SIZE (80)) OPTIONAL

}

LegacyDLBurstProfile ::= SEQUENCE {

diuc	INTEGER (0..15),

mcs	INTEGER (0..255)

}

LegacyULBurstProfile ::= SEQUENCE {

uiuc	INTEGER (0..15),

mcs	INTEGER (0..255)

}

------------------------------------------------------ End of Proposed Text ------------------------------------------------------