| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comment for 16m Direct HO |  |
| Date Submitted | 2011-05-16 |  |
| Source(s) | Inuk Jung, Kiseon Ryu and Jin Sam Kwak LG Electronics | Email: inuk.jung@lge.com |
| Re: | Reply comment on contribution C802.16maint-11/0001 |  |
| Abstract | The contribution proposes text modification on proposed Direct HO from 16e to 16m networks |  |
| Purpose | To be discussed and adopted by Maintenance Task Group for IEEE Std 802.16-2009 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comment for 16m Direct HO

Inuk Jung, Kiseon Ryu and Jin Sam Kwak

LG Electronics Inc.

# Introduction

This document is a reply comment/contribution on comment #1 (IEEE802.16-2009) contribution C802.16maint-11/0001.

In perspective of overhead and deployment scenario, we think that providing a zone switch TLV (Inter-FA) with the carrier information of the 16m carrier is a simple approach to allow the MS to handover to a 16m only network.

# Proposed Text Change

Adopt the following Remedies in IEEE 802.16-2009.

[Remedy 1: Add new TLVs on page 95 in RNG-RSP message, as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.2.3.6 RNG-RSP (ranging response) message

…

Transmission Opportunity Offset

A unique transmission opportunity assigned to the MS, to be used for dedicated ranging, in units of symbol duration

The following TLV parameter may be included for the BS to notify the MS of known WirelessMAN-Advanced Air Interface carriers at a different FA of the serving BS:

BSID

The BSID of the ABS

SA-Preambleindex

SA-Preamble index at a different FA of the serving BS

Frequency offset of ABS

Offset in frequency band unit indicating frequency location of ABS from the current serving BS’s FA

Bandwidth

BW size of the neighbor ABS

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 2: Add new TLVs on page 200 in MOB_NBR-ADV message, as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.2.3.42 MOB_NBR-ADV (neighbor advertisement) message

…

MCID-Continuity and Transmission Info (see 11.1.12.2)

MCID-Continuity and Transmission Info is used by the BS’s in one MBS-Zone to show consistency of MCID’s mapping used in select other MBS Zones as determined by the serving MBS Zone.

The following TLV parameter may be included for the BS to provide WirelessMAN-Advanced Air Interface  information of neighbor ABS:

N_Neighbor ABS

Indicates the total number of neighbor ABSs.

The following TLV parameters may be included for each neighbor ABS

BSID

The BSID of the ABS

SA-Preambleindex

SA-Preamble index of the neighbor ABS

Center frequency

Center frequency (in unit of Hz) of the neighbor ABS

Bandwidth

BW size of the neighbor ABS

----------------------------------------------------- End of Proposed Text -----------------------------------------------------

[Remedy 3: Add new TLVs on page 246 in MOB_BSHO-RSP message, as follows:]

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

6.3.2.3.49 MOB_BSHO-RSP (BS HO response) message

…

New SAID

New SAIDs are enumerated by the ascending order of corresponding current SAIDs. The MS

shall store the SAIDs associated with the newly added BS and use the SAIDs when the newly

added BS becomes the anchor BS.

For mode 0b000 the following TLV parameter may be included for the BS to provide the MS with system information of target ABS:

Recommended Candidate ABS

Indicates the total number of candidate ABSs for handover.

For the first candidate ABS, the following TLV parameters may be included:

S-SFH change count

Indicates the value of S-SFH change count of this neighbor ABS that is same as S-SFH change count in the ABS’s P-SFH IE

S-SFH SP1 IE

See Table 833 (excluding MSB of superframe number and LSBs of 48 bit ABS MAC ID) in IEEE P80216m/D12 (for each parameter, byte alignment is required)

S-SFH SP2 IE

See Table 834 (excluding MSB of 48 bit ABS MAC ID) in IEEE P80216m/D12 (for each parameter, byte alignment is required)

S-SFH SP3 IE

See Table 835 in IEEE P80216m/D12 (for each parameter, byte alignment is required)

If there are multiple candidate ABSs, the following TLV parameter may be included for the candidate ABSs except the first candidate ABS:

Delta information

Indicates the delta encoding, between 1st neighbor ABS and current neighbor ABS. Delta information contains SFH SP1, SP2, and SP3 attributes as defined in Table 833, Table 834, and table 835 in IEEE P80216m/D12, respectively (for each parameter, byte alignment is required). Each delta information attribute is optional.

----------------------------------------------------- End of Proposed Text -----------------------------------------------------