| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for the usage of Deregistration identifier (DID) in AAI-RNG-REQ message |  |
| Date Submitted | 2011-07-11 |  |
| Source(s) | Hyunjeong Kang, Yeongmoon Son, Youngkyo Baek Samsung Electronics | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | A change request according to IEEE 802.16-06/046 (IEEE Std 802.16m-2011) |  |
| Abstract | Changes are provided to clarify the usage of Deregistration identifier in AAI-RNG-REQ message. |  |
| Purpose | Approve and add to Maintenance TG database |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications for the usage of DID in AAI-RNG-REQ

Hyunjeong Kang, Yeongmoon Son, Youngkyo Baek

Samsung Electronics

- Problem statement

In current AAI-RNG-REQ message, the conditions to use DID or AMS MAC address in AAI-RNG-REQ message include not only S-SFH Network Configuration bit but AMSID privacy in case that an AMS performs network reentry from idle mode or idle mode location update. However, Deregistration ID (DID) is assigned and used for AMS in idle mode regardless of AMSID privacy if S-SFH Network Configuration bit is set to 0b0. Thus the condition that AMSID privacy is disabled or enabled shall be removed from AAI-RNG-REQ message in case of idle mode.

Section 2 presents the required spec change in IEEE Std 802.16m-2011 to clarify the condition of DID or AMS MAC address in AAI-RNG-REQ message.

- Text changes in IEEE Std 802.16m-2011

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1:]

[Change Table 684 in subclause 16.2.3.1 as indicated:]

Change the texts in page 94 and the texts in page 95 in subclause 16.2.3.1 as indicated:

| } else if (Ranging Purpose Indication == 0b0010) { |  | //Network reentry from idle mode |  |
| --- | --- | --- | --- |
| if (S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled) { |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| } else { |  |  |  |
| Deregistration Identifier (DID) | 18 | The ID that the AMS is assigned for idle mode and currently maintains. |  |
| } |  |  |  |
| … |  |  |  |
| } else if (Ranging Purpose Indication == 0b0011 ¦ 0b0110 ¦ 0b0111 ¦ 0b1011) { |  | //Idle mode location update (and with other additional purpose) |  |
| If (S-SFH Network Configuration bit == 0b1 or AMSID privacy is disabled) { |  |  |  |
| AMS MAC address | 48 | AMS’s real MAC address |  |
| } else { |  |  |  |
| Deregistration Identifier (DID) | 18 | The ID that the AMS is assigned for idle mode and currently maintains. |  |
| } |  |  |  |

-------------------------------  Text End  ---------------------------------------------------