| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup AAI-LBS-IND message and corresponding operation description on AAI-LBS-IND message (IEEE Std 802.16m-2011) |  |
| Date Submitted | 2011-07-10 |  |
| Source(s) | Lei Zhou, Sangheon Kim, Hyunjeong Kang, Xufeng Zheng, Rakesh Taori, Jaeweon Cho Samsung Electronics | Email: l.zhou@samsung.com |
| Re: | Call for Change Requests regarding IEEE Std 802.16 (Volume: IEEE Std 802.16m-2011) |  |
| Abstract | The contribution proposes to Cleanup AAI-LBS-IND message and corresponding operation description on AAI-LBS-IND message |  |
| Purpose | To be discussed and adopted by Maintenance Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup AAI-LBS-IND message and corresponding operation description on AAI-LBS-IND message

Lei Zhou, Sangheon Kim, Hyunjeong Kang, Xufeng Zheng, Rakesh Taori, Jaeweon Cho

Samsung Electronics

- Introduction

This contribution proposes to correct size value in order to be consistent with that of corresponding parameter in AAI-HO-CMD. At the same time, for operation description on AAI-LBS-IND message, we correct the operation of serving ABS because location server is charge of calculating position of AMS.

- References

[1] IEEE Std 802.16m-2011, “Part 16: Air Interface for Broadband Wireless Access Systems Amendment 3: Advanced Air Interface” 2011-06-06

- Proposed Text Changes

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

----------------------------------------------- Start of Proposed Text Changes -----------------------------------------------

[Remedy#1: Adopt the following modification text on Table 763 from page 306, section 16.2.3.63, IEEE Std 802.16m-2011 as]

Table 763—AAI-LBS-IND Message Field Description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Action code | 1 | 0x0: Special U-TDOA 0x1: General U-TDOA |  |
| Frame identifier | 4 | Frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe |  |
| Dedicated CDMA ranging code for the serving ABS | 5 | Indicates the dedicated ranging code. |  |
| Ranging opportunity index for the serving ABS | 3 1 | Indicates the index of the allocated ranging opportunity. |  |
| Subframe Index for the serving ABS | 2 3 | Indicates the subframe index of the allocated ranging opportunity. |  |
| If (Action code == 0x1) { |  |  |  |
| Neighbor_ABS_Info_Array | 8 | List of neighbor ABS Info | Presented when Action code == 0x1 |
| For (i=0; i< Neighbor_ABS_Info_Array; i ++){ |  |  |  |
| Neighbor ABS index | 8 | ABS index corresponds to AAI-NBR-ADV message | Presented when Action code == 0x1 is used |
| Action time | 8 | Action Time included in this message is the absolute frame number at the serving ABS. | Presented when Action code == 0x1 is used |
| Dedicated CDMA ranging code of neighbor ABS | 5 | Indicates the dedicated ranging code | Presented when Action code == 0x1 is used |
| Ranging opportunity index of neighbor ABS | 3 1 | Indicates the index of the allocated ranging opportunity | Presented when Action code == 0x1 is used |
| Subframe Index of neighbor ABS | 2 3 | Indicates the subframe index of the allocated ranging opportunity | Presented when Action code == 0x1 is used |
| } |  |  |  |
| } |  |  |  |

[Remedy#2: Adopt the following modification text on Figure 694 and Figure 695 from page 931, section 16.8.2.4, IEEE Std 802.16m-2011 as]

Figure 609 Operation description on AAI for General U-TDOA

1. Serving BS requests neighbor BS to assign the dedicated ranging opportunity for the MS.

2. Neighbor BS confirms the allocation of the dedicated ranging opportunity for the MS and returns the related parameters used for dedicated ranging between the MS and the neighbor BS.

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

3. Serving BS allocates a CDMA code and dedicated ranging opportunity to MS.

4. AAI_LBS-IND message is sent to the MS to request the MS to initiate dedicated ranging signals for the serving BS and neighbors. The following parameters are included in this message:

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

5. MS sends dedicated ranging signals to the serving BS on the dedicated ranging opportunity.

6. Serving BS measures Timing Adjustment t1

7. MS sends dedicated ranging signal to the neighbor BS on the dedicated ranging opportunity.

8. Neighbor BS measures Timing Adjustment t2

9. Neighbor BS returns t2 to serving BS

10. Serving BS calculates the U-TDOA

△t=t1-t2

Figure 610 Operation description on AAI for Special U-TDOA

1. Serving BS and neighbor BS negotiate the allocation of a dedicated ranging region for the MS.

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

2. AAI_LBS-IND message is sent to the MS to request the MS to initiate dedicated ranging signals for the serving BS and neighbors. The following parameters are included in this message:

- Frame Number

-Subframe Index

- CDMA code

- Transmission opportunity index

3. MS sends dedicated ranging signal to the serving BS on the dedicated ranging opportunity. At the same time, the neighbor BS shall make no allocations in that dedicated ranging region, and the neighbor BS will listen to the dedicated ranging signal from the MS.

4. Serving BS measures Timing Adjustment t1

5. Neighbor BS listens to the dedicated ranging signal from the MS, and then measures Timing Adjustment t2

6. Neighbor BS returns t2 to serving BS

7. Serving BS calculates the U-TDOA

△t=t1-t2

------------------------------------------------ Proposed texts end --------------------------------------------------------