| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup text for MIMO feedback formats (IEEE Std 802.16m) |  |
| Date Submitted | 2011-07-12 |  |
| Source(s) | Jisoo Park, Sookjin Lee ETRI | jsp@etri.re.kr +82-42-860-5748 |
| Re: | Call for Change Requests regarding IEEE Std 802.16 (Volume: IEEE Std 802.16m) |  |
| Abstract | The contribution proposes the cleanup text for MIMO feedback formats |  |
| Purpose | To be discussed and adopted by Maintenance Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup text for MIMO feedback formats (IEEE Std 802.16m)

Jisoo Park, Sookjin Lee

ETRI

- Introduction

This contribution proposes the cleanup text related to MIMO feedback formats for the suitable usage of a MIMO parameter, STC rate. There are two types of STC rate according to the purpose of the usage of RU, that is to say, subband STC rate used for frequency selective scheduling of RU in localized permutation and wideband STC rate used for wideband beamforming of RU in diversity permutation. These are used as MIMO parameters through various sections in [1], but needed to clean up for the suitable usage as MIMO parameters. Then, this contribution provides some suitable represents as feedback fields of MIMO feedback formats including some editorial corrections.

- References

[1] IEEE Std 802.16m-2011, “IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Amendment 3: Advanced Air Interface,” 2011-05-06

- Proposed Text Changes

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

----------------------------------------------- Start of Proposed Text Changes -----------------------------------------------

[Remedy#1: Adopt the following modification text in page 824, section 16.3.8.3.1.5 Feedback formats]

- 16.3.8.3.1.5 Feedback formats

Feedback formats define the information content carried by the fast feedback channels. The format of the content of the PFBCH is determined by the PFBCH encoding type. The SFBCH payload information bits may carry following feedback fields; subbands index (Best_subbands_index), subband STC rate used for frequency selective scheduling of RU in localized permutation, wideband STC rate used for wideband beamforming of RU in diversity permutation, PFBCH indicator, subband CQI, subband avg_CQI, differential CQI, stream index, wideband PMI, subband PMI, base PMI, and differential PMI. The SFBCH payload information bits a0a1...a23 are defined in the order specified in Table 936 to Table 944.

Subband STC rate and wideband STC rate shall be encoded with 1, 2 or 3 bits when the ABS has 2, 4 or 8 transmit antennas, respectively.

Wideband CQI, Subband CQI and subband avg_CQI shall be encoded with 4 bits corresponding to the nominal MCS of Table 893935.

Differential CQI shall be encoded with 2 bits indicating values {-1, 0, +1, +2}.

…

[Remedy#2: Adopt the following modification text on Table 936, in page 824, section 16.3.8.3.1.5 Feedback formats]

Table 936 - Feedback formats for MIMO feedback mode 0, 4, and 7 (outside OL region)

| MFM | Feedback format | FFBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | PFBCH | 2 | Short | Wideband CQI and Wideband STC rate | Encoding type 0 |
|  |  |  |  | Long | Wideband CQI and Wideband STC rate | Encoding type 0 Long-term FPI for FFR No long-term report when q=0 |
| 4 | s0 | PFBCH | 2 | Short | Wideband CQI and Wideband STC rate or type of EDI | Encoding type 0 |
|  |  |  |  | Long | Wideband PMI | PMI Encoding type 2 No long-term report when q=0 |
|  | 1 | SFBCH | 1 | Short | Wideband CQI Wideband STC rate Wideband PMI PFBCH indicator |  |
| 7 | 0 | PFBCH | 2 | Short | Wideband CQI | STC rate = 1 Encoding type 0 |
|  |  |  |  | Long | Wideband PMI | PMI for rank 1 Encoding type 2 No long-term report when q=0 |
|  | 1 | SFBCH | 1 | Short | Wideband CQI Wideband PMI PFBCH indicator |  |

[Remedy#3: Adopt the following modification text on Table 937, in page 825, section 16.3.8.3.1.5 Feedback formats]

Table 937 - Feedback formats for MIMO feedback mode 0 and 1 (inside OL region)

| MFM | FFBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 1 | Long | Wideband CQI and Wideband STC rate or any type of EDI | Encoding type 0 |
| 1 | PFBCH | 1 | Long | Wideband CQI or any type of EDI | Encoding type 3 |

[Remedy#4: Adopt the following modification text on Table 938, in page 826, section 16.3.8.3.1.5 Feedback formats]

Table 938 - Feedback formats for MFM 2 (outside OL Region)

| Feedback Format | FFBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI and Subband STC rate or any type of EDI | Encoding Type 0 No short-term report when q=0 |
|  |  |  | Long | Best_subbands_index | Encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Long | Best_subbands_index Subband CQI Subband STC rate PFBCH indicator | Support of STC rate 1 to 8 |
| 2 (M=min{5,YSB}) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ Subband differential CQI } | No short-term report when q=0 |
|  |  |  | Long | Best_subbands_index WidebandSubband STC rate PFBCH indicator | Support of STC rate 1 to 8 |
| 3 (M=min{10,YSB}) | SFBCH | 2 | Short | Subband avg CQI for (m=1:M){ Subband differential CQI } | No short-term report when q=0 |
|  |  |  | Long | Best_subbands_index WidebandSubband STC rate PFBCH indicator | Support of STC rate 1 to 8 |

[Remedy#5: Adopt the following modification text on Table 940, in page 828, section 16.3.8.3.1.5 Feedback formats]

Table 940 - Feedback formats for MFM 3

| Feedback Format | FFBCH | Number of reports | Report period | Feedback Fields | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | Encoding Type 0 No short-term report when q=0 |
|  |  |  | Short | Subband PMI | Encoding Type 2 STC rate = 1 No short-term report when q=0 |
|  |  |  | Long | Best_subbands_index | Encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short-term report when q=0 |
|  |  |  | Long | Best_subbands_index WidebandSubband STC rate Subband avg CQI PFBCH indicator |  |
| 2 (M = 3) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short-term report when q=0 |
|  |  |  | Long | Best_subbands_index WidebandSubband STC rate Subband avg CQI PFBCH indicator |  |
| 3 (M = 4) | SFBCH | 2 | Short | for (m=1:M){ Subband differential CQI Subband PMI } | No short-term report when q=0 |
|  |  |  | Long | Best_subbands_index WidebandSubband STC rate Subband avg CQI PFBCH indicator |  |

[Remedy#6: Adopt the following modification text on Table 941, in page 829, section 16.3.8.3.1.5 Feedback formats]

Table 941 - Feedback formats for MFM 3 for differential codebook

| Feedback Format | FFBCH | Number of Reports | Feedback Fields |  | Description/Notes |
| --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI Padding | Padding 1 bit: If Nt=2 0 bit: Otherwise |
|  |  |  | Long | Best_subbands_index Subband STC rate Base PMI PFBCH indicator | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 1 (M=2) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI }, WidebandSubband STC rate | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 2 (M =3) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index WidebandSubband STC rate subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |
| 3 (M =4) | SFBCH | 3 | Short | for (m=1:M){ Subband differential CQI Subband Differential PMI } |  |
|  |  |  | Long | Best_subbands_index WidebandSubband STC rate subband avg_CQI PFBCH indicator |  |
|  |  |  | Long | for (m=1:M){ Base PMI } | PMI for reset. Base PMI is a PMI from the base codebook or the base codebook subset. |

------------------------------------------------ Proposed texts end --------------------------------------------f------------