| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for MIMO feedback header (IEEE Std 802.16m) |  |
| Date Submitted | 2011-07-12 |  |
| Source(s) | Jisoo Park, Sookjin Lee ETRI | jsp@etri.re.kr +82-42-860-5748 |
| Re: | Call for Change Requests regarding IEEE Std 802.16 (Volume: IEEE Std 802.16m) |  |
| Abstract | The contribution proposes the clarified text changes related to MIMO feedback header |  |
| Purpose | To be discussed and adopted by Maintenance Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for MIMO feedback header (IEEE Std 802.16m)

Jisoo Park, Sookjin Lee

ETRI

- Introduction

This contribution proposes the change of field size in MIMO feedback header. This signaling header is used as a response to feedback request for wideband information of MIMO feedback mode (MFM) 0, 4 and 7 with short period (q = 0) outside the OL region. In MFM 0 for MIMO mode 0 and mode 1, wideband STC rate of OL SU-MIMO is limited to 1 for SFBC or 2 ~ 4 for SM with Mt=2 for DLRU or 2≤Mt≤4 for NLRU. In MFM 4 for MIMO mode 2, wideband STC rate of CL SU-MIMO is limited to 1 ~ 4 for SM with Mt≤4 for NLRU. So, the size of wideband STC rate is needed as 2-bits size for MFM 0 and MFM 4, in MIMO feedback header. Then, this contribution provides the correction of the bit size of wideband STC rate in MIMO feedback header.

- References

[1] IEEE Std 802.16m-2011, “IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Amendment 3: Advanced Air Interface,” 2011-05-06

- Proposed Text Changes

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

----------------------------------------------- Start of Proposed Text Changes -----------------------------------------------

[Remedy#1: Adopt the following modification text on Table 670 in page 75, section 16.3.6.2.5.4 Downlink signaling support of DL-MIMO modes]

Table 670—MIMO feedback header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFH() { |  |  |
| FID | 4 | Flow Identifier. Set to 0b0010 |
| Type | 5 | MAC signaling header type = 0b00111 |
| Length | 3 | Indicates the length of the signaling header. |
| MFM bitmap | 3 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with current feedback allocations corresponding to the MFM requested by Feedback Polling A-MAP IE. LSB #0: MFM 0 LSB #1: MFM 4 LSB #2: MFM 7 |
| If (LSB#0 in MFM_bitmap == 1){ |  | MFM 0 |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 2 | ‘STC rate – 1’ mapped to a 3-bit unsigned integer (i.e., STC rate=1 as 0b000 ~ STC rate = 8 as 0b111) Indicates STC rate (R) used for wideband beamforming. 0b00: STC rate (R) = 1 0b01: STC rate (R) = 2 0b10: STC rate (R) = 3 0b11: STC rate (R) = 4 |
| } |  |  |
| If (LSB#1 in MFM_bitmap == 1){ |  | MFM 4 |
| Wideband CQI | 4 |  |
| Wideband STC rate | 3 2 | ‘STC rate – 1’ mapped to a 3-bit unsigned integer (i.e., STC rate=1 as 0b000 ~ STC rate = 8 as 0b111) Indicates STC rate (R) used for wideband beamforming. 0b00: STC rate (R) = 1 0b01: STC rate (R) = 2 0b10: STC rate (R) = 3 0b11: STC rate (R) = 4 |
| Wideband PMI | 6 | Wideband preferred matrix index (PMI), size of which is the number of PMI bits (‘NB’) used, mapped to NB LSB bits of this field, while the remaining MSB bit(s) set to zero(0) |
| } |  |  |
| If (LSB#2 in MFM_bitmap == 1){ |  | MFM 7 |
| Wideband CQI | 4 |  |
| Wideband PMI | 6 | Wideband preferred matrix index (PMI), size of which is the number of PMI bits (‘NB’) used, mapped to NB LSB bits of this field, while the remaining MSB bit(s) set to zero(0) |
| } |  |  |
| Reserved | variable | For byte alignment |
| } |  |  |

------------------------------------------------ Proposed texts end --------------------------------------------------------