| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text changes related to MIMO control parameters (IEEE Std 802.16m) |  |
| Date Submitted | 2011-07-12 |  |
| Source(s) | Jisoo Park, Sookjin Lee ETRI | jsp@etri.re.kr +82-42-860-5748 |
| Re: | Call for Change Requests regarding IEEE Std 802.16 (Volume: IEEE Std 802.16m) |  |
| Abstract | The contribution proposes the text changes related to MIMO control parameters |  |
| Purpose | To be discussed and adopted by Maintenance Task Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text changes related to MIMO control parameters (IEEE Std 802.16m)

Jisoo Park, Sookjin Lee

ETRI

- Introduction

This contribution proposes the text changes for the clarification related to two control parameters for indexing a stream of MU-MIMO. One is PSI for DL MU-MIMO, and another is SI for UL MU-MIMO.

- For DL MIMO, PSI

The pilot stream index, PSI is defined as DL MIMO stream index and the value of PSI is stream index starting from 1 and numbering up to 8 according to the number of streams in DL transmission. PSI is used to distinguish a DL MIMO stream in DL Basic Assignment A-MAP IE, DL Subband Assignment AMAP IE and DL Persistent Allocation A-MAP IE. However, it is differently used to indicate indexing the allocated pilot stream in DL Basic Assignment A-MAP IE and DL Persistent Allocation A-MAP IE. For example, PSI is in use with the index value by staring from 0(stream0) in DL Basic Assignment A-MAP IE, but starting from 1(stream1) in DL Persistent Allocation A-MAP IE.

- For UL MIMO, SI

The first pilot index, SI is defined as UL MIMO stream index and the value of SI is starting from 1, and it is numbering up to 8 according to the number of UL MIMO streams (TNS). SI is used to distinguish a UL MIMO stream in UL Persistent Allocation A-MAP IE as well as UL Basic Assignment A-MAP IE, UL Subband Assignment AMAP IE. But, in UL Persistent Allocation A-MAP IE, SI is directly set by mapping to the value of range from 0(0b00) to 3(0b11). So it is needed to be modified with additional indication of the UL MIMO stream index.

In the current standard [1], the some usages of these MIMO control parameters can make confusion about the numbering of stream index and naming PSI or SI. Then, this contribution provides the text changes related to MIMO control parameters for DL and UL and some editorial correction so that the usage of pilot stream index can be fixed and consistently used over several sections.

- References

[1] IEEE Std 802.16m-2011, “IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Amendment 3: Advanced Air Interface,” 2011-05-06

- Proposed Text Changes

[Added texts and figures marked in blue font with underline and removed texts and figures marked in red font with strikeout]

----------------------------------------------- Start of Proposed Text Changes -----------------------------------------------

[Remedy#1: Adopt the following modification text on Table 879 in page 727, section 16.3.6.2.5.4 Downlink signaling support of DL-MIMO modes]

Table 879—DL MIMO control parameters

| Parameter | Description | Value | Notes |
| --- | --- | --- | --- |
| … | … | … | … |
| MEF | MIMO encoder format | SFBC Vertical encoding Multi-layer encoding CDR | MIMO encoder format. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent Allocation A-MAP IE |
| Mt | Number of MIMO streams in transmission | 1 to 8 | Number of MIMO streams in the transmission. Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent Allocation A-MAP IE |
| SI PSI | Index of allocated pilot MIMO stream | 1 to 8 | SI PSI shall be indicated if MEF is ME (Multi-layer Encoding). Indicated in DL Basic Assignment A-MAP IE, DL Subband Assignment A-MAP IE, DL Persistent Allocation A-MAP IE |
| MFM | MIMO feedback mode | Refer to Table 882 Table 878 | To decide the feedback content and related MS processing Indicated in Feedback Allocation A-MAP IE, Feedback Polling A-MAP IE |
| … | … | … | … |

[Remedy#2: Adopt the following modification text on Table 852 in page 665, section 16.3.5.5.2.4.1 DL basic assignment A-MAP IE]

| Table 852— DL basic assignment A-MAP IEa |  |  |
| --- | --- | --- |
| Syntax | Size (bits) | Description/Notes |
| DL_Basic_Assignment_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| … | … | … |
| } else if(MEF == 0b10) { |  | Parameters for multi-layer encoding |
| if (Nt == 4){ |  | Parameters for multi-layer encoding for ABS with modulation constellation of paired user for up-to 4 stream transmission. |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO, and the modulation constellation of paired user 0b00000: 2 streams with PSI=stream0 stream1 and other modulation =QPSK 0b00001: 2 streams with PSI= stream0 stream1 and other modulation =16QAM 0b00010: 2 streams with PSI= stream0 stream1 and other modulation =64QAM 0b00011: 2 streams with PSI= stream0 stream1 and other modulation information not available 0b00100: 2 streams with PSI= stream1 stream2 and other modulation =QPSK 0b00101: 2 streams with PSI= stream1 stream2 and other modulation =16QAM 0b00110: 2 streams with PSI= stream1 stream2 and other modulation =64QAM 0b00111: 2 streams with PSI= stream1 stream2 and other modulation information not available 0b01000: 3 streams with PSI= stream0 stream1 0b01001: 3 streams with PSI= stream1 stream2 0b01010: 3 streams with PSI= stream2 stream3 0b01011: 4 streams with PSI= stream0 stream1 0b01100: 4 streams with PSI= stream1 stream2 0b01101: 4 streams with PSI= stream2 stream3 0b01110: 4 streams with PSI= stream3 stream4 0b01111: 3 streams with PSI= stream0 stream1 and stream1 stream2 0b10000: 4 streams with PSI= stream0 stream1 and stream1 stream2 0b10001: 4 streams with PSI= stream2 stream3 and stream3 stream4 0b10010: 3 streams with PSI= stream2 stream3 and other modulation =QPSK 0b10011: 3 streams with PSI= stream2 stream3 and other modulation =16QAM 0b10100: 3 streams with PSI= stream2 stream3 and other modulation =64QAM 0b10101: 3 streams with PSI= stream0 stream1 and stream1 stream2 and other modulation =QPSK 0b10110: 3 streams with PSI= stream0 stream1 and stream1 stream2 and other modulation =16QAM 0b10111: 3 streams with PSI= stream0 stream1 and stream1 stream2 and other modulation =64QAM 0b11000: 4 streams with PSI= stream0 stream1 and stream1 stream2 and other modulation =QPSK 0b11001: 4 streams with PSI= stream0 stream1 and stream1 stream2 and other modulation =16QAM 0b11010: 4 streams with PSI= stream0 stream1 and stream1 stream2 and other modulation =64QAM 0b11011: 4 streams with PSI= stream2 stream3 and stream3 stream4 and other modulation =QPSK 0b11100: 4 streams with PSI= stream2 stream3 and stream3 stream4 and other modulation =16QAM 0b11101: 4 streams with PSI= stream2 stream3 and stream3 stream4 and other modulation =64QAM 0b11110: reserved 0b11111: reserved |
| } else { |  | Parameters for multi-layer encoding for ABS for ABS with modulation constellation of paired user for up-to 8 stream transmission. |
| Si | 5 | Index used to identify the combination of the number of streams and the allocated pilot stream index(PSI) in a transmission with MU-MIMO, and the modulation constellation of paired user in the case of 2 stream transmission 0b00000: 2 streams with PSI= stream0 stream1 and other modulation =QPSK 0b00001: 2 streams with PSI= stream0 stream1 and other modulation =16QAM 0b00010: 2 streams with PSI= stream0 stream1 and other modulation =64QAM 0b00011: 2 streams with PSI= stream0 stream1 and other modulation information not available 0b00100: 2 streams with PSI= stream1 stream2 and other modulation =QPSK 0b00101: 2 streams with PSI= stream1 stream2 and other modulation =16QAM 0b00110: 2 streams with PSI= stream1 stream2 and other modulation =64QAM 0b00111: 2 streams with PSI= stream1 stream2 and other modulation information not available 0b01000: 3 streams with PSI= stream0 stream1 0b01001: 3 streams with PSI= stream1 stream2 0b01010: 3 streams with PSI= stream2 stream3 0b01011: 4 streams with PSI= stream0 stream1 0b01100: 4 streams with PSI= stream1 stream2 0b01101: 4 streams with PSI= stream2 stream3 0b01110: 4 streams with PSI= stream3 stream4 0b01111: 3 streams with PSI= stream0 stream1 and stream1 stream2 0b10000: 4 streams with PSI= stream0 stream1 and stream1 stream2 0b10001: 4 streams with PSI= stream2 stream3 and stream3 0b10010: 8 streams with PSI= stream0 stream1 0b10011: 8 streams with PSI= stream1 stream2 0b10100: 8 streams with PSI= stream2 stream3 0b10101: 8 streams with PSI= stream3 stream4 0b10110: 8 streams with PSI= stream4 stream5 0b10111: 8 streams with PSI= stream5 stream6 0b11000: 8 streams with PSI= stream6 stream7 0b11001: 8 streams with PSI= stream7 stream8 0b11010: 8 streams with PSI= stream0 stream1 and stream1 stream2 0b11011: 8 streams with PSI= stream2 stream3 and stream3 stream4 0b11100: 8 streams with PSI= stream4 stream5and stream5 stream6 0b11101: 8 streams with PSI= stream6 stream7and stream7 stream8 0b11110: reserved 0b11111: reserved |
| } |  |  |
| … | … | … |

[Remedy#3: Adopt the following modification text on Table 861 in page 690, section 16.3.5.5.2.4.9 UL persistent allocation A-MAP IE]

| Table 861—UL persistent allocation A-MAP IEa |  |  |
| --- | --- | --- |
| Syntax | Size (bits) | Description/Notes |
| UL_Persistent_Allocation_A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | UL Persistent Allocation A-MAP IE |
| … | … | … |
| if (TNS > Mt){ | — | — |
| SI | 2 | First pilot index for CSM with TNS = 2 streams: 0b00, 0b01 First pilot index for CSM with TNS = 3,4 streams: 0b00, 0b01, 0b10, 0b11 Indicates first pilot index for CSM with TNS = 2, 3 or 4 streams: 0b00: SI = 1 0b01: SI = 2 0b10: SI = 3 0b11: SI = 4 If TNS = 2 streams, 0b10 and 0b11 are not applicable. If TNS = 3 streams, 0b11 is not applicable. |
| } |  |  |
| … | … | … |

------------------------------------------------ Proposed texts end --------------------------------------------------------