| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of PGID-Info message (CR for IEEE Std 802.16m) |  |
| Date Submitted | Nov 06, 2011 |  |
| Source(s) | Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak LG Electronics | giwon.park.@lge.com |
| Re: | IEEE 802.16-11/0026, IEEE 802.16 Working Group Letter Ballot #32 |  |
| Abstract | Change is provided to clarify the PGID-Info message field description. |  |
| Purpose | Approve and add to Maintenance TG database |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of PGID-Info message (CR for IEEE Std 802.16m)

Giwon Park, Youngsoo Yuk, Inuk Jung, JinSam Kwak

LG Electronics

# Introduction

An AMS should monitor only a specific paging frame for its paging instead of the entire superframe in order to minimize battery usage. The solution is to use parameter ‘m’ in PGID_Info message. The ‘m’ value of PGID-Info message is used to determine the frame index of a superframe for paging message transmission of an idle mode AMS. Moreover, ABS set the m value to distribute its entire paging load at the paging message transmission time regardless of paging group. However, ‘m’ value is located within ‘for loop (Num-PGIDs)’ of PGID-Info message in current Std. P802161_D2. This causes unnecessary overhead [i.e., {(Num-PGIDs – 1) * m value} bits].

Thus, we propose to modify the PGID-Info message by removing “m” from being dependent on PGID.

# Proposed Text Change

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

[Remedy 1. Modify the table 50 in page 139, line 1 as follows:]

Table 50– PGID-Info message field description

| Field | Size | Value/Description | Condition |
| --- | --- | --- | --- |
| For(i=0; i<Num_PGIDs; i++) { |  | Indicate the number of PGID included in PGID-Info message 1..4 |  |
| PGID | 16 | Indicate Paging group identifier where the ABS belongs 0..-1 | Shall be present |
| m | 2 | Time domain hash parameter (1 ~ 4) used to determine the frame index of a superframe for paging message transmission of an idle mode AMS. Indicate m in the equation Npaging frame = AMS’s deregistration identifier mod m to determine a predefined paging frame for the AMS 1..4 | Shall be present |
| } |  |  |  |
| m | 2 | Time domain hash parameter (1 ~ 4) used to determine the frame index of a superframe for paging message transmission of an idle mode AMS. Indicate m in the equation = AMS’s deregistration identifier mod m to determine a predefined paging frame for the AMS 1..4 | Shall be present |
| … |  |  |  |

# References

[1] IEEE P802.16.1/D2 Oct_2011