| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Resource Index (RI) Encoding for Assignment IEs (6.3.5.5.2.4.1) |  |
| Date Submitted | 2011-11-04 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | 802.16 Working Group Letter Ballot #32 for IEEE P802.16.1/D2. |  |
| Abstract | The contribution proposes an alternative RI encoding scheme for Assignment IEs. |  |
| Purpose | To be discussed and adopted by 802.16.1 standard. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Resource Index (RI) Encoding for Assignment IEs (6.3.5.5.2.4.1)

Lei Wang

InterDigital Communications, LLC

- Introduction

During 16m development, I pointed out there is a design deficit in the 20MHz system bandwidth by the comment #554 in commentary database 802.16-10/0042 in session #68. Here's the original comment:

"In the 20MHz system bandwidth, there are 4656 possible combinations of (L, S), where L is the location of an allocation; and S is the size of an allocation. With 11-bit RI field, those 4656 combinations cannot be signaled by the RI field. Based on the text in line 9 to line 23 on page 560, the number of allowed S values is reduced. Basically, the allocation granularity is no long 1 LRU, it is actually 1, 2, 4, and 8, depending on the value of S.

Sacrificing the allocation granularity seems a very bad design choice, particularly at steps as big as 8 LRUs. Even with code-matching schemes, the offset of the required size to the nearest allowed S value can be up to 4 LRUs. This makes the ratio of the offset to the assigned size is greater than majority of the code steps based on the nominal MCS table given in Table 934, on page 729 in 16m/D6.

We would recommend reconsidering the RI field encoding issue, particularly for the 20MHz system bandwidth, instead of sacrificing the allocation granularity, looking for some other alternatives, e.g., change the RI field from 11 bits to 12 bits by using the 1 reserved bit, and/or consider the constraints of the allocations to remove those ones that do not need to be signaled by the assignment A-MAP IEs, e.g., the control channel occupied resources, and/or allocations spanning over multiple frequency partitions, etc."

The above comment was rejected, resubmitted, and rejected again, for multiple rounds until 16m completion. Here's the reason of rejection "This issue was analyzed in the original design. Refer to the analysis in section 4 of contribution C802.16m-09/1334r1. It has been shown that link adaptation with the granularity of feedback MCS levels as defined in the 802.16m is not adversely affected by the proposed reduction in assignable resource indices with 11 bits for 20MHz. The original analysis does require an update with delta_min = 31/256 based on Table 834, but this change does not change the final conclusion since 1/6 < 31/1422."

Note that 1/6 is not less than 31/1422. It is actually way bigger than 31/1422. Therefore the 16m 20MHz resource allocation design is based on a serious Math error. Such an obvious error really bothers me. I would like to re-trigger the discussions again about this issue, and hope we can fix it during this revision project.

I would recommend reconsidering the RI field encoding issue, particularly for the 20MHz system bandwidth, instead of sacrificing the allocation granularity,  looking for some other alternatives, e.g., change the RI field from 11 bits to 12 bits by using the 1 reserved bit, and/or consider the constraints of the allocations to remove those ones that do not need  to be signaled by the assignment A-MAP IEs, e.g., the control channel occupied resources, and/or allocations spanning over multiple frequency partitions, etc.

- Discussions

For simplicity and also for the purpose of triggering the discussion, this contribution proposes:

- Change the RI field from 11 bits to 12 bits by using the 1 reserved bit, which gives a total of 4096 RI codes;

- For 20MHz systems, reduce the allowed assignable sizes, i.e., allow all the resource sizes except the odd resource sizes in the range of 46 to 80 LRUs, which gives a total of 4078 combinations of allocation size (S) and location (L).

With the above proposal, the allocation granularity issue is significantly released, as the maximum step size is reduced from 8 LRUs to 2 LRUs and also the number of disallowed sizes is reduced from 61 to 17.  In addition, since we follow the same RI usage design in the current spec, the required changes are minimized.

- Suggested changes in IEEE P802.16.1/D2

The following is the proposed change in the P802.16.1/D2. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: line 21, page 622

Change line 21 to 28 on page 622 as follows:

•All resource sizes in increments of 1 LRU are assignable in the range of 1 to 46 12 LRUs.

•Only even resource sizes are assignable in increments of 2 LRUs in the range of 47 14 to 79 24 LRUs.

•Resource sizes are assignable in increments of 4 LRUs in the range of 28 to 48 LRUs .

•Resource sizes are assignable in increments of 8 LRUs in the range of 56 to 88 LRUs.

•All resource sizes in increments of 1 LRU are assignable in the range of 80 92 to 96 LRUs.

Suggested change #2: line 12, page 623, Formula (238)

Replace the formula (238) in line 12 on page 623 by the following:

Suggested change #3: line 7, page 621

Change the row of Resource Index in Table 195 as follows:

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Resource Index. | 12 11 | 5 MHz: 0 in first 3 2 MSB bits + 9 bits for resource index 10 MHz: 0 in first 1 MSB bit + 11 bits for resource index 20 MHz: 12 11 bits for resource index Resource index includes location and allocation size. |

Suggested change #4: line 37, page 621

Remove the row of the “Reserved” row.

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16.1/D2