| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SFBC support in UL SLRU |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Zheng Yan-Xiu Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | L802.16-11/0062, liaison from WiMAX TWG regarding interoperability problems. |  |
| Abstract | SFBC support in UL SLRU |  |
| Purpose | As basis of LB32 data base for IEEE 802.16.1/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SFBC support in UL SLRU

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

For increasing throughput with MIMO, SLRU may be usually used in DL/UL subframe. ABS may assign MIMO feedback message with Feedback Polling A-MAP IE in DLRU or NLRU, but MIMO feedback message can be assigned in SLRU if there is no available resource in DLRU or NLUR.

MIMO feedback message by Feedback Polling A-MAP IE can be transmitted with SFBC encoding or vertical encoding with Mt=2 for Nt=2 or 4 or vertical encoding with Mt=1 for Nt=1. If the MIMO feedback message assigned by Feedback Polling A-MAP IE is assigned in SLRU for an AMS with two or four transmit antenna, the MIMO feedback message shall be transmitted with vertical encoding with Mt=2 since SFBC encoding is not supported in UL SLRU.

The vertical encoding with Mt=2 may not be supportable for users in cell-edge. For reliable reception of MIMO feedback message assigned by Feedback Polling A-MAP IE in SLRU, SFBC encoding needs to be supported in UL SLRU

# Proposed Text

[Adopt the following change in section 6.3.9.1.2.1 on page 805 in IEEE 802.16.1/D2:]

Table 292—Codebook subsets used for non-adaptive precoding in UL SLRU

| MEF | RU with Mt pilot streams |
| --- | --- |
| SFBC | N/A Nt=2: Cbase,UL(2, 2, 3) Nt=4: Cbase,UL(4, 2, 6) |
| VE | Nt=2: Cbase,UL (2, 1, 4) if Mt=1 or Cbase,UL(2, 2, 3) if Mt=2 Nt=4: Cbase,UL (4, Mt, 6), Mt=1,2,3,4 |

[Adopt the following change in section 6.3.9.2.4 on page 809 in IEEE 802.16.1/D2:]

Table 297—Supported permutation for each UL MIMO mode

|  | DLRU | NLRU | SLRU |
| --- | --- | --- | --- |
| MIMO mode 0 | Yes | Yes | Yes No |
| MIMO mode 1 | Yes, with Mt ≤2 | Yes | Yes |
| MIMO mode 2 | Yes, with Mt ≤2 | Yes | Yes |
| MIMO mode 3 | Yes, with Mt = 1 | Yes | Yes |
| MIMO mode 4 | Yes, with Mt = 1 | Yes | Yes |