| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification for UL Burst Transmission Scheme by BR-ACK Assignment in 16m Systems |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Zheng Yan-Xiu, Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | L802.16-11/0062, liaison from WiMAX TWG regarding interoperability problems. |  |
| Abstract | Clarification for UL Burst Transmission Scheme by BR-ACK Assignment in 16m Systems |  |
| Purpose | As basis of LB32 data base for IEEE 802.16.1/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification for UL Burst Transmission Scheme by BR-ACK Assignment in 16m Systems

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

Through BR-ACK A-MAP IE, ABS allocates UL resources to an anonymous user that requested bandwidth using a random BR preamble code. The BR-ACK A-MAP IE indicates the decoding status of the BR preamble sequences transmitted through the BR channels and includes the UL resource allocation information for the fixed sized BR header.

However, in the standard IEEE Std 802.16.m-2011, there is no any description related to the UL burst transmission information such as the MIMO scheme (i.e., MEF) and modulation scheme (i.e., ISizeOffset) for the UL resource signaled by BR-ACK A-MAP IE. So, it is needed to add the specification of UL burst transmission information (i.e., MIMO scheme and modulation scheme) in the related section.

# Proposed Text

[Modify the section 6.3.5.5.2.4.12 on page 667, line 54 in IEEE802.16.1/D2 as follows:]

6.3.5.5.2.4.12 BR ACK A-MAP IE

…

The UL HARQ burst signaled via the BR-ACK A-MAP IE is always transmitted using MIMO mode 0 as the MIMO encoder format and QPSK as the modulation scheme.