| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL Power Control For Control Channel |  |
| Date Submitted | 2011-11-09 |  |
| Source(s) | Zheng Yan-Xiu, Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | L802.16-11/0062, liaison from WiMAX TWG regarding interoperability problems. |  |
| Abstract | UL Power Control For Control Channel |  |
| Purpose | As basis of LB32 data base for IEEE 802.16.1/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

UL Power Control For Control Channel

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

Uplink power control is supported for both an initial calibration and periodic adjustment on transmit power without loss of data. The uplink power control algorithm determines the transmission power of an OFDM symbol to compensate for the pathloss, shadowing, and fast fading.

The power per subcarrier and per stream shall be calculated according to Equation (302) in IEEE Std 802.16m-2011.

PdBm= L + SINRTarget + NI + Offset                                                          (302)

where offset is a correction term for AMS-specific power offset. It is controlled by the ABS through power control messages. There are two kinds of Offset values that are used for data and control, respectively.

For a data channel transmission, the Offset in Equation (302) shall be set to the value OffsetData conveyed in an AAI-UL_POWER_ADJ message. For a control channel transmission, the Offset in Equation (302) shall be set to the value OffsetControl conveyed as offsetControl in an AAI-UL_POWER_ADJ message.

In case of control channel transmission, the ABS may change the AMS’s TX power through direct power adjustment by PC-A-MAP. When AMS receives its PC-A-MAP IE from the ABS, it shall modify its OffsetControl value according to Equation (305) in IEEE Std 802.16m-2011.

OffsetControl = OffsetControl + △PowerAdjust                                (305)

where △PowerAdjust is the power correction value indicated by ABS through PC-A-MAP.

If AMS receives AAI-UL_POWER_ADJUST message, the AMS’s OffsetControl shall be replaced by offsetControl indicated by AAI-UL_POWER_ADJUST message.

The value of OffsetControl can be changed by PC-A-MAP IE, but there is no way to check if the AMS correctly received PC-A-MAP IE. Therefore, the ABS may not know the AMS’s exact value of OffsetControl.

In current AAI-UL_POWER_ADJ message, both offsetData and offsetControl are mandatory field. If ABS is not sure of current OffsetControl value, it is required to maintain its current value, so it needs to change offsetControl as an optional field.

# Proposed Text

[Adopt the following change in Table 59 on page 163 in section 6.2.3.33 in IEEE 802.16.1/D2:]

6.2.3.33 AAI-UL-POWER-ADJ message

An AAI-UL-POWER-ADJ message shall be transmitted by the ABS to control the transmit power level of

AMS.

Table 59—AAI-UL-POWER-ADJ message field description

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| offsetData | 6 | offsetData is the transmit power adjustment value transmitted by the ABS. It represents the value among –15.5 to 16 dB with 0.5 dB step. | N/A |
| offsetControl | 6 | offsetControl is the transmit power adjustment value transmitted by the ABS. It represents the value among –15.5 to 16 dB with 0.5 dB step. | N/APresent when ABS wants to change the transmit power adjust value of UL control channel. (More details can be found in 6.3.8.4.2) |
| channelIndex | 8 | This value corresponds to “Channel Index” in Feedback Allocation AMAP IE. | Present when ABS wants to change Fast Feedback Control Channel Allocation by using AAI-UL-POWER-ADJ message. (More details can be found in 6.3.5.5.2.4.5.) |
| shortTermFeedbackPeriod | 3 | This value corresponds to “Short-term feedback period (p)” in Feedback Allocation A-MAP IE. | Present when ABS wants to change Fast Feedback Control Channel allocation by using AAI-UL-POWER-ADJ message. (More details can be found in 6.3.5.5.2.4.5.) |