| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Correction on “Unsolicited Grant Interval” |  |
| Date Submitted | 2011-3-16 |  |
| Source(s) | Zheng Yan-Xiu, Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | L802.16-11/0005, liaison from WiMAX TWG regarding interoperability problems. |  |
| Abstract | Changes are provided to accommodate Correction on “Unsolicited Grant Interval” |  |
| Purpose | As basis of maintenance data base for IEEE 802.16-2009 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Correction on “Unsolicited Grant Interval”

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

This contribution is intended to incorporate changes agreed based on the liaison IEEE L802.16-11/0005.

# Proposed Text

[Modify section 6.3.14.4.1 on page 372 in IEEE 802.16-2009 as follows:]

Unsolicited Grant Interval

This parameter defines the nominal interval between successive data grant opportunities for a DUL service flow, which are associated with Uplink Grant Scheduling Type = UGS or ertPS (refer to the Table 189 and 11.13.19). If this parameter is set to zero, then there is no explicitly mandated unsolicited grant interval. The maximum unsolicited grant interval field specifies only a bound, not a guarantee that the rate is available. The algorithm for policing this parameter is left to vendor differentiation and is outside the scope of the standard.