| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Direct HO from 16m only ABS to 16e BS |  |
| Date Submitted | 2011-9-21 |  |
| Source(s) | Zheng Yan-Xiu, Maintenance Task Group | zhengyanxiu@itri.org.tw |
| Re: | L802.16-11/0053, liaison from WiMAX TWG regarding interoperability problems. |  |
| Abstract | Changes are provided to accommodate Direct HO from 16m only ABS to 16e BS |  |
| Purpose | As basis of maintenance data base for IEEE 802.16.1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Direct HO from 16m only ABS to 16e BS

Zheng Yan-Xiu

Maintenance Task Group

# Introduction

This contribution is intended to incorporate changes agreed based on the liaison IEEE L802.16-11/0053.

# Proposed Text

[Remedy 1:]

[Change subclause 6.2.6.4.1.2.2 on line 42 in page 338 in IEEE 802.16.1/D0 as follows:]

An AMS served by a R1 BS having neighboring AAI-only ABSs may discover an AAI-only ABS and decide to directly HO to this ABS. The AMS, while served by an R1 BS, may obtain target AAI-only ABS information by receiving an MOB_NBR-INFO message. In this case, the AMS performs HO decision and initiation with the serving R1 BS per section 6.3.21.2 in IEEE 802.16-2009, and then performs WirelessMAN-Advanced Air Interface System network reentry procedure to the T-ABS per 6.2.6.3.5.2. The AMS may obtain target AAI-only ABS information by blind scanning.

If T-ABS is capable of identifying AMS and retrieving its context, it may inform AMS to skip certain network reentry steps using “Reentry Process Optimization” bitmap in AAI-RNG-RSP. In this case, it is recommended that “Reentry Process Optimization” bitmap is set to 01010 to indicate that AAI-SBC-REQ/RSP and AAI-REG-REQ/RSP transactions are not omitted, PKM Authentication phase and IP address refresh are omitted.

Otherwise, the AMS shall perform a full network entry at the T-ABS.

[Remedy 2:]

[Change subclause 6.2.6.4.1.3 on line 7 in page 339 in IEEE 802.16.1/D0 as follows:]

With zone switch based handover, the context management process from serving R1 BS to the LZone of the T-ABS is specified in section 6.3.21.2.8.1 in IEEE 802.16-2009. This subclause describe the context mapping from the LZone to the MZone of the T-ABS during network reentry procedure in the MZone in the case of zone switch based handover, or context mapping from serving R1 BS to the T-ABS during network reentry procedure in the T-ABS in the case of direct handover.

The ARQ context and state machine associated with serving R1 BS or LZone shall not be used in MZone and may be discarded upon expiration of the ABS_Resource_Retain_Timer or completion of network reentry into MZone in both direct handover and zone switch cases. After the AMS completes network reentry into the T-ABS, the T-ABS initiates an AAI-DSC transaction to update ARQ parameters if ARQ is applied to the AMS. In this case, ARQ parameters for multiple service flows may be updated in a single AAI-DSC procedure using “Group Parameter Create/Change” parameter.

[Remedy 3:]

[Add the following texts in Table 743 AAI-DSC-REQ message in page 219 in IEEE Std 802.16.1/D0: ]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| … |  |  |  |
| } //end if QoS |  |  |  |
| If (ARQ parameters are needed) { |  |  | Present when an ABS initiates AAI-DSC transaction with an AMS that has performed network reentry with the ABS from direct HO or zone switch handover |
| ARQ_WINDOW_SIZE | 16 | >0 and (ARQ_BSN_MODULUS/2) | Present if needed |
| ARQ_BLOCK_LIFETIME | 16 | 0=Infinite 1-6553500s (100s granularity) | Present if needed |
| ARQ_SYNC_LOSS_TIMEOUT | 16 | 0=Infinite 1-6553500s (100s granularity) | Present if needed |
| ARQ_RX_PURGE_TIMEOUT | 16 | 0=Infinite 1-6553500s (100s granularity) | Present if needed |
| ARQ_SUB_BLOCK_SIZE | 3 | Bit 0-2: encoding for selected block size (P), where the selected block size is equal to 2(P+3), 0P7. ARQ sub-block size is byte unit | Present if needed |
| ARQ_ERROR_DETECTION_TIMEOUT | 16 | 0=Infinite 1-6553500s (100s granularity) | Present if needed |
| ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | 16 | 0=Infinite 1-6553500s (100s granularity) | Present if needed |
| } |  |  |  |
| If(ROHC Parameter is needed) { |  |  | Present if needed |
| … |  |  |  |

[Remedy 4:]

[Add the following texts in Table 696 AAI-NBR-ADV message on line 50 in page 105 in IEEE 802.16.1/D0: ]

| Field | Size (bits) | Value/Description | Condition |
| --- | --- | --- | --- |
| Change Count | 3 | AAI-NBR-ADV Change Count |  |
| … | … | … | … |
| for (i=0; i<N-NBR-R1-BSs;i++) { |  | N-NBR-R1-BSs is the number of neighbor Release 1 BSs. | Shall be present if R1 NBR BS list is available |
| { |  |  |  |
| … | … | … | … |
| R1 BS Center Frequency | 32 | Indicates center frequency (in unit of Hz) of the neighbor R1 BS. | Present when S-ABS carrier index is different from that of target R1 BS. |
| DCD configuration change count | 8 | This represents the neighbor R1 BS’ current DCD configuration change count | May be present |
| UCD configuration change count | 8 | This represents the neighbor R1 BS’ current UCD configuration change count, | May be present |
| if(i == 0) { |  |  |  |
| DCD_Info | Variable | The information from 1st neighbor R1 BS’ DCD message (Table 575 in IEEE 802.16-2009) | May be present. If this parameter present, it is only for 1st neighbor R1 BS |
| UCD_Info | Variable | The information from 1st neighbor R1 BS’ UCD message (Table 568 in IEEE 802.16-2009) | May be present. If this parameter present, it is only for 1st neighbor R1 BS |
| } else { |  |  |  |
| Delta_DCD | Variable | Delta encoding of DCD information (Table 575 in IEEE 802.16-2009) that is different from the 1st neighbor R1 BS | Present if DCD information is different from that of 1st neighbor R1 BS |
| Delta_UCD | Variable | Delta encoding of UCD information (Table 568 in IEEE 802.16-2009) that is different from the 1st neighbor R1 BS | Present if UCD information is different from that of 1st neighbor R1 BS |
| } |  |  |  |
| } // end of for N-NBR-R1-BSs |  |  |  |
| … |  |  |  |

[Remedy 5:]

[Add the following texts in Table 694 AAI-HO-REQ message on line 54 in page 95 in IEEE 802.16.1/D0: ]

16.2.3.11 AAI-HO-REQ

…

| Table 694—AAI-HO-REQ message field description |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| … | … | … | … |
| for(i=0; i < N_New_ABS_Full; i++) { |  | N_New_ABS_ID is the number of neighboring ABSs to be are identified by full considered for handover, which is 48-bit ABSID. Range: 0 ~ 256 |  |
| … | … | … | … |
| } |  |  |  |
| for(i=0; i < N_R1_BS_Index; i++) { |  | N_R1_ABS_Index is the number of neighboring R1 BS index to be considered for handover Range: 0 ~ 255 |  |
| Change Count | 3 | AAI-NBR-ADV Change Count |  |
| Neighbor BS Index | 8 | Indicates neighbor BS index index in AAI-NBR-ADV which contains the Change Count |  |
| BS CINR mean | 8 | CINR mean of the new BS | May be included if it is available at the MS. |
| BS RSSI mean | 6 | RSSI mean of the new BS | May be included if it is available at the MS. |
| } |  |  |  |

[Remedy 6:]

[Add the following texts in Table 695 AAI-HO-CMD message on line 10 in page 101 in IEEE 802.16.1/D0: ]

16.2.3.12 AAI-HO-CMD

…

| Table 695—AAI-HO-CMD message field description |  |  |  |
| --- | --- | --- | --- |
| Field | Size (bits) | Value/Description | Condition |
| … | … | … | … |
| If (Mode == 0b00) { |  |  |  |
| …. |  |  |  |
| for(i=0; i < N_Target_BS; i++) { |  | N_Target_BS is the number of T-ABSs or target legacy BSs included in this message. |  |
| targetBSID | 48 | BSID of the T-ABS or target legacy BS |  |
| … | … | … | … |
| S-SFH application indicator | 1 | Indicate if the SFH delta information at the T-ABS is applied at the Action Time 0b0: not applied 0b1: applied | Shall be included when SFH delta information is included |
| DCD Configuration Change Count | 8 | This represents the neighbor BS current DCD configuration change count. | May be included if target BS is legacy BS |
| UCD Configuration Change Count | 8 | This represents the neighbor BS current UCD configuration change count. | May be included if target BS is legacy BS |
| DCD_Info | Variable | The information from DCD message (Table 575 in IEEE 802.16-2009) of target BS with the targetBSID | May be present if target BS is legacy BS. It is included only when AAI-NBR-ADV did not contain DCD_info for the targetBSID or HO is issued by Serving ABS |
| UCD_Info | Variable | The information from UCD message (Table568 in IEEE 802.16-2009) of target BS with the targetBSID | May be present if target BS is legacy BS. It is included only when AAI-NBR-ADV did not contain UCD_info for the targetBSID or HO is issued by Serving ABS |
| }//end of for(i=0; i < N_Target_BS; i++) |  |  |  |
| }else if (Mode == 0b01) { |  |  |  |
| … | … | … | … |