| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose performance counter objects for 802.16 performance characterization |  |
| Date Submitted | 2007-09-14 |  |
| Source(s) | Joey Chou Intel Shawn Wallace Sprint Fran Burns Sprint | E-mail: joey.chou@intel.com E-mail: Shawn.Wallace@sprint.com E-mail: francis.burns@sprint.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25c, on P802.16i/D4, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes performance counter objects for 802.16 performance characterization. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Propose performance counter objects for 802.16 performance characterization

Joey Chou, Shawn Wallace, Fran Burns

Intel, Sprint

# Introduction

This document proposes the following performance counters objects that are important to characterize the 802.16 WMAN network performance.

- wmanIf2BsStatisticsConfigTable

- wmanIf2BsMsStartupStatisticsTable

- wmanIf2BsMsDlThroughputStatisticsTable

- wmanIf2BsMsUlThroughputStatisticsTable

# Proposed Text

# 2.1	MIB Module Description

[Change subclause 13.1.3.1.2 as the following:]

13.1.3.1.2	wmanIf2BsCps

Figure 4 shows the structure of wmanIf2BsCps subtree that contains BS managed objects related to the MAC CPS management entity layer.

Figure 4 – wmanIf2BsCps Structure

[Change subclause 13.1.3.1.2.3 as the following:]

13.1.3.1.2.3	wmanIf2BsStatistics

13.1.3.1.2.3.1	wmanIf2BsChannelMeasurementTable

wmanIf2BsChannelMeasurementTable contains channel measurement information on the uplink signal that were received from SS, and the downlink signal were obtained from SS using REP-REQ/RSP messages.

13.1.3.1.2.3.2	wmanIf2BsStatisticsConfigTable

wmanIf2BsStatisticsConfigTable contains the configuration of statistics information capture..

13.1.3.1.2.3.3	wmanIf2BsMsStartupStatisticsTable

wmanIf2BsMsStartupStatisticsTable contains statistical information that can be used to characterize MS' performance during the startup.

13.1.3.1.2.3.4	wmanIf2BsMsDlThroughputStatisticsTable

wmanIf2BsMsDlThroughputStatisticsTable contains the throughput statistics for the downlink connections.

13.1.3.1.2.3.5	wmanIf2BsMsUlThroughputStatisticsTable

wmanIf2BsMsUlThroughputStatisticsTable contains the throughput statistics for the downlink connections.

# 2.2	ASN.1 Notations

The ASN.1 code highlighted in blue is the proposed new code.

--

-- Base Station performance statistics

--

wmanIf2BsStatistics OBJECT IDENTIFIER ::= { wmanIf2BsCps 3 }

--

-- Base Station Channel Measurement Table

--

wmanIf2BsChannelMeasurementTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsChannelMeasurementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains channel measurement information as

derived from BS measurement of uplink signal from SS,

and the downlink signal as reported from SS using

REP-REQ/RSP messages. The table shall be maintained as

FIFO to store measurement samples that can be used to

create RSSI and CINR histogram report. When the

measurement entry for a SS reaches the limit, the oldest

entry shall be deleted as the new entry is added to the

table."

REFERENCE

"6.3.2.3.33 in IEEE Std 802.16-2004"

::= { wmanIf2BsStatistics 1 }

wmanIf2BsChannelMeasurementEntry OBJECT-TYPE

SYNTAX      WmanIf2BsChannelMeasurementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry in the table contains RSSI and CINR

signal quality measurement on signal received from the SS.

The primary index is the ifIndex with ifType of

ieee80216WMAN identifing the BS sector.

wmanIf2BsSsMacAddress identifies the SS from which the

signal was received. wmanIf2BsChannelDirection is the

index to the direction of the channel.

wmanIf2BsHistogramIndex is the index to histogram samples.

Since there is no time stamp in the table,

wmanIf2BsHistogramIndex should be increased monotonically,

and wraps around when it reaches the implementation

specific limit."

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsChannelDirection,

wmanIf2BsHistogramIndex }

::= { wmanIf2BsChannelMeasurementTable  1 }

WmanIf2BsChannelMeasurementEntry ::= SEQUENCE {

wmanIf2BsChannelDirection               WmanIf2LinkDirection,

wmanIf2BsHistogramIndex                 Unsigned32,

wmanIf2BsChannelNumber                  WmanIf2ChannelNumber,

wmanIf2BsStartFrame                     INTEGER,

wmanIf2BsDuration                       INTEGER,

wmanIf2BsBasicReport                    BITS,

wmanIf2BsMeanCinrReport                 INTEGER,

wmanIf2BsMeanRssiReport                 INTEGER,

wmanIf2BsStdDeviationCinrReport         INTEGER,

wmanIf2BsStdDeviationRssiReport         INTEGER}

wmanIf2BsChannelDirection OBJECT-TYPE

SYNTAX      WmanIf2LinkDirection

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"wmanIf2BsChannelDirection identifies the direction of a

channel where the measurement takes place."

::= { wmanIf2BsChannelMeasurementEntry 1 }

wmanIf2BsHistogramIndex OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsHistogramIndex identifies the histogram samples

in the table for each subscriber station."

::= { wmanIf2BsChannelMeasurementEntry 2 }

wmanIf2BsChannelNumber OBJECT-TYPE

SYNTAX      WmanIf2ChannelNumber

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Physical channel number to be reported on is only

applicable to license exempt band. For licensed band,

this parameter should be null."

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 3 }

wmanIf2BsStartFrame OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Frame number in which measurement for this channel

started."

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 4 }

wmanIf2BsDuration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 16777215)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Cumulative measurement duration on the channel in

multiples of Ts. For any value exceeding 0xFFFFFF,

report 0xFFFFFF."

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 5 }

wmanIf2BsBasicReport OBJECT-TYPE

SYNTAX      BITS {wirelessHuman(0),

unknownTransmission(1),

primaryUser(2),

channelNotMeasured(3)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Bit #0: WirelessHUMAN detected on the channel

Bit #1: Unknown transmissions detected on the channel

Bit #2: Primary User detected on the channel

Bit #3: Unmeasured. Channel not measured"

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 6 }

wmanIf2BsMeanCinrReport OBJECT-TYPE

SYNTAX      INTEGER (-20 .. 37)

UNITS       "dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Mean CINR report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 7 }

wmanIf2BsMeanRssiReport OBJECT-TYPE

SYNTAX      INTEGER (-123 .. -40)

UNITS       "dBm"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Mean RSSI report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 8 }

wmanIf2BsStdDeviationCinrReport OBJECT-TYPE

SYNTAX      INTEGER (0 .. 29)

UNITS       "dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Standard deviation CINR report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 9 }

wmanIf2BsStdDeviationRssiReport OBJECT-TYPE

SYNTAX      INTEGER (0 .. 42)

UNITS       "dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Standard deviation RSSI report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 10 }

--

--  Statistics Configuration Table

--

wmanIf2BsStatisticsConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsStatisticsConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration of statistics

information capture."

::= { wmanIf2BsStatistics 2 }

wmanIf2BsStatisticsConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsStatisticsConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector, and is

indexed by ifIndex."

INDEX     { ifIndex }

::= { wmanIf2BsStatisticsConfigTable  1 }

WmanIf2BsStatisticsConfigEntry ::= SEQUENCE {

wmanIf2BsDlHistogramSampleInterval    INTEGER,

wmanIf2BsUlHistogramSampleInterval    INTEGER}

wmanIf2BsDlHistogramSampleInterval OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "Second"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object determines the interval between two histogram

samples. "

::= { wmanIf2BsStatisticsConfigEntry 1 }

wmanIf2BsUlHistogramSampleInterval OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "Second"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object determines the interval between two histogram

samples. "

::= { wmanIf2BsStatisticsConfigEntry 2 }

--

-- Mobile Station startup statistics Table

--

wmanIf2BsMsStartupStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsMsStartupStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains statistical information that can be

used to characterize MS' performance during the startup."

::= { wmanIf2BsStatistics 3 }

wmanIf2BsMsStartupStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsMsStartupStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector, and is

indexed by ifIndex."

INDEX     { ifIndex }

::= { wmanIf2BsMsStartupStatisticsTable  1 }

WmanIf2BsMsStartupStatisticsEntry ::= SEQUENCE {

wmanIf2BsMsAuthenAttempt                Unsigned32,

wmanIf2BsMsAuthenSuccess                Unsigned32,

wmanIf2BsMsNetwkEntryAttempt            Unsigned32,

wmanIf2BsMsNetwkEntrySuccess            Unsigned32,

wmanIf2BsMsStartupStatReset             INTEGER,

wmanIf2BsMsStartupStatResetTime         TimeStamp}

wmanIf2BsMsAuthenAttempt OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The MS authentication setup success rate % =

wmanIf2BsMsAuthenSuccess  / wmanIf2BsMsAuthenAttempt * 100

This object counts the number of MS authentication

attempt. The BS increments by one each time the BS

receives:

1) PKMv1 Auth Request,

2) PKMv2 RSA-Request,

3) PKMv2 EAP start message."

::= { wmanIf2BsMsStartupStatisticsEntry 1 }

wmanIf2BsMsAuthenSuccess OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of successful MS

authentication. The BS increments by one each time the BS

sends:

1) PKMv1 Auth Reply,

2) PKMv2 RSA-Reply,

3) PKMv2 EAP complete with EAP-success payload message."

::= { wmanIf2BsMsStartupStatisticsEntry 2 }

wmanIf2BsMsNetwkEntryAttempt OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The MS network entry success rate % =

wmanIf2BsMsNetwkEntrySuccess / wmanIf2BsMsNetwkEntryAttempt

* 100.

This object counts the number of MS network entry

attempt. The BS increments by one each time the BS receives

the RNG-REQ message."

::= { wmanIf2BsMsStartupStatisticsEntry 3 }

wmanIf2BsMsNetwkEntrySuccess OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of successful MS network

entry. The increments by one each time the BS sends the

RNG-RSP message."

::= { wmanIf2BsMsStartupStatisticsEntry 4 }

wmanIf2BsMsStartupStatReset OBJECT-TYPE

SYNTAX      INTEGER {null(0),

resetStat(1)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"When this attribute is SET to resetCounter(1), the

corresponding entry of startup statistics will be reset.

A GET operation performed on this object will always

return null(0). The statistics is normally reset after

the packet count information is retrieved. "

::= { wmanIf2BsMsStartupStatisticsEntry 5 }

wmanIf2BsMsStartupStatResetTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the statistics is reset."

::= { wmanIf2BsMsStartupStatisticsEntry 6 }

--

-- Mobile Station Downlink Throughput Statistics Table

--

wmanIf2BsMsDlThroughputStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsMsDlThroughputStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the throughput statistics for the

downlink connections. The table shall be maintained as

a FIFO to store histograms samples that can be used to

generate peak or average throughput on per MS or BS sector

basis. When the throughput entry for a MS reaches the limit

, the oldest entry shall be deleted as the new entry is

added to the table."

::= { wmanIf2BsStatistics 4 }

wmanIf2BsMsDlThroughputStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsMsDlThroughputStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry in the table contains a histogram sample that

captures the throughput data in an interval determined by

the wmanIf2BsDlHistogramSampleInterval.

The primary index is the ifIndex with ifType of

ieee80216WMAN identifing the BS sector.

wmanIf2BsSsMacAddress identifies the MS from which the

throughput is captured.

wmanIf2BsHistogramIndex is the index to histogram samples.

Since there is no time stamp in the table,

wmanIf2BsMsDlHistogramIndex should be increased

monotonically, and wraps around when it reaches the

implementation specific limit."

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsMsDlHistogramIndex }

::= { wmanIf2BsMsDlThroughputStatisticsTable 1 }

WmanIf2BsMsDlThroughputStatisticsEntry ::= SEQUENCE {

wmanIf2BsMsDlHistogramIndex             Unsigned32,

wmanIf2BsMsDlMacSduOctetCount           Counter64}

wmanIf2BsMsDlHistogramIndex OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object identifies the histogram samples."

::= { wmanIf2BsMsDlThroughputStatisticsEntry 1 }

wmanIf2BsMsDlMacSduOctetCount OBJECT-TYPE

SYNTAX      Counter64

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of octets of MAC SDUs

that have been transmitted or received."

::= { wmanIf2BsMsDlThroughputStatisticsEntry 2 }

--

-- Mobile Station Uplink Throughput Statistics Table

--

wmanIf2BsMsUlThroughputStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsMsUlThroughputStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the throughput statistics for the

uplink connections. The table shall be maintained as

a FIFO to store histograms samples that can be used to

generate peak or average throughput on per MS or BS sector

basis. When the throughput entry for a MS reaches the limit

, the oldest entry shall be deleted as the new entry is

added to the table."

::= { wmanIf2BsStatistics 5 }

wmanIf2BsMsUlThroughputStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsMsUlThroughputStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry in the table contains a histogram sample that

captures the throughput data in an interval determined by

the wmanIf2BsUlHistogramSampleInterval.

The primary index is the ifIndex with ifType of

ieee80216WMAN identifing the BS sector.

wmanIf2BsSsMacAddress identifies the MS from which the

throughput is captured.

wmanIf2BsHistogramIndex is the index to histogram samples.

Since there is no time stamp in the table,

wmanIf2BsMsUlHistogramIndex should be increased

monotonically, and wraps around when it reaches the

implementation specific limit."

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsMsDlHistogramIndex }

::= { wmanIf2BsMsUlThroughputStatisticsTable 1 }

WmanIf2BsMsUlThroughputStatisticsEntry ::= SEQUENCE {

wmanIf2BsMsUlHistogramIndex             Unsigned32,

wmanIf2BsMsUlMacSduOctetCount           Counter64}

wmanIf2BsMsUlHistogramIndex OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object identifies the histogram samples."

::= { wmanIf2BsMsUlThroughputStatisticsEntry 1 }

wmanIf2BsMsUlMacSduOctetCount OBJECT-TYPE

SYNTAX      Counter64

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of octets of MAC SDUs

that have been transmitted or received."

::= { wmanIf2BsMsUlThroughputStatisticsEntry 2 }