| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Moving the Service Flow Status Objects to wmanIf2sMib |  |
| Date Submitted | 2007-09-19 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25c, on P802.16i/D4, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes moving Service flow status objects to wmanIf2sMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Moving the Service Flow Status Objects to wmanIf2sMib

Joey Chou

Intel

# Introduction

The SNMP based provisioning has been moved to wmanIf2sMib to avoid the conflict with the AAA based provisioning. In wmanIf2CommonObjects, there are wmanIf2CmnClassifierRuleTable, wmanIf2CmnPhsRuleTable, and wmanIf2CmnCpsServiceFlowTable that are used to monitor service flows after they have been admitted or activated. These tables have the following issue:

- These tables were derived from 802.16f and are tightly coupled with the SNMP based provisioning.

- They are used mainly for fixed BWA.

- The objects in these tables are provided in the wmanIf2mBsServiceFlowTable. So, it is redundant, and can result to inconsistency between these two sets of tables.

Therefore, it is proposed that they should be moved to wmanIf2sMib.

# Proposed Text

# 2.1	MIB Module Description

[Change subclause 13.1.5 as the following]

13.1.5	wmanIf2sMib

Figure 21 shows the high level MIB structure of wmanIf2sMib for 802.16. The MIB structure is organized based on the reference model as defined in IEEE 802.16-2004 standard.

Figure 21 – wmanIf2sMib Structure

[Add a new subclause 13.1.5.2 ]

13.1.5.2		wmanIf2sCommonObjects

13.1.5.2.1	wmanIf2sCmnClassifierRuleTable

wmanIf2CmnClassifierRuleTable contains runtime classifier rules screening criteria for each service flow.

13.1.5.2.2	wmanIf2sCmnPhsRuleTable

wmanIf2CmnPhsRuleTable contains PHS rule dictionary entries. Each entry contains the data of the header to be suppressed along with its identification - PHSI.

13.1.5.2.3	wmanIf2sCmnCpsServiceFlowTable

wmanIf2CmnCpsServiceFlowTable contains Service Flow managed objects that are common in BS and SS.

# 2.2	ASN.1 Notations

Add the following ASN.1 code to wmanIf2sMib..

# 13.2.4	wmanIf2sMib

--

-- Common object group – containing common tables and objects to be

-- implemented in both Base Station and Subscriber Station

--

wmanIf2sCmnClassifierRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sCmnClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains packet classifier rules associated

with service flows."

::= { wmanIf2sCommonObjects 1 }

wmanIf2sCmnClassifierRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2sCmnClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each packet classifier

rule, and is indexed by ifIndex, wmanIf2sCmnCpsSfId, and

wmanIf2sCmnClassifierRuleIndex. ifIndex is associated with

the BS sector. wmanIf2sCmnCpsSfId identifies the service

flow,  and wmanIf2sCmnClassifierRuleIndex identifies the

packet classifier rule."

INDEX     { ifIndex,

wmanIf2sCmnCpsSfId,

wmanIf2sCmnClassifierRuleIndex }

::= { wmanIf2sCmnClassifierRuleTable 1 }

WmanIf2sCmnClassifierRuleEntry::= SEQUENCE {

wmanIf2sCmnClassifierRuleIndex          Unsigned32,

wmanIf2sCmnClassifierRulePriority       INTEGER,

wmanIf2sCmnClassifierRuleIpTosLow       INTEGER,

wmanIf2sCmnClassifierRuleIpTosHigh      INTEGER,

wmanIf2sCmnClassifierRuleIpTosMask      INTEGER,

wmanIf2sCmnClassifierRuleIpProtocol     Integer32,

wmanIf2sCmnClassifierRuleIpSrcAddr      InetAddress,

wmanIf2sCmnClassifierRuleIpSrcMask      InetAddress,

wmanIf2sCmnClassifierRuleIpDestAddr     InetAddress,

wmanIf2sCmnClassifierRuleIpDestMask     InetAddress,

wmanIf2sCmnClassifierRuleSrcPortStart   Integer32,

wmanIf2sCmnClassifierRuleSrcPortEnd     Integer32,

wmanIf2sCmnClassifierRuleDestPortStart  Integer32,

wmanIf2sCmnClassifierRuleDestPortEnd    Integer32,

wmanIf2sCmnClassifierRuleDestMacAddr    MacAddress,

wmanIf2sCmnClassifierRuleDestMacMask    MacAddress,

wmanIf2sCmnClassifierRuleSrcMacAddr     MacAddress,

wmanIf2sCmnClassifierRuleSrcMacMask     MacAddress,

wmanIf2sCmnClassifierRuleEnetProtType   INTEGER,

wmanIf2sCmnClassifierRuleEnetProtocol   Integer32,

wmanIf2sCmnClassifierRuleUserPriLow     Integer32,

wmanIf2sCmnClassifierRuleUserPriHigh    Integer32,

wmanIf2sCmnClassifierRuleVlanId         Integer32,

wmanIf2sCmnClassifierRulePkts           Counter64,

wmanIf2sCmnClassifierRuleIpv6FlowLabel  WmanIf2sIpv6FlowLabel,

wmanIf2sCmnClassifierRuleActionRule     WmanIf2sActionRule,

wmanIf2sCmnClassifierRuleBitMap         WmanIf2sClassifierMap}

wmanIf2sCmnClassifierRuleIndex  OBJECT-TYPE

SYNTAX      Unsigned32 (1..4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index is assigned to each classifier in the classifiers

table"

REFERENCE

"Subclause 11.13.19.3.4.14 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 1 }

wmanIf2sCmnClassifierRulePriority OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value specifies the order of evaluation of the

classifiers. The higher the value the higher the

priority. The value of 0 is used as default in

provisioned service flows classifiers. The default

value of 64 is used for dynamic service flow classifiers.

If the referenced parameter is not present in a classifier

, this object reports the default value as defined above"

REFERENCE

"Subclause 11.13.19.3.4.1 in IEEE Std 802.16-2004"

DEFVAL      { 0 }

::= { wmanIf2sCmnClassifierRuleEntry 2 }

wmanIf2sCmnClassifierRuleIpTosLow OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The low value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 3 }

wmanIf2sCmnClassifierRuleIpTosHigh OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The 8-bit high value of a range of TOS byte values.

If the referenced parameter is not present in a classifier

, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 4 }

wmanIf2sCmnClassifierRuleIpTosMask OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The mask value is bitwise ANDed with TOS byte in an IP

packet and this value is used for the range checking of

TosLow and TosHigh. If the referenced parameter is not

present in a classifier, this object reports the value

of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 5 }

wmanIf2sCmnClassifierRuleIpProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the value of the IP Protocol field

required for IP packets to match this rule. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 6 }

wmanIf2sCmnClassifierRuleIpSrcAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Source Address

required for packets to match this rule. An IP packet

matches the rule when the packet ip source address bitwise

ANDed with the wmanIf2sCmnClassifierRuleIpSrcMask value

equals the wmanIf2sCmnClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 7 }

wmanIf2sCmnClassifierRuleIpSrcMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP Source

Address that are compared to match this rule. An IP packet

matches the rule when the packet source address bitwise

ANDed with the

wmanIf2sCmnClassifierRuleIpSrcMask value equals the

wmanIf2sCmnClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 8 }

wmanIf2sCmnClassifierRuleIpDestAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Destination

Address required for packets to match this rule. An IP

packet matches the rule when the packet IP destination

address bitwise ANDed with the

wmanIf2sCmnClassifierRuleIpDestMask value equals the

wmanIf2sCmnClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a

classifier, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 9 }

wmanIf2sCmnClassifierRuleIpDestMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP

Destination Address that are compared to match this rule.

An IP packet matches the rule when the packet destination

address bitwise ANDed with the

wmanIf2sCmnClassifierRuleIpDestMask value equals the

wmanIf2sCmnClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 10 }

wmanIf2sCmnClassifierRuleSrcPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP source port numbers to which a packet is compared

. This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 11 }

wmanIf2sCmnClassifierRuleSrcPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a classifier,

this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 12 }

wmanIf2sCmnClassifierRuleDestPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 13 }

wmanIf2sCmnClassifierRuleDestPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of

65535."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 14 }

wmanIf2sCmnClassifierRuleDestMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sCmnClassifierRuleDestMacMask equals the value of

wmanIf2sCmnClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 15 }

wmanIf2sCmnClassifierRuleDestMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sCmnClassifierRuleDestMacMask equals the value of

wmanIf2sCmnClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 16 }

wmanIf2sCmnClassifierRuleSrcMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches this entry when its source

MAC address  bitwise ANDed with

wmanIf2sCmnClassifierRuleSrcMacMask equals the value

of wmanIf2sCmnClassifierRuleSrcMacAddr. If the

referenced parameter is not present in a classifier,

this object reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 17 }

wmanIf2sCmnClassifierRuleSrcMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sCmnClassifierRuleSrcMacMask equals the value of

wmanIf2sCmnClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 18 }

wmanIf2sCmnClassifierRuleEnetProtType OBJECT-TYPE

SYNTAX      INTEGER {none(0),

ethertype(1),

dsap(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the format of the layer 3 protocol

id in the Ethernet packet. A value of none(0) means that

the rule does not use the layer 3 protocol type as a

matching criteria. A value of ethertype(1) means that the

rule applies only to frames which contains an EtherType

value. Ethertype values are contained in packets using

the Dec-Intel-Xerox (DIX) encapsulation or the RFC1042

Sub-Network Access Protocol (SNAP) encapsulation formats.

A value of dsap(2) means that the rule applies only to

frames using the IEEE802.3 encapsulation format with a

Destination Service Access Point (DSAP) other than 0xAA

(which is reserved for SNAP). If the Ethernet frame

contains an 802.1P/Q Tag header (i.e. EtherType 0x8100),

this object applies to the embedded EtherType field within

the 802.1P/Q header. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 19 }

wmanIf2sCmnClassifierRuleEnetProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If wmanIf2sCmnClassifierRuleEnetProtocolTyp is none(0),

this object is ignored when considering whether a packet

matches the current rule.

If wmanIf2sCmnClassifierRuleEnetProtocolTyp is ethertype(1)

,this object gives the 16-bit value of the EtherType that

the packet must match in order to match the rule.

If wmanIf2sCmnClassifierRuleEnetProtocolTyp is dsap(2), the

lower 8 bits of this object's value must match the DSAP

byte of the packet in order to match the rule.

If the Ethernet frame contains an 802.1P/Q Tag header

(i.e. EtherType 0x8100), this object applies to the

embedded EtherType field within the 802.1P/Q header.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 20 }

wmanIf2sCmnClassifierRuleUserPriLow OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority field

within the range of wmanIf2sCmnClassifierRulePriLow and

wmanIf2sCmnClassifierRulePriHigh in order to match this

rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 21 }

wmanIf2sCmnClassifierRuleUserPriHigh OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority

field within the range of wmanIf2sCmnClassifierRulePriLow

and wmanIf2sCmnClassifierRulePriHigh in order to match

this rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 7."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 22 }

wmanIf2sCmnClassifierRuleVlanId OBJECT-TYPE

SYNTAX      Integer32 (0..4095)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header.

If this object's value is nonzero, tagged packets must

have a VLAN Identifier that matches the value in order

to match the rule.

Only the least significant 12 bits of this object's

value are valid.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.12 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnClassifierRuleEntry 23 }

wmanIf2sCmnClassifierRulePkts OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of packets that have

been classified using this entry."

::= { wmanIf2sCmnClassifierRuleEntry 25 }

wmanIf2sCmnClassifierRuleIpv6FlowLabel OBJECT-TYPE

SYNTAX      WmanIf2sIpv6FlowLabel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2sCmnClassifierRuleEntry 26 }

wmanIf2sCmnClassifierRuleActionRule OBJECT-TYPE

SYNTAX      WmanIf2sActionRule

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule. If this classification action rule

exists, its action shall be applied on the packets that

match this classifier rule."

::= { wmanIf2sCmnClassifierRuleEntry 27 }

wmanIf2sCmnClassifierRuleBitMap OBJECT-TYPE

SYNTAX      WmanIf2sClassifierMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2sCmnClassifierRuleEntry 28 }

wmanIf2sCmnPhsRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sCmnPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When a classifier is deleted, any associated

PHS rule shall also be deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2sCommonObjects 2 }

wmanIf2sCmnPhsRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2sCmnPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data. It is indexed by IfIndex,

wmanIf2sCmnCpsSfId, and wmanIf2sCmnPhsIndex. The table is

read-only for NMS. "

INDEX     { ifIndex, wmanIf2sCmnCpsSfId,

wmanIf2sCmnPhsRulePhsIndex }

::= { wmanIf2sCmnPhsRuleTable 1 }

WmanIf2sCmnPhsRuleEntry::= SEQUENCE {

wmanIf2sCmnPhsRulePhsIndex              INTEGER,

wmanIf2sCmnPhsRulePhsField              OCTET STRING,

wmanIf2sCmnPhsRulePhsMask               OCTET STRING,

wmanIf2sCmnPhsRulePhsSize               Integer32,

wmanIf2sCmnPhsRulePhsVerify             WmanIf2sPhsRuleVerify}

wmanIf2sCmnPhsRulePhsIndex OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnPhsRuleEntry 1 }

wmanIf2sCmnPhsRulePhsField OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnPhsRuleEntry 2 }

wmanIf2sCmnPhsRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2sCmnPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnPhsRuleEntry 3 }

wmanIf2sCmnPhsRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2sCmnPhsRuleEntry 4 }

wmanIf2sCmnPhsRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2sPhsRuleVerify

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2sCmnPhsRuleEntry 5 }

wmanIf2sCmnCpsServiceFlowTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sCmnCpsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains Service Flow managed objects that

are common in BS and SS."

::= { wmanIf2sCommonObjects 3 }

wmanIf2sCmnCpsServiceFlowEntry OBJECT-TYPE

SYNTAX      WmanIf2sCmnCpsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each created service

flow for a given MacAddress, and is indexed by ifIndex,

wmanIf2sCmnCpsCpsSfMacAddress, and wmanIf2sCmnCpsSfId.

IfIndex is associated with the BS sector."

INDEX     { ifIndex,

wmanIf2sCmnCpsSfMacAddress,

wmanIf2sCmnCpsSfId }

::= { wmanIf2sCmnCpsServiceFlowTable 1 }

WmanIf2sCmnCpsServiceFlowEntry::= SEQUENCE {

wmanIf2sCmnCpsSfMacAddress              MacAddress,

wmanIf2sCmnCpsSfId                      Unsigned32,

wmanIf2sCmnCpsSfCid                     WmanIf2sCidType,

wmanIf2sCmnCpsSfDirection               INTEGER,

wmanIf2sCmnCpsSfState                   WmanIf2sSfState,

wmanIf2sCmnCpsTrafficPriority           INTEGER,

wmanIf2sCmnCpsMaxSustainedRate          Unsigned32,

wmanIf2sCmnCpsMaxTrafficBurst           Unsigned32,

wmanIf2sCmnCpsMinReservedRate           Unsigned32,

wmanIf2sCmnCpsToleratedJitter           Unsigned32,

wmanIf2sCmnCpsMaxLatency                Unsigned32,

wmanIf2sCmnCpsFixedVsVariableSduInd     INTEGER,

wmanIf2sCmnCpsSduSize                   Unsigned32,

wmanIf2sCmnCpsSfSchedulingType          WmanIf2sSchedulingType,

wmanIf2sCmnCpsArqEnable                 TruthValue,

wmanIf2sCmnCpsArqWindowSize             INTEGER,

wmanIf2sCmnCpsArqBlockLifetime          INTEGER,

wmanIf2sCmnCpsArqSyncLossTimeout        INTEGER,

wmanIf2sCmnCpsArqDeliverInOrder         TruthValue,

wmanIf2sCmnCpsArqRxPurgeTimeout         INTEGER,

wmanIf2sCmnCpsArqBlockSize              INTEGER,

wmanIf2sCmnCpsMinRsvdTolerableRate      Unsigned32,

wmanIf2sCmnCpsReqTxPolicy               BITS,

wmanIf2sCmnSfCsSpecification            WmanIf2sCsType,

wmanIf2sCmnCpsTargetSaid                INTEGER}

wmanIf2sCmnCpsSfMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"When this table is implemented on the basestation, this

object contains the SS Mac address, the reported service

flow was created for. On the SS, the value returned is

the SS's own Mac address."

::= { wmanIf2sCmnCpsServiceFlowEntry 1 }

wmanIf2sCmnCpsSfId OBJECT-TYPE

SYNTAX      Unsigned32 ( 1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 32 bit quantity that uniquely identifies a service flow

to both the subscriber station and base station (BS)."

::= { wmanIf2sCmnCpsServiceFlowEntry 2 }

wmanIf2sCmnCpsSfCid OBJECT-TYPE

SYNTAX      WmanIf2sCidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 16 bit channel identifier to identify the connection

being created by DSA."

::= { wmanIf2sCmnCpsServiceFlowEntry 3 }

wmanIf2sCmnCpsSfDirection OBJECT-TYPE

SYNTAX      INTEGER {downstream(1),

upstream(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An attribute indicating the service flow is downstream or

upstream."

::= { wmanIf2sCmnCpsServiceFlowEntry 4 }

wmanIf2sCmnCpsSfState OBJECT-TYPE

SYNTAX      WmanIf2sSfState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2sCmnCpsSfState indicates the service flow state:

Authorized (1), Admitted (2), and Active (3) service

flow state."

REFERENCE

"Subclause 6.3.14.6, in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 5 }

wmanIf2sCmnCpsTrafficPriority OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the priority

assigned to a service flow. For uplink service flows,

the BS should use this parameter when determining

precedence in request service and grant generation,

and the SS shall preferentially select contention

Request opportunities for Priority Request CIDs

based on this priority"

REFERENCE

"Subclause 11.13.5 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 6 }

wmanIf2sCmnCpsMaxSustainedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the peak information rate

of the service. The rate is expressed in bits per

second and pertains to the SDUs at the input to

the system."

REFERENCE

"Subclause 11.13.6 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 7 }

wmanIf2sCmnCpsMaxTrafficBurst OBJECT-TYPE

SYNTAX      Unsigned32

UNITS	    "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the maximum burst size that

must be accommodated for the service."

REFERENCE

"Subclause 11.13.7 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 8 }

wmanIf2sCmnCpsMinReservedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the minimum rate reserved

for this service flow."

REFERENCE

"Subclause 11.13.8 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 9 }

wmanIf2sCmnCpsToleratedJitter OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the Maximum delay

variation (jitter) for the connection."

REFERENCE

"Subclause 11.13.13 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 10 }

wmanIf2sCmnCpsMaxLatency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the maximum

latency between the reception of a packet by the BS

or SS on its network interface and the forwarding

of the packet to its RF Interface."

REFERENCE

"Subclause 11.13.14 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 11 }

wmanIf2sCmnCpsFixedVsVariableSduInd OBJECT-TYPE

SYNTAX      INTEGER {variableLength(0),

fixedLength(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies whether the SDUs

on the service flow are variable-length (0) or

fixed-length (1). The parameter is used only if

packing is on for the service flow. The default value

is 0, i.e.,variable-length SDUs."

REFERENCE

"Subclause 11.13.15 in IEEE Std 802.16-2004"

DEFVAL      { variableLength }

::= { wmanIf2sCmnCpsServiceFlowEntry 12 }

wmanIf2sCmnCpsSduSize OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the length of the

SDU for a fixed-length SDU service flow. This parameter

is used only if packing is on and the service flow is

indicated as carrying fixed-length SDUs. The default

value is 49 bytes, i.e., VC-switched ATM cells with PHS.

The parameter is relevant for both ATM and Packet

Convergence Sublayers."

REFERENCE

"Subclause 11.13.16 in IEEE Std 802.16-2004"

DEFVAL      { 49 }

::= { wmanIf2sCmnCpsServiceFlowEntry 13 }

wmanIf2sCmnCpsSfSchedulingType OBJECT-TYPE

SYNTAX      WmanIf2sSchedulingType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the upstream scheduling service used for

upstream service flow. If the referenced parameter

is not present in the corresponding 802.16 QOS

Parameter Set of an upstream service flow, the

default value of this object is bestEffort(2)."

REFERENCE

"Subclause 11.13.11 in IEEE Std 802.16-2004"

DEFVAL         { bestEffort }

::= { wmanIf2sCmnCpsServiceFlowEntry 14 }

wmanIf2sCmnCpsArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the connection."

::= { wmanIf2sCmnCpsServiceFlowEntry 15 }

wmanIf2sCmnCpsArqWindowSize	 OBJECT-TYPE

SYNTAX	    INTEGER (1..1024)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged

fragments at any time."

::= { wmanIf2sCmnCpsServiceFlowEntry 16 }

wmanIf2sCmnCpsArqBlockLifetime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be

managed by the transmitter ARQ machine, once

initial transmission of the fragment has occured.

If transmission or retransmission of the fragment

is not acknowledged by the receiver before the

time limit is reached, the fragment is discarded.

A value of 0 means Infinite."

::= { wmanIf2sCmnCpsServiceFlowEntry 17 }

wmanIf2sCmnCpsArqSyncLossTimeout OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535 )

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss

of synchronization of the sender and receiver

state machines. A value of 0 means Infinite."

::= { wmanIf2sCmnCpsServiceFlowEntry 18 }

wmanIf2sCmnCpsArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered

by the receiving MAC to its client application

in the order in which data was handed off to the

originating MAC."

::= { wmanIf2sCmnCpsServiceFlowEntry 19 }

wmanIf2sCmnCpsArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means

Infinite."

::= { wmanIf2sCmnCpsServiceFlowEntry 20 }

wmanIf2sCmnCpsArqBlockSize OBJECT-TYPE

SYNTAX	    INTEGER (1..2040)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value of this parameter specifies the size of an

ARQ block. This parameter shall be established by

negotiation during the connection creation dialog."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 21 }

wmanIf2sCmnCpsMinRsvdTolerableRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Minimum Tolerable Traffic Rate = R (bits/sec) with

time base T(sec) means the following. Let S denote

additional demand accumulated at the MAC SAP of the

transmitter during an arbitrary time interval of the

length T. Then the amount of data forwarded at the

receiver to CS (in bits) during this interval should

be not less than min {S, R * T}."

REFERENCE

"Subclause 11.13.9 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 22 }

wmanIf2sCmnCpsReqTxPolicy OBJECT-TYPE

SYNTAX      BITS {noBroadcastBwReq(0),

reserved1(1),

noPiggybackReq(2),

noFragmentData(3),

noPHS(4),

noSduPacking(5),

noCrc(6),

reserved2(7)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter provides the capability to

specify certain attributes for the associated service

flow. An attribute is enabled by setting the

corresponding bit position to 1."

REFERENCE

"Subclause 11.13.12 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 23 }

wmanIf2sCmnSfCsSpecification OBJECT-TYPE

SYNTAX      WmanIf2sCsType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the convergence sublayer

encapsulation mode."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 24 }

wmanIf2sCmnCpsTargetSaid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The target SAID parameter indicates the SAID onto

which the service flow being set up shall be mapped."

REFERENCE

"Subclause 11.13.17 in IEEE Std 802.16-2004"

::= { wmanIf2sCmnCpsServiceFlowEntry 25 }