| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose additional performance statistics objects for 802.16 performance characterization |  |
| Date Submitted | 2007-11-10 |  |
| Source(s) | Joey Chou Intel Shawn Wallace Sprint Fran Burns Sprint | E-mail: joey.chou@intel.com E-mail: Shawn.Wallace@sprint.com E-mail: francis.burns@sprint.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes performance counter objects for 802.16 performance characterization. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Propose performance statistics objects for 802.16 performance characterization

Joey Chou, Shawn Wallace, Fran Burns

Intel, Sprint

# Introduction

This document proposes the following performance counters objects that are important to characterize the 802.16 WMAN network performances.

- wmanIf2BsDataRateStatisticsTable

- wmanIf2BsNetworkEntryStatisticsTable

- wmanIf2BsPacketErrorRateTable

# Proposed Text

# 2.1	MIB Module Description

13.1.3.1.2	wmanIf2BsCps

[Change Figure 4 as the following:]

Figure 4 – wmanIf2BsCps Structure

13.1.3.1.2.3	wmanIf2BsStatistics

[Add subclauses 13.1.3.1.2.4 and 13.1.3.1.2.5 as the following:]

13.1.3.1.2.3.4	wmanIf2BsDataRateStatisticsTable

wmanIf2BsDataRateStatisticsTable contains the average and peak data rate statistics at the BS sector level.

13.1.3.1.2.3.5	wmanIf2BsNetworkEntryStatisticsTable

# wmanIf2BsNetworkEntryStatisticsTable contains the statistics for the network and network re-entry.

13.1.3.1.2.3.6	wmanIf2BsPacketErrorRateTable

# wmanIf2BsPacketErrorRateTable contains the statistics for the packet error rate.

# 2.2	ASN.1 Notations – wmanIf2BsDataRateStatisticsTable

It proposes the following new code be added to the wmanIf2Mib.

-- XXX

-- Base Station Data Rate Statistics Table

--

wmanIf2BsDataRateStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsDataRateStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the average and peak data rate

statistics at the BS sector level. The BS measures the

number of bytes that are sent or received withing each

interval set by the wmanIf2BsDataSampleInterval object.

For example:

S = the number of bytes measured in an interval

N = the number of measurements

Avg(N) = the average data rate after N measurements

Avg(0) = S, the 1st measurement

Avg(N) = (Avg(N-1)*(N-1) + S) / N

Peak(N) = the peak data rate after N measurement

Peak(0) = S, the 1st measurement

Peak(N) = S if S > Peak(N-1)

= Peak(N -1) if S <= Peak(N-1)

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsStatistics 4 }

wmanIf2BsDataRateStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsDataRateStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsDataRateStatisticsTable 1 }

WmanIf2BsDataRateStatisticsEntry ::= SEQUENCE {

wmanIf2BsAvgDlUserThroughput            Counter32,

wmanIf2BsAvgUlUserThroughput            Counter32,

wmanIf2BsAvgDlMacThroughput             Counter32,

wmanIf2BsAvgUlMacThroughput             Counter32,

wmanIf2BsAvgDlPhyThroughput             Counter32,

wmanIf2BsAvgUlPhyThroughput             Counter32,

wmanIf2BsPeakDlUserThroughput           Counter32,

wmanIf2BsPeakUlUserThroughput           Counter32,

wmanIf2BsPeakDlMacThroughput            Counter32,

wmanIf2BsPeakUlMacThroughput            Counter32,

wmanIf2BsPeakDlPhyThroughput            Counter32,

wmanIf2BsPeakUlPhyThroughput            Counter32,

wmanIf2BsAvgDlCellEdgeThroughput        Counter32,

wmanIf2BsAvgUlCellEdgeThroughput        Counter32,

wmanIf2BsDataRateMeasurements           Counter64}

wmanIf2BsAvgDlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average downlink user throughput

Avg(N). Each measurement indicates the number of octets of

MAC SDUs that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 1 }

wmanIf2BsAvgUlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average uplink user throughput

Avg(N). Each measurement indicates the number of octets of

MAC SDUs that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 2 }

wmanIf2BsAvgDlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average downlink MAC throughput

Avg(N). Each measurement indicates the number of octets of

MAC PDUs (i.e. user data, MAC headers, and MAC management

messages) that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 3 }

wmanIf2BsAvgUlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average uplink MAC throughput

Avg(N). Each measurement indicates the number of octets of

MAC PDUs (i.e. user data, MAC headers, and MAC management

messages) that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 4 }

wmanIf2BsAvgDlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average downlink PHY throughput

Avg(N). Each measurement indicates the number of octets of

bursts (i.e. MAC PDU + PHY overheads) that are sent within

the wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 5 }

wmanIf2BsAvgUlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average uplink PHY throughput

Avg(N). Each measurement indicates the number of octets of

bursts (i.e. MAC PDU + PHY overheads) that are received

within the wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 6 }

wmanIf2BsPeakDlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak downlink user throughput

Peak(N). Each measurement indicates the number of octets of

MAC SDUs that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 7 }

wmanIf2BsPeakUlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink user throughput Peak(N)

. Each measurement indicates the number of octets of MAC

SDUs that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 8 }

wmanIf2BsPeakDlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak downlink MAC throughput

Peak(N). Each measurement indicates the number of octets of

MAC PDUs (i.e. user data, MAC headers, and MAC management

messages) that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 9 }

wmanIf2BsPeakUlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink MAC throughput Peak(N).

Each measurement indicates the number of octets of MAC PDUs

(i.e. user data, MAC headers, and MAC management messages)

that are received within the wmanIf2BsDataSampleInterval

interval."

::= { wmanIf2BsDataRateStatisticsEntry 10 }

wmanIf2BsPeakDlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak downlink PHY throughput

Peak(N). Each measurement indicates the number of octets of

bursts (i.e. MAC PDU + PHY overheads) that are sent within

the wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 11 }

wmanIf2BsPeakUlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink PHY throughput Peak(N).

Each measurement indicates the number of octets of bursts

(i.e. MAC PDU + PHY overheads) that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 12 }

wmanIf2BsAvgDlCellEdgeThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the Average downlink MAC throughput

Avg(N). Each measurement indicates the number of octets of

MAC PDU that are sent within the

wmanIf2BsDataSampleInterval interval using the most robust

coding (i.e. QPSK)."

::= { wmanIf2BsDataRateStatisticsEntry 13 }

wmanIf2BsAvgUlCellEdgeThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink PHY throughput Peak(N).

Each measurement indicates the number of octets of MAC PDU

that are received within the wmanIf2BsDataSampleInterval

interval using the most robust coding (i.e. QPSK)."

::= { wmanIf2BsDataRateStatisticsEntry 14 }

wmanIf2BsDataRateMeasurements OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object tracks the number of measurements 'N'."

::= { wmanIf2BsDataRateStatisticsEntry 15 }

# 2.3	ASN.1 Notations – wmanIf2BsNetworkEntryStatisticsTable

It proposes the following new code be added to the wmanIf2Mib.

-- XXX

-- Base Station Network Entry Statistics Table

--

wmanIf2BsNetworkEntryStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsNetworkEntryStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the average and maximum amount of time

for network and network re-entry.

For example:

T = a) Network entry time is the amount of time between:

1. BS received the 1st RNG-REQ from a MS

2. BS sends the DSA-RSP to such MS

b) Network re-entry time is the amount of time between:

1. BS received the 1st RNG-REQ from a MS with the

Ranging Purpose Indication TLV to indicate that

the MS is leaving the idle mode

2. BS sends the DSA-RSP to such MS

N = the number of network entries

Avg(N) = the average amount of time after N network entries

Avg(0) = T, the 1st network entry

Avg(N) = (Avg(N-1)*(N-1) + T) / N

Max(N) = the maximum amount of time after N network entries

Max(0) = T, the 1st network entry

Max(N) = T if T > Max(N-1)

= Max(N-1) if T <= Max(N-1)

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsStatistics 5 }

wmanIf2BsNetworkEntryStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsNetworkEntryStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsNetworkEntryStatisticsTable 1 }

WmanIf2BsNetworkEntryStatisticsEntry ::= SEQUENCE {

wmanIf2BsAvgNetworkEntryTime            INTEGER,

wmanIf2BsMaxNetworkEntryTime            INTEGER,

wmanIf2BsAvgNetworkReEntryTime          INTEGER,

wmanIf2BsMaxNetworkReEntryTime          INTEGER,

wmanIf2BsNumOfNetworkEntries            Counter64,

wmanIf2BsNumOfNetworkReEntries          Counter64}

wmanIf2BsAvgNetworkEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average network entry time Avg(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 1 }

wmanIf2BsMaxNetworkEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the maximum network entry time Max(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 2 }

wmanIf2BsAvgNetworkReEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average network re-entry time

Avg(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 3 }

wmanIf2BsMaxNetworkReEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the maximum network re-entry time

Max(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 4 }

wmanIf2BsNumOfNetworkEntries OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object tracks the number of network entries 'N'."

::= { wmanIf2BsNetworkEntryStatisticsEntry 5 }

wmanIf2BsNumOfNetworkReEntries OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object tracks the number of network re-entries 'N'."

::= { wmanIf2BsNetworkEntryStatisticsEntry 6 }

# 2.4	ASN.1 Notations – wmanIf2BsPacketErrorRateTable

It proposes the following new code be added to the wmanIf2Mib.

-- XXX

-- Mobile Packet Error Rate Table

--

wmanIf2BsPacketErrorRateTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsPacketErrorRateEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the packet error rate information.

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap"

::= { wmanIf2BsStatistics 6 }

wmanIf2BsPacketErrorRateEntry OBJECT-TYPE

SYNTAX      WmanIf2BsPacketErrorRateEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsPacketErrorRateTable  1 }

WmanIf2BsPacketErrorRateEntry ::= SEQUENCE {

wmanIf2BsDlPacketsSend                  Counter64,

wmanIf2BsDlPacketsErrored               Counter64,

wmanIf2BsDlPacketErrorRate              Unsigned32}

wmanIf2BsDlPacketsSend OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the total number of MAC SDUs that a BS

has sent."

::= { wmanIf2BsPacketErrorRateEntry 1 }

wmanIf2BsDlPacketsErrored OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the total number of MAC SDUs including

at least an ARQ block that has not been successful

acknowledged (i.e. timeout or NAK)."

::= { wmanIf2BsPacketErrorRateEntry 2 }

wmanIf2BsDlPacketErrorRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "1x10E-7"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsDlPacketErrorRate = (wmanIf2BsDlPacketsErrored /

wmanIf2BsDlPacketsSend) * 10000000"

::= { wmanIf2BsPacketErrorRateEntry 3 }

# 2.5	ASN.1 Notations – wmanIf2BsPerformanceCountersTrap

It proposes the wmanIf2BsPerformanceCountersTrap object to be added to wamnIf2Mib.

-- XXXX

wmanIf2BsPerformanceCountersTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2BsSsAuthenAttempt,

wmanIf2BsSsAuthenSuccess,

wmanIf2BsSsAuthenSuccessRate,

wmanIf2BsSsRangingAttempt,

wmanIf2BsSsRangingSuccess,

wmanIf2BsSsRangingSuccessRate,

wmanIf2BsAvgDlUserThroughput,

wmanIf2BsAvgUlUserThroughput,

wmanIf2BsAvgDlMacThroughput,

wmanIf2BsAvgUlMacThroughput,

wmanIf2BsAvgDlPhyThroughput,

wmanIf2BsAvgUlPhyThroughput,

wmanIf2BsPeakDlUserThroughput,

wmanIf2BsPeakUlUserThroughput,

wmanIf2BsPeakDlMacThroughput,

wmanIf2BsPeakUlMacThroughput,

wmanIf2BsPeakDlPhyThroughput,

wmanIf2BsPeakUlPhyThroughput,

wmanIf2BsAvgDlCellEdgeThroughput,

wmanIf2BsAvgUlCellEdgeThroughput,

wmanIf2BsDataRateMeasurements,

wmanIf2BsAvgNetworkEntryTime,

wmanIf2BsMaxNetworkEntryTime,

wmanIf2BsAvgNetworkReEntryTime,

wmanIf2BsMaxNetworkReEntryTime,

wmanIf2BsNumOfNetworkEntries,

wmanIf2BsNumOfNetworkReEntries,

wmanIf2BsDlPacketsSend,

wmanIf2BsDlPacketsErrored,

wmanIf2BsDlPacketErrorRate,

wmanIf2BsEventNotificationTime}

STATUS      current

DESCRIPTION

"This event should be automatically generated every

wmanIf2BsCountersReportInterval interval to report the

performance counters for a given BS sector identified by

ifIndex. The performance counters will be reset after this

event being sent to EMS "

::= { wmanIf2BsTrapPrefix 6 }