| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose objects to support MOB_SCN-REQ, MOB_SCN-RSP, and MOB_SCN-REP |  |
| Date Submitted | 2007-11-10 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes objects to support MOB_SCN-REQ, MOB_SCN-RSP, and MOB_SCN-REP. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Propose objects to support MOB_SCN-REQ, MOB_SCN-RSP, and MOB_SCN-REP

Joey Chou

Intel

# Introduction

This document proposes the following tables to support MOB_SCN-REQ, MOB_SCN-RSP, and MOB_SCN-REP.

- wmanIf2mBsMobileScanRequestTable

- wmanIf2mBsMobileScanResponseTable

- wmanIf2mBsNeighborBsInfoTable

- wmanIf2mBsDiversityBsInfoTable

# Proposed Text

# 2.1	MIB Module Description

13.1.4.1.2	wmanIf2mBsPm

[Change Figure 4 as the following:]

Figure 14 – wmanIf2mBsPm Structure

[Add the following subclauses:]

13.1.4.1.2.2	wmanIf2mBsMobileScanRequestTable

wmanIf2mBsMobileScanRequestTable contains the attributes that are sent in the MOB_SCN-REQ message.

          - wmanIf2mBsMobileScanResponseTable

# wmanIf2mBsMobileScanResponseTable contains the attributes that are sent in the MOB_SCN-RSP message.

          - wmanIf2mBsNeighborBsInfoTable

wmanIf2mBsNeighborBsInfoTable contains the neighbor BS information that is sent in the MOB_SCN-RSP and MOB_SCN-REP messages

          - wmanIf2mBsDiversityBsInfoTable

# wmanIf2mBsDiversityBsInfoTable contains the diversity BS information that is sent in the MOB_SCN-REP messages.

# 2.2	ASN.1 Notations

It proposes the following new code be added to the wmanIf2mMib.

-- XXX

WmanIf2mReportMode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Action code for an MS's report of CINR measurement:

0b00: The MS measures channel quality of the Available BSs

without reporting.

0b01: The MS reports the result of the measurement to

Serving BS periodically. The period of reporting is

different from that of scanning.

0b10: The MS reports the result of the measurement to

Serving BS after each measurement."

REFERENCE

"Subclause 6.3.2.3.49 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noReport(0),

periodicReport(1),

eventTriggeredReport(2)}

-- XXX

WmanIf2mReportMatric ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Bitmap indicator of trigger metrics that the serving BS

requests the MS to report. Serving BS shall indicate only

the trigger metrics agreed during SBC-REQ/RSP negotiation.

Each bit indicates whether reports will be initiated by

trigger based on the corresponding metric:

Bit 0:    BS CINR mean

Bit 1:    BS RSSI mean

Bit 2:    Relative delay

Bit 3:    BS RTD; this metric shall be only measured on

serving BS/anchor BS"

REFERENCE

"Subclause 6.3.2.3.48 in IEEE Std 802.16-2004"

SYNTAX      BITS {bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRtd(3)}

-- XXX

WmanIf2mFullBsId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The 48 bit field sent in the DL-MAP message to identifies

the BS."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(6))

-- XXX

WmanIf2mScanType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of scanning or association used by the MS and

coordinated by the Serving BS:

0b000: Scanning without Association Scanning

0b001: Scanning with Association level 0: association

without coordination

0b010: Scanning with Association level 1: association

with coordination

0b011: Scanning with Association level 2: network assisted

association"

REFERENCE

"Subclause 6.3.2.3.49 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {scanWoAssociation(0),

scanWithAssociation0(1),

scanWithAssociation1(2),

scanWithAssociation2(3)}

-- XXX

-- wmanIf2mBsMobileScanRequestTable

--

wmanIf2mBsMobileScanRequestTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsMobileScanRequestEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes that are sent in the

MOB_SCN-REQ message."

REFERENCE

"Subclause 6.3.2.3.48, Table 109h in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 2 }

wmanIf2mBsMobileScanRequestEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsMobileScanRequestEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex, wmanIf2mBsSsMacAddress }

::= { wmanIf2mBsMobileScanRequestTable 1 }

WmanIf2mBsMobileScanRequestEntry ::= SEQUENCE {

wmanIf2mBsScanReqDuration               INTEGER,

wmanIf2mBsScanReqInterleavingInterval   INTEGER,

wmanIf2mBsScanReqIteration              INTEGER,

wmanIf2mBsNumOfRecommendedBs            INTEGER,

wmanIf2mBsScanConfigChangeCount         INTEGER}

wmanIf2mBsScanReqDuration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Duration of the requested scanning period."

::= { wmanIf2mBsMobileScanRequestEntry 1 }

wmanIf2mBsScanReqInterleavingInterval OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The period of MS's Normal Operation which is interleaved

between Scanning Durations."

::= { wmanIf2mBsMobileScanRequestEntry 2 }

wmanIf2mBsScanReqIteration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The requested number of iterating scanning interval by an

MS."

::= { wmanIf2mBsMobileScanRequestEntry 3 }

wmanIf2mBsNumOfRecommendedBs OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of neighboring BS to be scanned or associated, which

are included in MOB_NBR-ADV message."

::= { wmanIf2mBsMobileScanRequestEntry 4 }

wmanIf2mBsScanConfigChangeCount OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of Configuration Change Count in MOB_NBR-ADV

message referred in order to compress neighbor BSID."

::= { wmanIf2mBsMobileScanRequestEntry 5 }

-- XXX

-- wmanIf2mBsMobileScanResponseTable

--

wmanIf2mBsMobileScanResponseTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsMobileScanResponseEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the attributes that are sent in the

MOB_SCN-RSP message."

REFERENCE

"Subclause 6.3.2.3.49, Table 109i in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 3 }

wmanIf2mBsMobileScanResponseEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsMobileScanResponseEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex, wmanIf2mBsSsMacAddress }

::= { wmanIf2mBsMobileScanResponseTable 1 }

WmanIf2mBsMobileScanResponseEntry ::= SEQUENCE {

wmanIf2mBsScanRspDuration               INTEGER,

wmanIf2mBsScanRspInterleavingInterval   INTEGER,

wmanIf2mBsScanRspIteration              INTEGER,

wmanIf2mBsReportMode                    WmanIf2mReportMode,

wmanIf2mBsReportPeriod                  INTEGER,

wmanIf2mBsReportMatric                  WmanIf2mReportMatric,

wmanIf2mBsStartFrame                    INTEGER}

wmanIf2mBsScanRspDuration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Duration where the MS may perform scanning or association

for Available BSs. When MOB_SCN-RSP is sent in response to

MOB_SCN-REQ, setting Scan duration to zero indicates the

request for an allocation of scanning interval is denied."

::= { wmanIf2mBsMobileScanResponseEntry 1 }

wmanIf2mBsScanRspInterleavingInterval OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The period interleaved between Scanning Intervals when MS

shall perform Normal Operation."

::= { wmanIf2mBsMobileScanResponseEntry 2 }

wmanIf2mBsScanRspIteration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of iterating scanning interval."

::= { wmanIf2mBsMobileScanResponseEntry 3 }

wmanIf2mBsReportMode OBJECT-TYPE

SYNTAX      WmanIf2mReportMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Action code for an MS's report of CINR measurement."

::= { wmanIf2mBsMobileScanResponseEntry 4 }

wmanIf2mBsReportPeriod OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The period of MSis report of CINR measurement when the MS

is required to report the value periodically."

::= { wmanIf2mBsMobileScanResponseEntry 5 }

wmanIf2mBsReportMatric OBJECT-TYPE

SYNTAX      WmanIf2mReportMatric

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Bitmap indicator of trigger metrics that the serving BS

requests the MS to report."

::= { wmanIf2mBsMobileScanResponseEntry 6 }

wmanIf2mBsStartFrame OBJECT-TYPE

SYNTAX      INTEGER (0 .. 15)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Measured from the frame in which this message was received.

A value of zero means that first Scanning Interval starts

in the next frame."

::= { wmanIf2mBsMobileScanResponseEntry 7 }

-- XXX

-- wmanIf2mBsNeighborBsInfoTable

--

wmanIf2mBsNeighborBsInfoTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsNeighborBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the neighbor BS information that is

sent in the MOB_SCN-RSP and MOB_SCN-REP messages."

REFERENCE

"Subclause 6.3.2.3.49 and 6.3.2.3.50, Table 109i and 109j

in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 4 }

wmanIf2mBsNeighborBsInfoEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsNeighborBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsNeighbirBsIndex }

::= { wmanIf2mBsNeighborBsInfoTable 1 }

WmanIf2mBsNeighborBsInfoEntry ::= SEQUENCE {

wmanIf2mBsNeighbirBsIndex               INTEGER,

wmanIf2mBsFullBsId                      WmanIf2mFullBsId,

wmanIf2mBsScanningType                  WmanIf2mScanType,

wmanIf2mBsRendezvousTime                INTEGER,

wmanIf2mBsScanCdmaCode                  INTEGER,

wmanIf2mBsTxOpportunityOffset           INTEGER,

wmanIf2mBsCinrMean                      INTEGER,

wmanIf2mBsRssiMean                      INTEGER,

wmanIf2mBsRelativeDelay                 INTEGER}

wmanIf2mBsNeighbirBsIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Index to the neighbor BS."

::= { wmanIf2mBsNeighborBsInfoEntry 1 }

wmanIf2mBsFullBsId OBJECT-TYPE

SYNTAX      WmanIf2mFullBsId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object contains the BS ID if full 48 bits BS ID is

used to scan the neighbor BS. This object returns NULL if

the Neighbor_BS_index as defined in MOB_SCN-RSP is used

instead."

::= { wmanIf2mBsNeighborBsInfoEntry 2 }

wmanIf2mBsScanningType OBJECT-TYPE

SYNTAX      WmanIf2mScanType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Type of scanning or association used by the MS and

coordinated by the Serving BS."

::= { wmanIf2mBsNeighborBsInfoEntry 3 }

wmanIf2mBsRendezvousTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This is offset, measured in units of frame duration (of

Serving BS), when the corresponding Recommended BS is

expected to provide non-contention-based ranging

opportunity for the MS."

::= { wmanIf2mBsNeighborBsInfoEntry 4 }

wmanIf2mBsScanCdmaCode OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A unique code assigned to the MS, to be used for

association with the neighbor BS. Code is from the initial

ranging codeset."

::= { wmanIf2mBsNeighborBsInfoEntry 5 }

wmanIf2mBsTxOpportunityOffset OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A unique transmission opportunity assigned to the MS, to be

used for association with the Target BS in units of symbol

duration."

::= { wmanIf2mBsNeighborBsInfoEntry 6 }

wmanIf2mBsCinrMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.5 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS CINR mean parameter indicates the CINR measured by

the MS from the particular BS. The value shall be

interpreted as a signed byte with units of 0.5 dB. The

measurement shall be performed on the subcarriers of the

frame preamble that are active in the particular BS's

segment and averaged over the measurement period."

::= { wmanIf2mBsNeighborBsInfoEntry 7 }

wmanIf2mBsRssiMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.25 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS RSSI mean parameter indicates the Received Signal

Strength measured by the MS from the particular BS. The

value shall be interpreted as an unsigned byte with units

of 0.25 dB."

::= { wmanIf2mBsNeighborBsInfoEntry 8 }

wmanIf2mBsRelativeDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "samples"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter indicates the delay of neighbor DL signals

relative to the serving BS, as measured by the MS for the

particular BS. The value shall be interpreted as a signed

integer in units of samples."

::= { wmanIf2mBsNeighborBsInfoEntry 9 }

-- XXX

-- wmanIf2mBsDiversityBsInfoTable

--

wmanIf2mBsDiversityBsInfoTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsDiversityBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the diversity BS information that is

sent in the MOB_SCN-REP messages."

REFERENCE

"Subclause 6.3.2.3.49 and 6.3.2.3.50, Table 109i and 109j

in IEEE Std 802.16e-2005"

::= { wmanIf2mBsPm 5 }

wmanIf2mBsDiversityBsInfoEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsDiversityBsInfoEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsTempBsIndex }

::= { wmanIf2mBsDiversityBsInfoTable 1 }

WmanIf2mBsDiversityBsInfoEntry ::= SEQUENCE {

wmanIf2mBsTempBsIndex                   INTEGER,

wmanIf2mBsFbssMdhoCinrMean              INTEGER,

wmanIf2mBsFbssMdhoRssiMean              INTEGER,

wmanIf2mBsFbssMdhoRelativeDelay         INTEGER,

wmanIf2mBsFbssMdhoRtd                   INTEGER}

wmanIf2mBsTempBsIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 15)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Diversity set member ID assigned to this BS. When the MS

has an empty diversity set or FBSS/MDHO is not supported,

Temp BSID shall be set to 0.."

::= { wmanIf2mBsDiversityBsInfoEntry 1 }

wmanIf2mBsFbssMdhoCinrMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.5 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS CINR mean parameter indicates the CINR measured by

the MS from the particular BS. The value shall be

interpreted as a signed byte with units of 0.5 dB. The

measurement shall be performed on the subcarriers of the

frame preamble that are active in the particular BS's

segment and averaged over the measurement period."

::= { wmanIf2mBsDiversityBsInfoEntry 2 }

wmanIf2mBsFbssMdhoRssiMean OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "0.25 dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS RSSI mean parameter indicates the Received Signal

Strength measured by the MS from the particular BS. The

value shall be interpreted as an unsigned byte with units

of 0.25 dB."

::= { wmanIf2mBsDiversityBsInfoEntry 3 }

wmanIf2mBsFbssMdhoRelativeDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "samples"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter indicates the delay of neighbor DL signals

relative to the serving BS, as measured by the MS for the

particular BS. The value shall be interpreted as a signed

integer in units of samples."

::= { wmanIf2mBsDiversityBsInfoEntry 4 }

wmanIf2mBsFbssMdhoRtd OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "1/Fs"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The BS RTD parameter indicates the round trip delay (RTD)

measured by the MS from the serving BS. RTD can be given by

the latest time advance taken by MS. The value shall be

interpreted as an unsigned byte with units of 1/Fs (see

10.3.4.3). This parameter shall be only measured on serving

BS/anchor BS."

::= { wmanIf2mBsDiversityBsInfoEntry 5 }