| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | BS and SS Capability Tables Simplification |  |
| Date Submitted | 2007-11-10 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes tables to simplify BS and SS Capability Tables. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# BS and SS Capability Tables Simplification

Joey Chou

Intel

# Introduction

This document proposes wmanIf2BsSsCapabilitiesTable, wmanIf2BsBasicCapabilitiesTable, wmanIf2mBsSsReqCapabilitiesTable, and wmanIf2mBsBasicCapabilitiesTable to replace the following tables.

- wmanIf2BsSsReqCapabilitiesTable

- wmanIf2BsSsRspCapabilitiesTable

- wmanIf2BsBasicCapabilitiesTable

- wmanIf2BsCapabilitiesConfigTable

- wmanIf2mBsSsReqCapabilitiesTable

- wmanIf2mBsSsRspCapabilitiesTable

- wmanIf2mBsBasicCapabilitiesTable

- wmanIf2mBsCapabilitiesConfigTable

# Proposed ASN.1 Notations

# 2.1	wmanIf2BsSsCapabilitiesTable

It proposes the following code be added to the wmanIf2Mib.

WmanIf2BsSsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"xxxReq: Indicates that SS reports its capabilities in

RNG-REQ, SBC-REQ, or REG-REQ messages.

xxxRsp: Indicates that BS confirms SS capabilities in

RNG-RSP, SBC-RSP, or REG-RSP messages."

SYNTAX INTEGER {xxxReq (1),

xxxRsp (2)}

It proposes the following code be included in wmanIf2Mib to replace wmanIf2BsSsReqCapabilitiesTable and wmanIf2BsSsRspCapabilitiesTable.

wmanIf2BsSsCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information of SSs

that have been negotiated and agreed between BS and SS via

RNG-REQ/RSP, SBC-REQ/RSP and REG-REQ/RSP messages."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsCapabilities 1 }

wmanIf2BsSsCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table has triple indexes:

1) the ifIndex with an ifType of ieee80216WMAN, indicating

the BS sector with which the SS is associated

2) wmanIf2BsSsMacAddress identifies the SS

3) wmanIf2BsSsCapabilityType indicates the entry is

associated with either xxx-REQ or xxx-RSP message"

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsSsCapabilityType }

::= { wmanIf2BsSsCapabilitiesTable 1 }

WmanIf2BsSsCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsSsCapabilityType               WmanIf2BsSsCapType,

wmanIf2BsSsUplinkCidSupport             WmanIf2NumOfCid,

wmanIf2BsSsArqSupport                   WmanIf2ArqSupportType,

wmanIf2BsSsDsxFlowControl               WmanIf2MaxDsxFlowType,

wmanIf2BsSsMacCrcSupport                WmanIf2MacCrcSupport,

wmanIf2BsSsMcaFlowControl               WmanIf2MaxMcaFlowType,

wmanIf2BsSsMcpGroupCidSupport           WmanIf2MaxMcpGroupCid,

wmanIf2BsSsPkmFlowControl               WmanIf2MaxPkmFlowType,

wmanIf2BsSsAuthPolicyControl            WmanIf2AuthPolicyType,

wmanIf2BsSsMaxNumOfSupportedSA          WmanIf2MaxNumOfSaType,

wmanIf2BsSsIpVersion                    WmanIf2IpVersionType,

wmanIf2BsSsMacCsSupportBitMap           WmanIf2MacCsBitMap,

wmanIf2BsSsMaxNumOfClassifier           WmanIf2MaxClassifiers,

wmanIf2BsSsPhsSupport                   WmanIf2PhsSupportType,

wmanIf2BsSsBandwidthAllocSupport        WmanIf2BwAllocSupport,

wmanIf2BsSsPduConstruction              WmanIf2PduConstruction,

wmanIf2BsSsTtgTransitionGap             WmanIf2SsTransitionGap,

wmanIf2BsSsRtgTransitionGap             WmanIf2SsTransitionGap,

wmanIf2BsSsDownlinkCidSupport           WmanIf2NumOfCid,

wmanIf2BsSsPackingSupport               WmanIf2PackingSupport,

wmanIf2BsSsExtendedRtpsSupport          WmanIf2ExtRtpsSupport,

wmanIf2BsSsMaxNumBurstToMs              INTEGER,

wmanIf2BsSsIpAddrAllocMethod            WmanIf2IpAllocMethod,

wmanIf2BsSsArqAckType                   WmanIf2ArqAckType,

wmanIf2BsSsMacHeader                    WmanIf2MacHeaderSupp,

wmanIf2BsSsMaxMacLevelDlFrame           WmanIf2MaxMacLevel,

wmanIf2BsSsMaxMacLevelUlFrame           WmanIf2MaxMacLevel,

wmanIf2BsSsNumOfProvisionedSf           Unsigned32,

wmanIf2BsSsPkmVersion                   WmanIf2PkmVersion,

wmanIf2BsSsAuthPolicy                   WmanIf2AuthPolicy,

wmanIf2BsSsMacMode                      WmanIf2MacMode,

wmanIf2BsSsPnWindowSize                 INTEGER,

wmanIf2BsSsExtCapability                WmanIf2ExtCapability}

wmanIf2BsSsCapabilityType OBJECT-TYPE

SYNTAX      WmanIf2BsSsCapType

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This index determines the corresponding entry indicates

the capabilities contained either in RNG/SBC/REG-REQ or

RNG/SBC/REG-RSP."

::= { wmanIf2BsSsCapabilitiesEntry 1 }

wmanIf2BsSsUplinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated number of Uplink CIDs the SS can support."

::= { wmanIf2BsSsCapabilitiesEntry 2 }

wmanIf2BsSsArqSupport OBJECT-TYPE

SYNTAX      WmanIf2ArqSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether the SS is allowed to use ARQ

as a result of the capabilities negotiation."

::= { wmanIf2BsSsCapabilitiesEntry 3 }

wmanIf2BsSsDsxFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxDsxFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of concurrent DSA, DSC, or DSD

transactions that may be outstanding."

::= { wmanIf2BsSsCapabilitiesEntry 4 }

wmanIf2BsSsMacCrcSupport OBJECT-TYPE

SYNTAX      WmanIf2MacCrcSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether or not the SS is allowed to

use MAC level CRC as a result of the capabilities

negotiation."

DEFVAL		{ macCrcSupport }

::= { wmanIf2BsSsCapabilitiesEntry 5 }

wmanIf2BsSsMcaFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxMcaFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of concurrent MCA transactions

that may be outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsSsCapabilitiesEntry 6 }

wmanIf2BsSsMcpGroupCidSupport OBJECT-TYPE

SYNTAX      WmanIf2MaxMcpGroupCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of simultaneous Multicast Polling

Groups the SS is capable of belonging to."

DEFVAL		{ 0 }

::= { wmanIf2BsSsCapabilitiesEntry 7 }

wmanIf2BsSsPkmFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxPkmFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of concurrent PKM transactions

that may be outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsSsCapabilitiesEntry 8 }

wmanIf2BsSsAuthPolicyControl OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicyType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies negotiated authorization policy. A

bit value of 0 = 'not supported', 1 = 'supported'. If this

field is omitted, then both SS and BS shall use the IEEE

802.16 security, constituting X.509 digital certificates

and the RSA public key encryption algorithm, as

authorization policy."

::= { wmanIf2BsSsCapabilitiesEntry 9 }

wmanIf2BsSsMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX      WmanIf2MaxNumOfSaType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of supported security association

of the SS."

DEFVAL		{ 1 }

::= { wmanIf2BsSsCapabilitiesEntry 10 }

wmanIf2BsSsIpVersion OBJECT-TYPE

SYNTAX      WmanIf2IpVersionType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated version of IP used on the 2nd Management

Connection. The value should be undefined if the 2nd

management CID doesn't exist."

::= { wmanIf2BsSsCapabilitiesEntry 11 }

wmanIf2BsSsMacCsSupportBitMap OBJECT-TYPE

SYNTAX      WmanIf2MacCsBitMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated set of MAC convergence sublayer support. When a

bit is set, it indicates the corresponding CS feature is

supported."

::= { wmanIf2BsSsCapabilitiesEntry 12 }

wmanIf2BsSsMaxNumOfClassifier OBJECT-TYPE

SYNTAX      WmanIf2MaxClassifiers

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of admitted Classifiers that the

SS is allowed to have."

DEFVAL      { 0 }

::= { wmanIf2BsSsCapabilitiesEntry 13 }

wmanIf2BsSsPhsSupport OBJECT-TYPE

SYNTAX      WmanIf2PhsSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the negotiated level of PHS support."

DEFVAL      { noPhsSupport }

::= { wmanIf2BsSsCapabilitiesEntry 14 }

wmanIf2BsSsBandwidthAllocSupport OBJECT-TYPE

SYNTAX      WmanIf2BwAllocSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates negotiated properties of the SS for

bandwidth allocation purposes. The usage is defined by

WmanIf2BwAllocSupport."

::= { wmanIf2BsSsCapabilitiesEntry 15 }

wmanIf2BsSsPduConstruction OBJECT-TYPE

SYNTAX      WmanIf2PduConstruction

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies negotiated capabilities for construction and

transmission of MAC PDUs. The usage is defined by

WmanIf2PduConstruction."

::= { wmanIf2BsSsCapabilitiesEntry 16 }

wmanIf2BsSsTtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the negotiated transition speed SSTTG

for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsCapabilitiesEntry 17 }

wmanIf2BsSsRtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the negotiated transition speed SSRTG

for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsCapabilitiesEntry 18 }

wmanIf2BsSsDownlinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object shows the number of Downlink CIDs the SS can

support."

::= { wmanIf2BsSsCapabilitiesEntry 19 }

wmanIf2BsSsPackingSupport OBJECT-TYPE

SYNTAX      WmanIf2PackingSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Packing."

REFERENCE

"Subclause 11.7.8.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 20 }

wmanIf2BsSsExtendedRtpsSupport OBJECT-TYPE

SYNTAX      WmanIf2ExtRtpsSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicate the availability of SS support for extended rtPs."

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 21 }

wmanIf2BsSsMaxNumBurstToMs OBJECT-TYPE

SYNTAX      INTEGER (1..16)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 22 }

wmanIf2BsSsIpAddrAllocMethod OBJECT-TYPE

SYNTAX      WmanIf2IpAllocMethod

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the method of allocating IP address for the

secondary management connection."

REFERENCE

"Subclause 11.7.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 23 }

wmanIf2BsSsArqAckType OBJECT-TYPE

SYNTAX      WmanIf2ArqAckType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the ARQ ACK type

supported by the MS."

REFERENCE

"Subclause 11.7.23 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 24 }

wmanIf2BsSsMacHeader OBJECT-TYPE

SYNTAX      WmanIf2MacHeaderSupp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not the MS and BS support various

types of MAC header and extended subheaders."

REFERENCE

"Subclause 11.7.25 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 25 }

wmanIf2BsSsMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of the

physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsSsCapabilitiesEntry 26 }

wmanIf2BsSsMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of the

physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsSsCapabilitiesEntry 27 }

wmanIf2BsSsNumOfProvisionedSf OBJECT-TYPE

SYNTAX      Unsigned32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted. This object should return NULL, if accessing

the given entry with wmanIf2BsSsCapabilityType = xxx-Req."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 28 }

wmanIf2BsSsPkmVersion OBJECT-TYPE

SYNTAX      WmanIf2PkmVersion

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a PKM version that SS or BS supports."

REFERENCE

"Subclause 11.8.4.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 29 }

wmanIf2BsSsAuthPolicy OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates authorization policy used by the MS and BS to

negotiate and synchronize."

REFERENCE

"Subclause 11.8.4.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 30 }

wmanIf2BsSsMacMode OBJECT-TYPE

SYNTAX      WmanIf2MacMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a MAC (Message Authentication Code) mode that MS

supports."

REFERENCE

"Subclause 11.8.4.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 31 }

wmanIf2BsSsPnWindowSize OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 32 }

wmanIf2BsSsExtCapability OBJECT-TYPE

SYNTAX      WmanIf2ExtCapability

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the extended subheader format support."

REFERENCE

"Subclause 11.8.6 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsCapabilitiesEntry 33 }

# 2.2	wmanIf2BsBasicCapabilitiesTable

It proposes the following new code be added to the wmanIf2Mib.

WmanIf2BsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"inheritedBsCap: Indicates BS' basic HW and SW capabilities.

configedBsCap: Indicates BS' capabilities that has been

configured."

SYNTAX      INTEGER {inheritedBsCap (1),

configedBsCap (2)}

It proposes the following code be included in wmanIf2Mib to replace wmanIf2BsBasicCapabilitiesTable with the following code, and delete wmanIf2BsCapabilitiesConfigTable.

wmanIf2BsBasicCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capabilities of the BS as

implemented in BS HW/SW or configured by EMS that are

determined by INDEX wmanIf2BsCapabilityType:

inheritedBsCap: basic BS HW/SW capabilities

configedBsCap:  configured BS capabilities

These capabilities are used for negotiation of basic

capabilities with SS using RNG-RSP, SBC-RSP and REG-RSP

messages. The negotiated capabilities are obtained

by interSubclause of SS raw reported capabilities, BS raw

capabilities and BS configured capabilities.

The objects in the table have read-write access. But

writing objects in an entry indexed by 'inheritedBsCap'

does not change the value of such object."

::= { wmanIf2BsCapabilities 2 }

wmanIf2BsBasicCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table has double indexes:

1) the ifIndex with an ifType of ieee80216WMAN, indicating

the BS sector with which the SS is associated

2) wmanIf2BsSsCapabilityType indicates the entry is

associated with inherited or configured BS capabilities"

INDEX     { ifIndex, wmanIf2BsCapabilityType }

::= { wmanIf2BsBasicCapabilitiesTable 1 }

WmanIf2BsBasicCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsCapabilityType                 WmanIf2BsCapType,

wmanIf2BsUplinkCidSupport               WmanIf2NumOfCid,

wmanIf2BsArqSupport                     WmanIf2ArqSupportType,

wmanIf2BsDsxFlowControl                 WmanIf2MaxDsxFlowType,

wmanIf2BsMacCrcSupport                  WmanIf2MacCrcSupport,

wmanIf2BsMcaFlowControl                 WmanIf2MaxMcaFlowType,

wmanIf2BsMcpGroupCidSupport             WmanIf2MaxMcpGroupCid,

wmanIf2BsPkmFlowControl                 WmanIf2MaxPkmFlowType,

wmanIf2BsAuthPolicyControl              WmanIf2AuthPolicyType,

wmanIf2BsMaxNumOfSupportedSA            WmanIf2MaxNumOfSaType,

wmanIf2BsIpVersion                      WmanIf2IpVersionType,

wmanIf2BsMacCsSupportBitMap             WmanIf2MacCsBitMap,

wmanIf2BsMaxNumOfClassifier             WmanIf2MaxClassifiers,

wmanIf2BsPhsSupport                     WmanIf2PhsSupportType,

wmanIf2BsBandwidthAllocSupport          WmanIf2BwAllocSupport,

wmanIf2BsPduConstruction                WmanIf2PduConstruction,

wmanIf2BsTtgTransitionGap               WmanIf2SsTransitionGap,

wmanIf2BsRtgTransitionGap               WmanIf2SsTransitionGap,

wmanIf2BsDownlinkCidSupport             WmanIf2NumOfCid,

wmanIf2BsPackingSupport                 WmanIf2PackingSupport,

wmanIf2BsExtendedRtpsSupport            WmanIf2ExtRtpsSupport,

wmanIf2BsMaxNumBurstToMs                INTEGER,

wmanIf2BsIpAddrAllocMethod              WmanIf2IpAllocMethod,

wmanIf2BsArqAckType                     WmanIf2ArqAckType,

wmanIf2BsMacHeader                      WmanIf2MacHeaderSupp,

wmanIf2BsMaxMacLevelDlFrame             WmanIf2MaxMacLevel,

wmanIf2BsMaxMacLevelUlFrame             WmanIf2MaxMacLevel,

wmanIf2BsNumOfProvisionedSf             Unsigned32,

wmanIf2BsPkmVersion                     WmanIf2PkmVersion,

wmanIf2BsAuthPolicy                     WmanIf2AuthPolicy,

wmanIf2BsMacMode                        WmanIf2MacMode,

wmanIf2BsPnWindowSize                   INTEGER,

wmanIf2BsExtCapability                  WmanIf2ExtCapability}

wmanIf2BsCapabilityType OBJECT-TYPE

SYNTAX      WmanIf2BsCapType

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This index determines the corresponding entry indicates the

BS capabilities that are either supported or configured."

::= { wmanIf2BsBasicCapabilitiesEntry 1 }

wmanIf2BsUplinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object shows the number of Uplink CIDs the BS can

support."

::= { wmanIf2BsBasicCapabilitiesEntry 2 }

wmanIf2BsArqSupport OBJECT-TYPE

SYNTAX      WmanIf2ArqSupportType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates whether the BS supports ARQ."

::= { wmanIf2BsBasicCapabilitiesEntry 3 }

wmanIf2BsDsxFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxDsxFlowType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent DSx

transactions that BS allows each SS to have outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 4 }

wmanIf2BsMacCrcSupport OBJECT-TYPE

SYNTAX      WmanIf2MacCrcSupport

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates whether or not the BS supports MAC

level CRC."

DEFVAL		{ macCrcSupport }

::= { wmanIf2BsBasicCapabilitiesEntry 5 }

wmanIf2BsMcaFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxMcaFlowType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent MCA

transactions that BS allows each SS to have."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 6 }

wmanIf2BsMcpGroupCidSupport OBJECT-TYPE

SYNTAX      WmanIf2MaxMcpGroupCid

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the maximum number of simultaneous Multicast

Polling Groups the BS allows each SS to belong to."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 7 }

wmanIf2BsPkmFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxPkmFlowType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Specifies the maximum number of concurrent PKM

transactions that BS allows each SS to have."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 8 }

wmanIf2BsAuthPolicyControl OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicyType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object specifies authorization policy that BS is

capable of. A bit value of 0 = 'not supported', 1 =

'supported'. If this field is omitted, then both SS and

BS shall use the IEEE 802.16 security, constituting X.509

digital certificates and the RSA public key encryption

algorithm, as authorization policy."

::= { wmanIf2BsBasicCapabilitiesEntry 9 }

wmanIf2BsMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX      WmanIf2MaxNumOfSaType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field specifies maximum number of supported security

associations per SS that the BS allows."

DEFVAL		{ 1 }

::= { wmanIf2BsBasicCapabilitiesEntry 10 }

wmanIf2BsIpVersion OBJECT-TYPE

SYNTAX      WmanIf2IpVersionType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates the version of IP BS allows each SS

to use on the 2nd Management Connection. The value

'undefined' should not be used for this field."

REFERENCE

"Subclause 11.7.4 in IEEE Std 802.16-2004"

::= { wmanIf2BsBasicCapabilitiesEntry 11 }

wmanIf2BsMacCsSupportBitMap OBJECT-TYPE

SYNTAX      WmanIf2MacCsBitMap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates BS set of MAC convergence sublayer

support. When a bit is set, it indicates the corresponding

CS feature is supported."

::= { wmanIf2BsBasicCapabilitiesEntry 12 }

wmanIf2BsMaxNumOfClassifier OBJECT-TYPE

SYNTAX      WmanIf2MaxClassifiers

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates the maximum number of admitted

Classifiers per SS that the BS allows."

DEFVAL      { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 13 }

wmanIf2BsPhsSupport OBJECT-TYPE

SYNTAX      WmanIf2PhsSupportType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates the level of BS support for PHS."

DEFVAL      { noPhsSupport }

::= { wmanIf2BsBasicCapabilitiesEntry 14 }

wmanIf2BsBandwidthAllocSupport OBJECT-TYPE

SYNTAX      WmanIf2BwAllocSupport

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the bandwidth allocation properties

that the BS permits SSs to use."

::= { wmanIf2BsBasicCapabilitiesEntry 15 }

wmanIf2BsPduConstruction OBJECT-TYPE

SYNTAX      WmanIf2PduConstruction

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Specifie the capabilities for construction and transmission

of MAC PDUs allowed by the BS."

::= { wmanIf2BsBasicCapabilitiesEntry 16 }

wmanIf2BsTtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the transition speed SSTTG for TDD and

H-FDD SSs allowed by the BS."

::= { wmanIf2BsBasicCapabilitiesEntry 17 }

wmanIf2BsRtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the transition speed SSRTG for TDD and

H-FDD SSs allowed by the BS."

::= { wmanIf2BsBasicCapabilitiesEntry 18 }

wmanIf2BsDownlinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object shows the number of Downlink CIDs the SS can

support."

::= { wmanIf2BsBasicCapabilitiesEntry 19 }

wmanIf2BsPackingSupport OBJECT-TYPE

SYNTAX      WmanIf2PackingSupport

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Packing."

REFERENCE

"Subclause 11.7.8.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 20 }

wmanIf2BsExtendedRtpsSupport OBJECT-TYPE

SYNTAX      WmanIf2ExtRtpsSupport

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicate the availability of SS support for extended rtPs."

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 21 }

wmanIf2BsMaxNumBurstToMs OBJECT-TYPE

SYNTAX      INTEGER (1..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 22 }

wmanIf2BsIpAddrAllocMethod OBJECT-TYPE

SYNTAX      WmanIf2IpAllocMethod

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the method of allocating IP address for the

secondary management connection."

REFERENCE

"Subclause 11.7.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 23 }

wmanIf2BsArqAckType OBJECT-TYPE

SYNTAX      WmanIf2ArqAckType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The value of this parameter specifies the ARQ ACK type

supported by the MS."

REFERENCE

"Subclause 11.7.23 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 24 }

wmanIf2BsMacHeader OBJECT-TYPE

SYNTAX      WmanIf2MacHeaderSupp

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates whether or not the MS and BS support various

types of MAC header and extended subheaders."

REFERENCE

"Subclause 11.7.25 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 25 }

wmanIf2BsMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 26 }

wmanIf2BsMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of the

physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 27 }

wmanIf2BsNumOfProvisionedSf OBJECT-TYPE

SYNTAX      Unsigned32 (0 .. 255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 28 }

wmanIf2BsPkmVersion OBJECT-TYPE

SYNTAX      WmanIf2PkmVersion

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates a PKM version that SS or BS supports."

REFERENCE

"Subclause 11.8.4.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 29 }

wmanIf2BsAuthPolicy OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicy

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates authorization policy used by the MS and BS to

negotiate and synchronize."

REFERENCE

"Subclause 11.8.4.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 30 }

wmanIf2BsMacMode OBJECT-TYPE

SYNTAX      WmanIf2MacMode

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates a MAC (Message Authentication Code) mode that MS

supports."

REFERENCE

"Subclause 11.8.4.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 31 }

wmanIf2BsPnWindowSize OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 32 }

wmanIf2BsExtCapability OBJECT-TYPE

SYNTAX      WmanIf2ExtCapability

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the extended subheader format support."

REFERENCE

"Subclause 11.8.6 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 33 }

# 2.3	wmanIf2mBsSsCapabilitiesTable

It proposes the following new code be added to the wmanIf2mMib.

WmanIf2mBsSsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"xxxReq: Indicates that SS reports its capabilities in

RNG-REQ, SBC-REQ, or REG-REQ messages.

xxxRsp: Indicates that BS confirms SS capabilities in

RNG-RSP, SBC-RSP, or REG-RSP messages."

SYNTAX INTEGER {xxxReq (1),

xxxRsp (2)}

It proposes the following code be included in wmanIf2mMib to replace wmanIf2mBsSsReqCapabilitiesTable and wmanIf2mBsSsRspCapabilitiesTable.

wmanIf2mBsSsCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsSsCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information of SSs

that are necessary for supporting mobility, and have been

negotiated and agreed between BS and SS via RNG-REQ/RSP,

SBC-REQ/RSP and REG-REQ/RSP messages."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsCapabilities 1 }

wmanIf2mBsSsCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsSsCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table has triple indexes:

1) the ifIndex with an ifType of ieee80216WMAN, indicating

the BS sector with which the SS is associated

2) wmanIf2mBsSsMacAddress identifies the SS

3) wmanIf2mBsSsCapabilityType indicates the entry is

associated with either xxx-REQ or xxx-RSP message"

INDEX     { ifIndex,

wmanIf2mBsSsMacAddress,

wmanIf2mBsSsCapabilityType }

::= { wmanIf2mBsSsCapabilitiesTable 1 }

WmanIf2mBsSsCapabilitiesEntry ::= SEQUENCE {

wmanIf2mBsSsCapabilityType              WmanIf2mBsSsCapType,

wmanIf2mBsSsHandoverSupported           WmanIf2mHandoverType,

wmanIf2mBsSsHoProcessTimer              Unsigned32,

wmanIf2mBsSsMobilityFeature             WmanIf2mOfdmaMobility,

wmanIf2mBsSsSleepRecoveryTime           Unsigned32,

wmanIf2mBsSsPreviousIpAddr              OCTET STRING,

wmanIf2mBsSsIdleModeTimeout             Unsigned32,

wmanIf2mBsSsHoConnProcessTime           Unsigned32,

wmanIf2mBsSsHoTekProcessTime            Unsigned32,

wmanIf2mBsSsPowerSavingType             WmanIf2mPowerSaveType,

wmanIf2mBsSsNumOfPsClassIandII          INTEGER,

wmanIf2mBsSsNumOfPsClassIII             INTEGER,

wmanIf2mBsSsHoTrigMatrix                WmanIf2mHoTrigMatrix,

wmanIf2mBsSsAssociationType             WmanIf2mAssociationTyp}

wmanIf2mBsSsCapabilityType OBJECT-TYPE

SYNTAX      WmanIf2mBsSsCapType

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This index determines the corresponding entry indicates

the capabilities contained either in RNG/SBC/REG-REQ or

RNG/SBC/REG-RSP."

::= { wmanIf2mBsSsCapabilitiesEntry 1 }

wmanIf2mBsSsHandoverSupported OBJECT-TYPE

SYNTAX      WmanIf2mHandoverType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS or MS supports."

REFERENCE

"Subclause 11.7.12 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 2 }

wmanIf2mBsSsHoProcessTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration in frames the MS shall wait until receipt of

the next unsolicited network re-entry MAC management

message as indicated in the HO Process Optimization element

of the RNG-RSP message."

REFERENCE

"Subclause 11.7.13.2 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 3 }

wmanIf2mBsSsMobilityFeature OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaMobility

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The field indicates whether or not the MS supports mobility

hand-over, Slee-pmode, and Idle-mode."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 4 }

wmanIf2mBsSsSleepRecoveryTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object indicates the time required for an MS that is in

a sleep mode to return to awake-mode. This may be used by

the BS to determine sleep interval window sizes when

initiating sleep mode with an MS."

REFERENCE

"Subclause 11.7.15 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 5 }

wmanIf2mBsSsPreviousIpAddr OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object indicates the IP address that the MS was

assigned on the secondary management connection based on an

association with its last serving BS. An IPv4 address shall

be specified in conventional dotted format; e.g.,

'134.234.2.3'. An IPv6 address may be expressed in abridged

or unabridged form; however, the form chosen shall be

consistent with RFC4291."

REFERENCE

"Subclause 11.7.16 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 6 }

wmanIf2mBsSsIdleModeTimeout OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Max time interval between MS Idle Mode Location Updates."

REFERENCE

"Subclause 11.7.20.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 7 }

wmanIf2mBsSsHoConnProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to process information on

connections provided in RNG-RSP or REG-RSP message during

Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 8 }

wmanIf2mBsSsHoTekProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to completely process TEK

information during Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 9 }

wmanIf2mBsSsPowerSavingType OBJECT-TYPE

SYNTAX      WmanIf2mPowerSaveType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 10 }

wmanIf2mBsSsNumOfPsClassIandII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 1 and 2."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 11 }

wmanIf2mBsSsNumOfPsClassIII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 3."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 12 }

wmanIf2mBsSsHoTrigMatrix OBJECT-TYPE

SYNTAX      WmanIf2mHoTrigMatrix

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates trigger metrics that MS or BS supports."

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 13 }

wmanIf2mBsSsAssociationType OBJECT-TYPE

SYNTAX      WmanIf2mAssociationTyp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the association level supported by the MS or the

BS."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsSsCapabilitiesEntry 14 }

# 2.4	wmanIf2mBsBasicCapabilitiesTable

It proposes the following new code be added to the wmanIf2mMib.

WmanIf2mBsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"inheritedBsCap: Indicates BS' basic HW and SW capabilities.

configedBsCap: Indicates BS' capabilities that has been

configured."

SYNTAX      INTEGER {inheritedBsCap (1),

configedBsCap (2)}

It proposes the following code be included in wmanIfm2Mib to replace wmanIf2mBsBasicCapabilitiesTable with the following code, and delete wmanIf2mBsCapabilitiesConfigTable

wmanIf2mBsBasicCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mBsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capabilities of the BS as

implemented in BS HW/SW or configured by EMS that are

determined by INDEX wmanIf2BsCapabilityType:

inheritedBsCap: basic BS HW/SW capabilities

configedBsCap:  configured BS capabilities

These capabilities are used for negotiation of basic

capabilities with SS using RNG-RSP, SBC-RSP and REG-RSP

messages. The negotiated capabilities are obtained

by interSubclause of SS raw reported capabilities, BS raw

capabilities and BS configured capabilities.

The objects in the table have read-write access. But

writing objects in an entry indexed by 'inheritedBsCap'

does not change the value of such object."

::= { wmanIf2mBsCapabilities 2 }

wmanIf2mBsBasicCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2mBsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table has double indexes:

1) the ifIndex with an ifType of ieee80216WMAN, indicating

the BS sector with which the SS is associated

2) wmanIf2mBsCapabilityType indicates the entry is

associated with inherited or configured BS capabilities"

INDEX     { ifIndex, wmanIf2mBsCapabilityType }

::= { wmanIf2mBsBasicCapabilitiesTable 1 }

WmanIf2mBsBasicCapabilitiesEntry ::= SEQUENCE {

wmanIf2mBsCapabilityType                WmanIf2mBsCapType,

wmanIf2mBsHandoverSupported             WmanIf2mHandoverType,

wmanIf2mBsRetrainTime                   Unsigned32,

wmanIf2mBsHoProcessTimer                Unsigned32,

wmanIf2mBsRetransmissionTimer           Unsigned32,

wmanIf2mBsMobilityFeature               WmanIf2mOfdmaMobility,

wmanIf2mBsIdleModeTimeout               Unsigned32,

wmanIf2mBsHoConnProcessTime             Unsigned32,

wmanIf2mBsHoTekProcessTime              Unsigned32,

wmanIf2mBsPowerSavingType               WmanIf2mPowerSaveType,

wmanIf2mBsNumOfPsClassIandII            INTEGER,

wmanIf2mBsNumOfPsClassIII               INTEGER,

wmanIf2mBsHoTrigMatrix                  WmanIf2mHoTrigMatrix,

wmanIf2mBsAssociationType               WmanIf2mAssociationTyp}

wmanIf2mBsCapabilityType OBJECT-TYPE

SYNTAX      WmanIf2mBsCapType

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This index determines the corresponding entry indicates

the BS capabilities that are either supported or configured

."

::= { wmanIf2mBsBasicCapabilitiesEntry 1 }

wmanIf2mBsHandoverSupported OBJECT-TYPE

SYNTAX      WmanIf2mHandoverType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS or MS supports."

REFERENCE

"Subclause 11.7.12 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 2 }

wmanIf2mBsRetrainTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "100 milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the duration for MS's connection information that

will be retained in serving BS. BS shall start

Resource_Retain_Time timer at MS notification of pending HO

attempt through MOB_HO-IND or by detecting an MS drop."

REFERENCE

"Subclause 11.7.13.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 3 }

wmanIf2mBsHoProcessTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration in frames the MS shall wait until receipt of

the next unsolicited network re-entry MAC management

message as indicated in the HO Process Optimization element

of the RNG-RSP message."

REFERENCE

"Subclause 11.7.13.2 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 4 }

wmanIf2mBsRetransmissionTimer OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "frames"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"When an MS transmits MOB_MSHO-REQ to initiate a handover

process, it shall start MS Handover Retransmission Timer

and shall not transmit another MOB_MSHO-REQ until the

expiration of the MS Handover Retransmission Timer."

REFERENCE

"Subclause 11.7.13.3 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 5 }

wmanIf2mBsMobilityFeature OBJECT-TYPE

SYNTAX      WmanIf2mOfdmaMobility

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The field indicates the mobility hand-over, Sleep-mode, and

Idle-mode supported by BS."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 6 }

wmanIf2mBsIdleModeTimeout OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Max time interval between MS Idle Mode Location Updates."

REFERENCE

"Subclause 11.7.20.1 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 7 }

wmanIf2mBsHoConnProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to process information on

connections provided in RNG-RSP or REG-RSP message during

Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 8 }

wmanIf2mBsHoTekProcessTime OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "milliseconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The duration that the MS needs to completely process TEK

information during Handoff."

REFERENCE

"Subclause 11.7.24 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 9 }

wmanIf2mBsPowerSavingType OBJECT-TYPE

SYNTAX      WmanIf2mPowerSaveType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 10 }

wmanIf2mBsNumOfPsClassIandII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 1 and 2."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 11 }

wmanIf2mBsNumOfPsClassIII OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of power save class instances supported from class

types 3."

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 12 }

wmanIf2mBsHoTrigMatrix OBJECT-TYPE

SYNTAX      WmanIf2mHoTrigMatrix

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates trigger metrics that MS or BS supports."

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 13 }

wmanIf2mBsAssociationType OBJECT-TYPE

SYNTAX      WmanIf2mAssociationTyp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the association level supported by the MS or the

BS."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

::= { wmanIf2mBsBasicCapabilitiesEntry 14 }