| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | wmanIf2BsMib |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes wmanIf2BsMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# wmanIf2BsMib

Joey Chou

Intel

# Introduction

This document proposes wmanIf2BsMib.

# Proposed ASN.1 Notations

WMAN-IF2-BS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF

ifIndex

FROM IF-MIB;

wmanIf2BsMib MODULE-IDENTITY

LAST-UPDATED    "200711090000Z" -- November 09, 2007

ORGANIZATION    "IEEE 802.16"

CONTACT-INFO

"WG E-mail: stds-802-16@ieee.org

WG Chair:  Roger B. Marks

Postal:    NextWave Broadband, Inc.

E-mail:    r.b.marks@ieee.org

TG Chair:  Phillip Barber

Postal:    Huawei Technologies Co., Ltd

E-mail:    pbarber@huawei.com

Editor:    Joey Chou

Postal:    Intel Corporation

5000 W. Chandler Blvd,

Chandler, AZ 85227, USA

E-mail:    joey.chou@intel.com"

DESCRIPTION

"This material is from IEEE Std 802.16i

Copyright (c) 2007 IEEE.

This MIB Module defines managed objects for Base Station

based on IEEE Std 802.16

. The MIB contains managed objects that are

common for both fixed and mobile Broadband Wireless

Networks."

REVISION        "200711090000Z"

DESCRIPTION

"The first revision of WMAN-IF2-BS-MIB module that is enhanced

to support IEEE 802.16e-2005 standard."

::= { iso std(0) iso8802(8802) wman(16) 2 }

wmanIf2MibObjects    OBJECT IDENTIFIER ::= { wmanIf2BsMib 1 }

wmanIf2BsFm          OBJECT IDENTIFIER ::= { wmanIf2MibObjects 1 }

wmanIf2BsCm          OBJECT IDENTIFIER ::= { wmanIf2MibObjects 2 }

wmanIf2BsAm          OBJECT IDENTIFIER ::= { wmanIf2MibObjects 3 }

wmanIf2BsPm          OBJECT IDENTIFIER ::= { wmanIf2MibObjects 4 }

wmanIf2BsSm          OBJECT IDENTIFIER ::= { wmanIf2MibObjects 5 }

-- Textual Conventions

WmanIf2SfSchedulingType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The scheduling service provided by a SC for an upstream

service flow. If the parameter is omitted from an upstream

QOS Parameter Set, this object takes the value of

bestEffort (2). This parameter must be reported as

undefined (1) for downstream QOS Parameter Sets."

SYNTAX      INTEGER {undefined(1),

bestEffort(2),

nonRealTimePollingService(3),

realTimePollingService(4),

extRealTimePollingService(5),

unsolicitedGrantService(6)}

WmanIf2PhsRuleVerify ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression. If PHSV is

enabled, the sender shall compare the bytes in the packet

header with the bytes in the PHSF that are to be

suppressed as indicated by the PHSM."

REFERENCE

"Subclause 11.13.19.3.7.5 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {phsVerifyEnable(0),

phsVerifyDisable(1)}

WmanIf2ClassifierBitMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"A bit of of this object is set to 1 if the parameter

indicated by the comment was present in the classifier

encoding, and 0 otherwise.

Note: that BITS are encoded most significant bit first,

so that if e.g. bits 6 and 7 are set, this object is

encoded as the octet string '030000'H."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16e-2005"

SYNTAX      BITS {priority(0),

ipTos(1),

ipProtocol(2),

ipMaskedSrcAddr(3),

ipMaskedDestAddr(4),

srcPort(5),

destPort(6),

destMacAddr(7),

srcMacAddr(8),

ethernetProtocol(9),

userPriority(10),

vlanId(11),

ipv6FlowLabel(12),

actionRule(13)}

WmanIf2SfState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the state of a service flow."

SYNTAX      INTEGER {authorized(1),

admitted(2),

active(3)}

WmanIf2ServClassName ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the type of service class name."

SYNTAX      OCTET STRING (SIZE(2..128))

WmanIf2CsSpecification ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the types of convergence sublayer."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {reserved(0),

packetIpV4(1),

packetIpV6(2),

packet802dot3Ethernet(3),

packet802dot1QVlan(4),

packetIpV4Over802dot3(5),

packetIpV6Over802dot3(6),

packetIpV4Over802dot1Q(7),

packetIpV6Over802dot1Q(8),

atm(9),

packet802dot3EthernetRohcHc(10),

packet802dot3EthernetEcrtpHc(11),

packetIp2RohcHc(12),

packetIp2EcrtpHc(13)}

WmanIf2MacVersion ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Version number of IEEE 802.16."

SYNTAX      INTEGER {ieee802Dot16Of2001(1),

ieee802Dot16cOf2002(2),

ieee802Dot16aOf2003(3),

ieee802Dot16Of2004(4),

ieee802Dot16e(5),

tbd(6)}

WmanIf2CidType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of CID."

SYNTAX      INTEGER (0 .. 65535)

WmanIf2DataEncryptAlgId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Data encryption algorithm identifiers."

REFERENCE

"Table 375 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {none(0),

des56BitCbcMode(1),

aes128BitCcmMode(2),

aes128BitCbcMode(3),

aes128BitCtrMode(128)}

WmanIf2DataAuthAlgId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Data authentication algorithm identifiers."

REFERENCE

"Table 376 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noDataAuthentication(0),

aes128BitCcmMode(1)}

WmanIf2TekEncryptAlgId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"TEK encryption algorithm identifiers."

REFERENCE

"Table 377 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {tripleDes128BitKey(1),

rsa1024BitKey(2),

aes128BitKeyEcbMode(3),

aes128BitKeyWrap(4)}

WmanIf2PkmErrorCode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This error code provides further information about an

Authorization Reject, Key Reject, Authorization Invalid,

or TEK Invalid.

0 - no failure

1 - unauthorized SS

2 - unauthorized SAID

3 - unsolicited

4 - invalid key sequence

5 - key request authentication failure

The following are error code for permanent authorization

failure that indicates any reattempts at authorization

would continue to result in Authorization Rejects.

6 - the BS does not have the CA certificate belonging

to the issuer of an SS certificate

7 - SS certificate has an invalid signature

8 - ASN.1 parsing failure during verification of SS

certificate

9 - SS certificate is on the 'hot list'

10 - inconsistencies between certificate data and data

in accompanying PKM attributes

11 - SS and BS have incompatible security capabilities"

REFERENCE

"Subclause 11.9.10 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {noFailure(0),

unauthorizedSs(1),

unauthorizedSaid(2),

unsolicited(3),

invalidKeySequence(4),

keyReqAuthFailure(5),

umknownManufactur(6),

invalidSignature(7),

asn1ParsingFailure(8),

ssCaOnHotList(9),

dataInconsistency(10),

ssBsIncompatibleSc(11)}

WmanIf2SaType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The type of Security Association (SA)."

REFERENCE

"Table 379 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {primarySa(0),

staticSa(1),

dynamicSa(2)}

WmanIf2TekState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"TEK State."

REFERENCE

"Subclause 7.2.5.1 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {start(1),

opWait(2),

opReauthWait(3),

operational(4),

rekeyWait(5),

rekeyReauthWait(6)}

WmanIf2CertificateStat ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The reason why a SS's certificate is deemed valid

or invalid:

0 - return unknown if the SS is running PKM mode

1 - means the certificate is valid because it chains

to a valid certificate

2 - means the certificate is valid because it has been

provisioned to be trusted

3 - means the certificate is invalid because it has been

provisioned to be untrusted.

4 - means the certificate is invalid because it chains

to an untrusted certificate.

5 - refer to errors in parsing, validity periods, etc,

of SS certificate

6 - refer to errors in parsing, validity periods, etc,

of CA certificate"

REFERENCE

"Subclause 7.2.5.1 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {unknown (0),

validSsChained (1),

validSsTrusted (2),

invalidSsUntrusted (3),

invalidCAUntrusted (4),

invalidSsOther (5),

invalidCAOther (6)}

WmanIf2ChannelNumber ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Physical channel number"

SYNTAX      INTEGER (0 .. 199)

WmanIf2OfdmFecCodeType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"FEC code type and modulation type"

REFERENCE

"Table 356 and Table 362 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {bpskCc1Over2(0),

qpskRsCcCc1Over2(1),

qpskRsCcCc3Over4(2),

sixteenQamRsCcCc1Over2(3),

sixteenQamRsCcCc3Over4(4),

sixtyFourQamRsCcCc2Over3(5),

sixtyFourQamRsCcCc3Over4(6),

qpskBtc1Over2(7),

qpskBtc3Over4(8),

sixteenQamBtc3Over4(9),

sixteenQamBtc4Over5(10),

sixtyFourQamBtc2Over3(11),

sixtyFourQamBtc5Over6(12),

qpskCtc1Over2(13),

qpskCtc2Over3(14),

qpskCtc3Over4(15),

sixteenQamCtc1Over2(16),

sixteenQamCtc3Over4(17),

sixtyFourQamCtc2Over3(18),

sixtyFourQamCtc3Over4(19)}

WmanIf2OfdmaFecCodeType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"FEC code type and modulation type"

REFERENCE

"Table 356 and Table 362 in IEEE Std 802.16-2004"

SYNTAX 	    INTEGER {qpskCc1Over2(0),

qpskCc3Over4(1),

sixteenQamCc1Over2(2),

sixteenQamCc3Over4(3),

sixtyFourQamCc2Over3(4),

sixtyFourQamCc3Over4(5),

qpskBtc1Over2(6),

qpskBtc2Over3(7),

sixteenQamBtc3Over5(8),

sixteenQamBtc4Over5(9),

sixtyFourQamBtc5Over8(10),

sixtyFourQamBtc4Over5(11),

qpskCtc1Over2(12),

qpskCtc2Over3(13),

qpskCtc3Over4(14),

sixteenQamCtc1Over2(15),

sixteenQamCtc3Over4(16),

sixtyFourQamCtc2Over3(17),

sixtyFourQamCtc3Over4(18),

sixtyFourQamCtc5Over6(19),

qpskZtCc1Over2(20),

qpskZtCc3Over4(21),

sixteenQamZtCc1Over2(22),

sixteenQamZtCc3Over4(23),

sixtyFourQamZtCc2Over3(24),

sixtyFourQamZtCc3Over4(25)}

WmanIf2OfdmaUcdFecCode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"UCD FEC code type and modulation type"

REFERENCE

"Table 357 in IEEE Std 802.16e-2005"

SYNTAX 	    INTEGER {qpskCc1Over2(0),

qpskCc3Over4(1),

sixteenQamCc1Over2(2),

sixteenQamCc3Over4(3),

sixtyFourQamCc1Over2(4),

sixtyFourQamCc2Over3(5),

sixtyFourQamCc3Over4(6),

qpskBtc1Over2(7),

qpskBtc3Over4(8),

sixteenQamBtc3Over5(9),

sixteenQamBtc4Over5(10),

sixtyFourQamBtc5Over8(11),

sixtyFourQamBtc4Over5(12),

qpskCtc1Over2(13),

reserved(14),

qpskCtc3Over4(15),

sixteenQamCtc1Over2(16),

sixteenQamCtc3Over4(17),

sixtyFourQamCtc1Over2(18),

sixtyFourQamCtc2Over3(19),

sixtyFourQamCtc3Over4(20),

sixtyFourQamCtc5Over6(21),

qpskZtCc1Over2(22),

qpskZtCc3Over4(23),

sixteenQamZtCc1Over2(24),

sixteenQamZtCc3Over4(25),

sixtyFourQamZtCc1Over2(26),

sixtyFourQamZtCc2Over3(27),

sixtyFourQamZtCc3Over4(28),

qpskLdpc1over2(29),

qpskLdpc2over3A(30),

qpskLdpc3over4A(31),

sixteenQamLdpc1over2(32),

sixteenQamLdpc2over3A(33),

sixteenQamLdpc3over4A(34),

sixtyFourQamLdpc1over2(35),

sixtyFourQamLdpc2over3A(36),

sixtyFourQamLdpc3over4A(37),

qpskLdpc2over3B(38),

qpskLdpc3over4B(39),

sixteenQamLdpc2over3B(40),

sixteenQamLdpc3over4B(41),

sixtyFourQamLdpc2over3B(42),

sixtyFourQamLdpc3over4B(43),

qpskCcOptIntv1over2(44),

qpskCcOptIntv3over4(45),

sixteenQamCcOptIntv1over2(46),

sixteenQamCcOptIntv3over4(47),

sixtyFourQamCcOptIntv2over3(48),

sixtyFourQamCcOptIntv3over4(49),

qpskLdpc5over6(50),

sixteenQamLdpc5over6(51),

sixtyFourQamLdpc5over6(52)}

WmanIf2OfdmaDcdFecCode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"DCD FEC code type and modulation type"

REFERENCE

"Table 363 in IEEE Std 802.16e-2005"

SYNTAX 	    INTEGER {qpskCc1Over2(0),

qpskCc3Over4(1),

sixteenQamCc1Over2(2),

sixteenQamCc3Over4(3),

sixtyFourQamCc1Over2(4),

sixtyFourQamCc2Over3(5),

sixtyFourQamCc3Over4(6),

qpskBtc1Over2(7),

qpskBtc3Over4Or2Over3(8),

sixteenQamBtc3Over5(9),

sixteenQamBtc4Over5(10),

sixtyFourQamBtc2Over3Or5Over8(11),

sixtyFourQamBtc5Over6Or4Over5(12),

qpskCtc1Over2(13),

reserved14(14),

qpskCtc3Over4(15),

sixteenQamCtc1Over2(16),

sixteenQamCtc3Over4(17),

sixtyFourQamCtc1Over2(18),

sixtyFourQamCtc2Over3(19),

sixtyFourQamCtc3Over4(20),

sixtyFourQamCtc5Over6(21),

qpskZtCc1Over2(22),

qpskZtCc3Over4(23),

sixteenQamZtCc1Over2(24),

sixteenQamZtCc3Over4(25),

sixtyFourQamZtCc1Over2(26),

sixtyFourQamZtCc2Over3(27),

sixtyFourQamZtCc3Over4(28),

reserved29(29),

reserved30(30),

reserved31(31),

reserved32(32),

reserved33(33),

reserved34(34),

reserved35(35),

reserved36(36),

reserved37(37),

reserved38(38),

reserved39(39),

reserved40(40),

reserved41(41),

reserved42(42),

reserved43(43),

qpskCcOptIntv1over2(44),

qpskCcOptIntv3over4(45),

sixteenQamCcOptIntv1over2(46),

sixteenQamCcOptIntv3over4(47),

sixtyFourQamCcOptIntv2over3(48),

sixtyFourQamCcOptIntv3over4(49)}

-- Textual convention for capabilities encodings

WmanIf2NumOfCid ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type shows the number of CIDs that

SS can support."

REFERENCE

"Subclause 11.7.6 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER (2..65535)

WmanIf2ArqSupportType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type indicates whether the SS support

ARQ."

REFERENCE

"Subclause 11.7.8.1 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {arqNotSupported(0),

arqSupported(1)}

WmanIf2MaxDsxFlowType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type specifies the maximum number of

concurrent DSA, DSC, or DSD transactions that may be

outstanding."

REFERENCE

"Subclause 11.7.8.2 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..255)

WmanIf2MacCrcSupport ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type indicates whether or not the SS

supports MAC level CRC."

REFERENCE

"Subclause 11.7.8.3 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {noMacCrcSupport(0),

macCrcSupport(1)}

WmanIf2MaxMcaFlowType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type specifies the maximum number of

concurrent MCA transactions that may be outstanding."

REFERENCE

"Subclause 11.7.8.4 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..255)

WmanIf2MaxMcpGroupCid ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type indicates the maximum number of

simultaneous Multicast Polling Groups the SS is

capable of belonging to."

REFERENCE

"Subclause 11.7.8.5 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..255)

WmanIf2MaxPkmFlowType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type specifies the maximum number of

concurrent PKM transactions that may be outstanding."

REFERENCE

"Subclause 11.7.8.6 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..255)

WmanIf2AuthPolicyType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type specifies authorization policy

that both SS and BS need to negotiate and synchronize.

A bit value of 0 = 'not supported', 1 = 'supported'. If

this field is omitted, then both SS and BS shall use the

IEEE 802.16 security, constituting X.509 digital

certificates and the RSA public key encryption

algorithm, as authorization policy."

REFERENCE

"Subclause 11.7.8.7 in IEEE Std 802.16-2004"

SYNTAX      BITS {ieee802Dot16PrivacySupported(0),

reserved1(1),

reserved2(2),

reserved3(3),

reserved4(4),

reserved5(5),

reserved6(6),

reserved7(7)}

WmanIf2MaxNumOfSaType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field specifies maximum number of supported

security association of the SS."

REFERENCE

"Subclause 11.7.8.8 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..255)

WmanIf2IpVersionType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type indicates the version of IP used

on the Secondary Management Connection. The valus should

be undefined if the 2nd management CID doesn't exist."

REFERENCE

"Subclause 11.7.4 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {undefined(0),

ipv4(1),

ipv6(2)}

WmanIf2MacCsBitMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type indicates the set of MAC

convergence sublayer support. When a bit is set, it

indicates the corresponding CS feature is supported."

REFERENCE

"Subclause 11.7.7.1 in IEEE Std 802.16e-2005"

SYNTAX      BITS {atm(0),

packetIpv4(1),

packetIpv6(2),

packet802Dot3(3),

packet802Dot1Q(4),

packetIpv4Over802Dot3(5),

packetIpv6Over802Dot3(6),

packetIpv4Over802Dot1Q(7),

packetIpv6Over802Dot1Q(8),

packet802dot3EthernetRohcHc(9),

packet802dot3EthernetEcrtpHc(10),

packetIpv4Orv6RohcHc(11),

packetIpv4Orv6EcrtpHc(12)}

WmanIf2MaxClassifiers ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type indicates the maximum number of

admitted Classifiers that the SS is allowed to have."

REFERENCE

"Subclause 11.7.7.2 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..65535)

WmanIf2PhsSupportType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The object of this type indicates the level

of PHS support."

REFERENCE

"Subclause 11.7.7.3 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noPhsSupport(0),

atmPhsSupport(1),

packetPhsSupport(2),

atmAndPacketPhsSupport(3)}

WmanIf2BwAllocSupport ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates properties of the SS that the BS

needs to know for bandwidth allocation purposes. When

a bit is set, it indicates the corresponding feature

is supported. All unspecified and reserved bits should

be set to zero."

REFERENCE

"Subclause 11.8.1 in IEEE Std 802.16-2004"

SYNTAX      BITS {reserved(0),

halfDuplexFdd(1),

fullDuplexFdd(2)}

WmanIf2PduConstruction ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Specifies capabilities for construction and transmission

of MAC PDUs. When piggybackedRequests bit is set, it

indicates that the piggybacked requests are supported. The

fsnValuesSize bit is coded as follows:

0 - only 3-bit FSN values are supported

1 - only 11-bit FSN values are supported

All unspecified and reserved bits should be set to zero."

REFERENCE

"Subclause 11.8.2 in IEEE Std 802.16e-2005"

SYNTAX      BITS {piggybackedRequests(0),

fsnValuesSize(1)}

WmanIf2SsTransitionGap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the transition speed SSTTG and SSRTG

for TDD and H-FDD SSs. Allowed values are:

OFDM mode: TDD and H-FDD 0..100

Other modes: TDD: 0..50; H-FDD: 0..100"

REFERENCE

"Subclause 11.8.3.1 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..100)

WmanIf2MaxTxPowerType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This type is used to define maximum available power for

BPSK, QPSK, 16-QAM and 64-QAM constellations. The maximum

power parameters are reported in dBm and quantized in 0.5

dBm steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

(encoded 0xFF). Values outside this range shall be

assigned the closest extreme. SSs that do not support

QAM64 shall report the value of 0x00 in the maximum QAM64

power field."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

SYNTAX      INTEGER (0..255)

WmanIf2OfdmFftSizes ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes supported by the SS.

For each FFT size, a bit value of 0 indicates 'not

supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.6.1 in IEEE 802.16-2004"

SYNTAX      BITS {fft256(0),

fft2048(1)}

WmanIf2OfdmSsDeModType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

supported by a WirelessMAN-OFDM PHY SS for downlink. This

field is not used for other PHY specifications. A bit

value of 0 indicates 'not supported' while 1 indicates

'supported'."

REFERENCE

"Subclause 11.8.3.6.2 in IEEE Std 802.16e-2005"

SYNTAX      BITS {qam64(0),

btc(1),

ctc(2),

stc(3),

aas(4),

subchannelization(5)}

WmanIf2OfdmSsModType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

supported by a WirelessMAN-OFDM PHY SS for uplink. This

field is not used for other PHY specifications. A bit

value of 0 indicates 'not supported' while 1 indicates

'supported'."

REFERENCE

"Subclause 11.8.3.6.3 in IEEE Std 802.16e-2005"

SYNTAX      BITS {qam64(0),

btc(1),

ctc(2),

subchannelization(3),

focusedCtBwReq(4),

ulCyclicDelay(5)}

WmanIf2OfdmFocusedCt ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates whether the SS supports Focused

Contention (see 8.3.7.3.3). A bit value of 0 indicates

'not supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.6.4 in IEEE Std 802.16-2004"

SYNTAX      BITS {focusedCtSupport(0)}

WmanIf2OfdmTcSublayer ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates whether or not the SS supports the

TC sublayer (see 8.3.4). A bit value of 0 indicates

'not supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.6.5 in IEEE Std 802.16-2004"

SYNTAX      BITS {tcSublayerSupport(0)}

WmanIf2OfdmPrivMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates if the private map parameters are

supported. A bit value of 0 indicates 'not supported'

while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.6.6 in IEEE Std 802.16e-2005"

SYNTAX      BITS {regularMap(0),

compressedMap(1)}

WmanIf2OfdmUlPower ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the uplink power control options

supported by a WirelessMAN-OFDM PHY SS for uplink

transmission. A bit value of 0 indicates 'not supported'

while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.10 in IEEE Std 802.16e-2005"

SYNTAX      BITS {ulOpenLoopPwrCntl(0),

ulAasPreamblePwrCntl(1)}

WmanIf2BsIdType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the encoding of BSID. The BSID is a 6 byte number

and follows the encoding rules of MacAddress textual

convention, i.e. as if it were transmitted

least-significant bit first. The value should be displayed

with 2 parts clearly separated by a colon e.g:

001DFF:00003A. The most significant part is representing

the Operator ID. "

SYNTAX      OCTET STRING (SIZE(6))

WmanIf2Ipv6FlowLabel ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field. As the flow label field has a

length of 20 bits, the first 4 bits of the most significant

byte shall be set to 0x0 and disregarded."

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2OfdmaFftSizes ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes supported by the SS/MS.

For each FFT size, a bit value of 0 indicates 'not

supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.1 in IEEE 802.16e-2005"

SYNTAX      BITS {fft256(0),

fft2048(1),

fft128(2),

fft512(3),

fft1024(4)}

WmanIf2OfdmaMsDeModType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

supported by a WirelessMAN-OFDMA PHY SS for downlink.

A bit value of 0 indicates 'not supported' while 1

indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.2 in IEEE Std 802.16e-2005"

SYNTAX      BITS {qam64(0),

btc(1),

ctc(2),

stc(3),

ccWithInterleacer(4),

harqChase(5),

harqCtcIr(6),

reserved(7),

harqCcIr(8),

ldpc(9),

dedicatedPilots(10)}

WmanIf2OfdmaMsModType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

supported by a WirelessMAN-OFDMA PHY SS for uplink. A bit

value of 0 indicates 'not supported' while 1 indicates

'supported'."

REFERENCE

"Subclause 11.8.3.7.3 in IEEE Std 802.16e-2005"

SYNTAX      BITS {qam64(0),

btc(1),

ctc(2),

stc(3),

harqChase(4),

ctcIr(5),

ccIr(6),

ldpc(7)}

WmanIf2OfdmaPermutation ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the OFDMA SS Permutation support.

A bit value of 0 indicates 'not supported' while 1

indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.4 in IEEE 802.16e"

SYNTAX      BITS {optionalPuscSupport(0),

optionalFuscSupport(1),

amcOneBySixSupport(2),

amcTwoByThreeSupport(3),

amcThreeByTwoSupport(4),

amcSupportWithHarqMap(5),

tusc1Support(6),

tusc2Support(7)}

WmanIf2OfdmaDemMimo ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA MS

demodulator. A bit value of 0 indicates 'not supported'

while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

SYNTAX      BITS {twoAntStcMatrixA(0),

twoAntStcMatrixBVCoding(1),

twoAntStcMatrixBHCoding(2),

fourAntStcMatrixA(3),

fourAntStcMatrixBVCoding(4),

fourAntStcMatrixBHCoding(5),

fourAntStcMatrixCVCoding(6),

fourAntStcMatrixCHCodingt(7)}

WmanIf2OfdmaMimoCap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA MS

demodulator."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

SYNTAX      BITS {twoAntStcMatrixA(0),

twoAntStcMatrixBVCoding(1),

fourRxAntenna(2),

fourAntStcMatrixA(3),

fourAntStcMatrixBVCoding(4),

fourAntStcMatrixBHCoding(5),

fourAntStcMatrixCVCoding(6),

fourAntStcMatrixCHCodingt(7),

threeAntStcMatrixA(8),

threeAntStcMatrixB(9),

threeAntStcMatrixCVCoding(10),

threeAntStcMatrixCHCodingt(11),

calculatingPrecodingWeight(12),

adaptiveRateControl(13),

calculatingChannelMatrix(14),

antennaGrouping(15),

antennaSelection(16),

codebookBasedPrecoding(17),

longTermPrecoding(18),

mimoMidamble(19)}

WmanIf2OfdmaUlMimo ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the different MIMO options supported

by a WirelessMAN-OFDMA PHY SS in the uplink. A bit value of

0 indicates 'not supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.6 in IEEE 802.16e"

SYNTAX      BITS {twoAntSttd(0),

twoAntSmVCoding(1),

oneAntCooperativeSm(2)}

WmanIf2OfdmaPrivMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the AAS private map parameters

supported by a WirelessMAN-OFDMA SS. A bit value of 0

indicates 'not supported' while 1 indicates 'supported' for

most bits, except chainConcurrency0, chainConcurrency1 that

indicates how many parallel private map chains can be

supported by an SS.

0:    no limit

1..3: maximum concurrent private map chains"

REFERENCE

"Subclause 11.8.3.7.7 in IEEE Std 802.16e-2005"

SYNTAX      BITS {harqMap(0),

privMap(1),

reducedPrivMap(2),

privMapChainEnable(3),

privMapDlFrameOffset(4),

privMapUlFrameOffset(5),

chainConcurrency0(6),

chainConcurrency1(7)}

WmanIf2OfdmaAasCap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the different AAS options supported by

a WirelessMAN-OFDMA PHY SS in the downlink. A bit value of

0 indicates 'not supported' while 1 indicates 'supported'

for most bits."

REFERENCE

"Subclause 11.8.3.7.8 in IEEE Std 802.16e-2005"

SYNTAX      BITS {aasZone(0),

aasDiversityMapScan(1),

aasFbckRsp(2),

dlAasPreamble(3),

ulAasPreamble(4)}

WmanIf2OfdmaCinrCap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the CINR measurement capability

supported by a WirelessMAN-OFDMA PHY SS in the downlink.

A bit value of 0 indicates 'not supported' while 1

indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.9 in IEEE Std 802.16e-2005"

SYNTAX      BITS {phyCinrPreamble(0),

phyCinrPilotSubc(1),

phyCinrDataSubc(2),

effectiveCinrPreamble(3),

effectiveCinrPilotSubc(4),

effectiveCinrDataSubc(5),

twoCqiChannel(6),

freqSelectivityReport(7)}

WmanIf2OfdmaUlPower ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the power control options supported by

a WirelessMAN-OFDMA PHY SS for uplink transmission. A bit

value of 0 indicates 'not supported' while 1 indicates

'supported'."

REFERENCE

"Subclause 11.8.3.7.11 in IEEE Std 802.16e-2005"

SYNTAX      BITS {ulOpenLoopPwrCntl(0),

ulAasPreamblePwrCntl(1)}

WmanIf2OfdmaMapCap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the different MAP options supported

by a WirelessMAN-OFDMA PHY SS. A bit value of 0 indicates

'not supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.12 in IEEE Std 802.16e-2005"

SYNTAX      BITS {harqMap(0),

extendedHarqIe(1),

subMapFor1stZone(2),

subMapForOtherZone(3),

dlRegionDefinition(4)}

WmanIf2OfdmaUlCntlCh ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates different uplink control channels

supported by a WirelessMAN-OFDMA PHY SS. A bit value of 0

indicates 'not supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.8.3.7.13 in IEEE Std 802.16e-2005"

SYNTAX      BITS {threeBitMimoFastFeedback(0),

enhancedFastFeedback(1),

ulAck(2),

reserved(3),

uepFastFeedback(4),

fastDlMeasurementFeedback(5),

priSecFastFeedback(6),

diucCqiFastFeedback(7)}

WmanIf2OfdmaMsCistCap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates MS capability of supporting CSIT

(uplink sounding). A bit value of 0 indicates 'not

supported' while 1 indicates 'supported'.

Bits 3..5: Time needed for SS to respond to a sounding

command transmitted by the BS

000   0.5ms

001   0.75ms

010   1ms

011   1.25ms

100   1.5ms

101   min(2ms, Next Frame)

110   min(5ms, Next Frame)

111   Next Frame

Bits 6..9: Max number of simultaneous sounding

instructions (0 = unlimited)"

REFERENCE

"Subclause 11.8.3.7.14 in IEEE Std 802.16e-2005"

SYNTAX      BITS {csitTypeA(0),

csitTypeB(1),

powerAssignment(2),

soundingRspTime0(3),

soundingRspTime1(4),

soundingRspTime2(5),

maxSimuSoundInst0(6),

maxSimuSoundInst1(7),

maxSimuSoundInst2(8),

maxSimuSoundInst3(9),

noP9Or18ForCsitTypeA(10),

csitNotSupported(11)}

WmanIf2OfdmaMaxHarq ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the maximum number of UL/DL HARQ burst

allocations for the SS in a single UL/DL subframe.

Bits 0..2: Maximum number of UL HARQ bursts per HARQ

enabled MS per frame

0b000 = 1 (default)

Bit     3: Indicates whether the maximum number of UL HARQ

bursts per frame in bits 0-2 includes the one

Non-HARQ burst.

0 = not included (default)

1 = included

Bits 4..7: Maximum number of DL HARQ bursts per HARQ

enabled MS per frame.

0b0000 = 1 (default)"

REFERENCE

"Subclause 11.8.3.7.15 in IEEE Std 802.16e-2005"

SYNTAX      BITS {maxUlHarqBurst0(0),

maxUlHarqBurst1(1),

maxUlHarqBurst2(2),

nonHarqBurstInUl(3),

maxDlHarqBurst0(4),

maxDlHarqBurst1(5),

maxDlHarqBurst2(6),

maxDlHarqBurst3(7)}

WmanIf2OfdmaModMimo ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA SS

modulator. A bit value of 0 indicates 'not supported'

while 1 indicates 'supported'"

REFERENCE

"Subclause 11.8.3.7.16 in IEEE Std 802.16e-2005"

SYNTAX      BITS {twoTxAntenna(0),

txDiversity(1),

spatialMultiplexing(2),

beamforming(3),

adaptiveRateControl(4),

singleAntenna(5),

twoAntenna(6)}

WmanIf2SdmaPilotCap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates SDMA pilot pattern support for AMC

zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noSupport(0),

sdmaPilotAandB(1),

allSdmaPilotPatterns(2)}

WmanIf2MultiBurst ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates whether multiple FEC types are

supported in DL/UL burst profiles. A bit value of 0

indicates 'not supported' while 1 indicates

'supported'"

REFERENCE

"Subclause 11.8.3.7.18 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {dlWithMultiFecType(0),

ulWithMultiFecType(1)}

WmanIf2IncrHarqBuf ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data bits the SS

is able to use for buffering for NEP/NSCH based incremental

redundancy CTC in downlink and uplink transmissions.

Bits 0..3: NEP value indicating downlink HARQ buffering

capability for incremental redundancy CTC

Bit     4: Aggregation Flag for DL

0 = the number of bits is counted separately for

each channel

1 = buffering capability may be shared between

channels

Bits 5..7: reserved

Bits 8..11: NEP value indicating uplink HARQ buffering

capability for incremental redundancy CTC

Bit    12: Aggregation Flag for UL

0 = the number of bits is counted separately for

each channel

1 = buffering capability may be shared between

channels"

REFERENCE

"Subclause 11.8.3.7.19 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {dlNep0(0),

dlNep1(1),

dlNep2(2),

dlNep3(3),

dlAggFlag(4),

reserved0(5),

reserved1(6),

reserved2(7),

ulNep0(8),

ulNep1(9),

ulNep2(10),

ulNep3(11),

ulAggFlag(12)}

WmanIf2ChaseHarqBuf ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data bits the SS

is able to use for buffering for DIUC/duration based HARQ

methods (Chase combining and CC-IR)in downlink and uplink

transmissions.

Bits 0..5: Downlink HARQ buffering capability for

chase combining (K)

Bit     6: Aggregation Flag for DL

0 = the number of bits is counted separately for

each channel

1 = buffering capability may be shared between

channels

Bits    7: reserved

Bits 8..13: Uplink HARQ buffering capability for chase

combining (K)

Bit    14: Aggregation Flag for UL

0 = the number of bits is counted separately for

each channel

1 = buffering capability may be shared between

channels"

REFERENCE

"Subclause 11.8.3.7.19 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {dlChaseComb0(0),

dlChaseComb1(1),

dlChaseComb2(2),

dlChaseComb3(3),

dlChaseComb4(4),

dlChaseComb5(5),

dlAggFlag(6),

reserved(7),

ulChaseComb0(8),

ulChaseComb1(9),

ulChaseComb2(10),

ulChaseComb3(11),

ulChaseComb4(12),

ulChaseComb5(13),

ulAggFlag(14)}

WmanIf2PkmVersion ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates a PKM version that SS or BS supports.

A bit 0 - 'not supported'

1 - 'supported'

Both an SS and a BS should negotiate only one PKM version."

REFERENCE

"Subclause 11.8.4.1 in IEEE Std 802.16e-2005"

SYNTAX      BITS {pkmVersion1(0),

pkmVersion2(1)}

WmanIf2AuthPolicy ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates authorization policy used by the MS and

BS to negotiate and synchronize.

A bit 0 - 'not supported'

1 - 'supported'"

REFERENCE

"Subclause 11.8.4.2 in IEEE Std 802.16e-2005"

SYNTAX      BITS {rsaBasedAuthInitNtwkEntry(0),

eapBasedAuthInitNtwkEntry(1),

authenEapBasedAuthInitNtwkEntry(2),

reserved0(3),

rsaBasedAuthReentry(4),

eapBasedAuthReentry(5),

authenEapBasedAuthReentry(6),

reserved1(7)}

WmanIf2MacMode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates a MAC (Message Authentication Code)

mode that MS supports. Both MS and BS shall determine and

use a MAC mode.

A bit 0 - 'not supported'

1 - 'supported'"

REFERENCE

"Subclause 11.8.4.3 in IEEE Std 802.16e-2005"

SYNTAX      BITS {hmac(0),

cmac(1),

sixtyfourBitShortHmaca(2),

eightyBitShortHmaca(3),

nintysixBitShortHmaca(4)}

WmanIf2ExtCapability ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the extended subheader format support.

A bit 0 - 'not supported'

1 - 'supported'"

REFERENCE

"Subclause 11.8.6 in IEEE Std 802.16e-2005"

SYNTAX      BITS {extSubheader(0)}

WmanIf2PackingSupport ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Packing"

REFERENCE

"Subclause 11.7.8.11 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noPackingSupport(0),

packingSupported(1)}

WmanIf2ExtRtpsSupport ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Extended

rtPS."

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noExtendedRtpsSupport(0),

extendedRtpsSupported(1)}

WmanIf2IpAllocMethod ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates the method of allocating IP address for the

secondary management connection. A bit value of 0

indicates 'not supported' while 1 indicates 'supported'."

REFERENCE

"Subclause 11.7.11 in IEEE Std 802.16e-2005"

SYNTAX      BITS {dhcp(0),

mobileIpv4(1),

dhcpV6(2),

ipv6Autoconfig(3)}

WmanIf2ArqAckType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Specifies the ARQ ACK type supported by the MS."

REFERENCE

"Subclause 11.7.23 in IEEE Std 802.16e-2005"

SYNTAX      BITS {selectiveAck(0),

cumulativeAck(1),

cumWithSelAck(2),

cumWithBlockSeqAck(3)}

WmanIf2MacHeaderSupp ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates whether or not the MS and BS support various

types of MAC header and extended subheaders. A bit value of

0 indicates 'not supported', while 1 indicates 'supported'.

Bits 8-10: parameters of SDU_SN extended subheader that

represent the period of SDU_SN transmission for

connection with ARQ disabled = once every 2^p MAC

PDUs."

REFERENCE

"Subclause 11.7.25 in IEEE Std 802.16e-2005"

SYNTAX      BITS {bwReqUlTxPowerReport(0),

bwReqCinrReport(1),

cqichAlloationReq(2),

phyChannelReport(3),

bwReqUlSleepCntl(4),

snReport(5),

feedbackReport(6),

sduSn(7),

sdnSnPeriod0(8),

sdnSnPeriod1(9),

sdnSnPeriod2(10),

dlSleepControl(11),

feedbackRequest(12),

mimcModeFeedback(13),

ulTxPowerReport(14),

miniFeedback(15),

snRequest(16),

shortPduSn(17),

longPduSn(18)}

WmanIf2HarqAckDelay ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"HARQ ACK delay for UL and DL bursts

1 = one frame offset

2 = two frames offset

3 = three frames offset"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {oneframeoffset(1),

twoframesoffset(2),

threeframesoffset(3)}

WmanIf2AasBeamSel ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Boolean to indicate whether unsolicited AAS Beam Select

messages (see 6.3.2.3.41 in IEEE 802.16e-2005) should be

sent by the MS.

0: MS should not send AAS Beam Select Messages

1: MS may send AAS Beam Select Messages"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {notAllowed(0),

allowed(1)}

WmanIf2MaxMacLevel ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"maximum amount of MAC level data including MAC headers and

HARQ retransmission bursts the MS is capable of processing

in the DL/UL part of a single MAC frame."

REFERENCE

"Subclause 11.7.8.10 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER (0..65535)

WmanIfPermutationType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Permutation type for broadcast region in HARQ zone"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {pusc(1),

fusc(2),

optionalFusc(3),

amc(4)}

WmanIf2HoSupportType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The types of handover supported."

REFERENCE

"Table 358 in IEEE Std 802.16e-2005"

SYNTAX      BITS {handover(0),

mdHandover(1),

fbssHandover(2)}

WmanIf2ActionRule ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule

Bit #0: 0 = none.

1 = Discard packet"

REFERENCE

"Subclause 11.13.19.3.4.17 in IEEE Std 802.16e-2005"

SYNTAX      BITS {discardPacket(0)}

WmanIf2LinkDirection ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The direction of a connection."

SYNTAX      INTEGER {downstream(1),

upstream(2)}

WmanIf2SaServiceType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This attribute indicates service types of the corresponding

SA type. This attribute shall be defined only when the SA

type is Static SA or Dynamic SA. The GTEK shall be used to

encrypt connection for group multicast service. The MTK

shall be used to encrypt connection for MBS service."

REFERENCE

"Subclause 11.9.35 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {unicastService(0),

groupMulticastService(1),

mbsService(2)}

WmanIf2GlobalSrvClass ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Global Service Class Name contains 8 information fields

that map to predefined QoS attributes as shown in

subclause 6.3.14.4.1.

bit#0:     Uplink/Downlink indicator

0 - uplink

1 - downlink

bit#1-6:   Maximum sustained traffic rate in bps that is

defined in Table 124b

bit#7:     0 - no traffic indication

1 - traffic indication

bit#8-13:  Maximum traffic burst defines the maximum burst

size that must be accommodated for the service.

bit#14-19: Minimum reserved traffic rate parameter

specifies the minimum rate, in bits per second,

reserved for this service flow.

bit#20-25: Maximum latency specifies the maximum interval

between the reception of a packet at CS of BS

or SS and the arrival of the packet to the peer

device.

bit#26:    SDU indicator specifies whether the SDUs on the

service flow are fixed-length or variable-length.

0 - variable length

1 - fixed length

bit#27:    Paging indicator of an MS preference for the

reception of paging advisory messages during

idle mode. When set, it indicates that the BS

may present paging advisory messages or other

indicative messages to the MS when data SDUs

bound for the MS are present while the MS is in

Idle Mode.

0 - no paging generation

1 - paging generation"

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

SYNTAX      BITS {ulDlIndicator(0),

maxSustainedRate0(1),

maxSustainedRate1(2),

maxSustainedRate2(3),

maxSustainedRate3(4),

maxSustainedRate4(5),

maxSustainedRate5(6),

trafficIndication(7),

maxTrafficBurst0(8),

maxTrafficBurst1(9),

maxTrafficBurst2(10),

maxTrafficBurst3(11),

maxTrafficBurst4(12),

maxTrafficBurst5(13),

minReservedRate0(14),

minReservedRate1(15),

minReservedRate2(16),

minReservedRate3(17),

minReservedRate4(18),

minReservedRate5(19),

maxLatency0(20),

maxLatency1(21),

maxLatency2(22),

maxLatency3(23),

maxLatency4(24),

maxLatency5(25),

sduIndicator(26),

pagingGeneration(27),

reserved0(28),

reserved1(29),

reserved2(30),

reserved3(31)}

WmanIf2OfdmaFftSize ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"FFT size for OFDMA PHY

0b000 = 2048

0b001 = 1024

0b010 = 512

0b100 = 128"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX INTEGER {fft2048(0),

fft1024(1),

fft512(2),

reserved(3),

fft128(4)}

WmanIf2OfdmFftSize ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"FFT size for OFDM PHY

0b000: 2048

0b011: 256"

REFERENCE

"Subclause 11.8.3.6.1, IEEE Std 802.16-2004"

SYNTAX INTEGER {fft2048(0),

fft256(3)}

-- XXX

WmanIf2BsSsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"xxxReq: Indicates that SS reports its capabilities in

RNG-REQ, SBC-REQ, or REG-REQ messages.

xxxRsp: Indicates that BS confirms SS capabilities in

RNG-RSP, SBC-RSP, or REG-RSP messages."

SYNTAX INTEGER {xxxReq (1),

xxxRsp (2)}

WmanIf2BsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"inheritedBsCap: Indicates BS' basic HW and SW capabilities.

configedBsCap: Indicates BS' capabilities that has been

configured."

SYNTAX      INTEGER {inheritedBsCap (1),

configedBsCap (2)}

--

-- wmanIf2BsFm group – containing tables and objects related to Fault

--                     Management (i.e. BS SNMP Trap objects)

--

wmanIf2BsTrapControl     OBJECT IDENTIFIER ::= {wmanIf2BsFm 1}

wmanIf2BsTrapDefinitions OBJECT IDENTIFIER ::= {wmanIf2BsFm 2}

-- This object groups all NOTIFICATION-TYPE objects for BS.

-- It is defined following RFC2758 sections 8.5 and 8.6

-- for the compatibility with SNMPv1.

wmanIf2BsTrapPrefix OBJECT IDENTIFIER ::= { wmanIf2BsTrapDefinitions 0 }

wmanIf2BsTrapControlRegister    OBJECT-TYPE

SYNTAX      BITS {wmanIf2BsSsStatusNotification (0),

wmanIf2BsSsDynamicServiceFail (1),

wmanIf2BsSsRssiStatusChange   (2),

wmanIf2BsSsRegister           (3),

wmanIf2BsSsPkmFail            (4),

wmanIf2BsPerformanceCounters  (5)}

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The object is used to enable or disable Base Station traps.

From left to right, the set bit indicates the corresponding

Base Station trap is enabled."

::= { wmanIf2BsTrapControl 1 }

wmanIf2BsStatusTrapControlRegister    OBJECT-TYPE

SYNTAX      BITS {unused(0),

ssInitRangingSucc(1),

ssInitRangingFail(2),

ssRegistered(3),

ssRegistrationFail(4),

ssDeregistered(5),

ssBasicCapabilitySucc(6),

ssBasicCapabilityFail(7),

ssAuthorizationSucc(8),

ssAuthorizationFail(9),

tftpSucc(10),

tftpFail(11),

sfCreationSucc(12),

sfCreationFail(13)}

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The object is used to enable or disable Base Station status

notification traps. The set bit indicates the corresponding

Base Station trap is enabled."

::= { wmanIf2BsTrapControl 2 }

--

-- BS threshold Definitions

--

wmanIf2BsThresholdConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains threshold objects that can be set

to detect the threshold crossing events."

::= { wmanIf2BsTrapControl 3 }

wmanIf2BsThresholdConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2BsThresholdConfigTable 1 }

WmanIf2BsThresholdConfigEntry ::= SEQUENCE {

wmanIf2BsRssiLowThreshold               Integer32,

wmanIf2BsRssiHighThreshold              Integer32}

wmanIf2BsRssiLowThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Low threshold for generating the RSSI alarm."

::= { wmanIf2BsThresholdConfigEntry 1 }

wmanIf2BsRssiHighThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"High threshold for clearing the RSSI alarm."

::= { wmanIf2BsThresholdConfigEntry 2 }

--

-- Subscriber station Notification Objects Definitions

--

wmanIf2BsSsNotificationObjectsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains SS notification objects that have been

reported by the trap."

::= { wmanIf2BsTrapDefinitions 1 }

wmanIf2BsSsNotificationObjectsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has

generated traps."

INDEX     { ifIndex, wmanIf2BsSsNotificationMacAddr }

::= { wmanIf2BsSsNotificationObjectsTable 1 }

WmanIf2BsSsNotificationObjectsEntry ::= SEQUENCE {

wmanIf2BsSsNotificationMacAddr          MacAddress,

wmanIf2BsSsStatusValue                  INTEGER,

wmanIf2BsSsStatusInfo                   OCTET STRING,

wmanIf2BsDynamicServiceType             INTEGER,

wmanIf2BsDynamicServiceFailReason       OCTET STRING,

wmanIf2BsSsRssiStatus                   INTEGER,

wmanIf2BsSsRssiStatusInfo               OCTET STRING,

wmanIf2BsSsRegisterStatus               INTEGER,

wmanIf2BsDynamicServiceFailSfid         Unsigned32,

wmanIf2BsEventNotificationTime          TimeStamp}

wmanIf2BsSsNotificationMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The MAC address of the SS, reporting the notification."

::= { wmanIf2BsSsNotificationObjectsEntry 1 }

wmanIf2BsSsStatusValue  OBJECT-TYPE

SYNTAX      INTEGER {ssInitRangingSucc(1),

ssInitRangingFail(2),

ssRegistered(3),

ssRegistrationFail(4),

ssDeregistered(5),

ssBasicCapabilitySucc(6),

ssBasicCapabilityFail(7),

ssAuthorizationSucc(8),

ssAuthorizationFail(9),

tftpSucc(10),

tftpFail(11),

sfCreationSucc(12),

sfCreationFail(13)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the status of a SS, as it goes

through network entry and initialization procedure."

::= { wmanIf2BsSsNotificationObjectsEntry 2 }

wmanIf2BsSsStatusInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the reason of SS's status change."

::= { wmanIf2BsSsNotificationObjectsEntry 3 }

wmanIf2BsDynamicServiceType  OBJECT-TYPE

SYNTAX      INTEGER {bsSfCreationReq(1),

bsSfCreationRsp(2),

bsSfCreationAck(3)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the dynamic service flow

creation command type."

::= { wmanIf2BsSsNotificationObjectsEntry 4 }

wmanIf2BsDynamicServiceFailReason  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the reason why the service flow

creation has failed."

::= { wmanIf2BsSsNotificationObjectsEntry 5 }

wmanIf2BsSsRssiStatus  OBJECT-TYPE

SYNTAX      INTEGER {bsRssiAlarm(1),

bsRssiNoAlarm(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A RSSI alarm is generated when RSSI becomes lower than

wmanIf2BsLowRssiThreshold and is cleared when RSSI becomes

higher than wmanIf2BsLowRssiThreshold."

::= { wmanIf2BsSsNotificationObjectsEntry 6 }

wmanIf2BsSsRssiStatusInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the reason why RSSI alarm is

generated."

::= { wmanIf2BsSsNotificationObjectsEntry 7 }

wmanIf2BsSsRegisterStatus OBJECT-TYPE

SYNTAX      INTEGER {ssRegister(1),

ssDeregister(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the status of SS registration."

::= { wmanIf2BsSsNotificationObjectsEntry 8 }

wmanIf2BsDynamicServiceFailSfid OBJECT-TYPE

SYNTAX      Unsigned32 (1..4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object identifies the dynamic service flow

for notification purposes."

::= { wmanIf2BsSsNotificationObjectsEntry 9 }

wmanIf2BsEventNotificationTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the event notification

trap was generated."

::= { wmanIf2BsSsNotificationObjectsEntry 10 }

--

-- Subscriber station Notification Trap Definitions

--

wmanIf2BsSsStatusNotificationTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2BsSsNotificationMacAddr,

wmanIf2BsSsStatusValue,

wmanIf2BsSsStatusInfo}

STATUS      current

DESCRIPTION

"This trap reports the status of a SS. Based on this

notification the NMS will issue an alarm with certain

severity depending on the status and the reason received."

::= { wmanIf2BsTrapPrefix 1 }

wmanIf2BsSsRssiStatusChangeTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2BsSsNotificationMacAddr,

wmanIf2BsSsRssiStatus,

wmanIf2BsSsRssiStatusInfo}

STATUS      current

DESCRIPTION

"An event to report that the uplink RSSI is below

wmanIf2BsLowRssiThreshold, or above

wmanIf2BsHighRssiThreshold after restore."

::= { wmanIf2BsTrapPrefix 2 }

wmanIf2BsSsPkmFailTrap NOTIFICATION-TYPE

OBJECTS     {wmanIf2BsSsNotificationMacAddr}

STATUS      current

DESCRIPTION

"An event to report the failure of a Pkm operation."

::= { wmanIf2BsTrapPrefix 3 }

wmanIf2BsSsDynamicServiceFailTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2BsSsNotificationMacAddr,

wmanIf2BsDynamicServiceType,

wmanIf2BsDynamicServiceFailReason,

wmanIf2BsDynamicServiceFailSfid}

STATUS      current

DESCRIPTION

"An event reporting failure of DSx operation for a service

flow identified by wmanIf2BsDynamicServiceFailSfid and

detected in the Bs side."

::= { wmanIf2BsTrapPrefix 4 }

wmanIf2BsSsRegisterTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2BsSsNotificationMacAddr,

wmanIf2BsSsRegisterStatus}

STATUS      current

DESCRIPTION

"An event to report SS registration status for a given sector

identified by ifIndex."

::= { wmanIf2BsTrapPrefix 5 }

-- XXXX

wmanIf2BsPerformanceCountersTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2BsSsAuthenAttempt,

wmanIf2BsSsAuthenSuccess,

wmanIf2BsSsAuthenSuccessRate,

wmanIf2BsSsRangingAttempt,

wmanIf2BsSsRangingSuccess,

wmanIf2BsSsRangingSuccessRate,

wmanIf2BsAvgDlUserThroughput,

wmanIf2BsAvgUlUserThroughput,

wmanIf2BsAvgDlMacThroughput,

wmanIf2BsAvgUlMacThroughput,

wmanIf2BsAvgDlPhyThroughput,

wmanIf2BsAvgUlPhyThroughput,

wmanIf2BsPeakDlUserThroughput,

wmanIf2BsPeakUlUserThroughput,

wmanIf2BsPeakDlMacThroughput,

wmanIf2BsPeakUlMacThroughput,

wmanIf2BsPeakDlPhyThroughput,

wmanIf2BsPeakUlPhyThroughput,

wmanIf2BsAvgDlCellEdgeThroughput,

wmanIf2BsAvgUlCellEdgeThroughput,

wmanIf2BsDataRateMeasurements,

wmanIf2BsAvgNetworkEntryTime,

wmanIf2BsMaxNetworkEntryTime,

wmanIf2BsAvgNetworkReEntryTime,

wmanIf2BsMaxNetworkReEntryTime,

wmanIf2BsNumOfNetworkEntries,

wmanIf2BsNumOfNetworkReEntries,

wmanIf2BsDlPacketsSend,

wmanIf2BsDlPacketsErrored,

wmanIf2BsDlPacketErrorRate,

wmanIf2BsEventNotificationTime}

STATUS      current

DESCRIPTION

"This event should be automatically generated every

wmanIf2BsCountersReportInterval interval to report the

performance counters for a given BS sector identified by

ifIndex. The performance counters will be reset after this

event being sent to EMS."

::= { wmanIf2BsTrapPrefix 6 }

--

-- wmanIf2BsCm group – containing tables and objects related to

--                     Configuration Management

--

wmanIf2BsRegisteredSsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsRegisteredSsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information

of SSs that have been negotiated and agreed between

BS and SS via REG-REQ and REG-RSP messages. An entry

in this table indicates the SS has entered and registered

into the BS."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsCm 1 }

wmanIf2BsRegisteredSsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsRegisteredSsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. The primary index is the ifIndex

with an ifType of ieee80216WMAN, indicating the BS sector

with which the SS is associated. wmanIf2BsSsMacAddress

identifies the SS being registered."

INDEX     { ifIndex, wmanIf2BsSsMacAddress }

::= { wmanIf2BsRegisteredSsTable 1 }

WmanIf2BsRegisteredSsEntry ::= SEQUENCE {

wmanIf2BsSsMacAddress                   MacAddress,

wmanIf2BsSsBasicCid                     WmanIf2CidType,

wmanIf2BsSsPrimaryCid                   WmanIf2CidType,

wmanIf2BsSsSecondaryCid                 WmanIf2CidType,

wmanIf2BsSsManagementSupport            INTEGER,

wmanIf2BsSsIpManagementMode             INTEGER,

wmanIf2BsSs2ndMgmtArqEnable             TruthValue,

wmanIf2BsSs2ndMgmtArqWindowSize         INTEGER,

wmanIf2BsSs2ndMgmtArqDnLinkTxDelay      INTEGER,

wmanIf2BsSs2ndMgmtArqUpLinkTxDelay      INTEGER,

wmanIf2BsSs2ndMgmtArqDnLinkRxDelay      INTEGER,

wmanIf2BsSs2ndMgmtArqUpLinkRxDelay      INTEGER,

wmanIf2BsSs2ndMgmtArqBlockLifetime      INTEGER,

wmanIf2BsSs2ndMgmtArqSyncLossTimeout    INTEGER,

wmanIf2BsSs2ndMgmtArqDeliverInOrder     TruthValue,

wmanIf2BsSs2ndMgmtArqRxPurgeTimeout     INTEGER,

wmanIf2BsSs2ndMgmtArqBlockSize          INTEGER,

wmanIf2BsSsVendorIdEncoding             OCTET STRING,

wmanIf2BsSsAasBroadcastPermission       INTEGER,

wmanIf2BsSsMaxTxPowerBpsk               WmanIf2MaxTxPowerType,

wmanIf2BsSsMaxTxPowerQpsk               WmanIf2MaxTxPowerType,

wmanIf2BsSsMaxTxPower16Qam              WmanIf2MaxTxPowerType,

wmanIf2BsSsMaxTxPower64Qam              WmanIf2MaxTxPowerType,

wmanIf2BsSsMacVersion                   WmanIf2MacVersion}

wmanIf2BsSsMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The MAC address of SS is received from the RNG-REQ

message. When SS registers, this MAC address is entered

into the table, and used as the identifier to the SS."

REFERENCE

"Subclause 6.3.2.3.5 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 1 }

wmanIf2BsSsBasicCid OBJECT-TYPE

SYNTAX      WmanIf2CidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object indicates the SS's basic CID

that was sent in the RNG-RSP message."

REFERENCE

"Subclause 6.3.2.3.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 2 }

wmanIf2BsSsPrimaryCid OBJECT-TYPE

SYNTAX      WmanIf2CidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object indicates the primary CID of the

SS received from the RNG-RSP message."

REFERENCE

"Subclause 6.3.2.3.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 3 }

wmanIf2BsSsSecondaryCid OBJECT-TYPE

SYNTAX      WmanIf2CidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object indicates the secondary

management CID present in the REG-RSP message. The value

should be null if the 2nd management connection is not

available."

REFERENCE

"Subclause 6.4.2.3.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 4 }

wmanIf2BsSsManagementSupport OBJECT-TYPE

SYNTAX      INTEGER {unmanagedSs(0),

managedSs(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether or not the SS is managed."

REFERENCE

"Subclause 11.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 5 }

wmanIf2BsSsIpManagementMode OBJECT-TYPE

SYNTAX      INTEGER {unmanaged(0),

ipManaged(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The IP management mode parameter dictates whether

the provider intends to manage the SS on an ongoing

basis via IP-based mechanisms."

REFERENCE

"Subclause 11.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 6 }

wmanIf2BsSs2ndMgmtArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the 2nd

management channel."

REFERENCE

"Subclause 11.13.18.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 7 }

wmanIf2BsSs2ndMgmtArqWindowSize	 OBJECT-TYPE

SYNTAX      INTEGER (1 .. 1024)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged

fragments at any time for 2nd management connection."

REFERENCE

"Subclause 11.13.18.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 8 }

wmanIf2BsSs2ndMgmtArqDnLinkTxDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object defines the ARQ transmitter delay for

downlink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 9 }

wmanIf2BsSs2ndMgmtArqUpLinkTxDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object defines the ARQ transmitter delay for

uplink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 10 }

wmanIf2BsSs2ndMgmtArqDnLinkRxDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object defines the ARQ receiver delay for

downlink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 11 }

wmanIf2BsSs2ndMgmtArqUpLinkRxDelay OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The object defines the ARQ receiver delay for

uplink transmission."

REFERENCE

"Subclause 11.13.18.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 12 }

wmanIf2BsSs2ndMgmtArqBlockLifetime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be

managed by the transmitter ARQ machine, once

initial transmission of the fragment has occured.

If transmission or retransmission of the fragment

is not acknowledged by the receiver before the

time limit is reached, the fragment is discarded.

A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.4 in IEEE Std 802.16-2004"

DEFVAL		{0}

::= { wmanIf2BsRegisteredSsEntry 13 }

wmanIf2BsSs2ndMgmtArqSyncLossTimeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss

of synchronization of the sender and receiver

state machines. A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.5 in IEEE Std 802.16-2004"

DEFVAL	    {0}

::= { wmanIf2BsRegisteredSsEntry 14 }

wmanIf2BsSs2ndMgmtArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered

by the receiving MAC to its client application

in the order in which data was handed off to the

originating MAC."

REFERENCE

"Subclause 11.13.18.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 15 }

wmanIf2BsSs2ndMgmtArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18.7 in IEEE Std 802.16-2004"

DEFVAL	    {0}

::= { wmanIf2BsRegisteredSsEntry 16 }

wmanIf2BsSs2ndMgmtArqBlockSize OBJECT-TYPE

SYNTAX      INTEGER (1 .. 2040)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the size of a ARQ block. This

parameter shall be established by negotiation during the

connection setup. The requester includes its desired

setting in the REQ message. The receiver of the REQ

message shall take the smaller of the value it prefers and

value in the REQ message. The minimum value is included in

the RSP message."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 17 }

wmanIf2BsSsVendorIdEncoding OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(3))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value field contains the vendor identification

specified by the 3 byte vendor-specific organizationally

unique identifier of the SS or BS MAC address.A vendor ID

used in a REG-REQ shall be the Vendor ID of the SS sending

the request. A vendor ID used in a REG-RSP shall be the

Vendor ID of the BS sending the response."

REFERENCE

"Subclause 11.1.5 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 18 }

wmanIf2BsSsAasBroadcastPermission OBJECT-TYPE

SYNTAX	    INTEGER {contBasedBwReqPermitted(0),

contBasedBwReqNotPermitted(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies if SS can issue contention-based

bandwidth request or not."

REFERENCE

"Subclause 11.6 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 19 }

wmanIf2BsSsMaxTxPowerBpsk OBJECT-TYPE

SYNTAX      WmanIf2MaxTxPowerType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum available power for BPSK. The maximum power

parameters are reported in dBm and quantized in 0.5 dBm

steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

(encoded 0xFF). Values outside this range shall be assigned

the closest extreme. This parameter is only applicable to

systems supporting the SCa, OFDM or OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 20 }

wmanIf2BsSsMaxTxPowerQpsk OBJECT-TYPE

SYNTAX      WmanIf2MaxTxPowerType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum available power for QPSK. The maximum power

parameters are reported in dBm and quantized in 0.5 dBm

steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm

(encoded 0xFF). Values outside this range shall be assigned

to closest extreme. This parameter is only applicable to

systems supporting the SCa, OFDM or OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 21 }

wmanIf2BsSsMaxTxPower16Qam OBJECT-TYPE

SYNTAX      WmanIf2MaxTxPowerType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum available power for 16-QAM constellations.

The maximum power parameters are reported in dBm and

quantized in 0.5 dBm steps ranging from -64 dBm (encoded

0x00) to 63.5 dBm (encoded 0xFF). Values outside this

range shall be assigned the closest extreme. This parameter

is only applicable to systems supporting the SCa, OFDM or

OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 22 }

wmanIf2BsSsMaxTxPower64Qam OBJECT-TYPE

SYNTAX      WmanIf2MaxTxPowerType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum available power for 64-QAM constellations.

The maximum power parameters are reported in dBm and

quantized in 0.5 dBm steps ranging from -64 dBm (encoded

0x00) to 63.5 dBm (encoded 0xFF). Values outside this

range shall be assigned the closest extreme. SSs that do

not support QAM64 shall report the value of 0x00. This

parameter is only applicable to systems supporting the SCa,

OFDM or OFDMA PHY."

REFERENCE

"Subclause 11.8.3.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 23 }

wmanIf2BsSsMacVersion OBJECT-TYPE

SYNTAX      WmanIf2MacVersion

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the version of 802.16 to which the

message originator conforms."

REFERENCE

"Subclause 11.1.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsRegisteredSsEntry 24 }

--

-- wmanIf2BsConfigurationTable contains global parameters common in BS

--

wmanIf2BsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector that

contains the BS system parameters as defined in Subclause

10.1 of [3]. The objects in this table define the default

behaviour of the BS for 2nd Management connection

scheduling and SFID allocation as well as configuration

parameters of the CPS scheduler and AAS system."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCm 2 }

wmanIf2BsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2BsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2BsConfigurationTable 1 }

WmanIf2BsConfigurationEntry ::= SEQUENCE {

wmanIf2BsDcdInterval                    INTEGER,

wmanIf2BsUcdInterval                    INTEGER,

wmanIf2BsUcdTransition                  INTEGER,

wmanIf2BsDcdTransition                  INTEGER,

wmanIf2BsInitialRangingInterval         INTEGER,

wmanIf2BsSsULMapProcTime                Unsigned32,

wmanIf2BsSsRangRespProcTime             Unsigned32,

wmanIf2BsT9Timeout                      INTEGER,

wmanIf2BsT13Timeout                     INTEGER,

wmanIf2BsT15Timeout                     INTEGER,

wmanIf2BsT17Timeout                     INTEGER,

wmanIf2BsT27IdleTimer                   Unsigned32,

wmanIf2BsT27ActiveTimer                 Unsigned32,

wmanIf2Bs2ndMgmtDlQoSProfileIndex       INTEGER,

wmanIf2Bs2ndMgmtUlQoSProfileIndex       INTEGER,

wmanIf2BsAutoSfidEnabled                INTEGER,

wmanIf2BsAutoSfidRangeMin               Unsigned32,

wmanIf2BsAutoSfidRangeMax               Unsigned32,

wmanIf2BsAasChanFbckReqFreq             INTEGER,

wmanIf2BsAasBeamSelectFreq              INTEGER,

wmanIf2BsAasChanFbckReqResolution       INTEGER,

wmanIf2BsAasBeamReqResolution           INTEGER,

wmanIf2BsAasNumOptDiversityZones        INTEGER,

wmanIf2BsResetSector                    INTEGER,

wmanIf2BsSaChallengeTimer               INTEGER,

wmanIf2BsSaChallengeMaxResends          INTEGER,

wmanIf2BsSaTekTimer                     INTEGER,

wmanIf2BsSaTekReqMaxResends             INTEGER,

wmanIf2Bs2ndEapTimeout                  INTEGER,

wmanIf2BsEapCompleteResends             INTEGER,

wmanIf2BsInvitedRangRetries             INTEGER,

wmanIf2BsDSxReqRetries                  Unsigned32,

wmanIf2BsDSxRespRetries                 Unsigned32,

wmanIf2BsT7Timeout                      INTEGER,

wmanIf2BsT8Timeout                      INTEGER,

wmanIf2BsT10Timeout                     INTEGER,

wmanIf2BsT22Timeout                     INTEGER}

wmanIf2BsDcdInterval OBJECT-TYPE

SYNTAX      INTEGER (0..10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time between transmission of DCD messages in ms."

::= { wmanIf2BsConfigurationEntry 1 }

wmanIf2BsUcdInterval OBJECT-TYPE

SYNTAX      INTEGER (0..10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time between transmission of UCD messages in ms."

::= { wmanIf2BsConfigurationEntry 2 }

wmanIf2BsUcdTransition OBJECT-TYPE

SYNTAX      INTEGER (2..65535)

UNITS       "Number of MAC Frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The number of frames from the end of the frame carrying

the UCD message the BS shall wait after transmitting a

UCD message with an incremented Configuration Change

Count before issuing a UL-MAP message referring to

Uplink_Burst_Profiles defined in that UCD message.

Minimum value = 20ms following the last fragment of the

message"

::= { wmanIf2BsConfigurationEntry 3 }

wmanIf2BsDcdTransition OBJECT-TYPE

SYNTAX      INTEGER (2..65535)

UNITS       "Number of MAC Frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The number of frames from the end of the frame carrying

the DCD message the BS shall wait after transmitting a

DCD message with an incremented Configuration Change

Count before issuing a DL-MAP message referring to

Downlink_Burst_Profiles defined in that DCD message.

Minimum value = 20ms following the last fragment of the

message"

::= { wmanIf2BsConfigurationEntry 4 }

wmanIf2BsInitialRangingInterval OBJECT-TYPE

SYNTAX      INTEGER(0..2000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time between Initial Ranging regions assigned by the BS

in ms."

::= { wmanIf2BsConfigurationEntry 5 }

wmanIf2BsSsULMapProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (200 .. 4294967295)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time provided between arrival of the last bit of a UL-MAP

at an SS and effectiveness of that map in us. For OFDMA

mode, the time shall be counted starting from the end of

the burst carrying the UL-MAP.

Minimum value: SC = 200us

OFDM = 1ms

OFDMA = frame duration"

::= { wmanIf2BsConfigurationEntry 6 }

wmanIf2BsSsRangRespProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (10000 .. 4294967295)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an SS following receipt of a ranging

response before it is expected to reply to an invited

ranging request in us."

::= { wmanIf2BsConfigurationEntry 7 }

wmanIf2BsT9Timeout OBJECT-TYPE

SYNTAX      INTEGER (300 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Registration Timeout, the time allowed between the BS

sending a RNG-RSP (success) to an SS, and receiving a

SBC-REQ from that same SS in ms."

::= { wmanIf2BsConfigurationEntry 9 }

wmanIf2BsT13Timeout OBJECT-TYPE

SYNTAX      INTEGER (15 .. 65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time allowed for an SS, following receipt of a

REG-RSP message to send a TFTP-CPLT message to the BS

in min."

::= { wmanIf2BsConfigurationEntry 10 }

wmanIf2BsT15Timeout OBJECT-TYPE

SYNTAX      INTEGER (20 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for MCA-RSP in ms."

::= { wmanIf2BsConfigurationEntry 11 }

wmanIf2BsT17Timeout OBJECT-TYPE

SYNTAX      INTEGER (5 .. 65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for SS to complete SS Authorization and

Key Exchange in minutes."

::= { wmanIf2BsConfigurationEntry 12 }

wmanIf2BsT27IdleTimer OBJECT-TYPE

SYNTAX      Unsigned32 (10000 .. 4294967295)

UNITS       "microsecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum time between unicast grants to SS when BS believes

SS uplink transmission quality is good enough."

::= { wmanIf2BsConfigurationEntry 13 }

wmanIf2BsT27ActiveTimer OBJECT-TYPE

SYNTAX      Unsigned32 (10000 .. 4294967295)

UNITS       "microsecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum time between unicast grants to SS when BS believes

SS uplink transmission quality is not good enough."

::= { wmanIf2BsConfigurationEntry 14 }

wmanIf2Bs2ndMgmtDlQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the index of a row in

wmanIf2BsServiceClassTable which is used to obtain all QoS

parameters required for the BS downlink scheduler to

properly allocate and manage the bandwidth and schedule

the 2nd Management Connection traffic. The 2nd Management

Connection traffic doesn't differ from Traffic Connection

traffic in the area of QoS management."

::= { wmanIf2BsConfigurationEntry 15 }

wmanIf2Bs2ndMgmtUlQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the index of a row in

wmanIf2BsServiceClassTable which is used to obtain all QoS

parameters required for the BS uplink scheduler to

properly allocate and manage the bandwidth and schedule

the 2nd Management Connection traffic. The 2nd Management

Connection traffic doesn't differ from Traffic Connection

traffic in the area of QoS management."

::= { wmanIf2BsConfigurationEntry 16 }

wmanIf2BsAutoSfidEnabled OBJECT-TYPE

SYNTAX      INTEGER {autoSfidDisabled(0),

autoSfidEnabled(1)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines whether the BS is allowed to

autonomously allocate SFIDs. When the object is set to

autoSfidEnabled, the BS is allowed to autonomously allocate

SFIDs from the range of allowed values defined by

wmanIf2BsConfigExtAutoSfidRangeMin and

wmanIf2BsConfigExtAutoSfidRangeMax. A SF is created

autonomously when it has not been provisioned in the

wmanIf2BsProvisionedSfTable and may be initiated by either

the SS or BS. The BS should always initiate SF creation

based on the provisioned Service flows configured in

wmanIf2BsProvisionedSfTable."

REFERENCE

"Subclause 11.13.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 17 }

wmanIf2BsAutoSfidRangeMin OBJECT-TYPE

SYNTAX      Unsigned32 ( 1 .. 4294967295)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the minimum value of the range of SFID

values allocated for the BS sector for the purpose of

autonomous creation of service flows. This value is used

when the object wmanIf2BsAutoSfidEnabled allows

autonomous creation of SFIDs."

REFERENCE

"Subclause 11.13.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 18 }

wmanIf2BsAutoSfidRangeMax OBJECT-TYPE

SYNTAX      Unsigned32 ( 1 .. 4294967295)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the maximum value of the range of SFID

values allocated for the BS sector for the purpose of

autonomous creation of the service flows. This value is

used when the object wmanIf2BsAutoSfidEnabled allows

autonomous creation of SFIDs."

REFERENCE

"Subclause 11.13.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 19 }

wmanIf2BsAasChanFbckReqFreq OBJECT-TYPE

SYNTAX      INTEGER (5..10000)

UNITS       "millisecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines AAS channel feedback request frequency.

It controls the frequency of downlink beam measurements.

The relevant MAC messages are AAS-FBCK-REQ/RSP"

REFERENCE

"Subclause 6.3.2.3.40 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 20 }

wmanIf2BsAasBeamSelectFreq OBJECT-TYPE

SYNTAX      INTEGER (5..10000)

UNITS       "millisecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines AAS beam select frequency.

It controls how often SS issues beam select messages.

The relevant MAC message is AAS_Beam_Select"

REFERENCE

"Subclause 6.3.2.3.41 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 21 }

wmanIf2BsAasChanFbckReqResolution OBJECT-TYPE

SYNTAX      INTEGER {aasChanFbckRes00(0),

aasChanFbckRes01(1),

aasChanFbckRes10(2),

aasChanFbckRes11(3)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines AAS feedback request frequency

measurements resolution. It is coded as follows:

aasChanFbckRes00 - every 4th carrier

(-100, -96, -92, .., 100)

aasChanFbckRes01 - every 8th carrier

(-100, -92, -84, .., 100)

aasChanFbckRes10 - every 16th carrier

(-100, -84, -68, .., 100)

aasChanFbckRes11 - every 32th carrier

(-100, -68, -36, .., 100)"

REFERENCE

"Subclause 8.3.6.4 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 22 }

wmanIf2BsAasBeamReqResolution OBJECT-TYPE

SYNTAX      INTEGER {aasBeamReqRes000(0),

aasBeamReqRes001(1),

aasBeamReqRes010(2),

aasBeamReqRes011(3),

aasBeamReqRes100(4)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines AAS beam select request resolution

parameter. It is coded as follows:

aasBeamReqRes000 - every 4th carrier

aasBeamReqRes001 - every 8th carrier

aasBeamReqRes010 - every 16th carrier

aasBeamReqRes011 - every 32th carrier

aasBeamReqRes100 - every 64th carrier"

REFERENCE

"Subclause 8.3.6.5 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 23 }

wmanIf2BsAasNumOptDiversityZones OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the number of optional diversity zones

transmitted in downlink."

REFERENCE

"Figure 209 in IEEE Std 802.16-2004"

::= { wmanIf2BsConfigurationEntry 24 }

wmanIf2BsResetSector OBJECT-TYPE

SYNTAX      INTEGER {noAction(0),

sectorReset(1)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object should be implemented as follows:

- When set to actionsResetSector value, instructs BS to

reset the sector identified by ifIndex. As a result of

this action the PHY and MAC of this sector should be

reinitialised.

- When set to value different than actionsResetSector it

should be ignored

- When read it should return actionsResetSectorNoAction"

::= { wmanIf2BsConfigurationEntry 25 }

wmanIf2BsSaChallengeTimer OBJECT-TYPE

SYNTAX      INTEGER (500 .. 2000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Challenge."

DEFVAL      { 1000 }

::= { wmanIf2BsConfigurationEntry 27 }

wmanIf2BsSaChallengeMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Challenge."

DEFVAL      { 3 }

::= { wmanIf2BsConfigurationEntry 28 }

wmanIf2BsSaTekTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Request."

DEFVAL      { 300 }

::= { wmanIf2BsConfigurationEntry 29 }

wmanIf2BsSaTekReqMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Request."

DEFVAL      { 3 }

::= { wmanIf2BsConfigurationEntry 30 }

wmanIf2Bs2ndEapTimeout OBJECT-TYPE

SYNTAX      INTEGER (300 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time, in seconds, to wait for PKMv2_EAP_Start or

PKMv2_Authenticated_EAP_Start after the success of the

first EAP in double EAP mode."

DEFVAL      { 1000 }

::= { wmanIf2BsConfigurationEntry 31 }

wmanIf2BsEapCompleteResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Total number of sending PKMv2_EAP_Complete message in

double EAP mode."

DEFVAL      { 3 }

::= { wmanIf2BsConfigurationEntry 32 }

wmanIf2BsInvitedRangRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on inviting Ranging Requests."

::= { wmanIf2BsConfigurationEntry 33 }

wmanIf2BsDSxReqRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Requests."

DEFVAL      { 3 }

::= { wmanIf2BsConfigurationEntry 34 }

wmanIf2BsDSxRespRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Responses."

DEFVAL      { 3 }

::= { wmanIf2BsConfigurationEntry 35 }

wmanIf2BsT7Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Response Timeout in ms."

::= { wmanIf2BsConfigurationEntry 36 }

wmanIf2BsT8Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 300)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Acknowledge Timeout in ms."

::= { wmanIf2BsConfigurationEntry 37 }

wmanIf2BsT10Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 3000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for Transaction End timeout in ms."

::= { wmanIf2BsConfigurationEntry 38 }

wmanIf2BsT22Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 500)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for ARQ Reset in ms."

::= { wmanIf2BsConfigurationEntry 39 }

wmanIf2BsSsReqCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information of SSs

that have been reported by SSs to BS using RNG-REQ, SBC-REQ

and REG-REQ messages. Entries in this table should be

created when an SS registers with a BS."

::= { wmanIf2BsCm 3 }

wmanIf2BsSsReqCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. This table augments the table

wmanIf2BsRegisteredSsTable."

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsReqCapabilitiesTable 1 }

WmanIf2BsSsReqCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsSsReqCapUplinkCidSupport       WmanIf2NumOfCid,

wmanIf2BsSsReqCapArqSupport             WmanIf2ArqSupportType,

wmanIf2BsSsReqCapDsxFlowControl         WmanIf2MaxDsxFlowType,

wmanIf2BsSsReqCapMacCrcSupport          WmanIf2MacCrcSupport,

wmanIf2BsSsReqCapMcaFlowControl         WmanIf2MaxMcaFlowType,

wmanIf2BsSsReqCapMcpGroupCidSupport     WmanIf2MaxMcpGroupCid,

wmanIf2BsSsReqCapPkmFlowControl         WmanIf2MaxPkmFlowType,

wmanIf2BsSsReqCapAuthPolicyControl      WmanIf2AuthPolicyType,

wmanIf2BsSsReqCapMaxNumOfSupportedSA    WmanIf2MaxNumOfSaType,

wmanIf2BsSsReqCapIpVersion              WmanIf2IpVersionType,

wmanIf2BsSsReqCapMacCsSupportBitMap     WmanIf2MacCsBitMap,

wmanIf2BsSsReqCapMaxNumOfClassifier     WmanIf2MaxClassifiers,

wmanIf2BsSsReqCapPhsSupport             WmanIf2PhsSupportType,

wmanIf2BsSsReqCapBandwidthAllocSupport  WmanIf2BwAllocSupport,

wmanIf2BsSsReqCapPduConstruction        WmanIf2PduConstruction,

wmanIf2BsSsReqCapTtgTransitionGap       WmanIf2SsTransitionGap,

wmanIf2BsSsReqCapRtgTransitionGap       WmanIf2SsTransitionGap,

wmanIf2BsSsReqCapDownlinkCidSupport     WmanIf2NumOfCid,

wmanIf2BsSsReqCapPackingSupport         WmanIf2PackingSupport,

wmanIf2BsSsReqCapExtendedRtpsSupport    WmanIf2ExtRtpsSupport,

wmanIf2BsSsReqCapMaxNumBurstToMs        INTEGER,

wmanIf2BsSsReqCapIpAddrAllocMethod      WmanIf2IpAllocMethod,

wmanIf2BsSsReqCapArqAckType             WmanIf2ArqAckType,

wmanIf2BsSsReqCapMacHeader              WmanIf2MacHeaderSupp,

wmanIf2BsSsReqCapMaxMacLevelDlFrame     WmanIf2MaxMacLevel,

wmanIf2BsSsReqCapMaxMacLevelUlFrame     WmanIf2MaxMacLevel,

wmanIf2BsSsReqCapPkmVersion             WmanIf2PkmVersion,

wmanIf2BsSsReqCapAuthPolicy             WmanIf2AuthPolicy,

wmanIf2BsSsReqCapMacMode                WmanIf2MacMode,

wmanIf2BsSsReqCapPnWindowSize           INTEGER,

wmanIf2BsSsReqCapExtCapability          WmanIf2ExtCapability}

wmanIf2BsSsReqCapUplinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object shows the number of Uplink CIDs the SS can

support."

::= { wmanIf2BsSsReqCapabilitiesEntry 1 }

wmanIf2BsSsReqCapArqSupport OBJECT-TYPE

SYNTAX      WmanIf2ArqSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether the SS supports ARQ."

::= { wmanIf2BsSsReqCapabilitiesEntry 2 }

wmanIf2BsSsReqCapDsxFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxDsxFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent

DSA, DSC, or DSD transactions that SS is capable of having

outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 3 }

wmanIf2BsSsReqCapMacCrcSupport OBJECT-TYPE

SYNTAX      WmanIf2MacCrcSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether or not the SS supports MAC

level CRC."

DEFVAL		{ macCrcSupport }

::= { wmanIf2BsSsReqCapabilitiesEntry 4 }

wmanIf2BsSsReqCapMcaFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxMcaFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent MCA

transactions that SS is capable of having outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 5 }

wmanIf2BsSsReqCapMcpGroupCidSupport OBJECT-TYPE

SYNTAX      WmanIf2MaxMcpGroupCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the maximum number of

simultaneous Multicast Polling Groups the SS is

capable of belonging to."

DEFVAL		{ 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 6 }

wmanIf2BsSsReqCapPkmFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxPkmFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent PKM

transactions that SS is capable of having outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 7 }

wmanIf2BsSsReqCapAuthPolicyControl OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicyType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies authorization policy that SS is

capable of. A bit value of 0 = 'not supported', 1 =

'supported'. If this field is omitted, then both SS and

BS shall use the IEEE 802.16 security, constituting X.509

digital certificates and the RSA public key encryption

algorithm, as authorization policy."

::= { wmanIf2BsSsReqCapabilitiesEntry 8 }

wmanIf2BsSsReqCapMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX      WmanIf2MaxNumOfSaType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field specifies the maximum number of supported

security associations of the SS."

DEFVAL		{ 1 }

::= { wmanIf2BsSsReqCapabilitiesEntry 9 }

wmanIf2BsSsReqCapIpVersion OBJECT-TYPE

SYNTAX      WmanIf2IpVersionType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the version of IP used on the 2nd

Management Connection. The value should be undefined

if the 2nd management CID doesn't exist."

::= { wmanIf2BsSsReqCapabilitiesEntry 10 }

wmanIf2BsSsReqCapMacCsSupportBitMap OBJECT-TYPE

SYNTAX      WmanIf2MacCsBitMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates SS reported set of MAC convergence

sublayer support. When a bit is set, it indicates

the corresponding CS feature is supported."

::= { wmanIf2BsSsReqCapabilitiesEntry 11 }

wmanIf2BsSsReqCapMaxNumOfClassifier OBJECT-TYPE

SYNTAX      WmanIf2MaxClassifiers

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the maximum number of admitted

Classifiers that the SS can support."

DEFVAL      { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 12 }

wmanIf2BsSsReqCapPhsSupport OBJECT-TYPE

SYNTAX      WmanIf2PhsSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates indicates the level of SS support

for PHS."

DEFVAL      { noPhsSupport }

::= { wmanIf2BsSsReqCapabilitiesEntry 13 }

wmanIf2BsSsReqCapBandwidthAllocSupport OBJECT-TYPE

SYNTAX      WmanIf2BwAllocSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the bandwidth allocation

capabilities of the SS. The usage is defined by

WmanIf2BwAllocSupport."

::= { wmanIf2BsSsReqCapabilitiesEntry 14 }

wmanIf2BsSsReqCapPduConstruction OBJECT-TYPE

SYNTAX      WmanIf2PduConstruction

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the SS's capabilities for

construction and transmission of MAC PDUs. The usage

is defined by WmanIf2PduConstruction."

::= { wmanIf2BsSsReqCapabilitiesEntry 15 }

wmanIf2BsSsReqCapTtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the SS's transition speed SSTTG

for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsReqCapabilitiesEntry 16 }

wmanIf2BsSsReqCapRtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the SS's transition speed SSRTG

for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsReqCapabilitiesEntry 17 }

wmanIf2BsSsReqCapDownlinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object shows the number of Downlink CIDs the SS can

support."

::= { wmanIf2BsSsReqCapabilitiesEntry 18 }

wmanIf2BsSsReqCapPackingSupport OBJECT-TYPE

SYNTAX      WmanIf2PackingSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Packing."

REFERENCE

"Subclause 11.7.8.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 19 }

wmanIf2BsSsReqCapExtendedRtpsSupport OBJECT-TYPE

SYNTAX      WmanIf2ExtRtpsSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of SS support for extended

rtPs."

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 20 }

wmanIf2BsSsReqCapMaxNumBurstToMs OBJECT-TYPE

SYNTAX      INTEGER (1..16)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS's CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 21 }

wmanIf2BsSsReqCapIpAddrAllocMethod OBJECT-TYPE

SYNTAX      WmanIf2IpAllocMethod

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the method of allocating IP address for the

secondary management connection."

REFERENCE

"Subclause 11.7.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 22 }

wmanIf2BsSsReqCapArqAckType OBJECT-TYPE

SYNTAX      WmanIf2ArqAckType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the ARQ ACK type

supported by the MS."

REFERENCE

"Subclause 11.7.23 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 23 }

wmanIf2BsSsReqCapMacHeader OBJECT-TYPE

SYNTAX      WmanIf2MacHeaderSupp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not the MS and BS support various

types of MAC header and extended subheaders."

REFERENCE

"Subclause 11.7.25 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 24 }

wmanIf2BsSsReqCapMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame."

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 25 }

wmanIf2BsSsReqCapMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame."

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsSsReqCapabilitiesEntry 26 }

wmanIf2BsSsReqCapPkmVersion OBJECT-TYPE

SYNTAX      WmanIf2PkmVersion

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a PKM version that SS or BS supports."

REFERENCE

"Subclause 11.8.4.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 27 }

wmanIf2BsSsReqCapAuthPolicy OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates authorization policy used by the MS and BS to

negotiate and synchronize."

REFERENCE

"Subclause 11.8.4.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 28 }

wmanIf2BsSsReqCapMacMode OBJECT-TYPE

SYNTAX      WmanIf2MacMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a MAC (Message Authentication Code) mode that MS

supports."

REFERENCE

"Subclause 11.8.4.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 29 }

wmanIf2BsSsReqCapPnWindowSize OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 30 }

wmanIf2BsSsReqCapExtCapability OBJECT-TYPE

SYNTAX      WmanIf2ExtCapability

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the extended subheader format support."

REFERENCE

"Subclause 11.8.6 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsReqCapabilitiesEntry 31 }

wmanIf2BsSsRspCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information of SSs

that have been negotiated and agreed between BS and SS via

RNG-REQ/RSP, SBC-REQ/RSP and REG-REQ/RSP messages.

This table augments the wmanIf2BsRegisteredSsTable."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsCm 4 }

wmanIf2BsSsRspCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. This table augments the

wmanIf2BsRegisteredSsTable. "

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsRspCapabilitiesTable 1 }

WmanIf2BsSsRspCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsSsRspCapUplinkCidSupport       WmanIf2NumOfCid,

wmanIf2BsSsRspCapArqSupport             WmanIf2ArqSupportType,

wmanIf2BsSsRspCapDsxFlowControl         WmanIf2MaxDsxFlowType,

wmanIf2BsSsRspCapMacCrcSupport          WmanIf2MacCrcSupport,

wmanIf2BsSsRspCapMcaFlowControl         WmanIf2MaxMcaFlowType,

wmanIf2BsSsRspCapMcpGroupCidSupport     WmanIf2MaxMcpGroupCid,

wmanIf2BsSsRspCapPkmFlowControl         WmanIf2MaxPkmFlowType,

wmanIf2BsSsRspCapAuthPolicyControl      WmanIf2AuthPolicyType,

wmanIf2BsSsRspCapMaxNumOfSupportedSA    WmanIf2MaxNumOfSaType,

wmanIf2BsSsRspCapIpVersion              WmanIf2IpVersionType,

wmanIf2BsSsRspCapMacCsSupportBitMap     WmanIf2MacCsBitMap,

wmanIf2BsSsRspCapMaxNumOfClassifier     WmanIf2MaxClassifiers,

wmanIf2BsSsRspCapPhsSupport             WmanIf2PhsSupportType,

wmanIf2BsSsRspCapBandwidthAllocSupport  WmanIf2BwAllocSupport,

wmanIf2BsSsRspCapPduConstruction        WmanIf2PduConstruction,

wmanIf2BsSsRspCapTtgTransitionGap       WmanIf2SsTransitionGap,

wmanIf2BsSsRspCapRtgTransitionGap       WmanIf2SsTransitionGap,

wmanIf2BsSsRspCapDownlinkCidSupport     WmanIf2NumOfCid,

wmanIf2BsSsRspCapPackingSupport         WmanIf2PackingSupport,

wmanIf2BsSsRspCapExtendedRtpsSupport    WmanIf2ExtRtpsSupport,

wmanIf2BsSsRspCapMaxNumBurstToMs        INTEGER,

wmanIf2BsSsRspCapIpAddrAllocMethod      WmanIf2IpAllocMethod,

wmanIf2BsSsRspCapArqAckType             WmanIf2ArqAckType,

wmanIf2BsSsRspCapMacHeader              WmanIf2MacHeaderSupp,

wmanIf2BsSsRspCapMaxMacLevelDlFrame     WmanIf2MaxMacLevel,

wmanIf2BsSsRspCapMaxMacLevelUlFrame     WmanIf2MaxMacLevel,

wmanIf2BsSsRspCapNumOfProvisionedSf     Unsigned32,

wmanIf2BsSsRspCapPkmVersion             WmanIf2PkmVersion,

wmanIf2BsSsRspCapAuthPolicy             WmanIf2AuthPolicy,

wmanIf2BsSsRspCapMacMode                WmanIf2MacMode,

wmanIf2BsSsRspCapPnWindowSize           INTEGER,

wmanIf2BsSsRspCapExtCapability          WmanIf2ExtCapability}

wmanIf2BsSsRspCapUplinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated number of Uplink CIDs the SS can support."

::= { wmanIf2BsSsRspCapabilitiesEntry 1 }

wmanIf2BsSsRspCapArqSupport OBJECT-TYPE

SYNTAX      WmanIf2ArqSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether the SS is allowed to use ARQ

as a result of the capabilities negotiation."

::= { wmanIf2BsSsRspCapabilitiesEntry 2 }

wmanIf2BsSsRspCapDsxFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxDsxFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of concurrent DSA, DSC, or DSD

transactions that may be outstanding."

::= { wmanIf2BsSsRspCapabilitiesEntry 3 }

wmanIf2BsSsRspCapMacCrcSupport OBJECT-TYPE

SYNTAX      WmanIf2MacCrcSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether or not the SS is allowed to

use MAC level CRC as a result of the capabilities

negotiation."

DEFVAL		{ macCrcSupport }

::= { wmanIf2BsSsRspCapabilitiesEntry 4 }

wmanIf2BsSsRspCapMcaFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxMcaFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of concurrent

MCA transactions that may be outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 5 }

wmanIf2BsSsRspCapMcpGroupCidSupport OBJECT-TYPE

SYNTAX      WmanIf2MaxMcpGroupCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of simultaneous Multicast

Polling Groups the SS is capable of belonging to."

DEFVAL		{ 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 6 }

wmanIf2BsSsRspCapPkmFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxPkmFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of concurrent PKM

transactions that may be outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 7 }

wmanIf2BsSsRspCapAuthPolicyControl OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicyType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies negotiated authorization policy.

A bit value of 0 = 'not supported', 1 = 'supported'. If

this field is omitted, then both SS and BS shall use the

IEEE 802.16 security, constituting X.509 digital

certificates and the RSA public key encryption

algorithm, as authorization policy."

::= { wmanIf2BsSsRspCapabilitiesEntry 8 }

wmanIf2BsSsRspCapMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX      WmanIf2MaxNumOfSaType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of supported security

association of the SS."

DEFVAL		{ 1 }

::= { wmanIf2BsSsRspCapabilitiesEntry 9 }

wmanIf2BsSsRspCapIpVersion OBJECT-TYPE

SYNTAX      WmanIf2IpVersionType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated version of IP used on the 2nd Management

Connection. The value should be undefined if the 2nd

management CID doesn't exist."

::= { wmanIf2BsSsRspCapabilitiesEntry 10 }

wmanIf2BsSsRspCapMacCsSupportBitMap OBJECT-TYPE

SYNTAX      WmanIf2MacCsBitMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated set of MAC convergence sublayer support.

When a bit is set, it indicates the corresponding CS

feature is supported."

::= { wmanIf2BsSsRspCapabilitiesEntry 11 }

wmanIf2BsSsRspCapMaxNumOfClassifier OBJECT-TYPE

SYNTAX      WmanIf2MaxClassifiers

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Negotiated maximum number of admitted Classifiers

that the SS is allowed to have."

DEFVAL      { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 12 }

wmanIf2BsSsRspCapPhsSupport OBJECT-TYPE

SYNTAX      WmanIf2PhsSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the negotiated level of PHS

support."

DEFVAL      { noPhsSupport }

::= { wmanIf2BsSsRspCapabilitiesEntry 13 }

wmanIf2BsSsRspCapBandwidthAllocSupport OBJECT-TYPE

SYNTAX      WmanIf2BwAllocSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates negotiated properties of the SS

for bandwidth allocation purposes. The usage is defined

by WmanIf2BwAllocSupport."

::= { wmanIf2BsSsRspCapabilitiesEntry 14 }

wmanIf2BsSsRspCapPduConstruction OBJECT-TYPE

SYNTAX      WmanIf2PduConstruction

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies negotiated capabilities for construction and

transmission of MAC PDUs. The usage is defined by

WmanIf2PduConstruction."

::= { wmanIf2BsSsRspCapabilitiesEntry 15 }

wmanIf2BsSsRspCapTtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the negotiated transition speed

SSTTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsRspCapabilitiesEntry 16 }

wmanIf2BsSsRspCapRtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the negotiated transition speed

SSRTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsSsRspCapabilitiesEntry 17 }

wmanIf2BsSsRspCapDownlinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object shows the number of Downlink CIDs the SS can

support."

::= { wmanIf2BsSsRspCapabilitiesEntry 18 }

wmanIf2BsSsRspCapPackingSupport OBJECT-TYPE

SYNTAX      WmanIf2PackingSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Packing."

REFERENCE

"Subclause 11.7.8.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 19 }

wmanIf2BsSsRspCapExtendedRtpsSupport OBJECT-TYPE

SYNTAX      WmanIf2ExtRtpsSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of SS support for extended

rtPs."

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 20 }

wmanIf2BsSsRspCapMaxNumBurstToMs OBJECT-TYPE

SYNTAX      INTEGER (1..16)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 21 }

wmanIf2BsSsRspCapIpAddrAllocMethod OBJECT-TYPE

SYNTAX      WmanIf2IpAllocMethod

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the method of allocating IP address for the

secondary management connection."

REFERENCE

"Subclause 11.7.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 22 }

wmanIf2BsSsRspCapArqAckType OBJECT-TYPE

SYNTAX      WmanIf2ArqAckType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the ARQ ACK type

supported by the MS."

REFERENCE

"Subclause 11.7.23 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 23 }

wmanIf2BsSsRspCapMacHeader OBJECT-TYPE

SYNTAX      WmanIf2MacHeaderSupp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not the MS and BS support various

types of MAC header and extended subheaders."

REFERENCE

"Subclause 11.7.25 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 24 }

wmanIf2BsSsRspCapMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 25 }

wmanIf2BsSsRspCapMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsSsRspCapabilitiesEntry 26 }

wmanIf2BsSsRspCapNumOfProvisionedSf OBJECT-TYPE

SYNTAX      Unsigned32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 27 }

wmanIf2BsSsRspCapPkmVersion OBJECT-TYPE

SYNTAX      WmanIf2PkmVersion

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a PKM version that SS or BS supports."

REFERENCE

"Subclause 11.8.4.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 28 }

wmanIf2BsSsRspCapAuthPolicy OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates authorization policy used by the MS and BS to

negotiate and synchronize."

REFERENCE

"Subclause 11.8.4.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 29 }

wmanIf2BsSsRspCapMacMode OBJECT-TYPE

SYNTAX      WmanIf2MacMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a MAC (Message Authentication Code) mode that MS

supports."

REFERENCE

"Subclause 11.8.4.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 30 }

wmanIf2BsSsRspCapPnWindowSize OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 31 }

wmanIf2BsSsRspCapExtCapability OBJECT-TYPE

SYNTAX      WmanIf2ExtCapability

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the extended subheader format support."

REFERENCE

"Subclause 11.8.6 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsRspCapabilitiesEntry 32 }

wmanIf2BsBasicCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capabilities of the BS as

implemented in BS hardware and software. These capabilities

along with the configuration for them

(wmanIf2BsCapabilitiesConfigTable) are used for negotiation

of basic capabilities with SS using RNG-RSP, SBC-RSP and

REG-RSP messages. The negotiated capabilities are obtained

by interSubclause of SS raw reported capabilities, BS raw

capabilities and BS configured capabilities. The objects in

the table have read-only access. The table is maintained

by BS."

::= { wmanIf2BsCm 5 }

wmanIf2BsBasicCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsBasicCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsBasicCapabilitiesTable 1 }

WmanIf2BsBasicCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsCapUplinkCidSupport            WmanIf2NumOfCid,

wmanIf2BsCapArqSupport                  WmanIf2ArqSupportType,

wmanIf2BsCapDsxFlowControl              WmanIf2MaxDsxFlowType,

wmanIf2BsCapMacCrcSupport               WmanIf2MacCrcSupport,

wmanIf2BsCapMcaFlowControl              WmanIf2MaxMcaFlowType,

wmanIf2BsCapMcpGroupCidSupport          WmanIf2MaxMcpGroupCid,

wmanIf2BsCapPkmFlowControl              WmanIf2MaxPkmFlowType,

wmanIf2BsCapAuthPolicyControl           WmanIf2AuthPolicyType,

wmanIf2BsCapMaxNumOfSupportedSA         WmanIf2MaxNumOfSaType,

wmanIf2BsCapIpVersion                   WmanIf2IpVersionType,

wmanIf2BsCapMacCsSupportBitMap          WmanIf2MacCsBitMap,

wmanIf2BsCapMaxNumOfClassifier          WmanIf2MaxClassifiers,

wmanIf2BsCapPhsSupport                  WmanIf2PhsSupportType,

wmanIf2BsCapBandwidthAllocSupport       WmanIf2BwAllocSupport,

wmanIf2BsCapPduConstruction             WmanIf2PduConstruction,

wmanIf2BsCapTtgTransitionGap            WmanIf2SsTransitionGap,

wmanIf2BsCapRtgTransitionGap            WmanIf2SsTransitionGap,

wmanIf2BsCapDownlinkCidSupport          WmanIf2NumOfCid,

wmanIf2BsCapPackingSupport              WmanIf2PackingSupport,

wmanIf2BsCapExtendedRtpsSupport         WmanIf2ExtRtpsSupport,

wmanIf2BsCapMaxNumBurstToMs             INTEGER,

wmanIf2BsCapIpAddrAllocMethod           WmanIf2IpAllocMethod,

wmanIf2BsCapArqAckType                  WmanIf2ArqAckType,

wmanIf2BsCapMacHeader                   WmanIf2MacHeaderSupp,

wmanIf2BsCapMaxMacLevelDlFrame          WmanIf2MaxMacLevel,

wmanIf2BsCapMaxMacLevelUlFrame          WmanIf2MaxMacLevel,

wmanIf2BsCapNumOfProvisionedSf          Unsigned32,

wmanIf2BsCapPkmVersion                  WmanIf2PkmVersion,

wmanIf2BsCapAuthPolicy                  WmanIf2AuthPolicy,

wmanIf2BsCapMacMode                     WmanIf2MacMode,

wmanIf2BsCapPnWindowSize                INTEGER,

wmanIf2BsCapExtCapability               WmanIf2ExtCapability}

wmanIf2BsCapUplinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object shows the number of Uplink CIDs the BS can

support per SS."

::= { wmanIf2BsBasicCapabilitiesEntry 1 }

wmanIf2BsCapArqSupport OBJECT-TYPE

SYNTAX      WmanIf2ArqSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether the BS supports ARQ."

::= { wmanIf2BsBasicCapabilitiesEntry 2 }

wmanIf2BsCapDsxFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxDsxFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent

DSA, DSC, or DSD transactions that BS allows each SS to

have outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 3 }

wmanIf2BsCapMacCrcSupport OBJECT-TYPE

SYNTAX      WmanIf2MacCrcSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates whether or not the BS supports MAC

level CRC."

DEFVAL		{ macCrcSupport }

::= { wmanIf2BsBasicCapabilitiesEntry 4 }

wmanIf2BsCapMcaFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxMcaFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent

MCA transactions that BS allows each SS to have."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 5 }

wmanIf2BsCapMcpGroupCidSupport OBJECT-TYPE

SYNTAX      WmanIf2MaxMcpGroupCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the maximum number of simultaneous

Multicast Polling Groups the BS allows each SS to belong

to."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 6 }

wmanIf2BsCapPkmFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxPkmFlowType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent

PKM transactions that BS allows each SS to have."

DEFVAL		{ 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 7 }

wmanIf2BsCapAuthPolicyControl OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicyType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies authorization policy that BS is

capable of. A bit value of 0 = 'not supported', 1 =

'supported'. If this field is omitted, then both SS and

BS shall use the IEEE 802.16 security, constituting X.509

digital certificates and the RSA public key encryption

algorithm, as authorization policy."

::= { wmanIf2BsBasicCapabilitiesEntry 8 }

wmanIf2BsCapMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX      WmanIf2MaxNumOfSaType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field specifies maximum number of supported security

associations per SS that the BS allows."

DEFVAL		{ 1 }

::= { wmanIf2BsBasicCapabilitiesEntry 9 }

wmanIf2BsCapIpVersion OBJECT-TYPE

SYNTAX      WmanIf2IpVersionType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the version of IP BS allows each SS

to use on the 2nd Management Connection. The value

'undefined' should not be used for this field."

REFERENCE

"Subclause 11.7.4 in IEEE Std 802.16-2004"

::= { wmanIf2BsBasicCapabilitiesEntry 10 }

wmanIf2BsCapMacCsSupportBitMap OBJECT-TYPE

SYNTAX      WmanIf2MacCsBitMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates BS set of MAC convergence

sublayer support. When a bit is set, it indicates

the corresponding CS feature is supported."

::= { wmanIf2BsBasicCapabilitiesEntry 11 }

wmanIf2BsCapMaxNumOfClassifier OBJECT-TYPE

SYNTAX      WmanIf2MaxClassifiers

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the maximum number of admitted

Classifiers per SS that the BS allows."

DEFVAL      { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 12 }

wmanIf2BsCapPhsSupport OBJECT-TYPE

SYNTAX      WmanIf2PhsSupportType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the level of BS support for PHS.

The usage is defined by WmanIf2PhsSupportType."

DEFVAL      { noPhsSupport }

::= { wmanIf2BsBasicCapabilitiesEntry 13 }

wmanIf2BsCapBandwidthAllocSupport OBJECT-TYPE

SYNTAX      WmanIf2BwAllocSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the bandwidth allocation properties

that the BS permits SSs to use. The usage is defined by

WmanIf2BwAllocSupport."

::= { wmanIf2BsBasicCapabilitiesEntry 14 }

wmanIf2BsCapPduConstruction OBJECT-TYPE

SYNTAX      WmanIf2PduConstruction

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the capabilities for construction and

transmission of MAC PDUs allowed by the BS. The usage is

defined by WmanIf2PduConstruction."

::= { wmanIf2BsBasicCapabilitiesEntry 15 }

wmanIf2BsCapTtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the transition speed SSTTG for TDD

and H-FDD SSs allowed by the BS. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsBasicCapabilitiesEntry 16 }

wmanIf2BsCapRtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the transition speed SSRTG for TDD

and H-FDD SSs allowed by the BS. The usage is defined

by WmanIf2SsTransitionGap."

::= { wmanIf2BsBasicCapabilitiesEntry 17 }

wmanIf2BsCapDownlinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object shows the number of Downlink CIDs the SS can

support."

::= { wmanIf2BsBasicCapabilitiesEntry 18 }

wmanIf2BsCapPackingSupport OBJECT-TYPE

SYNTAX      WmanIf2PackingSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Packing."

REFERENCE

"Subclause 11.7.8.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 19 }

wmanIf2BsCapExtendedRtpsSupport OBJECT-TYPE

SYNTAX      WmanIf2ExtRtpsSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of SS support for extended

rtPs."

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 20 }

wmanIf2BsCapMaxNumBurstToMs OBJECT-TYPE

SYNTAX      INTEGER (1..16)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 21 }

wmanIf2BsCapIpAddrAllocMethod OBJECT-TYPE

SYNTAX      WmanIf2IpAllocMethod

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the method of allocating IP address for the

secondary management connection."

REFERENCE

"Subclause 11.7.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 22 }

wmanIf2BsCapArqAckType OBJECT-TYPE

SYNTAX      WmanIf2ArqAckType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the ARQ ACK type

supported by the MS."

REFERENCE

"Subclause 11.7.23 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 23 }

wmanIf2BsCapMacHeader OBJECT-TYPE

SYNTAX      WmanIf2MacHeaderSupp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not the MS and BS support various

types of MAC header and extended subheaders."

REFERENCE

"Subclause 11.7.25 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 24 }

wmanIf2BsCapMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 25 }

wmanIf2BsCapMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsBasicCapabilitiesEntry 26 }

wmanIf2BsCapNumOfProvisionedSf OBJECT-TYPE

SYNTAX      Unsigned32 (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 27 }

wmanIf2BsCapPkmVersion OBJECT-TYPE

SYNTAX      WmanIf2PkmVersion

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a PKM version that SS or BS supports."

REFERENCE

"Subclause 11.8.4.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 28 }

wmanIf2BsCapAuthPolicy OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates authorization policy used by the MS and BS to

negotiate and synchronize."

REFERENCE

"Subclause 11.8.4.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 29 }

wmanIf2BsCapMacMode OBJECT-TYPE

SYNTAX      WmanIf2MacMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a MAC (Message Authentication Code) mode that MS

supports."

REFERENCE

"Subclause 11.8.4.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 30 }

wmanIf2BsCapPnWindowSize OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 31 }

wmanIf2BsCapExtCapability OBJECT-TYPE

SYNTAX      WmanIf2ExtCapability

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the extended subheader format support."

REFERENCE

"Subclause 11.8.6 in IEEE Std 802.16e-2005"

::= { wmanIf2BsBasicCapabilitiesEntry 32 }

wmanIf2BsCapabilitiesConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration for basic

capabilities of BS. The table is intended to be used to

restrict the Capabilities implemented by BS, for example in

order to comply with local regulatory requirements. The BS

should use the configuration along with the implemented

Capabilities (wmanIf2BsBasicCapabilitiesTable) for

negotiation of basic capabilities with SS using RNG-RSP,

SBC-RSP and REG-RSP messages. The negotiated capabilities

are obtained by interSubclause of SS reported capabilities,

BS raw capabilities and BS configured capabilities. The

objects in the table have read-write access. The rows are

created by BS as a copy of wmanIf2BsBasicCapabilitiesTable

and can be modified by NMS."

::= { wmanIf2BsCm 6 }

wmanIf2BsCapabilitiesConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsCapabilitiesConfigTable 1 }

WmanIf2BsCapabilitiesConfigEntry ::= SEQUENCE {

wmanIf2BsCapCfgUplinkCidSupport         WmanIf2NumOfCid,

wmanIf2BsCapCfgArqSupport               WmanIf2ArqSupportType,

wmanIf2BsCapCfgDsxFlowControl           WmanIf2MaxDsxFlowType,

wmanIf2BsCapCfgMacCrcSupport            WmanIf2MacCrcSupport,

wmanIf2BsCapCfgMcaFlowControl           WmanIf2MaxMcaFlowType,

wmanIf2BsCapCfgMcpGroupCidSupport       WmanIf2MaxMcpGroupCid,

wmanIf2BsCapCfgPkmFlowControl           WmanIf2MaxPkmFlowType,

wmanIf2BsCapCfgAuthPolicyControl        WmanIf2AuthPolicyType,

wmanIf2BsCapCfgMaxNumOfSupportedSA      WmanIf2MaxNumOfSaType,

wmanIf2BsCapCfgIpVersion                WmanIf2IpVersionType,

wmanIf2BsCapCfgMacCsSupportBitMap       WmanIf2MacCsBitMap,

wmanIf2BsCapCfgMaxNumOfClassifier       WmanIf2MaxClassifiers,

wmanIf2BsCapCfgPhsSupport               WmanIf2PhsSupportType,

wmanIf2BsCapCfgBandwidthAllocSupport    WmanIf2BwAllocSupport,

wmanIf2BsCapCfgPduConstruction          WmanIf2PduConstruction,

wmanIf2BsCapCfgTtgTransitionGap         WmanIf2SsTransitionGap,

wmanIf2BsCapCfgRtgTransitionGap         WmanIf2SsTransitionGap,

wmanIf2BsCapCfgDownlinkCidSupport       WmanIf2NumOfCid,

wmanIf2BsCapCfgPackingSupport           WmanIf2PackingSupport,

wmanIf2BsCapCfgExtendedRtpsSupport      WmanIf2ExtRtpsSupport,

wmanIf2BsCapCfgMaxNumBurstToMs          INTEGER,

wmanIf2BsCapCfgIpAddrAllocMethod        WmanIf2IpAllocMethod,

wmanIf2BsCapCfgArqAckType               WmanIf2ArqAckType,

wmanIf2BsCapCfgMacHeader                WmanIf2MacHeaderSupp,

wmanIf2BsCapCfgMaxMacLevelDlFrame       WmanIf2MaxMacLevel,

wmanIf2BsCapCfgMaxMacLevelUlFrame       WmanIf2MaxMacLevel,

wmanIf2BsCapCfgNumOfProvisionedSf       Unsigned32,

wmanIf2BsCapCfgPkmVersion               WmanIf2PkmVersion,

wmanIf2BsCapCfgAuthPolicy               WmanIf2AuthPolicy,

wmanIf2BsCapCfgMacMode                  WmanIf2MacMode,

wmanIf2BsCapCfgPnWindowSize             INTEGER,

wmanIf2BsCapCfgExtCapability            WmanIf2ExtCapability}

wmanIf2BsCapCfgUplinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object shows the configured number of Uplink CIDs the

BS can support per SS."

::= { wmanIf2BsCapabilitiesConfigEntry 1 }

wmanIf2BsCapCfgArqSupport OBJECT-TYPE

SYNTAX      WmanIf2ArqSupportType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates whether the BS is configured to

support ARQ."

::= { wmanIf2BsCapabilitiesConfigEntry 2 }

wmanIf2BsCapCfgDsxFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxDsxFlowType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object specifies the configured maximum number of

concurrent DSA, DSC, or DSD transactions that BS allows

each SS to have outstanding."

DEFVAL		{ 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 3 }

wmanIf2BsCapCfgMacCrcSupport OBJECT-TYPE

SYNTAX      WmanIf2MacCrcSupport

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates whether BS is configured to support

MAC level CRC."

DEFVAL		{ macCrcSupport }

::= { wmanIf2BsCapabilitiesConfigEntry 4 }

wmanIf2BsCapCfgMcaFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxMcaFlowType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent

MCA transactions that BS is configured to allow each SS to

have."

DEFVAL		{ 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 5 }

wmanIf2BsCapCfgMcpGroupCidSupport OBJECT-TYPE

SYNTAX      WmanIf2MaxMcpGroupCid

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates the maximum number of simultaneous

Multicast Polling Groups the BS is configured to allow

each SS to belong to."

DEFVAL		{ 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 6 }

wmanIf2BsCapCfgPkmFlowControl OBJECT-TYPE

SYNTAX      WmanIf2MaxPkmFlowType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object specifies the maximum number of concurrent

PKM transactions that BS is configured to allow each SS

to have."

DEFVAL		{ 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 7 }

wmanIf2BsCapCfgAuthPolicyControl OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicyType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object specifies authorization policy that BS is

configured to be capable of. A bit value of 0 = 'not

supported', 1 = 'supported'. If this field is omitted,

then both SS and BS shall use the IEEE 802.16 security,

constituting X.509 digital certificates and the RSA

public key encryption algorithm, as authorization policy."

::= { wmanIf2BsCapabilitiesConfigEntry 8 }

wmanIf2BsCapCfgMaxNumOfSupportedSA OBJECT-TYPE

SYNTAX      WmanIf2MaxNumOfSaType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field specifies configured maximum number of supported

security association per SS."

DEFVAL		{ 1 }

::= { wmanIf2BsCapabilitiesConfigEntry 9 }

wmanIf2BsCapCfgIpVersion OBJECT-TYPE

SYNTAX      WmanIf2IpVersionType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates the configured version of IP that the

BS allows each SS to use on the 2nd Management Connection.

The value 'undefined' should not be used in this field."

::= { wmanIf2BsCapabilitiesConfigEntry 10 }

wmanIf2BsCapCfgMacCsSupportBitMap OBJECT-TYPE

SYNTAX      WmanIf2MacCsBitMap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates BS configured set of MAC convergence

sublayer support. When a bit is set, it indicates

the corresponding CS feature is supported."

::= { wmanIf2BsCapabilitiesConfigEntry 11 }

wmanIf2BsCapCfgMaxNumOfClassifier OBJECT-TYPE

SYNTAX      WmanIf2MaxClassifiers

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates the configured maximum number of

admitted Classifiers per SS that the BS can support."

DEFVAL      { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 12 }

wmanIf2BsCapCfgPhsSupport OBJECT-TYPE

SYNTAX      WmanIf2PhsSupportType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates the configured level of BS support

for PHS."

DEFVAL      { noPhsSupport }

::= { wmanIf2BsCapabilitiesConfigEntry 13 }

wmanIf2BsCapCfgBandwidthAllocSupport OBJECT-TYPE

SYNTAX      WmanIf2BwAllocSupport

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates configured properties of the BS for

bandwidth allocation purposes. The usage is defined by

WmanIf2CapBwAllocSupport."

::= { wmanIf2BsCapabilitiesConfigEntry 14 }

wmanIf2BsCapCfgPduConstruction OBJECT-TYPE

SYNTAX      WmanIf2PduConstruction

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Specifies configured capabilities for construction and

transmission of MAC PDUs. The usage is defined by

WmanIf2PduConstruction."

::= { wmanIf2BsCapabilitiesConfigEntry 15 }

wmanIf2BsCapCfgTtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the configured transition speed

SSTTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsCapabilitiesConfigEntry 16 }

wmanIf2BsCapCfgRtgTransitionGap OBJECT-TYPE

SYNTAX      WmanIf2SsTransitionGap

UNITS       "microsecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the configured transition speed

SSRTG for TDD and H-FDD SSs. The usage is defined by

WmanIf2SsTransitionGap."

::= { wmanIf2BsCapabilitiesConfigEntry 17 }

wmanIf2BsCapCfgDownlinkCidSupport OBJECT-TYPE

SYNTAX      WmanIf2NumOfCid

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object shows the number of Downlink CIDs the SS can

support."

::= { wmanIf2BsCapabilitiesConfigEntry 18 }

wmanIf2BsCapCfgPackingSupport OBJECT-TYPE

SYNTAX      WmanIf2PackingSupport

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the availability of MS support for Packing."

REFERENCE

"Subclause 11.7.8.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 19 }

wmanIf2BsCapCfgExtendedRtpsSupport OBJECT-TYPE

SYNTAX      WmanIf2ExtRtpsSupport

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the availability of SS support for extended

rtPs."

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 20 }

wmanIf2BsCapCfgMaxNumBurstToMs OBJECT-TYPE

SYNTAX      INTEGER (1..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of bursts transmitted concurrently to the MS

, including all bursts without CID or with CIDs matching

the MS CIDs."

REFERENCE

"Subclause 11.7.8.13 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 21 }

wmanIf2BsCapCfgIpAddrAllocMethod OBJECT-TYPE

SYNTAX      WmanIf2IpAllocMethod

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the method of allocating IP address for the

secondary management connection."

REFERENCE

"Subclause 11.7.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 22 }

wmanIf2BsCapCfgArqAckType OBJECT-TYPE

SYNTAX      WmanIf2ArqAckType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The value of this parameter specifies the ARQ ACK type

supported by the MS."

REFERENCE

"Subclause 11.7.23 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 23 }

wmanIf2BsCapCfgMacHeader OBJECT-TYPE

SYNTAX      WmanIf2MacHeaderSupp

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates whether or not the MS and BS support various

types of MAC header and extended subheaders."

REFERENCE

"Subclause 11.7.25 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 24 }

wmanIf2BsCapCfgMaxMacLevelDlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per DL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 25 }

wmanIf2BsCapCfgMaxMacLevelUlFrame OBJECT-TYPE

SYNTAX      WmanIf2MaxMacLevel

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum amount of MAC level data the MS is capable of

processing per UL frame. A value of 0 indicates such

limitation does not exist, except the limitation of

the physical medium"

REFERENCE

"Subclause 11.7.8.10.1 in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsCapabilitiesConfigEntry 26 }

wmanIf2BsCapCfgNumOfProvisionedSf OBJECT-TYPE

SYNTAX      Unsigned32 (0 .. 255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"When a BS is to transmit multiple DSA transactions for

provisioned service flows, this object indicates how many

DSA transactions with provisioned service flows will be

transmitted."

REFERENCE

"Subclause 11.7.19 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 27 }

wmanIf2BsCapCfgPkmVersion OBJECT-TYPE

SYNTAX      WmanIf2PkmVersion

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a PKM version that SS or BS supports."

REFERENCE

"Subclause 11.8.4.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 28 }

wmanIf2BsCapCfgAuthPolicy OBJECT-TYPE

SYNTAX      WmanIf2AuthPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates authorization policy used by the MS and BS to

negotiate and synchronize."

REFERENCE

"Subclause 11.8.4.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 29 }

wmanIf2BsCapCfgMacMode OBJECT-TYPE

SYNTAX      WmanIf2MacMode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates a MAC (Message Authentication Code) mode that MS

supports."

REFERENCE

"Subclause 11.8.4.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 30 }

wmanIf2BsCapCfgPnWindowSize OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the size capability of the receiver PN window for

SAs and management connections. The receiver shall track

PNs within this window to prevent replay attacks (see

7.5.1.2.4)."

REFERENCE

"Subclause 11.8.4.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 31 }

wmanIf2BsCapCfgExtCapability OBJECT-TYPE

SYNTAX      WmanIf2ExtCapability

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the extended subheader format support."

REFERENCE

"Subclause 11.8.6 in IEEE Std 802.16e-2005"

::= { wmanIf2BsCapabilitiesConfigEntry 32 }

wmanIf2BsSsActionsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsActionsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains all the actions specified for SSs in

the standard. The actions are routed down to SS using

unsolicited MAC messages: REG-RSP, DREG-REQ and RES-CMD.

The table also contains the parameters of the actions in

cases where they are specified by the standard."

::= { wmanIf2BsCm 7 }

wmanIf2BsSsActionsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsActionsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The action can be requested for SS in any state not only

those registered. However BS will decide whether the action

is applicable to the SS based on its current state and

execute it or skip it as defined in each action definition.

"

INDEX     { wmanIf2BsSsActionsMacAddress }

::= { wmanIf2BsSsActionsTable 1 }

WmanIf2BsSsActionsEntry ::= SEQUENCE {

wmanIf2BsSsActionsMacAddress            MacAddress,

wmanIf2BsSsActionsResetSs               INTEGER,

wmanIf2BsSsActionsAbortSs               INTEGER,

wmanIf2BsSsActionsOverrideDnFreq        Unsigned32,

wmanIf2BsSsActionsOverrideChannelId     INTEGER,

wmanIf2BsSsActionsDeReRegSs             INTEGER,

wmanIf2BsSsActionsDeReRegSsCode         INTEGER,

wmanIf2BsSsActionsRowStatus             RowStatus}

wmanIf2BsSsActionsMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This object uniquely identifies the SS as an action

target."

::= { wmanIf2BsSsActionsEntry 1 }

wmanIf2BsSsActionsResetSs OBJECT-TYPE

SYNTAX      INTEGER {actionsResetSsNoAction(0),

actionsResetSs(1)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object should be implemented as follows:

- When set to actionsResetSs value, instructs BS to send

RES-CMD to SS

- When set to value different than actionsResetSs it

should be ignored

- When read it should return actionsResetSsNoAction

The RES-CMD message shall be transmitted by the BS on an

SS Basic CID to force the SS to reset itself,

reinitialize its MAC, and repeat initial system access."

REFERENCE

"Subclause 6.3.2.3.22 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsActionsEntry 2 }

wmanIf2BsSsActionsAbortSs OBJECT-TYPE

SYNTAX      INTEGER {actionsAbortSsNoAction(0),

actionsAbortSs(1),

actionAbortSsParams(2)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object should be implemented as follows:

- When set to actionsAbortSs value, it instructs BS to send

unsolicited RNG-RSP with Ranging Status equal to 'abort'

without override parameters

- When set to actionAbortSsParams value, it instructs BS to

send unsolicited RNG-RSP with Ranging Status equal to

'abort' and with 'Downlink Frequency Override' and

'Uplink Channel ID Override' parameters.

- When set to any other value it should be ignored

- When read it should returned actionsAbortSsNoAction"

REFERENCE

"Subclause 11.6, Table 365 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsActionsEntry 3 }

wmanIf2BsSsActionsOverrideDnFreq OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "kHz"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used as a parmeter of the AbortSs action

with the code actionAbortSsParams. It is used for licensed

bands only. It defines the Center frequency, in kHz, of

new downlink channel where the SS should redo initial

ranging."

REFERENCE

"Subclause 11.6, Table 365 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsActionsEntry 4 }

wmanIf2BsSsActionsOverrideChannelId OBJECT-TYPE

SYNTAX      INTEGER (0..199)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used as a parmeter of the AbortSs action

with the code actionAbortSsParams. It is coded as follows:

- Licensed bands: The identifier of the uplink channel

with which the SS is to redo initial ranging (not used

with PHYs without channelized uplinks).

- License-exempt bands: The Channel Nr (see 8.5.1) where

the SS should redo initial ranging."

REFERENCE

"Subclause 11.6, Table 365 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsActionsEntry 5 }

wmanIf2BsSsActionsDeReRegSs OBJECT-TYPE

SYNTAX      INTEGER {actionsDeReRegSsNoAction(0),

actionsDeReRegSs(1)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object should be implemented as follows:

- When set to actionsDeReRegSs value, instructs BS to

send DREG-CMD to SS with specified action code

- When set to value different than actionsDeReRegSs it

should be ignored

- When read it should return actionsDeReRegSsNoAction

The DREG-CMD message shall be transmitted by the BS on an

SS Basic CID to force the SS to change its access state.

Upon receiving a DREG-CMD, the SS shall take the action

indicated by the action code defined by

wmanIf2BsSsActionsDeReRegSsCode."

REFERENCE

"Subclause 6.3.2.3.26 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsActionsEntry 6 }

wmanIf2BsSsActionsDeReRegSsCode OBJECT-TYPE

SYNTAX      INTEGER {actionsDeReRegSsCodeChangeChan(0),

actionsDeReRegSsCodeNoTransmit(1),

actionsDeReRegSsCodeLtdTransmit(2),

actionsDeReRegSsCodeResume(3)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object defines the action code for

wmanIf2BsSsActionsDeReRegSs action. The codes are defined

as follows:

actionsDeReRegSsCodeChangeChan - SS shall leave the

current channel and attempt to access another channel.

actionsDeReRegSsCodeNoTransmit - SS shall listen to the

current channel but shall not transmit until an

RES-CMD message or DREG_CMD with an Action Code that

allows transmission is received.

actionsDeReRegSsCodeLtdTransmit - SS shall listen to the

current channel but only transmit on the Basic,

Primary Management and 2nd Management Connections.

actionsDeReRegSsCodeResume - SS shall return to normal

operation and may transmit on any of its active

connections."

REFERENCE

"Subclause 6.3.2.3.26, Table 55 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsActionsEntry 7 }

wmanIf2BsSsActionsRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to ensure that the write operation to

multiple columns is guaranteed to be treated as atomic

operation by agent."

::= { wmanIf2BsSsActionsEntry 8 }

wmanIf2BsMulticastPollingTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsMulticastPollingEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the multicast polling group information

. BS can send MCA-REQ message to assign/remove a SS to/from

a multicast polling group. An entry is created when a SS is

assigned to a multicast polling group; and deleted when a

SS is removed from a multicast polling group."

REFERENCE

"Subclause 6.3.2.3.18 in IEEE Std 802.16-2004"

::= { wmanIf2BsCm 8 }

wmanIf2BsMulticastPollingEntry OBJECT-TYPE

SYNTAX      WmanIf2BsMulticastPollingEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { wmanIf2BsMulticastPollingCid,

wmanIf2BsSsMacAddress }

::= { wmanIf2BsMulticastPollingTable 1 }

WmanIf2BsMulticastPollingEntry ::= SEQUENCE {

wmanIf2BsMulticastPollingCid            WmanIf2CidType,

wmanIf2BsMulticastGroupType             INTEGER,

wmanIf2BsPeriodAllocationParameterM     INTEGER,

wmanIf2BsPeriodAllocationParameterK     INTEGER,

wmanIf2BsPeriodAllocationParameterN     INTEGER,

wmanIf2BsPeriodicAllocationType         INTEGER}

wmanIf2BsMulticastPollingCid OBJECT-TYPE

SYNTAX      WmanIf2CidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 16 bit channel identifier points to the connection being

created by DSA for this service flow. When '0' is returned

from reading this object, it means no CID has been assigned

to this service flow yet."

::= { wmanIf2BsMulticastPollingEntry 1 }

wmanIf2BsMulticastGroupType OBJECT-TYPE

SYNTAX      INTEGER {regular(0),

aas(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Multicast group type."

REFERENCE

"Subclause 11.10, Table 382 in IEEE Std 802.16-2004"

DEFVAL         { 0 }

::= { wmanIf2BsMulticastPollingEntry 2 }

wmanIf2BsPeriodAllocationParameterM OBJECT-TYPE

SYNTAX      INTEGER ( 0 .. 255 )

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Periodic allocation parameter = 'm'

Parameters m, k have the following meaning: multicast group

gets a multicast polling allocation at the end of the frame

#N if N mod k = m; size of the allocation is n."

REFERENCE

"Subclause 11.10, Table 382 in IEEE Std 802.16-2004"

::= { wmanIf2BsMulticastPollingEntry 3 }

wmanIf2BsPeriodAllocationParameterK OBJECT-TYPE

SYNTAX      INTEGER ( 0 .. 255 )

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Periodic allocation parameter = 'k'

Parameters m, k have the following meaning: multicast group

gets a multicast polling allocation at the end of the frame

#N if N mod k = m; size of the allocation is n."

REFERENCE

"Subclause 11.10, Table 382 in IEEE Std 802.16-2004"

::= { wmanIf2BsMulticastPollingEntry 4 }

wmanIf2BsPeriodAllocationParameterN OBJECT-TYPE

SYNTAX      INTEGER ( 0 .. 255 )

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Periodic allocation parameter = 'n'

Parameters m, k have the following meaning: multicast group

gets a multicast polling allocation at the end of the frame

#N if N mod k = m; size of the allocation is n."

REFERENCE

"Subclause 11.10, Table 382 in IEEE Std 802.16-2004"

::= { wmanIf2BsMulticastPollingEntry 5 }

wmanIf2BsPeriodicAllocationType OBJECT-TYPE

SYNTAX      INTEGER {reqRegionFull(0),

regRegionFocused(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Periodic allocation type. Applicable for OFDM PHY only."

REFERENCE

"Subclause 11.10, Table 382 in IEEE Std 802.16-2004"

::= { wmanIf2BsMulticastPollingEntry 6 }

--

-- Base station PHY Group

--

wmanIf2BsPhy OBJECT IDENTIFIER ::= { wmanIf2BsCm 9 }

--

-- BS Common PHY objects

--

wmanIf2BsCmnPhy OBJECT IDENTIFIER ::= { wmanIf2BsPhy 1 }

wmanIf2BsCmnPhyUplinkChannelTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsCmnPhyUplinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains common channel attributes that

characterize the uplink channels"

REFERENCE

"Table 349 and Table 352, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhy 1 }

wmanIf2BsCmnPhyUplinkChannelEntry OBJECT-TYPE

SYNTAX      WmanIf2BsCmnPhyUplinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each uplink channel of

multi-sector BS. An entry in this table exists for each

ifEntry of BS with an ifType of ieee80216WMAN."

INDEX     { ifIndex }

::= { wmanIf2BsCmnPhyUplinkChannelTable 1 }

WmanIf2BsCmnPhyUplinkChannelEntry ::= SEQUENCE {

wmanIf2BsCmnPhyCtBasedResvTimeout       INTEGER,

wmanIf2BsCmnPhyUplinkCenterFreq         Unsigned32}

wmanIf2BsCmnPhyCtBasedResvTimeout OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The number of UL-MAPs to receive before contention-based

reservation is attempted again for the same connection."

REFERENCE

"Table 349, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyUplinkChannelEntry 1 }

wmanIf2BsCmnPhyUplinkCenterFreq OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "kHz"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

" Uplink center frequency (kHz)"

REFERENCE

"Table 349, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyUplinkChannelEntry 2 }

wmanIf2BsCmnPhyDownlinkChannelTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsCmnPhyDownlinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains common channel attributes that

charaterize the downlink channels"

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhy 2 }

wmanIf2BsCmnPhyDownlinkChannelEntry OBJECT-TYPE

SYNTAX      WmanIf2BsCmnPhyDownlinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each downlink channel of

multi-sector BS. An entry in this table exists for each

ifEntry of BS with an ifType of ieee80216WMAN."

INDEX     { ifIndex }

::= { wmanIf2BsCmnPhyDownlinkChannelTable 1 }

WmanIf2BsCmnPhyDownlinkChannelEntry ::= SEQUENCE {

wmanIf2BsCmnPhyBsEIRP                   INTEGER,

wmanIf2BsCmnPhyChannelNumber            WmanIf2ChannelNumber,

wmanIf2BsCmnPhyTTG                      INTEGER,

wmanIf2BsCmnPhyRTG                      INTEGER,

wmanIf2BsCmnPhyInitRngMaxRSS            INTEGER,

wmanIf2BsCmnPhyDownlinkCenterFreq       Unsigned32,

wmanIf2BsCmnPhyBsId                     WmanIf2BsIdType,

wmanIf2BsCmnPhyMacVersion               WmanIf2MacVersion,

wmanIf2BsCmnPhyCyclicPrefix             INTEGER}

wmanIf2BsCmnPhyBsEIRP OBJECT-TYPE

SYNTAX      INTEGER (-32768..32767)

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The EIRP is the equivalent isotropic radiated power of

the base station, which is computed for a simple

single-antenna transmitter."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 1 }

wmanIf2BsCmnPhyChannelNumber OBJECT-TYPE

SYNTAX      WmanIf2ChannelNumber

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Downlink channel number as defined in 8.5.

Used for license-exempt operation only."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 2 }

wmanIf2BsCmnPhyTTG OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Transmit / Receive Transition Gap."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 3 }

wmanIf2BsCmnPhyRTG OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Receive / Transmit Transition Gap."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 4 }

wmanIf2BsCmnPhyInitRngMaxRSS OBJECT-TYPE

SYNTAX      INTEGER (-32768..32767)

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial Ranging Max. equivalent isotropic received power

at BS Signed in units of 1 dBm."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 5 }

wmanIf2BsCmnPhyDownlinkCenterFreq OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "kHz"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Downlink center frequency (kHz)."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 6 }

wmanIf2BsCmnPhyBsId OBJECT-TYPE

SYNTAX      WmanIf2BsIdType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Base station ID."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 7 }

wmanIf2BsCmnPhyMacVersion OBJECT-TYPE

SYNTAX      WmanIf2MacVersion

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This parameter specifies the version of 802.16 to which

the message originator conforms."

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 8 }

wmanIf2BsCmnPhyCyclicPrefix OBJECT-TYPE

SYNTAX INTEGER {ratio1To4(0),

ratio1To8(1),

ratio1To16(2),

ratio1To32(3)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The ratio of CP time to 'useful' time.Values

are 1/4, 1/8, 1/16 or 1/32."

REFERENCE

"Subclause 8.3.1.1.1 in IEEE Std 802.16-2004"

DEFVAL      { ratio1To4 }

::= { wmanIf2BsCmnPhyDownlinkChannelEntry 9 }

--

-- BS OFDM PHY objects

--

wmanIf2BsOfdmPhy OBJECT IDENTIFIER ::= { wmanIf2BsPhy 2 }

wmanIf2BsOfdmUplinkChannelTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmUplinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains UCD channel attributes, defining the

transmission characteristics of uplink channels"

REFERENCE

"Table 349 and Table 352, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmPhy 1 }

wmanIf2BsOfdmUplinkChannelEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmUplinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each uplink channel of

multi-sector BS. An entry in this table exists for each

ifEntry of BS with an ifType of ieee80216WMAN."

INDEX     { ifIndex }

::= { wmanIf2BsOfdmUplinkChannelTable 1 }

WmanIf2BsOfdmUplinkChannelEntry ::= SEQUENCE {

wmanIf2BsOfdmBwReqOppSize               INTEGER,

wmanIf2BsOfdmRangReqOppSize             INTEGER,

wmanIf2BsOfdmNumSubChReqRegionFull      INTEGER,

wmanIf2BsOfdmNumSymbolsReqRegionFull    INTEGER,

wmanIf2BsOfdmSubChFocusCtCode           INTEGER}

wmanIf2BsOfdmBwReqOppSize OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "PS"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Size (in units of PS) of PHY payload that SS may use to

format and transmit a bandwidth request message in a

contention request opportunity. The value includes all

PHY overhead as well as allowance for the MAC data the

message may hold."

REFERENCE

"Table 349, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUplinkChannelEntry 1 }

wmanIf2BsOfdmRangReqOppSize OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "PS"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Size (in units of PS) of PHY payload that SS may use to

format and transmit a RNG-REQ message in a contention

request opportunity. The value includes all PHY overhead

as well as allowance for the MAC data the message may

hold and the maxiumum SS/BS roundtrip propagation delay."

REFERENCE

"Table 349, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUplinkChannelEntry 2 }

wmanIf2BsOfdmNumSubChReqRegionFull OBJECT-TYPE

SYNTAX      INTEGER {oneSubchannel(0),

twoSubchannels(1),

fourSubchannels(2),

eightSubchannels(3),

sixteenSubchannels(4)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of subchannels used by each transmit opportunity

when REQ Region-Full is allocated in subchannelization

region."

REFERENCE

"Table 352, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUplinkChannelEntry 3 }

wmanIf2BsOfdmNumSymbolsReqRegionFull OBJECT-TYPE

SYNTAX      INTEGER (0..31)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of OFDM symbols used by each transmit

opportunity when REQ Region-Full is allocated in

subchannelization region."

REFERENCE

"Table 352, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUplinkChannelEntry 4 }

wmanIf2BsOfdmSubChFocusCtCode OBJECT-TYPE

SYNTAX      INTEGER (0..8)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of contention codes (CSE) that shall only be used to

request a subchannelized allocation. Default value 0.

Allowed values 0-8."

REFERENCE

"Table 352, in IEEE Std 802.16-2004"

DEFVAL         { 0 }

::= { wmanIf2BsOfdmUplinkChannelEntry 5 }

wmanIf2BsOfdmDownlinkChannelTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmDownlinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains DCD channel attributes, defining the

transmission characteristics of downlink channels"

REFERENCE

"Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmPhy 2 }

wmanIf2BsOfdmDownlinkChannelEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmDownlinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each downlink channel of

multi-sector BS. An entry in this table exists for each

ifEntry of BS with an ifType of ieee80216WMAN."

INDEX     { ifIndex }

::= { wmanIf2BsOfdmDownlinkChannelTable 1 }

WmanIf2BsOfdmDownlinkChannelEntry ::= SEQUENCE {

wmanIf2BsOfdmFrameDurationCode          INTEGER,

wmanIf2BsOfdmFftSize                    WmanIf2OfdmFftSize}

wmanIf2BsOfdmFrameDurationCode OBJECT-TYPE

SYNTAX      INTEGER {duration2dot5ms(0),

duration4ms(1),

duration5ms(2),

duration8ms(3),

duration10ms(4),

duration12dot5ms(5),

duration20ms(6)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration of the frame. The frame duration code

values are specified in Table 230."

REFERENCE

"Subclause 11.4.1, Table 230, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDownlinkChannelEntry 1 }

wmanIf2BsOfdmFftSize OBJECT-TYPE

SYNTAX      WmanIf2OfdmFftSize

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the channel BW in units of 125 kHz

for OFDM PHY."

REFERENCE

"Subclause 11.8.3.6.1, IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDownlinkChannelEntry 2 }

wmanIf2BsOfdmUcdBurstProfileTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmUcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains UCD burst profiles for each uplink

channel"

REFERENCE

"Table 356, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmPhy 3 }

wmanIf2BsOfdmUcdBurstProfileEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmUcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each UCD burst profile."

INDEX     { ifIndex, wmanIf2BsOfdmUiucIndex }

::= { wmanIf2BsOfdmUcdBurstProfileTable 1 }

WmanIf2BsOfdmUcdBurstProfileEntry ::= SEQUENCE {

wmanIf2BsOfdmUiucIndex                  INTEGER,

wmanIf2BsOfdmUcdFecCodeType             WmanIf2OfdmFecCodeType,

wmanIf2BsOfdmFocusCtPowerBoost          INTEGER,

wmanIf2BsOfdmUcdTcsEnable               INTEGER,

wmanIf2BsOfdmUcdBurstProfileRowStatus   RowStatus}

wmanIf2BsOfdmUiucIndex OBJECT-TYPE

SYNTAX      INTEGER (5 .. 12)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The Uplink Interval Usage Code indicates the uplink burst

profile in the UCD message, and is used along with ifIndex

to identify an entry in the

wmanIf2BsOfdmUcdBurstProfileTable."

REFERENCE

"Subclause 8.3.6.3.1, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUcdBurstProfileEntry 1 }

wmanIf2BsOfdmUcdFecCodeType OBJECT-TYPE

SYNTAX      WmanIf2OfdmFecCodeType

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Uplink FEC code type and modulation type"

REFERENCE

"Table 356, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUcdBurstProfileEntry 2 }

wmanIf2BsOfdmFocusCtPowerBoost OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The power boost in dB of focused contention carriers, as

described in 8.3.6.3.3."

REFERENCE

"Table 356, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUcdBurstProfileEntry 3 }

wmanIf2BsOfdmUcdTcsEnable OBJECT-TYPE

SYNTAX      INTEGER {tcsDisabled(0),

tcsEnabled(1)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This parameter determines the transmission convergence

sublayer, as described in 8.1.4.3, can be enabled on a

per-burst basis for both uplink and downlink. through

DIUC/UIUC messages."

REFERENCE

"Table 356, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmUcdBurstProfileEntry 4 }

wmanIf2BsOfdmUcdBurstProfileRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or

delete an existing row in this table.

If the implementator of this MIB has choosen not

to implement 'dynamic assignment' of profiles, this

object is not useful and should return noSuchName

upon SNMP request."

::= { wmanIf2BsOfdmUcdBurstProfileEntry 5 }

wmanIf2BsOfdmDcdBurstProfileTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmDcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each DCD burst profile."

REFERENCE

"Table 362, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmPhy 4 }

wmanIf2BsOfdmDcdBurstProfileEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmDcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each DCD burst profile."

INDEX     { ifIndex, wmanIf2BsOfdmDiucIndex }

::= { wmanIf2BsOfdmDcdBurstProfileTable 1 }

WmanIf2BsOfdmDcdBurstProfileEntry ::= SEQUENCE {

wmanIf2BsOfdmDiucIndex                  INTEGER,

wmanIf2BsOfdmDownlinkFrequency          Unsigned32,

wmanIf2BsOfdmDcdFecCodeType             WmanIf2OfdmFecCodeType,

wmanIf2BsOfdmDiucMandatoryExitThresh    INTEGER,

wmanIf2BsOfdmDiucMinEntryThresh         INTEGER,

wmanIf2BsOfdmTcsEnable                  INTEGER,

wmanIf2BsOfdmDcdBurstProfileRowStatus   RowStatus}

wmanIf2BsOfdmDiucIndex OBJECT-TYPE

SYNTAX      INTEGER (1..11)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The Downlink Interval Usage Code indicates the downlink

burst profile in the DCD message, and is used along with

ifIndex to identify an entry in the

wmanIf2BsOfdmDcdBurstProfileTable."

REFERENCE

"Subclause 8.3.6.3.1, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDcdBurstProfileEntry 1 }

wmanIf2BsOfdmDownlinkFrequency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "kHz"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Downlink Frequency (kHz)."

REFERENCE

"Table 359, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDcdBurstProfileEntry 2 }

wmanIf2BsOfdmDcdFecCodeType OBJECT-TYPE

SYNTAX      WmanIf2OfdmFecCodeType

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Downlink FEC code type and modulation type"

REFERENCE

"Table 362, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDcdBurstProfileEntry 3 }

wmanIf2BsOfdmDiucMandatoryExitThresh OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"DIUC mandatory exit threshold: 0 - 63.75 dB CINR at or

below where this DIUC can no longer be used and where this

change to a more robust DIUC is required in 0.25 dB units."

REFERENCE

"Table 362, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDcdBurstProfileEntry 4 }

wmanIf2BsOfdmDiucMinEntryThresh OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"DIUC minimum entry threshold: 0 - 63.75 dB The minimum CINR

required to start using this DIUC when changing from a more

robust DIUC is required, in 0.25 dB units."

REFERENCE

"Table 362, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDcdBurstProfileEntry 5 }

wmanIf2BsOfdmTcsEnable OBJECT-TYPE

SYNTAX      INTEGER {tcsDisabled (0),

tcsEnabled (1)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Indicates whether Transmission COnvergence Sublayer

is enabled or disabled."

REFERENCE

"Table 362, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmDcdBurstProfileEntry 6 }

wmanIf2BsOfdmDcdBurstProfileRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or

delete an existing row in this table.

If the implementator of this MIB has choosen not

to implement 'dynamic assignment' of profiles, this

object is not useful and should return noSuchName

upon SNMP request."

::= { wmanIf2BsOfdmDcdBurstProfileEntry 7 }

wmanIf2BsOfdmConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains BS configuration objects, specific to

OFDM PHY."

::= { wmanIf2BsOfdmPhy 5 }

wmanIf2BsOfdmConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2BsOfdmConfigurationTable 1 }

WmanIf2BsOfdmConfigurationEntry ::= SEQUENCE {

wmanIf2BsOfdmMinReqRegionFullTxOpp      INTEGER,

wmanIf2BsOfdmMinFocusedCtTxOpp          INTEGER,

wmanIf2BsOfdmMaxRoundTripDelay          INTEGER,

wmanIf2BsOfdmRangeAbortTimingThold      INTEGER,

wmanIf2BsOfdmRangeAbortPowerThold       INTEGER,

wmanIf2BsOfdmRangeAbortFreqThold        INTEGER,

wmanIf2BsOfdmDnlkRateId                 INTEGER}

wmanIf2BsOfdmMinReqRegionFullTxOpp OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "n per second"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The minimum number of Full bandwidth Req-Region Full

Transmit opportunities scheduled in the UL per second."

REFERENCE

"Subclause 6.3.7.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmConfigurationEntry 1 }

wmanIf2BsOfdmMinFocusedCtTxOpp OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

UNITS       "1/sec"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The minimum number of focused contention Transmit

opportunities scheduled in the UL per second. The value may

be 0 if the focused contention is not implemented."

REFERENCE

"Subclauses 6.3.6.4 and 8.3.7.3.3 in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmConfigurationEntry 2 }

wmanIf2BsOfdmMaxRoundTripDelay OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "microsecond"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum supported round trip delay.

It is required to limit the cell size."

REFERENCE

"Subclause 8.3.5.1 in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmConfigurationEntry 3 }

wmanIf2BsOfdmRangeAbortTimingThold OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "1/Fs"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines Tolerable Timing Offset. BS performs

Initial Ranging until the SS transmissions are within

limits that are deemed tolerable by the BS. If the SS does

not transmit within these limits after a number of

correction attempts then the BS aborts Initial Ranging."

REFERENCE

"Figure 63 and Table 365 in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmConfigurationEntry 4 }

wmanIf2BsOfdmRangeAbortPowerThold OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "0.25dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines Tolerable Power Offset. BS performs

Initial Ranging until the SS transmissions are within

limits that are deemed tolerable by the BS. If the SS does

not transmit within these limits after a number of

correction attempts then the BS aborts Initial Ranging."

REFERENCE

"Figure 63 and Table 365 in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmConfigurationEntry 5 }

wmanIf2BsOfdmRangeAbortFreqThold OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "Hz"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines Tolerable Frequency Offset. BS performs

Initial Ranging until the SS transmissions are within

limits that are deemed tolerable by the BS. If the SS does

not transmit within these limits after a number of

correction attempts then the BS aborts Initial Ranging."

REFERENCE

"Figure 63 and Table 365 in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmConfigurationEntry 6 }

wmanIf2BsOfdmDnlkRateId OBJECT-TYPE

SYNTAX      INTEGER {dnlkRateIdBpsk1Over2(0),

dnlkRateIdQpsk1Over2(1),

dnlkRateIdQpsk3Over4(2),

dnlkRateId16Qam1Over2(3),

dnlkRateId16Qam3Over4(4),

dnlkRateId64Qam2Over3(5),

dnlkRateId64Qam3Over4(6)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The Rate ID to be used in the first downlink burst

immediately following the FCH. The Rate ID encoding is

static and cannot be changed during system operation. The

change of the Rate ID should be applied on system

re-intialisation (e.g. following sector or BS reset)."

REFERENCE

"Subclause 8.3.3.4.3 in IEEE Std 802.16-2004"

DEFVAL      { dnlkRateIdBpsk1Over2 }

::= { wmanIf2BsOfdmConfigurationEntry 7 }

wmanIf2BsSsOfdmReqCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsOfdmReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information,

specific to OFDM PHY, of SSs that have been reported by

SSs to BS using RNG-REQ, SBC-REQ and REG-REQ messages.

Entries in this table should be created when an SS

registers with a BS."

::= { wmanIf2BsOfdmPhy 6 }

wmanIf2BsSsOfdmReqCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsOfdmReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. This table augments the table

wmanIf2BsRegisteredSsTable."

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsOfdmReqCapabilitiesTable 1 }

WmanIf2BsSsOfdmReqCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsSsOfdmReqCapFftSizes           WmanIf2OfdmFftSizes,

wmanIf2BsSsOfdmReqCapSsDemodulator      WmanIf2OfdmSsDeModType,

wmanIf2BsSsOfdmReqCapSsModulator        WmanIf2OfdmSsModType,

wmanIf2BsSsOfdmReqCapFocusedCtSupport   WmanIf2OfdmFocusedCt,

wmanIf2BsSsOfdmReqCapTcSublayerSupport  WmanIf2OfdmTcSublayer,

wmanIf2BsSsOfdmReqCapPrivteMapSupport   WmanIf2OfdmPrivMap,

wmanIf2BsSsOfdmReqCapUlPowerControl     WmanIf2OfdmUlPower,

wmanIf2BsSsOfdmReqCapLoopPwrControlSw   Unsigned32}

wmanIf2BsSsOfdmReqCapFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmFftSizes

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes supported by SS.

The usage is defined by WmanIf2OfdmFftSizes."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 1 }

wmanIf2BsSsOfdmReqCapSsDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsDeModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

supported by SS for downlink.

The usage is defined by WmanIf2OfdmSsDeModType."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 2 }

wmanIf2BsSsOfdmReqCapSsModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

supported by SS for uplink.

The usage is defined by WmanIf2OfdmSsModType."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 3 }

wmanIf2BsSsOfdmReqCapFocusedCtSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmFocusedCt

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates whether the SS supports Focused

Contention. The usage is defined by

WmanIf2OfdmFocusedCt."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 4 }

wmanIf2BsSsOfdmReqCapTcSublayerSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmTcSublayer

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates whether or not the SS supports

the TC sublayer. The usage is defined by

WmanIf2OfdmTcSublayer."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 5 }

wmanIf2BsSsOfdmReqCapPrivteMapSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmPrivMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates if the private map parameters

are supported."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 6 }

wmanIf2BsSsOfdmReqCapUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmUlPower

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the uplink power control options

supported by SS."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 7 }

wmanIf2BsSsOfdmReqCapLoopPwrControlSw OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsSsOfdmReqCapabilitiesEntry 8 }

wmanIf2BsSsOfdmRspCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsOfdmRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information,

specific to OFDM PHY, of SSs that have been negotiated

and agreed between BS and SS via RNG-REQ/RSP,

SBC-REQ/RSP and REG-REQ/RSP messages. This table

augments the wmanIf2BsRegisteredSsTable."

REFERENCE

"Subclause 6.3.2.3.7 in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmPhy 7 }

wmanIf2BsSsOfdmRspCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsOfdmRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has been

registered in the BS. This table augments the

wmanIf2BsRegisteredSsTable. "

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsOfdmRspCapabilitiesTable 1 }

WmanIf2BsSsOfdmRspCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsSsOfdmRspCapFftSizes           WmanIf2OfdmFftSizes,

wmanIf2BsSsOfdmRspCapSsDemodulator      WmanIf2OfdmSsDeModType,

wmanIf2BsSsOfdmRspCapSsModulator        WmanIf2OfdmSsModType,

wmanIf2BsSsOfdmRspCapFocusedCtSupport   WmanIf2OfdmFocusedCt,

wmanIf2BsSsOfdmRspCapTcSublayerSupport  WmanIf2OfdmTcSublayer,

wmanIf2BsSsOfdmRspCapPrivteMapSupport   WmanIf2OfdmPrivMap,

wmanIf2BsSsOfdmRspCapUlPowerControl     WmanIf2OfdmUlPower,

wmanIf2BsSsOfdmRspCapLoopPwrControlSw   Unsigned32}

wmanIf2BsSsOfdmRspCapFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmFftSizes

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes negotiated with the

SS. The usage is defined by WmanIf2OfdmFftSizes."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 1 }

wmanIf2BsSsOfdmRspCapSsDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsDeModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

negotiated for SS for downlink. The usage is defined by

WmanIf2OfdmSsDeModType."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 2 }

wmanIf2BsSsOfdmRspCapSsModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

negotiated for SS for uplink. The usage is defined by

WmanIf2OfdmSsModType."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 3 }

wmanIf2BsSsOfdmRspCapFocusedCtSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmFocusedCt

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates whether the SS has negotiated the

support for Focused Contention. The usage is defined by

WmanIf2OfdmFocusedCt."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 4 }

wmanIf2BsSsOfdmRspCapTcSublayerSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmTcSublayer

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates whether the SS has negotiated

support for the TC sublayer. The usage is defined by

WmanIf2OfdmTcSublayer."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 5 }

wmanIf2BsSsOfdmRspCapPrivteMapSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmPrivMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field iindicates if the private map parameters

are supported."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 6 }

wmanIf2BsSsOfdmRspCapUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmUlPower

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the uplink power control options

supported by SS."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 7 }

wmanIf2BsSsOfdmRspCapLoopPwrControlSw OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsSsOfdmRspCapabilitiesEntry 8 }

wmanIf2BsOfdmCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capabilities, specific to

OFDM PHY, of the BS as implemented in BS hardware and

software. These capabilities along with the configuration

for them (wmanIf2BsOfdmCapabilitiesConfigTable) are used

for negotiation of basic capabilities with SS using

RNG-RSP, SBC-RSP and REG-RSP messages. The negotiated

capabilities are obtained by interSubclause of SS raw

reported capabilities, BS raw capabilities and BS

configured capabilities. The objects in the table have

read-only access. The table is maintained by BS."

::= { wmanIf2BsOfdmPhy 8 }

wmanIf2BsOfdmCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2BsOfdmCapabilitiesTable 1 }

WmanIf2BsOfdmCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsOfdmCapFftSizes                WmanIf2OfdmFftSizes,

wmanIf2BsOfdmCapSsDemodulator           WmanIf2OfdmSsDeModType,

wmanIf2BsOfdmCapSsModulator             WmanIf2OfdmSsModType,

wmanIf2BsOfdmCapFocusedCtSupport        WmanIf2OfdmFocusedCt,

wmanIf2BsOfdmCapTcSublayerSupport       WmanIf2OfdmTcSublayer,

wmanIf2BsOfdmCapPrivteMapSupport        WmanIf2OfdmPrivMap,

wmanIf2BsSsOfdmCapUlPowerControl        WmanIf2OfdmUlPower,

wmanIf2BsSsOfdmCapLoopPwrControlSw      Unsigned32}

wmanIf2BsOfdmCapFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmFftSizes

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes supported by the BS.

The usage is defined by WmanIf2OfdmFftSizes."

::= { wmanIf2BsOfdmCapabilitiesEntry 1 }

wmanIf2BsOfdmCapSsDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsDeModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different BS demodulator options

for uplink supported by the BS. The usage is defined by

WmanIf2OfdmSsDeModType."

::= { wmanIf2BsOfdmCapabilitiesEntry 2 }

wmanIf2BsOfdmCapSsModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different BS modulator options

for downlink supported by the BS. The usage is defined by

WmanIf2OfdmSsModType."

::= { wmanIf2BsOfdmCapabilitiesEntry 3 }

wmanIf2BsOfdmCapFocusedCtSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmFocusedCt

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the BS support for Focused

Contention. The usage is defined by

WmanIf2OfdmFocusedCt."

::= { wmanIf2BsOfdmCapabilitiesEntry 4 }

wmanIf2BsOfdmCapTcSublayerSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmTcSublayer

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the BS supports for TC sublayer. The

usage is defined by WmanIf2OfdmTcSublayer."

::= { wmanIf2BsOfdmCapabilitiesEntry 5 }

wmanIf2BsOfdmCapPrivteMapSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmPrivMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field iindicates if the private map parameters

are supported."

::= { wmanIf2BsOfdmCapabilitiesEntry 6 }

wmanIf2BsSsOfdmCapUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmUlPower

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the uplink power control options

supported by SS."

::= { wmanIf2BsOfdmCapabilitiesEntry 7 }

wmanIf2BsSsOfdmCapLoopPwrControlSw OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsOfdmCapabilitiesEntry 8 }

wmanIf2BsOfdmCapabilitiesConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration for basic

capabilities of BS, specific to OFDM PHY. The table is

intended to be used to restrict the Capabilities

implemented by BS, for example in order to comply with

local regulatory requirements. The BS should use the

configuration along with the implemented Capabilities

(wmanIf2BsOfdmPhyTable) for negotiation of basic

capabilities with SS using RNG-RSP, SBC-RSP and REG-RSP

messages. The negotiated capabilities are obtained by

interSubclause of SS reported capabilities, BS raw

capabilities and BS configured capabilities. The objects

in the table have read-write access. The rows are created

by BS as a copy of wmanIf2BsBasicCapabilitiesTable

and can be modified by NMS."

::= { wmanIf2BsOfdmPhy 9 }

wmanIf2BsOfdmCapabilitiesConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2BsOfdmCapabilitiesConfigTable 1 }

WmanIf2BsOfdmCapabilitiesConfigEntry ::= SEQUENCE {

wmanIf2BsOfdmCapCfgFftSizes             WmanIf2OfdmFftSizes,

wmanIf2BsOfdmCapCfgSsDemodulator        WmanIf2OfdmSsDeModType,

wmanIf2BsOfdmCapCfgSsModulator          WmanIf2OfdmSsModType,

wmanIf2BsOfdmCapCfgFocusedCtSupport     WmanIf2OfdmFocusedCt,

wmanIf2BsOfdmCapCfgTcSublayerSupport    WmanIf2OfdmTcSublayer,

wmanIf2BsOfdmCapCfgPrivteMapSupport     WmanIf2OfdmPrivMap,

wmanIf2BsSsOfdmCapCfgUlPowerControl     WmanIf2OfdmUlPower,

wmanIf2BsSsOfdmCapCfgLoopPwrControlSw   Unsigned32}

wmanIf2BsOfdmCapCfgFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmFftSizes

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes support configured for

the BS. The usage is defined by WmanIf2OfdmFftSizes."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 1 }

wmanIf2BsOfdmCapCfgSsDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsDeModType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the different BS demodulator options

configured for uplink. The usage is defined by

WmanIf2OfdmSsDeModType."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 2 }

wmanIf2BsOfdmCapCfgSsModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmSsModType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the different BS modulator options

configured for downlink. The usage is defined by

WmanIf2OfdmSsModType."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 3 }

wmanIf2BsOfdmCapCfgFocusedCtSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmFocusedCt

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the BS support configured for

Focused Contention. The usage is defined by

WmanIf2OfdmFocusedCt."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 4 }

wmanIf2BsOfdmCapCfgTcSublayerSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmTcSublayer

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the BS support configured for TC

sublayer. The usage is defined by

WmanIf2OfdmTcSublayer."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 5 }

wmanIf2BsOfdmCapCfgPrivteMapSupport OBJECT-TYPE

SYNTAX      WmanIf2OfdmPrivMap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field iindicates if the private map parameters

are supported."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 6 }

wmanIf2BsSsOfdmCapCfgUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmUlPower

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the uplink power control options

supported by SS."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 7 }

wmanIf2BsSsOfdmCapCfgLoopPwrControlSw OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates he minimum number of frames that

SS takes to switch from the open loop power control

scheme to the closed loop power control scheme or

vice versa."

::= { wmanIf2BsOfdmCapabilitiesConfigEntry 8 }

--

-- BS OFDMA PHY objects

--

wmanIf2BsOfdmaPhy OBJECT IDENTIFIER ::= { wmanIf2BsPhy 3 }

wmanIf2BsOfdmaUplinkChannelTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmaUplinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains UCD channel attributes, defining the

transmission characteristics of uplink channels"

REFERENCE

"Table 349 and Table 353, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaPhy 1 }

wmanIf2BsOfdmaUplinkChannelEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmaUplinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each uplink channel of

multi-sector BS. An entry in this table exists for each

ifEntry of BS with an ifType of ieee80216WMAN."

INDEX     { ifIndex }

::= { wmanIf2BsOfdmaUplinkChannelTable 1 }

WmanIf2BsOfdmaUplinkChannelEntry ::= SEQUENCE {

wmanIf2BsOfdmaStartOfRngCodes           INTEGER,

wmanIf2BsOfdmaPermutationBase           INTEGER,

wmanIf2BsOfdmaULAllocSubchBitmap        OCTET STRING,

wmanIf2BsOfdmaOptPermULAllocSubchBitmap OCTET STRING,

wmanIf2BsOfdmaBandAMCAllocThreshold     INTEGER,

wmanIf2BsOfdmaBandAMCReleaseThreshold   INTEGER,

wmanIf2BsOfdmaBandAMCAllocTimer         INTEGER,

wmanIf2BsOfdmaBandAMCReleaseTimer       INTEGER,

wmanIf2BsOfdmaBandStatRepMAXPeriod      INTEGER,

wmanIf2BsOfdmaBandAMCRetryTimer         INTEGER,

wmanIf2BsOfdmaPerRngBackoffStart        INTEGER,

wmanIf2BsOfdmaPerRngBackoffEnd          INTEGER,

wmanIf2BsOfdmaInitRngCodes              INTEGER,

wmanIf2BsOfdmaPeriodicRngCodes          INTEGER,

wmanIf2BsOfdmaBWReqCodes                INTEGER,

wmanIf2BsOfdmaHandoverRangingStart      INTEGER,

wmanIf2BsOfdmaHandoverRangingEnd        INTEGER,

wmanIf2BsOfdmaHARQAckDelayDLBurst       WmanIf2HarqAckDelay,

wmanIf2BsOfdmaUlAmcAlloPhyBandsBitmap   OCTET STRING,

wmanIf2BsOfdmaMaxRetransmission         INTEGER,

wmanIf2BsOfdmaNormalizedCnOverride      OCTET STRING,

wmanIf2BsOfdmaSizeOfCqichId             INTEGER,

wmanIf2BsOfdmaNormalizedCnValue         INTEGER,

wmanIf2BsOfdmaNormalizedCnOverride2     OCTET STRING,

wmanIf2BsOfdmaBandAmcEntryAvgCinr       INTEGER,

wmanIf2BsOfdmaAasPreambleUpperBond      INTEGER,

wmanIf2BsOfdmaAasPreambleLowerBond      INTEGER,

wmanIf2BsOfdmaAasBeamSelectAllowed      WmanIf2AasBeamSel,

wmanIf2BsOfdmaCqichIndicationFlag       OCTET STRING,

wmanIf2BsOfdmaUpPowerAdjStep            Unsigned32,

wmanIf2BsOfdmaDownPowerAdjStep          Unsigned32,

wmanIf2BsOfdmaMinPowerOffsetAdj         INTEGER,

wmanIf2BsOfdmaMaxPowerOffsetAdj         INTEGER,

wmanIf2BsOfdmaHandoverRngCodes          INTEGER,

wmanIf2BsOfdmaInitialRngInterval        INTEGER,

wmanIf2BsOfdmaTxPwrRepThreshold         INTEGER,

wmanIf2BsOfdmaTprPower                  INTEGER,

wmanIf2BsOfdmaAlphaPavg                 INTEGER,

wmanIf2BsOfdmaCqichTxPwrRepThreshold    INTEGER,

wmanIf2BsOfdmaCqichTprPower             INTEGER,

wmanIf2BsOfdmaCqichAlphaPavg            INTEGER,

wmanIf2BsOfdmaNormalizedCnChSounding    INTEGER,

wmanIf2BsOfdmaInitialRngBackoffStart    INTEGER,

wmanIf2BsOfdmaInitialRngBackoffEnd      INTEGER,

wmanIf2BsOfdmaBwRequestBackoffStart     INTEGER,

wmanIf2BsOfdmaBwRequestBackoffEnd       INTEGER}

wmanIf2BsOfdmaStartOfRngCodes OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the starting number, S, of the group of codes

used for this uplink.All the ranging codes used on this

uplink will be between S and ((S+N+M+L) mod 256). Where,

N: the number of initial-ranging codes

M: the number of periodic-ranging codes

L: the number of bandwidth-request codes

O: the number of handover-ranging codes"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

DEFVAL 	    { 0 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 1 }

wmanIf2BsOfdmaPermutationBase OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Determines the UL_PermBase parameter for the subcarrier

permutation to be used on this uplink channel.

UL_PermBase = 7 LSBs of Permutation base."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

DEFVAL 	    { 0 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 2 }

wmanIf2BsOfdmaULAllocSubchBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (9))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a bitmap describing the physical sub-channels

allocated to the segment in the UL, when using the uplink

PUSC permutation. The LSB of the first byte shall corrspond

to subchannel 0. For any bit that is not set, the

corresponding subchannel shall not be used by the SS on

that segment"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 3 }

wmanIf2BsOfdmaOptPermULAllocSubchBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (13))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a bitmap describing the sub-channels allocated to

the segment in the UL, when using the uplink optional PUSC

permutation (see 8.4.6.2.5 in IEEE Std 802.16-2004). The

LSB of the first byte shall correspond to subchannel 0.

For any bit that is not set, the corresponding subchannel

shall not be used by the SS on that segment. When this TLV

is not present, BS may allocate any subchannels to an SS."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaUplinkChannelEntry 4 }

wmanIf2BsOfdmaBandAMCAllocThreshold OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold of the maximum of the standard deviations of the

individual bands CINR measurements over time to trigger

mode transition from normal subchannel to Band AMC"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 5 }

wmanIf2BsOfdmaBandAMCReleaseThreshold OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold of the maximum of the standard deviations of the

individual bands CINR measurements over time to trigger

mode transition from Band AMC to normal subchannel"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 6 }

wmanIf2BsOfdmaBandAMCAllocTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum required number of frames to measure the average

and standard deviation for the event of Band AMC triggering"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 7 }

wmanIf2BsOfdmaBandAMCReleaseTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum required number of frames to measure the average

and standard deviation for the event triggering from Band

AMC to normal subchannel"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 8 }

wmanIf2BsOfdmaBandStatRepMAXPeriod OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum period between refreshing the Band CINR

measurement by the unsolicited REP-RSP"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 9 }

wmanIf2BsOfdmaPerRngBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS 	read-only

STATUS      current

DESCRIPTION

"Initial backoff window size for periodic ranging

contention, expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

DEFVAL 	    { 0 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 10 }

wmanIf2BsOfdmaPerRngBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (0 .. 15)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Final backoff window size for periodic ranging contention,

expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

DEFVAL 	    { 15 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 11 }

wmanIf2BsOfdmaBandAMCRetryTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

UNITS       "Frame"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Backoff timer between consecutive mode transitions from

normal subchannel to Band AMC when the previous request

is failed"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 12 }

wmanIf2BsOfdmaInitRngCodes OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of initial ranging CDMA codes. Possible values are

0..255. The total number of wmanIf2SsOfdmaInitRngCodes,

wmanIf2SsOfdmaPeriodicRngCodes and wmanIf2SsOfdmaBWReqCodes

shall be equal or less than 256."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

DEFVAL      { 30 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 13 }

wmanIf2BsOfdmaPeriodicRngCodes OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of periodic ranging CDMA codes. Possible values are

0..255. The total number of wmanIf2SsOfdmaInitRngCodes,

wmanIf2SsOfdmaPeriodicRngCodes and wmanIf2SsOfdmaBWReqCodes

shall be equal or less than 256."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

DEFVAL 	    { 30 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 14 }

wmanIf2BsOfdmaBWReqCodes OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Number of bandwidth request codes. Possible values are

0..255. The total number of wmanIf2SsOfdmaInitRngCodes,

wmanIf2SsOfdmaPeriodicRngCodes and wmanIf2SsOfdmaBWReqCodes

shall be equal or less than 256."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16-2004"

DEFVAL 	    { 30 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 15 }

wmanIf2BsOfdmaHandoverRangingStart OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial backoff window size for MS performing initial

ranging during handover process, expressed as a power

of 2."

REFERENCE

"Table 349, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 16 }

wmanIf2BsOfdmaHandoverRangingEnd OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Final backoff window size for MS performing initial

ranging during handover process, expressed as a power

of 2."

REFERENCE

"Table 349, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 17 }

wmanIf2BsOfdmaHARQAckDelayDLBurst OBJECT-TYPE

SYNTAX      WmanIf2HarqAckDelay

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the OFDMA H-ARQ ACK delay for DL

burst."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 18 }

wmanIf2BsOfdmaUlAmcAlloPhyBandsBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (6))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"A bitmap describing the physical bands allocated to the

segment in the UL, when using the optional AMC permutation

with regular MAPs (see 8.4.6.3). The LSB of the first byte

shall correspond to the physical band 0. For any bit that

is not set, the corresponding physical bands shall not be

used by the SS on that segment. When this TLV is not

present, BS may allocate any physical bands to an SS."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 19 }

wmanIf2BsOfdmaMaxRetransmission OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of retransmission in UL HARQ."

REFERENCE

"Table 353, in IEEE Std 802.16e-2005"

DEFVAL      { 4 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 20 }

wmanIf2BsOfdmaNormalizedCnOverride OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (8))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a list of numbers, where each number is encoded by

one nibble, and interpreted as a signed integer. The

nibbles correspond in order to the list define by Table

334, starting from the second line, such that the LS

nibble of the first byte corresponds to the second line in

the table. The number encoded by each nibble represents

the difference in normalized C/N relative to the previous

line in the table."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 21 }

wmanIf2BsOfdmaSizeOfCqichId OBJECT-TYPE

SYNTAX      INTEGER (0..7)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Size of CQICH ID field.

0 = 0 bits

1 = 3 bits

2 = 4 bits

3 = 5 bits

4 = 6 bits

5 = 7 bits

6 = 8 bits

7 = 9 bits"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

DEFVAL      { 0 }

::= { wmanIf2BsOfdmaUplinkChannelEntry 22 }

wmanIf2BsOfdmaNormalizedCnValue OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"It shall be interpreted as signed integer in dB. It

corresponds to the normalized C/N value in the first line

(counting except for header cell of table)"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 23 }

wmanIf2BsOfdmaNormalizedCnOverride2 OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (7))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This is a list of numbers, where each number is encoded

by one nibble, and interpreted as a signed integer. The

nibbles correspond in order to the list define by Table

334, starting from the second line (counting except for

the header cell of table), such that the LS nibble of

the first byte corresponds to the second line in the

table. The number encoded by each nibble represents the

difference in normalized C/N relative to the previous

line in the table."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 24 }

wmanIf2BsOfdmaBandAmcEntryAvgCinr OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold of the average CINR of the whole bandwidth to

trigger mode transition from normal subchannel to AMC"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 25 }

wmanIf2BsOfdmaAasPreambleUpperBond OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "0.25 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Upper bound of AAS preamble."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 26 }

wmanIf2BsOfdmaAasPreambleLowerBond OBJECT-TYPE

SYNTAX      INTEGER (-128..128)

UNITS       "0.25 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Lower bound of AAS preamble."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 27 }

wmanIf2BsOfdmaAasBeamSelectAllowed OBJECT-TYPE

SYNTAX      WmanIf2AasBeamSel

UNITS       "0.25 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicate whether unsolicited AAS Beam Select messages

(see 6.3.2.3.41 in IEEE 802.16e-2005) should be sent by

the MS."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

DEFVAL      { allowed }

::= { wmanIf2BsOfdmaUplinkChannelEntry 28 }

wmanIf2BsOfdmaCqichIndicationFlag OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (1))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The N MSB values of this field represents the N-bit

payload value on the Fast-Feedback channel reserved as

indication flag for MS to initiate feedback on the

Feedback header, where N is the number of payload bits

used for S/N measurement feedback on the Fast-Feedback

channel. The value shall not be set to all zeros."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 29 }

wmanIf2BsOfdmaUpPowerAdjStep OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "0.01 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MS-specific up power offset adjustment step"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 30 }

wmanIf2BsOfdmaDownPowerAdjStep OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "0.01 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MS-specific down power offset adjustment step"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 31 }

wmanIf2BsOfdmaMinPowerOffsetAdj OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "0.1 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum level of power offset adjustment"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 32 }

wmanIf2BsOfdmaMaxPowerOffsetAdj OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "0.1 dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum level of power offset adjustment"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 33 }

wmanIf2BsOfdmaHandoverRngCodes OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of handover ranging CDMA codes"

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 34 }

wmanIf2BsOfdmaInitialRngInterval OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of frames between initial ranging interval

allocation."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 35 }

wmanIf2BsOfdmaTxPwrRepThreshold OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report threshold.

wmanIf2BsOfdmaTxPwrRepThreshold = 0b1111 means infinite."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 36 }

wmanIf2BsOfdmaTprPower OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report interval = 2 ^ wmanIf2BsOfdmaTprPower.

The unit of Tx power report interval is frame.

wmanIf2BsOfdmaTprPower = 0b1111 means infinite."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 37 }

wmanIf2BsOfdmaAlphaPavg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Aplha p_avg parameter as shown in equation 138d in

IEEE 802.16e-2005 indicates the multiple of 1/16. For

example '0' means 1/16, 15 means 16/16. "

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 38 }

wmanIf2BsOfdmaCqichTxPwrRepThreshold OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report threshold.

wmanIf2BsOfdmaTxPwrRepThreshold = 0b1111 means infinite.

It shall be used when CQICH is allocated to the SS."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 39 }

wmanIf2BsOfdmaCqichTprPower OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Tx power report interval = 2 ^ wmanIf2BsOfdmaTprPower.

The unit of Tx power report interval is frame.

wmanIf2BsOfdmaTprPower = 0b1111 means infinite.

It shall be used when CQICH is allocated to the SS."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 40 }

wmanIf2BsOfdmaCqichAlphaPavg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Aplha p_avg parameter as shown in equation 138d in

IEEE 802.16e-2005 indicates the multiple of 1/16. For

example '0' means 1/16, 15 means 16/16. It shall be

used when CQICH is allocated to the SS."

REFERENCE

"Subclause 11.3.1, Table 353, and 8.4.10.3.2.1 in IEEE

Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 41 }

wmanIf2BsOfdmaNormalizedCnChSounding OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Signed integer for the required C/N (dB) for Channel

Sounding."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 42 }

wmanIf2BsOfdmaInitialRngBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial backoff window size for initial ranging

contention, expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 43 }

wmanIf2BsOfdmaInitialRngBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Final backoff window size for initial ranging

contention, expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 44 }

wmanIf2BsOfdmaBwRequestBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial backoff window size for contention BW requests,

expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 45 }

wmanIf2BsOfdmaBwRequestBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Final backoff window size for contention BW requests,

expressed as a power of 2."

REFERENCE

"Subclause 11.3.1, Table 353, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaUplinkChannelEntry 46 }

wmanIf2BsOfdmaDownlinkChannelTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmaDownlinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains DCD channel attributes, defining the

transmission characteristics of downlink channels"

REFERENCE

"Subclause 11.4.1, Table 358, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaPhy 2 }

wmanIf2BsOfdmaDownlinkChannelEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmaDownlinkChannelEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each downlink channel of

multi-sector BS. An entry in this table exists for each

ifEntry of BS with an ifType of ieee80216WMAN."

INDEX     { ifIndex }

::= { wmanIf2BsOfdmaDownlinkChannelTable 1 }

WmanIf2BsOfdmaDownlinkChannelEntry ::= SEQUENCE {

wmanIf2BsOfdmaFrameDurationCode         INTEGER,

wmanIf2BsOfdmaFftSize                   WmanIf2OfdmaFftSize,

wmanIf2BsOfdmaHARQAckDelayULBurst       WmanIf2HarqAckDelay,

wmanIf2BsOfdmaHarqZonePermutation       WmanIfPermutationType,

wmanIf2BsOfdmaHMaxRetransmission        INTEGER,

wmanIf2BsOfdmaCinrAlphaAvg              INTEGER,

wmanIf2BsOfdmaRssiAlphaAvg              INTEGER,

wmanIf2BsOfdmaDlAmcAlloPhyBandsBitmap   OCTET STRING,

wmanIf2BsOfdmaHandoverSupported         WmanIf2HoSupportType,

wmanIf2BsOfdmaThresholdAddBsDivSet      INTEGER,

wmanIf2BsOfdmaThresholdDelBsDivSet      INTEGER,

wmanIf2BsOfdmaAsrSlotLength             INTEGER,

wmanIf2BsOfdmaAsrSwitchingPeriod        INTEGER,

wmanIf2BsOfdmaHysteresisMargin          INTEGER,

wmanIf2BsOfdmaTimeToTrigger             INTEGER,

wmanIf2BsOfdmaRestartCount              INTEGER}

wmanIf2BsOfdmaFrameDurationCode OBJECT-TYPE

SYNTAX      INTEGER {aASGap(0),

duration2ms(1),

duration2dot5ms(2),

duration4ms(3),

duration5ms(4),

duration8ms(5),

duration10ms(6),

duration12dot5ms(7),

duration20ms(8)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration of the frame. The frame duration code values

are specified in Table 274."

REFERENCE

"Table 273, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 1 }

wmanIf2BsOfdmaFftSize OBJECT-TYPE

SYNTAX      WmanIf2OfdmaFftSize

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the channel BW in units of 125 kHz

for OFDMA PHY."

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 2 }

wmanIf2BsOfdmaHARQAckDelayULBurst OBJECT-TYPE

SYNTAX      WmanIf2HarqAckDelay

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the OFDMA H-ARQ ACK delay for UL

burst."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 3 }

wmanIf2BsOfdmaHarqZonePermutation OBJECT-TYPE

SYNTAX      WmanIfPermutationType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Permutation type for broadcast region in HARQ zone"

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 4 }

wmanIf2BsOfdmaHMaxRetransmission OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of retransmission in DL HARQ."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

DEFVAL      { 4 }

::= { wmanIf2BsOfdmaDownlinkChannelEntry 5 }

wmanIf2BsOfdmaCinrAlphaAvg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of Default RSSI and CINR averaging parameter

TLV.

Default averaging parameter Alpha Avg for physical

CINR measurements, in multiples of 1/16. For example

'0' means 1/16, 15 means 16/16."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

DEFVAL      { 3 }

::= { wmanIf2BsOfdmaDownlinkChannelEntry 6 }

wmanIf2BsOfdmaRssiAlphaAvg OBJECT-TYPE

SYNTAX      INTEGER (0..15)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of Default RSSI and CINR averaging parameter

TLV.

Default averaging parameter Alpha Avg for physical

RSSI measurements, in multiples of 1/16. For example

'0' means 1/16, 15 means 16/16."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

DEFVAL      { 3 }

::= { wmanIf2BsOfdmaDownlinkChannelEntry 7 }

wmanIf2BsOfdmaDlAmcAlloPhyBandsBitmap OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (6))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"A bitmap describing the physical bands allocated to the

segment in the DL, when allocating AMC subchannels

through the HARQ MAP, or through the Normal MAP, or for

Band-AMC CINR reports, or using the optional AMC

permutation (see 8.4.6.3). The LSB of the first byte

shall correspond to band 0. For any bit that is not set,

the corresponding band shall not be used by the SS on

that segment. When this TLV is not present, BS may

allocate any physical bands to an SS."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 8 }

wmanIf2BsOfdmaHandoverSupported OBJECT-TYPE

SYNTAX      WmanIf2HoSupportType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the types of handover supported.

Bit #0 = HO

Bit #1 = MDHO

Bit #2 = FBSS HO."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 9 }

wmanIf2BsOfdmaThresholdAddBsDivSet OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold used by the MS to add a neighbor BS to the

diversity set. When the CINR of a neighbor BS is higher

than H_Add_Threshold, the MS should send MOB_MSHO-REQ to

request adding this neighbor BS to the diversity set.

This threshold is used for the MS that is performing

MDHO/FBSS HO. If the BS does not support FBSS HO/MDHO,

this value is not set."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 10 }

wmanIf2BsOfdmaThresholdDelBsDivSet OBJECT-TYPE

SYNTAX      INTEGER (0..255)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Threshold used by the MS to delete a neighbor BS to the

diversity set. When the CINR of a neighbor BS is lower

than H_Add_Threshold, the MS should send MOB_MSHO-REQ to

request dropping this neighbor BS to the diversity set.

This threshold is used for the MS that is performing

MDHO/FBSS HO. If the BS does not support FBSS HO/MDHO,

this value is not set."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 12 }

wmanIf2BsOfdmaAsrSlotLength OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "Frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of ASR Slot Length and Switching Period.

For FBSS operation, the time axis is slotted by an ASR

(Anchor Switch Reporting) slot that is

wmanIf2BsOfdmaAsrSlotLength frame long."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 13 }

wmanIf2BsOfdmaAsrSwitchingPeriod OBJECT-TYPE

SYNTAX      INTEGER (0..15)

UNITS       "ASR slots"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Bit 0..3 of ASR Slot Length and Switching Period.

A switching period is introduced whose duration is equals

to wmanIf2BsOfdmaAsrSwitchingPeriod ASR slots that

should be long enough such that certain process (e.g.,

HARQ transmission, backhaul context transfer) can be

completed at the current anchor BS before the MS switches

to the new anchor BS."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 14 }

wmanIf2BsOfdmaHysteresisMargin OBJECT-TYPE

SYNTAX      INTEGER (0..57)

UNITS       "dB"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"When the CINR of a neighbor BS is larger than the sum of

the CINR of the current serving BS and

wmanIf2BsOfdmaHysteresisMargin for the time-to-trigger

duration, then the neighbor BS is included in the list

of possible target BSs in MOB_MSHO-REQ."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 15 }

wmanIf2BsOfdmaTimeToTrigger OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates the time duration for MS decides to select a

neighbor BS as a possible target BS. It is  applicable

only for HHO."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 16 }

wmanIf2BsOfdmaRestartCount OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value is incremented by one whenever BS restarts

(see 6.3.9.11). The value rolls over from 0 to 255."

REFERENCE

"Table 358, in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaDownlinkChannelEntry 17 }

wmanIf2BsOfdmaUcdBurstProfileTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmaUcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains UCD burst profiles for each uplink

channel"

REFERENCE

"Table 356, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaPhy 3 }

wmanIf2BsOfdmaUcdBurstProfileEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmaUcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each UCD burst profile."

INDEX     { ifIndex, wmanIf2BsOfdmaUiucIndex }

::= { wmanIf2BsOfdmaUcdBurstProfileTable 1 }

WmanIf2BsOfdmaUcdBurstProfileEntry ::= SEQUENCE {

wmanIf2BsOfdmaUiucIndex                 INTEGER,

wmanIf2BsOfdmaUcdFecCodeType            WmanIf2OfdmaUcdFecCode,

wmanIf2BsOfdmaRangingDataRatio          INTEGER,

wmanIf2BsOfdmaUcdBurstProfileRowStatus  RowStatus}

wmanIf2BsOfdmaUiucIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 10)

MAX-ACCESS 	not-accessible

STATUS      current

DESCRIPTION

"The Uplink Interval Usage Code indicates the uplink burst

profile in the UCD message, and is used along with ifIndex

to identify an entry in the

wmanIf2BsOfdmaUcdBurstProfileTable."

REFERENCE

"Subclause 8.4.5.4.1, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaUcdBurstProfileEntry 1 }

wmanIf2BsOfdmaUcdFecCodeType OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUcdFecCode

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Uplink FEC code type and modulation type"

REFERENCE

"Subclause 11.3.1.1, Table 357, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaUcdBurstProfileEntry 2 }

wmanIf2BsOfdmaRangingDataRatio OBJECT-TYPE

SYNTAX      INTEGER (-128 .. 127)

UNITS       "dB"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Reducing factor in units of 1 dB, between the power used

for this burst and power should be used for CDMA Ranging."

REFERENCE

"Subclause 11.3.1.1, Table 357, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaUcdBurstProfileEntry 3 }

wmanIf2BsOfdmaUcdBurstProfileRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or delete

an existing row in this table. If the implementator of this

MIB has choosen not to implement 'dynamic assignment' of

profiles, this object is not useful and should return

noSuchName upon SNMP request."

::= { wmanIf2BsOfdmaUcdBurstProfileEntry 5 }

wmanIf2BsOfdmaDcdBurstProfileTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmaDcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each DCD burst profile."

::= { wmanIf2BsOfdmaPhy 4 }

wmanIf2BsOfdmaDcdBurstProfileEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmaDcdBurstProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each DCD burst profile."

INDEX     { ifIndex, wmanIf2BsOfdmaDiucIndex }

::= { wmanIf2BsOfdmaDcdBurstProfileTable 1 }

WmanIf2BsOfdmaDcdBurstProfileEntry ::= SEQUENCE {

wmanIf2BsOfdmaDiucIndex                 INTEGER,

wmanIf2BsOfdmaDownlinkFrequency         Unsigned32,

wmanIf2BsOfdmaDcdFecCodeType            WmanIf2OfdmaDcdFecCode,

wmanIf2BsOfdmaDcdBurstProfileRowStatus  RowStatus}

wmanIf2BsOfdmaDiucIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 12)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The Downlink Interval Usage Code indicates the downlink

burst profile in the DCD message, and is used along with

ifIndex to identify an entry in the

wmanIf2BsOfdmaDcdBurstProfileTable."

REFERENCE

"Subclause 8.4.5.3.1, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaDcdBurstProfileEntry 1 }

wmanIf2BsOfdmaDownlinkFrequency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "kHz"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Downlink Frequency (kHz)."

REFERENCE

"Subclause 11.4.2, Table 359, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaDcdBurstProfileEntry 2 }

wmanIf2BsOfdmaDcdFecCodeType OBJECT-TYPE

SYNTAX      WmanIf2OfdmaDcdFecCode

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Downlink FEC code type and modulation type"

REFERENCE

"Subclause 11.4.2, Table 363, in IEEE Std 802.16-2004"

::= { wmanIf2BsOfdmaDcdBurstProfileEntry 3 }

wmanIf2BsOfdmaDcdBurstProfileRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or delete

an existing row in this table. If the implementator of this

MIB has choosen not to implement 'dynamic assignment' of

profiles, this object is not useful and should return

noSuchName upon SNMP request."

::= { wmanIf2BsOfdmaDcdBurstProfileEntry 6 }

wmanIf2BsSsOfdmaReqCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsOfdmaReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information,

specific to OFDMA PHY, of MSs that have been reported by

MSs to BS using RNG-REQ, SBC-REQ and REG-REQ messages.

Entries in this table should be created when an MS

registers with a BS."

::= { wmanIf2BsOfdmaPhy 5 }

wmanIf2BsSsOfdmaReqCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsOfdmaReqCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each MS that has been

registered in the BS. This table augments the table

wmanIf2BsRegisteredSsTable."

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsOfdmaReqCapabilitiesTable 1 }

WmanIf2BsSsOfdmaReqCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsSsOfdmaReqCapFftSizes          WmanIf2OfdmaFftSizes,

wmanIf2BsSsOfdmaReqCapDemodulator       WmanIf2OfdmaMsDeModType,

wmanIf2BsSsOfdmaReqCapModulator         WmanIf2OfdmaMsModType,

wmanIf2BsSsOfdmaReqCapNoHarqChannel     Unsigned32,

wmanIf2BsSsOfdmaReqCapPermutation       WmanIf2OfdmaPermutation,

wmanIf2BsSsOfdmaReqCapDemMimo           WmanIf2OfdmaDemMimo,

wmanIf2BsSsOfdmaReqCapMimoCapability    WmanIf2OfdmaMimoCap,

wmanIf2BsSsOfdmaReqCapUlMimo            WmanIf2OfdmaUlMimo,

wmanIf2BsSsOfdmaReqCapPrivateMap        WmanIf2OfdmaPrivMap,

wmanIf2BsSsOfdmaReqCapAasCapability     WmanIf2OfdmaAasCap,

wmanIf2BsSsOfdmaReqCapCinrMeasurement   WmanIf2OfdmaCinrCap,

wmanIf2BsSsOfdmaReqCapUlPowerControl    WmanIf2OfdmaUlPower,

wmanIf2BsSsOfdmaReqCapMapCapability     WmanIf2OfdmaMapCap,

wmanIf2BsSsOfdmaReqCapUlControlChannel  WmanIf2OfdmaUlCntlCh,

wmanIf2BsSsOfdmaReqCapCistCapability    WmanIf2OfdmaMsCistCap,

wmanIf2BsSsOfdmaReqCapMaxHarqBurst      WmanIf2OfdmaMaxHarq,

wmanIf2BsSsOfdmaReqCapModMimo           WmanIf2OfdmaModMimo,

wmanIf2BsSsOfdmaReqCapSdmaPilot         WmanIf2SdmaPilotCap,

wmanIf2BsSsOfdmaReqCapMultipleBurst     WmanIf2MultiBurst,

wmanIf2BsSsOfdmaReqCapIncrHarqBuffer    WmanIf2IncrHarqBuf,

wmanIf2BsSsOfdmaReqCapChaseHarqBuffer   WmanIf2ChaseHarqBuf}

wmanIf2BsSsOfdmaReqCapFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmaFftSizes

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes supported by MS."

REFERENCE

"Subclause 11.8.3.7.1 in IEEE 802.16e-2005"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 1 }

wmanIf2BsSsOfdmaReqCapDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsDeModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

supported by MS for downlink."

REFERENCE

"Subclause 11.8.3.7.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 2 }

wmanIf2BsSsOfdmaReqCapModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

supported by MS for uplink."

REFERENCE

"Subclause 11.8.3.7.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 3 }

wmanIf2BsSsOfdmaReqCapNoHarqChannel OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 4 }

wmanIf2BsSsOfdmaReqCapPermutation OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPermutation

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the OFDMA MS Permutation support."

REFERENCE

"Subclause 11.8.3.7.4 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 5 }

wmanIf2BsSsOfdmaReqCapDemMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaDemMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different MIMO options supported

by a WirelessMAN-OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 6 }

wmanIf2BsSsOfdmaReqCapMimoCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMimoCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA MS

demodulator."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 7 }

wmanIf2BsSsOfdmaReqCapUlMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates different MIMO options supported

by a OFDMA PHY SS in the uplink"

REFERENCE

"Subclause 11.8.3.7.6 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 8 }

wmanIf2BsSsOfdmaReqCapPrivateMap OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPrivMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates AAS private map parameters

supported by a OFDMA SS"

REFERENCE

"Subclause 11.8.3.7.7 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 9 }

wmanIf2BsSsOfdmaReqCapAasCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaAasCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates different AAS options

supported by a OFDMA PHY SS in the downlink"

REFERENCE

"Subclause 11.8.3.7.8 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 10 }

wmanIf2BsSsOfdmaReqCapCinrMeasurement OBJECT-TYPE

SYNTAX      WmanIf2OfdmaCinrCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the CINR measurement capability

supported by a OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.9 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 11 }

wmanIf2BsSsOfdmaReqCapUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlPower

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the power control options

supported by a OFDMA PHY SS for uplink transmission."

REFERENCE

"Subclause 11.8.3.7.11 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 12 }

wmanIf2BsSsOfdmaReqCapMapCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMapCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different MAP options supported

by a OFDMA PHY SS"

REFERENCE

"Subclause 11.8.3.7.12 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 13 }

wmanIf2BsSsOfdmaReqCapUlControlChannel OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlCntlCh

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different uplink control channels

supported by a OFDMA PHY SS."

REFERENCE

"Subclause 11.8.3.7.13 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 14 }

wmanIf2BsSsOfdmaReqCapCistCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsCistCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MS capability of supporting CSIT

(uplink sounding)."

REFERENCE

"Subclause 11.8.3.7.14 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 15 }

wmanIf2BsSsOfdmaReqCapMaxHarqBurst OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMaxHarq

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximum number of UL/DL HARQ

burst allocations for the SS in a single UL/DL subframe."

REFERENCE

"Subclause 11.8.3.7.15 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 16 }

wmanIf2BsSsOfdmaReqCapModMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaModMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA SS

modulator."

REFERENCE

"Subclause 11.8.3.7.16 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 17 }

wmanIf2BsSsOfdmaReqCapSdmaPilot OBJECT-TYPE

SYNTAX      WmanIf2SdmaPilotCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 18 }

wmanIf2BsSsOfdmaReqCapMultipleBurst OBJECT-TYPE

SYNTAX      WmanIf2MultiBurst

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates whether multiple FEC types are

supported in DL/UL burst profiles."

REFERENCE

"Subclause 11.8.3.7.18 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 19 }

wmanIf2BsSsOfdmaReqCapIncrHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2IncrHarqBuf

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for NEP/NSCH

based incremental redundancy CTC in downlink and uplink

transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 20 }

wmanIf2BsSsOfdmaReqCapChaseHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2ChaseHarqBuf

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for

DIUC/duration based HARQ methods (Chase combining and

CC-IR)in downlink and uplink transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaReqCapabilitiesEntry 21 }

wmanIf2BsSsOfdmaRspCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsOfdmaRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capability information,

specific to OFDMA PHY, of MSs that have been reported by

MSs to BS using RNG-REQ, SBC-REQ and REG-REQ messages.

Entries in this table should be created when an MS

registers with a BS."

::= { wmanIf2BsOfdmaPhy 6 }

wmanIf2BsSsOfdmaRspCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsOfdmaRspCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each MS that has been

registered in the BS. This table augments the table

wmanIf2BsRegisteredSsTable."

AUGMENTS { wmanIf2BsRegisteredSsEntry }

::= { wmanIf2BsSsOfdmaRspCapabilitiesTable 1 }

WmanIf2BsSsOfdmaRspCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsSsOfdmaRspCapFftSizes          WmanIf2OfdmaFftSizes,

wmanIf2BsSsOfdmaRspCapDemodulator       WmanIf2OfdmaMsDeModType,

wmanIf2BsSsOfdmaRspCapModulator         WmanIf2OfdmaMsModType,

wmanIf2BsSsOfdmaRspCapNoHarqChannel     Unsigned32,

wmanIf2BsSsOfdmaRspCapPermutation       WmanIf2OfdmaPermutation,

wmanIf2BsSsOfdmaRspCapDemMimo           WmanIf2OfdmaDemMimo,

wmanIf2BsSsOfdmaRspCapMimoCapability    WmanIf2OfdmaMimoCap,

wmanIf2BsSsOfdmaRspCapUlMimo            WmanIf2OfdmaUlMimo,

wmanIf2BsSsOfdmaRspCapPrivateMap        WmanIf2OfdmaPrivMap,

wmanIf2BsSsOfdmaRspCapAasCapability     WmanIf2OfdmaAasCap,

wmanIf2BsSsOfdmaRspCapCinrMeasurement   WmanIf2OfdmaCinrCap,

wmanIf2BsSsOfdmaRspCapUlPowerControl    WmanIf2OfdmaUlPower,

wmanIf2BsSsOfdmaRspCapMapCapability     WmanIf2OfdmaMapCap,

wmanIf2BsSsOfdmaRspCapUlControlChannel  WmanIf2OfdmaUlCntlCh,

wmanIf2BsSsOfdmaRspCapCistCapability    WmanIf2OfdmaMsCistCap,

wmanIf2BsSsOfdmaRspCapMaxHarqBurst      WmanIf2OfdmaMaxHarq,

wmanIf2BsSsOfdmaRspCapModMimo           WmanIf2OfdmaModMimo,

wmanIf2BsSsOfdmaRspCapSdmaPilot         WmanIf2SdmaPilotCap,

wmanIf2BsSsOfdmaRspCapMultipleBurst     WmanIf2MultiBurst,

wmanIf2BsSsOfdmaRspCapIncrHarqBuffer    WmanIf2IncrHarqBuf,

wmanIf2BsSsOfdmaRspCapChaseHarqBuffer   WmanIf2ChaseHarqBuf}

wmanIf2BsSsOfdmaRspCapFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmaFftSizes

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes negotiated with the

MS."

REFERENCE

"Subclause 11.8.3.7.1 in IEEE 802.16e-2005"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 1 }

wmanIf2BsSsOfdmaRspCapDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsDeModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

negotiated for MS for downlink."

REFERENCE

"Subclause 11.8.3.7.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 2 }

wmanIf2BsSsOfdmaRspCapModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

negotiated for MS for uplink."

REFERENCE

"Subclause 11.8.3.7.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 3 }

wmanIf2BsSsOfdmaRspCapNoHarqChannel OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 4 }

wmanIf2BsSsOfdmaRspCapPermutation OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPermutation

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the OFDMA MS Permutation support

negotiated for MS."

REFERENCE

"Subclause 11.8.3.7.4 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 5 }

wmanIf2BsSsOfdmaRspCapDemMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaDemMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different MIMO options supported

by a WirelessMAN-OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 6 }

wmanIf2BsSsOfdmaRspCapMimoCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMimoCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA MS

demodulator."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 7 }

wmanIf2BsSsOfdmaRspCapUlMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates different MIMO options supported

by a OFDMA PHY SS in the uplink"

REFERENCE

"Subclause 11.8.3.7.6 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 8 }

wmanIf2BsSsOfdmaRspCapPrivateMap OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPrivMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates AAS private map parameters

supported by a OFDMA SS"

REFERENCE

"Subclause 11.8.3.7.7 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 9 }

wmanIf2BsSsOfdmaRspCapAasCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaAasCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates different AAS options

supported by a OFDMA PHY SS in the downlink"

REFERENCE

"Subclause 11.8.3.7.8 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 10 }

wmanIf2BsSsOfdmaRspCapCinrMeasurement OBJECT-TYPE

SYNTAX      WmanIf2OfdmaCinrCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the CINR measurement capability

supported by a OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.9 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 11 }

wmanIf2BsSsOfdmaRspCapUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlPower

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the power control options

supported by a OFDMA PHY SS for uplink transmission."

REFERENCE

"Subclause 11.8.3.7.11 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 12 }

wmanIf2BsSsOfdmaRspCapMapCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMapCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different MAP options supported

by a OFDMA PHY SS"

REFERENCE

"Subclause 11.8.3.7.11 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 13 }

wmanIf2BsSsOfdmaRspCapUlControlChannel OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlCntlCh

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different uplink control channels

supported by a OFDMA PHY SS."

REFERENCE

"Subclause 11.8.3.7.13 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 14 }

wmanIf2BsSsOfdmaRspCapCistCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsCistCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MS capability of supporting CSIT

(uplink sounding)."

REFERENCE

"Subclause 11.8.3.7.14 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 15 }

wmanIf2BsSsOfdmaRspCapMaxHarqBurst OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMaxHarq

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximum number of UL/DL HARQ

burst allocations for the SS in a single UL/DL subframe."

REFERENCE

"Subclause 11.8.3.7.15 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 16 }

wmanIf2BsSsOfdmaRspCapModMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaModMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA SS

modulator."

REFERENCE

"Subclause 11.8.3.7.16 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 17 }

wmanIf2BsSsOfdmaRspCapSdmaPilot OBJECT-TYPE

SYNTAX      WmanIf2SdmaPilotCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 18 }

wmanIf2BsSsOfdmaRspCapMultipleBurst OBJECT-TYPE

SYNTAX      WmanIf2MultiBurst

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates whether multiple FEC types are

supported in DL/UL burst profiles."

REFERENCE

"Subclause 11.8.3.7.18 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 19 }

wmanIf2BsSsOfdmaRspCapIncrHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2IncrHarqBuf

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for NEP/NSCH

based incremental redundancy CTC in downlink and uplink

transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 20 }

wmanIf2BsSsOfdmaRspCapChaseHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2ChaseHarqBuf

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for

DIUC/duration based HARQ methods (Chase combining and

CC-IR)in downlink and uplink transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsSsOfdmaRspCapabilitiesEntry 21 }

wmanIf2BsOfdmaCapabilitiesTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmaCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the basic capabilities, specific to

OFDMA PHY, of the BS as implemented in BS hardware and

software. These capabilities along with the configuration

for them (wmanIf2BsOfdmaCapabilitiesConfigTable) are used

for negotiation of basic capabilities with SS using

RNG-RSP, SBC-RSP and REG-RSP messages. The negotiated

capabilities are obtained by interSubclause of MS raw

reported capabilities, BS raw capabilities and BS

configured capabilities. The objects in the table have

read-only access. The table is maintained by BS."

::= { wmanIf2BsOfdmaPhy 7 }

wmanIf2BsOfdmaCapabilitiesEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmaCapabilitiesEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsOfdmaCapabilitiesTable 1 }

WmanIf2BsOfdmaCapabilitiesEntry ::= SEQUENCE {

wmanIf2BsOfdmaCapFftSizes               WmanIf2OfdmaFftSizes,

wmanIf2BsOfdmaCapDemodulator            WmanIf2OfdmaMsDeModType,

wmanIf2BsOfdmaCapModulator              WmanIf2OfdmaMsModType,

wmanIf2BsOfdmaCapNoHarqChannel          Unsigned32,

wmanIf2BsOfdmaCapPermutation            WmanIf2OfdmaPermutation,

wmanIf2BsSsOfdmaCapDemMimo              WmanIf2OfdmaDemMimo,

wmanIf2BsSsOfdmaCapMimoCapability       WmanIf2OfdmaMimoCap,

wmanIf2BsSsOfdmaCapUlMimo               WmanIf2OfdmaUlMimo,

wmanIf2BsSsOfdmaCapPrivateMap           WmanIf2OfdmaPrivMap,

wmanIf2BsSsOfdmaCapAasCapability        WmanIf2OfdmaAasCap,

wmanIf2BsSsOfdmaCapCinrMeasurement      WmanIf2OfdmaCinrCap,

wmanIf2BsSsOfdmaCapUlPowerControl       WmanIf2OfdmaUlPower,

wmanIf2BsSsOfdmaCapMapCapability        WmanIf2OfdmaMapCap,

wmanIf2BsSsOfdmaCapUlControlChannel     WmanIf2OfdmaUlCntlCh,

wmanIf2BsSsOfdmaCapCistCapability       WmanIf2OfdmaMsCistCap,

wmanIf2BsSsOfdmaCapMaxHarqBurst         WmanIf2OfdmaMaxHarq,

wmanIf2BsSsOfdmaCapModMimo              WmanIf2OfdmaModMimo,

wmanIf2BsSsOfdmaCapSdmaPilot            WmanIf2SdmaPilotCap,

wmanIf2BsSsOfdmaCapMultipleBurst        WmanIf2MultiBurst,

wmanIf2BsSsOfdmaCapIncrHarqBuffer       WmanIf2IncrHarqBuf,

wmanIf2BsSsOfdmaCapChaseHarqBuffer      WmanIf2ChaseHarqBuf}

wmanIf2BsOfdmaCapFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmaFftSizes

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes supported by BS."

REFERENCE

"Subclause 11.8.3.7.1 in IEEE 802.16e-2005"

::= { wmanIf2BsOfdmaCapabilitiesEntry 1 }

wmanIf2BsOfdmaCapDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsDeModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

supported by BS."

REFERENCE

"Subclause 11.8.3.7.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaCapabilitiesEntry 2 }

wmanIf2BsOfdmaCapModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsModType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

supported by BS."

REFERENCE

"Subclause 11.8.3.7.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaCapabilitiesEntry 3 }

wmanIf2BsOfdmaCapNoHarqChannel OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsOfdmaCapabilitiesEntry 4 }

wmanIf2BsOfdmaCapPermutation OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPermutation

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the OFDMA MS Permutation support

supported by BS."

REFERENCE

"Subclause 11.8.3.7.4 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 5 }

wmanIf2BsSsOfdmaCapDemMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaDemMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different MIMO options supported

by a WirelessMAN-OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 6 }

wmanIf2BsSsOfdmaCapMimoCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMimoCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA MS

demodulator."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 7 }

wmanIf2BsSsOfdmaCapUlMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates different MIMO options supported

by a OFDMA PHY SS in the uplink"

REFERENCE

"Subclause 11.8.3.7.6 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 8 }

wmanIf2BsSsOfdmaCapPrivateMap OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPrivMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates AAS private map parameters

supported by a OFDMA SS"

REFERENCE

"Subclause 11.8.3.7.7 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 9 }

wmanIf2BsSsOfdmaCapAasCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaAasCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates different AAS options

supported by a OFDMA PHY SS in the downlink"

REFERENCE

"Subclause 11.8.3.7.8 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 10 }

wmanIf2BsSsOfdmaCapCinrMeasurement OBJECT-TYPE

SYNTAX      WmanIf2OfdmaCinrCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the CINR measurement capability

supported by a OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.9 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 11 }

wmanIf2BsSsOfdmaCapUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlPower

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the power control options

supported by a OFDMA PHY SS for uplink transmission."

REFERENCE

"Subclause 11.8.3.7.11 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 12 }

wmanIf2BsSsOfdmaCapMapCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMapCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different MAP options supported

by a OFDMA PHY SS"

REFERENCE

"Subclause 11.8.3.7.11 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 13 }

wmanIf2BsSsOfdmaCapUlControlChannel OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlCntlCh

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the different uplink control channels

supported by a OFDMA PHY SS."

REFERENCE

"Subclause 11.8.3.7.13 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 14 }

wmanIf2BsSsOfdmaCapCistCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsCistCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MS capability of supporting CSIT

(uplink sounding)."

REFERENCE

"Subclause 11.8.3.7.14 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 15 }

wmanIf2BsSsOfdmaCapMaxHarqBurst OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMaxHarq

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximum number of UL/DL HARQ

burst allocations for the SS in a single UL/DL subframe."

REFERENCE

"Subclause 11.8.3.7.15 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 16 }

wmanIf2BsSsOfdmaCapModMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaModMimo

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA SS

modulator."

REFERENCE

"Subclause 11.8.3.7.16 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 17 }

wmanIf2BsSsOfdmaCapSdmaPilot OBJECT-TYPE

SYNTAX      WmanIf2SdmaPilotCap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 18 }

wmanIf2BsSsOfdmaCapMultipleBurst OBJECT-TYPE

SYNTAX      WmanIf2MultiBurst

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates whether multiple FEC types are

supported in DL/UL burst profiles."

REFERENCE

"Subclause 11.8.3.7.18 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 19 }

wmanIf2BsSsOfdmaCapIncrHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2IncrHarqBuf

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for NEP/NSCH

based incremental redundancy CTC in downlink and uplink

transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 20 }

wmanIf2BsSsOfdmaCapChaseHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2ChaseHarqBuf

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for

DIUC/duration based HARQ methods (Chase combining and

CC-IR)in downlink and uplink transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesEntry 21 }

wmanIf2BsOfdmaCapabilitiesConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOfdmaCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration for basic

capabilities of BS, specific to OFDMA PHY. The table is

intended to be used to restrict the Capabilities

implemented by BS, for example in order to comply with

local regulatory requirements. The BS should use the

configuration along with the implemented Capabilities

(wmanIf2BsOfdmaPhyTable) for negotiation of basic

capabilities with SS using RNG-RSP, SBC-RSP and REG-RSP

messages. The negotiated capabilities are obtained by

interSubclause of MS reported capabilities, BS raw

capabilities and BS configured capabilities. The objects

in the table have read-write access. The rows are created

by BS as a copy of wmanIf2BsBasicCapabilitiesTable

and can be modified by NMS."

::= { wmanIf2BsOfdmaPhy 8 }

wmanIf2BsOfdmaCapabilitiesConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOfdmaCapabilitiesConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsOfdmaCapabilitiesConfigTable 1 }

WmanIf2BsOfdmaCapabilitiesConfigEntry ::= SEQUENCE {

wmanIf2BsOfdmaCapCfgFftSizes            WmanIf2OfdmaFftSizes,

wmanIf2BsOfdmaCapCfgDemodulator         WmanIf2OfdmaMsDeModType,

wmanIf2BsOfdmaCapCfgModulator           WmanIf2OfdmaMsModType,

wmanIf2BsOfdmaCapCfgNoHarqChannel       Unsigned32,

wmanIf2BsOfdmaCapCfgPermutation         WmanIf2OfdmaPermutation,

wmanIf2BsSsOfdmaCapCfgDemMimo           WmanIf2OfdmaDemMimo,

wmanIf2BsSsOfdmaCapCfgMimoCapability    WmanIf2OfdmaMimoCap,

wmanIf2BsSsOfdmaCapCfgUlMimo            WmanIf2OfdmaUlMimo,

wmanIf2BsSsOfdmaCapCfgPrivateMap        WmanIf2OfdmaPrivMap,

wmanIf2BsSsOfdmaCapCfgAasCapability     WmanIf2OfdmaAasCap,

wmanIf2BsSsOfdmaCapCfgCinrMeasurement   WmanIf2OfdmaCinrCap,

wmanIf2BsSsOfdmaCapCfgUlPowerControl    WmanIf2OfdmaUlPower,

wmanIf2BsSsOfdmaCapCfgMapCapability     WmanIf2OfdmaMapCap,

wmanIf2BsSsOfdmaCapCfgUlControlChannel  WmanIf2OfdmaUlCntlCh,

wmanIf2BsSsOfdmaCapCfgCistCapability    WmanIf2OfdmaMsCistCap,

wmanIf2BsSsOfdmaCapCfgMaxHarqBurst      WmanIf2OfdmaMaxHarq,

wmanIf2BsSsOfdmaCapCfgModMimo           WmanIf2OfdmaModMimo,

wmanIf2BsSsOfdmaCapCfgSdmaPilot         WmanIf2SdmaPilotCap,

wmanIf2BsSsOfdmaCapCfgMultipleBurst     WmanIf2MultiBurst,

wmanIf2BsSsOfdmaCapCfgIncrHarqBuffer    WmanIf2IncrHarqBuf,

wmanIf2BsSsOfdmaCapCfgChaseHarqBuffer   WmanIf2ChaseHarqBuf}

wmanIf2BsOfdmaCapCfgFftSizes OBJECT-TYPE

SYNTAX      WmanIf2OfdmaFftSizes

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the FFT sizes configured for the BS."

REFERENCE

"Subclause 11.8.3.7.1 in IEEE 802.16e-2005"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 1 }

wmanIf2BsOfdmaCapCfgDemodulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsDeModType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the different demodulator options

configured for the BS."

REFERENCE

"Subclause 11.8.3.7.2 in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 2 }

wmanIf2BsOfdmaCapCfgModulator OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsModType

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the different modulator options

configured for the BS."

REFERENCE

"Subclause 11.8.3.7.3 in IEEE Std 802.16e-2005"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 3 }

wmanIf2BsOfdmaCapCfgNoHarqChannel OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This field specifies the number of uplink H-ARQ

channels (n) the SS supports, where n = 1..16.

The value of this object should be 0..15."

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 4 }

wmanIf2BsOfdmaCapCfgPermutation OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPermutation

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the OFDMA MS Permutation support

configured for the BS."

REFERENCE

"Subclause 11.8.3.7.4 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 5 }

wmanIf2BsSsOfdmaCapCfgDemMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaDemMimo

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the different MIMO options supported

by a WirelessMAN-OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 6 }

wmanIf2BsSsOfdmaCapCfgMimoCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMimoCap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA MS

demodulator."

REFERENCE

"Subclause 11.8.3.7.5 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 7 }

wmanIf2BsSsOfdmaCapCfgUlMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlMimo

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates different MIMO options supported

by a OFDMA PHY SS in the uplink"

REFERENCE

"Subclause 11.8.3.7.6 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 8 }

wmanIf2BsSsOfdmaCapCfgPrivateMap OBJECT-TYPE

SYNTAX      WmanIf2OfdmaPrivMap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates AAS private map parameters

supported by a OFDMA SS"

REFERENCE

"Subclause 11.8.3.7.7 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 9 }

wmanIf2BsSsOfdmaCapCfgAasCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaAasCap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates different AAS options

supported by a OFDMA PHY SS in the downlink"

REFERENCE

"Subclause 11.8.3.7.8 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 10 }

wmanIf2BsSsOfdmaCapCfgCinrMeasurement OBJECT-TYPE

SYNTAX      WmanIf2OfdmaCinrCap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the CINR measurement capability

supported by a OFDMA PHY SS in the downlink."

REFERENCE

"Subclause 11.8.3.7.9 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 11 }

wmanIf2BsSsOfdmaCapCfgUlPowerControl OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlPower

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the power control options

supported by a OFDMA PHY SS for uplink transmission."

REFERENCE

"Subclause 11.8.3.7.11 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 12 }

wmanIf2BsSsOfdmaCapCfgMapCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMapCap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the different MAP options supported

by a OFDMA PHY SS"

REFERENCE

"Subclause 11.8.3.7.11 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 13 }

wmanIf2BsSsOfdmaCapCfgUlControlChannel OBJECT-TYPE

SYNTAX      WmanIf2OfdmaUlCntlCh

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the different uplink control channels

supported by a OFDMA PHY SS."

REFERENCE

"Subclause 11.8.3.7.13 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 14 }

wmanIf2BsSsOfdmaCapCfgCistCapability OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMsCistCap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the MS capability of supporting CSIT

(uplink sounding)."

REFERENCE

"Subclause 11.8.3.7.14 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 15 }

wmanIf2BsSsOfdmaCapCfgMaxHarqBurst OBJECT-TYPE

SYNTAX      WmanIf2OfdmaMaxHarq

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the maximum number of UL/DL HARQ

burst allocations for the SS in a single UL/DL subframe."

REFERENCE

"Subclause 11.8.3.7.15 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 16 }

wmanIf2BsSsOfdmaCapCfgModMimo OBJECT-TYPE

SYNTAX      WmanIf2OfdmaModMimo

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the MIMO capability of OFDMA SS

modulator."

REFERENCE

"Subclause 11.8.3.7.16 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 17 }

wmanIf2BsSsOfdmaCapCfgSdmaPilot OBJECT-TYPE

SYNTAX      WmanIf2SdmaPilotCap

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the SDMA pilot pattern support

for AMC zone."

REFERENCE

"Subclause 11.8.3.7.17 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 18 }

wmanIf2BsSsOfdmaCapCfgMultipleBurst OBJECT-TYPE

SYNTAX      WmanIf2MultiBurst

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates whether multiple FEC types are

supported in DL/UL burst profiles."

REFERENCE

"Subclause 11.8.3.7.18 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 19 }

wmanIf2BsSsOfdmaCapCfgIncrHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2IncrHarqBuf

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for NEP/NSCH

based incremental redundancy CTC in downlink and uplink

transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 20 }

wmanIf2BsSsOfdmaCapCfgChaseHarqBuffer OBJECT-TYPE

SYNTAX      WmanIf2ChaseHarqBuf

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This field indicates the maximal number of data

bits the SS is able to use for buffering for

DIUC/duration based HARQ methods (Chase combining and

CC-IR)in downlink and uplink transmissions."

REFERENCE

"Subclause 11.8.3.7.19 in IEEE 802.16e"

::= { wmanIf2BsOfdmaCapabilitiesConfigEntry 21 }

--

-- wmanIf2BsAm group – containing tables and objects related to Account

--                     Management

--

wmanIf2BsOtaUsageDataRecordTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsOtaUsageDataRecordEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains counters to keep track of the number

of packets and octets that have been received or

transmitted over the air interface. BS may delete some

OTA UDR in wmanIf2BsOtaUsageDataRecordTable after they

have been transferred to the AAA server."

::= { wmanIf2BsAm 1 }

wmanIf2BsOtaUsageDataRecordEntry OBJECT-TYPE

SYNTAX      WmanIf2BsOtaUsageDataRecordEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each session, and Since MAC

management CID (i.e. basic , primary, and 2nd management)

share the same CID for both UL and DL, it should use the QoS

parameter set to distinguish which entry is DL or UL."

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsCid,

wmanIf2BsSessionId }

::= { wmanIf2BsOtaUsageDataRecordTable 1 }

WmanIf2BsOtaUsageDataRecordEntry::= SEQUENCE {

wmanIf2BsSessionId                      Unsigned32,

wmanIf2BsCid                            INTEGER,

wmanIf2BsServiceFlowId                  Unsigned32,

wmanIf2BsMacSduCount                    Counter64,

wmanIf2BsOctetCount                     Counter64,

wmanIf2BsSessionEstablishTime           TimeStamp,

wmanIf2BsSessionTerminateTime           TimeStamp,

wmanIf2BsGlobalServiceClass             WmanIf2GlobalSrvClass,

wmanIf2BsQoSProfileIndex                INTEGER}

wmanIf2BsSessionId OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index identifies the accounting seesion within a CID.

An accounting session may be created or ended, based on

certain events, for example

- QoS parameter set change in a CID

- wmanIf2BsServiceFlowState is changed

- an SS registers at the BS

- an MS handoffs to another BS"

::= { wmanIf2BsOtaUsageDataRecordEntry 1 }

wmanIf2BsCid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A 16 bit channel identifier points to the connection being

created by DSA for this service flow."

::= { wmanIf2BsOtaUsageDataRecordEntry 2 }

wmanIf2BsServiceFlowId OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 32 bit quantity that uniquely identifies a service flow.

wmanIf2BsServiceFlowId should return '0' for MAC

management (i.e. basic, primary, and 2nd management CID)."

::= { wmanIf2BsOtaUsageDataRecordEntry 3 }

wmanIf2BsMacSduCount OBJECT-TYPE

SYNTAX      Counter64

UNITS       "SDU"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of MAC SDUs or MAC messages

that have been transmitted or received over the air

interface. For MAC management CID, wmanIf2BsMacSduCount

tracks SDU count on DL and UL."

::= { wmanIf2BsOtaUsageDataRecordEntry 4 }

wmanIf2BsOctetCount OBJECT-TYPE

SYNTAX      Counter64

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of octets of MAC SDUs or MAC

messages that have been transmitted or received over the

air interface."

::= { wmanIf2BsOtaUsageDataRecordEntry 5 }

wmanIf2BsSessionEstablishTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the session is established

."

::= { wmanIf2BsOtaUsageDataRecordEntry 6 }

wmanIf2BsSessionTerminateTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the session is terminated

."

::= { wmanIf2BsOtaUsageDataRecordEntry 7 }

wmanIf2BsGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2GlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the QoS parameter set used in this

session. When '0' is returned from reading this object, it

means either no global service class is available for this

session, or its Qos profile may be defined in the entry

pointed by wmanIf2BsQoSProfileIndex."

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

::= { wmanIf2BsOtaUsageDataRecordEntry 8 }

wmanIf2BsQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mCmnQoSProfileTable

that defines the the QoS parameter set used in this

session. When '0' is returned from reading this object, it

means the QoS profile either is not available for this

session."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2BsOtaUsageDataRecordEntry 9 }

--

-- wmanIf2BsPm group – containing tables and objects related to

--                     Performance Management

--

wmanIf2BsChannelMeasurementTable OBJECT-TYPE

SYNTAX      SEQUENCE OF   WmanIf2BsChannelMeasurementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains channel measurement information as

derived from BS measurement of uplink signal from SS,

and the downlink signal as reported from SS using

REP-REQ/RSP messages. The table shall be maintained as

FIFO to store measurement samples that can be used to

create RSSI and CINR histogram report. When the

measurement entry for a SS reaches the limit, the oldest

entry shall be deleted as the new entry is added to the

table."

REFERENCE

"6.3.2.3.33 in IEEE Std 802.16-2004"

::= { wmanIf2BsPm 1 }

wmanIf2BsChannelMeasurementEntry OBJECT-TYPE

SYNTAX      WmanIf2BsChannelMeasurementEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry in the table contains RSSI and CINR

signal quality measurement on signal received from the SS.

The primary index is the ifIndex with ifType of

ieee80216WMAN identifing the BS sector.

wmanIf2BsSsMacAddress identifies the SS from which the

signal was received. wmanIf2BsChannelDirection is the

index to the direction of the channel.

wmanIf2BsHistogramIndex is the index to histogram samples.

Since there is no time stamp in the table,

wmanIf2BsHistogramIndex should be increased monotonically,

and wraps around when it reaches the implementation

specific limit."

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsChannelDirection,

wmanIf2BsHistogramIndex }

::= { wmanIf2BsChannelMeasurementTable  1 }

WmanIf2BsChannelMeasurementEntry ::= SEQUENCE {

wmanIf2BsChannelDirection               WmanIf2LinkDirection,

wmanIf2BsHistogramIndex                 Unsigned32,

wmanIf2BsChannelNumber                  WmanIf2ChannelNumber,

wmanIf2BsStartFrame                     INTEGER,

wmanIf2BsDuration                       INTEGER,

wmanIf2BsBasicReport                    BITS,

wmanIf2BsMeanCinrReport                 INTEGER,

wmanIf2BsMeanRssiReport                 INTEGER,

wmanIf2BsStdDeviationCinrReport         INTEGER,

wmanIf2BsStdDeviationRssiReport         INTEGER}

wmanIf2BsChannelDirection OBJECT-TYPE

SYNTAX      WmanIf2LinkDirection

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"wmanIf2BsChannelDirection identifies the direction of a

a channel where the measurement takes place."

::= { wmanIf2BsChannelMeasurementEntry 1 }

wmanIf2BsHistogramIndex OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsHistogramIndex identifies the histogram samples

in the table for each subscriber station."

::= { wmanIf2BsChannelMeasurementEntry 2 }

wmanIf2BsChannelNumber OBJECT-TYPE

SYNTAX      WmanIf2ChannelNumber

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Physical channel number to be reported on is only

applicable to license exempt band. For licensed band,

this parameter should be null."

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 3 }

wmanIf2BsStartFrame OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Frame number in which measurement for this channel

started."

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 4 }

wmanIf2BsDuration OBJECT-TYPE

SYNTAX      INTEGER (0 .. 16777215)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Cumulative measurement duration on the channel in

multiples of Ts. For any value exceeding 0xFFFFFF,

report 0xFFFFFF."

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 5 }

wmanIf2BsBasicReport OBJECT-TYPE

SYNTAX      BITS {wirelessHuman(0),

unknownTransmission(1),

primaryUser(2),

channelNotMeasured(3)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Bit #0: WirelessHUMAN detected on the channel

Bit #1: Unknown transmissions detected on the channel

Bit #2: Primary User detected on the channel

Bit #3: Unmeasured. Channel not measured"

REFERENCE

"Subclause 11.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 6 }

wmanIf2BsMeanCinrReport OBJECT-TYPE

SYNTAX      INTEGER (-20 .. 37)

UNITS       "dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Mean CINR report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 7 }

wmanIf2BsMeanRssiReport OBJECT-TYPE

SYNTAX      INTEGER (-123 .. -40)

UNITS       "dBm"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Mean RSSI report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 8 }

wmanIf2BsStdDeviationCinrReport OBJECT-TYPE

SYNTAX      INTEGER (0 .. 29)

UNITS       "dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Standard deviation CINR report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 9 }

wmanIf2BsStdDeviationRssiReport OBJECT-TYPE

SYNTAX      INTEGER (0 .. 42)

UNITS       "dB"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Standard deviation RSSI report."

REFERENCE

"Subclause 8.2.2, 8.3.9 in IEEE Std 802.16-2004"

::= { wmanIf2BsChannelMeasurementEntry 10 }

-- XXX

-- Statistics Configuration Table

--

wmanIf2BsStatisticsConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsStatisticsConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration of statistics

information capture."

::= { wmanIf2BsPm 2 }

wmanIf2BsStatisticsConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsStatisticsConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsStatisticsConfigTable  1 }

WmanIf2BsStatisticsConfigEntry ::= SEQUENCE {

wmanIf2BsDataSampleInterval           INTEGER,

wmanIf2BsCountersReportInterval       INTEGER}

wmanIf2BsDataSampleInterval OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "Seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object determines the interval that the BS uses to

measure the peak and average data rate statistics to be

stored in wmanIf2BsDataRateStatisticsTable."

DEFVAL      { 1 }

::= { wmanIf2BsStatisticsConfigEntry 1 }

wmanIf2BsCountersReportInterval OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "Minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object determines the interval that the BS shall

generate an event trap to report the performance counters

to EMS. BS should reset all counters after they have been

reported to EMS via wmanIf2BsPerformanceCountersTrap."

DEFVAL      { 15 }

::= { wmanIf2BsStatisticsConfigEntry 2 }

-- XXX

-- Mobile Station startup statistics Table

--

wmanIf2BsSsStartupStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsStartupStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains statistical information that can be

used to characterize MS' performance during the startup.

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsPm 3 }

wmanIf2BsSsStartupStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsStartupStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsSsStartupStatisticsTable  1 }

WmanIf2BsSsStartupStatisticsEntry ::= SEQUENCE {

wmanIf2BsSsAuthenAttempt                Counter64,

wmanIf2BsSsAuthenSuccess                Counter64,

wmanIf2BsSsAuthenSuccessRate            INTEGER,

wmanIf2BsSsRangingAttempt               Counter64,

wmanIf2BsSsRangingSuccess               Counter64,

wmanIf2BsSsRangingSuccessRate           INTEGER}

wmanIf2BsSsAuthenAttempt OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of MS authentication

attempt. The BS increments by one each time the BS

receives:

1) PKMv1 Auth Request,

2) PKMv2 RSA-Request,

3) PKMv2 EAP start message."

::= { wmanIf2BsSsStartupStatisticsEntry 1 }

wmanIf2BsSsAuthenSuccess OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of successful MS

authentication. The BS increments by one each time the BS

sends:

1) PKMv1 Auth Reply,

2) PKMv2 RSA-Reply,

3) PKMv2 EAP complete with EAP-success payload message."

::= { wmanIf2BsSsStartupStatisticsEntry 2 }

wmanIf2BsSsAuthenSuccessRate OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsSsAuthenSuccessRate % =

wmanIf2BsSsAuthenSuccess  / wmanIf2BsSsAuthenAttempt * 100"

::= { wmanIf2BsSsStartupStatisticsEntry 3 }

wmanIf2BsSsRangingAttempt OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of MS Ranging attempt. The BS

increments by one each time the BS receives the RNG-REQ

message."

::= { wmanIf2BsSsStartupStatisticsEntry 4 }

wmanIf2BsSsRangingSuccess OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of successful MS Ranging.

The increments by one each time the BS sends the RNG-RSP

message."

::= { wmanIf2BsSsStartupStatisticsEntry 5 }

wmanIf2BsSsRangingSuccessRate OBJECT-TYPE

SYNTAX      INTEGER

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsSsRangingSuccessRate % =

wmanIf2BsSsNetwkEntrySuccess / wmanIf2BsSsNetwkEntryAttempt

* 100."

::= { wmanIf2BsSsStartupStatisticsEntry 6 }

-- XXX

-- Base Station Data Rate Statistics Table

--

wmanIf2BsDataRateStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsDataRateStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the average and peak data rate

statistics at the BS sector level. The BS measures the

number of bytes that are sent or received withing each

interval set by the wmanIf2BsDataSampleInterval object.

For example:

S = the number of bytes measured in an interval

N = the number of measurements

Avg(N) = the average data rate after N measurements

Avg(0) = S, the 1st measurement

Avg(N) = (Avg(N-1)*(N-1) + S) / N

Peak(N) = the peak data rate after N measurement

Peak(0) = S, the 1st measurement

Peak(N) = S if S > Peak(N-1)

= Peak(N -1) if S <= Peak(N-1)

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsPm 4 }

wmanIf2BsDataRateStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsDataRateStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsDataRateStatisticsTable 1 }

WmanIf2BsDataRateStatisticsEntry ::= SEQUENCE {

wmanIf2BsAvgDlUserThroughput            Counter32,

wmanIf2BsAvgUlUserThroughput            Counter32,

wmanIf2BsAvgDlMacThroughput             Counter32,

wmanIf2BsAvgUlMacThroughput             Counter32,

wmanIf2BsAvgDlPhyThroughput             Counter32,

wmanIf2BsAvgUlPhyThroughput             Counter32,

wmanIf2BsPeakDlUserThroughput           Counter32,

wmanIf2BsPeakUlUserThroughput           Counter32,

wmanIf2BsPeakDlMacThroughput            Counter32,

wmanIf2BsPeakUlMacThroughput            Counter32,

wmanIf2BsPeakDlPhyThroughput            Counter32,

wmanIf2BsPeakUlPhyThroughput            Counter32,

wmanIf2BsAvgDlCellEdgeThroughput        Counter32,

wmanIf2BsAvgUlCellEdgeThroughput        Counter32,

wmanIf2BsDataRateMeasurements           Counter64}

wmanIf2BsAvgDlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average downlink user throughput

Avg(N). Each measurement indicates the number of octets of

MAC SDUs that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 1 }

wmanIf2BsAvgUlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average uplink user throughput

Avg(N). Each measurement indicates the number of octets of

MAC SDUs that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 2 }

wmanIf2BsAvgDlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average downlink MAC throughput

Avg(N). Each measurement indicates the number of octets of

MAC PDUs (i.e. user data, MAC headers, and MAC management

messages) that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 3 }

wmanIf2BsAvgUlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average uplink MAC throughput

Avg(N). Each measurement indicates the number of octets of

MAC PDUs (i.e. user data, MAC headers, and MAC management

messages) that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 4 }

wmanIf2BsAvgDlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average downlink PHY throughput

Avg(N). Each measurement indicates the number of octets of

bursts (i.e. MAC PDU + PHY overheads) that are sent within

the wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 5 }

wmanIf2BsAvgUlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average uplink PHY throughput

Avg(N). Each measurement indicates the number of octets of

bursts (i.e. MAC PDU + PHY overheads) that are received

within the wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 6 }

wmanIf2BsPeakDlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak downlink user throughput

Peak(N). Each measurement indicates the number of octets of

MAC SDUs that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 7 }

wmanIf2BsPeakUlUserThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink user throughput Peak(N)

. Each measurement indicates the number of octets of MAC

SDUs that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 8 }

wmanIf2BsPeakDlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak downlink MAC throughput

Peak(N). Each measurement indicates the number of octets of

MAC PDUs (i.e. user data, MAC headers, and MAC management

messages) that are sent within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 9 }

wmanIf2BsPeakUlMacThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink MAC throughput Peak(N).

Each measurement indicates the number of octets of MAC PDUs

(i.e. user data, MAC headers, and MAC management messages)

that are received within the wmanIf2BsDataSampleInterval

interval."

::= { wmanIf2BsDataRateStatisticsEntry 10 }

wmanIf2BsPeakDlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak downlink PHY throughput

Peak(N). Each measurement indicates the number of octets of

bursts (i.e. MAC PDU + PHY overheads) that are sent within

the wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 11 }

wmanIf2BsPeakUlPhyThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink PHY throughput Peak(N).

Each measurement indicates the number of octets of bursts

(i.e. MAC PDU + PHY overheads) that are received within the

wmanIf2BsDataSampleInterval interval."

::= { wmanIf2BsDataRateStatisticsEntry 12 }

wmanIf2BsAvgDlCellEdgeThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the Average downlink MAC throughput

Avg(N). Each measurement indicates the number of octets of

MAC PDU that are sent within the

wmanIf2BsDataSampleInterval interval using the most robust

coding (i.e. QPSK)."

::= { wmanIf2BsDataRateStatisticsEntry 13 }

wmanIf2BsAvgUlCellEdgeThroughput OBJECT-TYPE

SYNTAX      Counter32

UNITS       "Octet"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the peak uplink PHY throughput Peak(N).

Each measurement indicates the number of octets of MAC PDU

that are received within the wmanIf2BsDataSampleInterval

interval using the most robust coding (i.e. QPSK)."

::= { wmanIf2BsDataRateStatisticsEntry 14 }

wmanIf2BsDataRateMeasurements OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object tracks the number of measurements 'N'."

::= { wmanIf2BsDataRateStatisticsEntry 15 }

-- XXX

-- Base Station Network Entry Statistics Table

--

wmanIf2BsNetworkEntryStatisticsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsNetworkEntryStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the average and maximum amount of time

for network and network re-entry.

For example:

T = a) Network entry time is the amount of time between:

1. BS received the 1st RNG-REQ from a MS

2. BS sends the DSA-RSP to such MS

b) Network re-entry time is the amount of time between:

1. BS received the 1st RNG-REQ from a MS with the

Ranging Purpose Indication TLV to indicate that

the MS is leaving the idle mode

2. BS sends the DSA-RSP to such MS

N = the number of network entries

Avg(N) = the average amount of time after N network entries

Avg(0) = T, the 1st network entry

Avg(N) = (Avg(N-1)*(N-1) + T) / N

Max(N) = the maximum amount of time after N network entries

Max(0) = T, the 1st network entry

Max(N) = T if T > Max(N-1)

= Max(N-1) if T <= Max(N-1)

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap."

::= { wmanIf2BsPm 5 }

wmanIf2BsNetworkEntryStatisticsEntry OBJECT-TYPE

SYNTAX      WmanIf2BsNetworkEntryStatisticsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsNetworkEntryStatisticsTable 1 }

WmanIf2BsNetworkEntryStatisticsEntry ::= SEQUENCE {

wmanIf2BsAvgNetworkEntryTime            INTEGER,

wmanIf2BsMaxNetworkEntryTime            INTEGER,

wmanIf2BsAvgNetworkReEntryTime          INTEGER,

wmanIf2BsMaxNetworkReEntryTime          INTEGER,

wmanIf2BsNumOfNetworkEntries            Counter64,

wmanIf2BsNumOfNetworkReEntries          Counter64}

wmanIf2BsAvgNetworkEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average network entry time Avg(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 1 }

wmanIf2BsMaxNetworkEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the maximum network entry time Max(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 2 }

wmanIf2BsAvgNetworkReEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the average network re-entry time

Avg(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 3 }

wmanIf2BsMaxNetworkReEntryTime OBJECT-TYPE

SYNTAX      INTEGER

UNITS       "second"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object records the maximum network re-entry time

Max(N)."

::= { wmanIf2BsNetworkEntryStatisticsEntry 4 }

wmanIf2BsNumOfNetworkEntries OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object tracks the number of network entries 'N'."

::= { wmanIf2BsNetworkEntryStatisticsEntry 5 }

wmanIf2BsNumOfNetworkReEntries OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object tracks the number of network re-entries 'N'."

::= { wmanIf2BsNetworkEntryStatisticsEntry 6 }

-- XXX

-- Mobile Packet Error Rate Table

--

wmanIf2BsPacketErrorRateTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsPacketErrorRateEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the packet error rate information.

BS should reset all counters after they have been reported

to EMS via wmanIf2BsPerformanceCountersTrap"

::= { wmanIf2BsPm 6 }

wmanIf2BsPacketErrorRateEntry OBJECT-TYPE

SYNTAX      WmanIf2BsPacketErrorRateEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each BS sector."

INDEX     { ifIndex }

::= { wmanIf2BsPacketErrorRateTable  1 }

WmanIf2BsPacketErrorRateEntry ::= SEQUENCE {

wmanIf2BsDlPacketsSend                  Counter64,

wmanIf2BsDlPacketsErrored               Counter64,

wmanIf2BsDlPacketErrorRate              Unsigned32}

wmanIf2BsDlPacketsSend OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the total number of MAC SDUs that a BS

has sent."

::= { wmanIf2BsPacketErrorRateEntry 1 }

wmanIf2BsDlPacketsErrored OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the total number of MAC SDUs including

at least an ARQ block that has not been successful

acknowledged (i.e. timeout or NAK)."

::= { wmanIf2BsPacketErrorRateEntry 2 }

wmanIf2BsDlPacketErrorRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "1x10E-7"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2BsDlPacketErrorRate = (wmanIf2BsDlPacketsErrored /

wmanIf2BsDlPacketsSend) * 10000000"

::= { wmanIf2BsPacketErrorRateEntry 3 }

--

-- wmanIf2BsSm group – containing tables and objects related to Security

--                     Management (i.e. Privacy Sublayer objects)

--

wmanIf2BsPkmSecurityCapabilityTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsPkmSecurityCapabilityEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the list of the cryptographic

suite(s) an BS supports."

REFERENCE

"Subclause 11.9.13 in IEEE Std 802.16-2004"

::= { wmanIf2BsSm 1 }

wmanIf2BsPkmSecurityCapabilityEntry OBJECT-TYPE

SYNTAX      WmanIf2BsPkmSecurityCapabilityEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2BsPkmSecurityCapIndex }

::= { wmanIf2BsPkmSecurityCapabilityTable  1 }

WmanIf2BsPkmSecurityCapabilityEntry ::= SEQUENCE {

wmanIf2BsPkmSecurityCapIndex            INTEGER,

wmanIf2BsPkmScDataEncryptAlgorithm      WmanIf2DataEncryptAlgId,

wmanIf2BsPkmScDataAuthentAlgorithm      WmanIf2DataAuthAlgId,

wmanIf2BsPkmScEncryptAlgorithm          WmanIf2TekEncryptAlgId}

wmanIf2BsPkmSecurityCapIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry

in the wmanIf2BsPkmSecurityCapabilityTable"

::= { wmanIf2BsPkmSecurityCapabilityEntry 1 }

wmanIf2BsPkmScDataEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object is the data encryption algorithm

being utilized."

REFERENCE

"Table 375, IEEE Std 802.16-2004"

::= { wmanIf2BsPkmSecurityCapabilityEntry 2 }

wmanIf2BsPkmScDataAuthentAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataAuthAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object is the data authentication

algorithm being utilized."

REFERENCE

"Table 376, IEEE Std 802.16-2004"

::= { wmanIf2BsPkmSecurityCapabilityEntry 3 }

wmanIf2BsPkmScEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2TekEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object is the TEK key encryption

algorithm being utilized."

REFERENCE

"Table 377, IEEE Std 802.16-2004"

::= { wmanIf2BsPkmSecurityCapabilityEntry 4 }

--

-- Table wmanIf2BsSsPkmSecurityCapabilityTable

--

wmanIf2BsSsPkmSecurityCapabilityTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsPkmSecurityCapabilityEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the SS's Security Capabilities that are

conveyed by the Auth Request or PKMv2 SA-TEK-Request

message. It contains the list of the cryptographic suite(s)

an SS supports."

REFERENCE

"Subclause 11.9.13 in IEEE Std 802.16-2004 and

Table 37i in IEEE Std 802.16e-2005"

::= { wmanIf2BsSm 2 }

wmanIf2BsSsPkmSecurityCapabilityEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsPkmSecurityCapabilityEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsSsPkmSecurityCapIndex }

::= { wmanIf2BsSsPkmSecurityCapabilityTable  1 }

WmanIf2BsSsPkmSecurityCapabilityEntry ::= SEQUENCE {

wmanIf2BsSsPkmSecurityCapIndex          INTEGER,

wmanIf2BsSsPkmScDataEncryptAlgorithm    WmanIf2DataEncryptAlgId,

wmanIf2BsSsPkmScDataAuthentAlgorithm    WmanIf2DataAuthAlgId,

wmanIf2BsSsPkmScEncryptAlgorithm        WmanIf2TekEncryptAlgId}

wmanIf2BsSsPkmSecurityCapIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry

in the wmanIf2BsSsPkmSecurityCapabilityTable"

::= { wmanIf2BsSsPkmSecurityCapabilityEntry 1 }

wmanIf2BsSsPkmScDataEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object is the data encryption algorithm

being utilized."

REFERENCE

"Table 375, IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmSecurityCapabilityEntry 2 }

wmanIf2BsSsPkmScDataAuthentAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataAuthAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object is the data authentication

algorithm being utilized."

REFERENCE

"Table 376, IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmSecurityCapabilityEntry 3 }

wmanIf2BsSsPkmScEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2TekEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this object is the TEK key encryption

algorithm being utilized."

REFERENCE

"Table 377, IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmSecurityCapabilityEntry 4 }

wmanIf2BsPkmV1Objects OBJECT IDENTIFIER ::= { wmanIf2BsSm 3 }

--

-- Table wmanIf2BsPkmV1ConfigTable

--

wmanIf2BsPkmV1ConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsPkmV1ConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration of the PKM

attributes that are to be used for BS and SS."

REFERENCE

"Table 343 in IEEE Std 802.16-2004 and 802.16e-2005"

::= { wmanIf2BsPkmV1Objects  1 }

wmanIf2BsPkmV1ConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsPkmV1ConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry contains objects that define the PKM attributes

of each BS wireless interface, and all SSs that are

connected with such BS."

INDEX     { ifIndex }

::= { wmanIf2BsPkmV1ConfigTable  1 }

WmanIf2BsPkmV1ConfigEntry ::= SEQUENCE {

wmanIf2BsPkmV1AkLifetime                Integer32,

wmanIf2BsPkmV1TekLifetime               Integer32,

wmanIf2BsPkmV1SelfSigManufCertTrust     INTEGER,

wmanIf2BsPkmV1AuthWaitTimeout           Integer32,

wmanIf2BsPkmV1ReauthWaitTimeout         Integer32,

wmanIf2BsPkmV1AuthGraceTime             Integer32,

wmanIf2BsPkmV1OpWaitTimeout             Integer32,

wmanIf2BsPkmV1RekeyWaitTimeout          Integer32,

wmanIf2BsPkmV1TekGraceTime              Integer32,

wmanIf2BsPkmV1AuthRejectWaitTimeout     Integer32,

wmanIf2BsPkmV1CheckCertValidityPeriods  TruthValue}

wmanIf2BsPkmV1AkLifetime OBJECT-TYPE

SYNTAX      Integer32 (86400 .. 6048000)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the lifetime of a newly assigned

authorization key."

REFERENCE

"Table 343 in IEEE Std 802.16-2004"

DEFVAL         { 604800 }

::= { wmanIf2BsPkmV1ConfigEntry 1 }

wmanIf2BsPkmV1TekLifetime OBJECT-TYPE

SYNTAX      Integer32 (1800 .. 604800)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the lifetime of a newly assigned

Traffic Encryption Key(TEK)."

REFERENCE

"Table 343 in IEEE Std 802.16-2004"

DEFVAL         { 43200 }

::= { wmanIf2BsPkmV1ConfigEntry 2 }

wmanIf2BsPkmV1SelfSigManufCertTrust OBJECT-TYPE

SYNTAX      INTEGER {trusted (1),

untrusted (2)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object determines the default trust of all (new)

self-signed manufacturer certificates obtained after

setting the object."

::= { wmanIf2BsPkmV1ConfigEntry 3 }

wmanIf2BsPkmV1AuthWaitTimeout OBJECT-TYPE

SYNTAX      Integer32 (2 .. 30)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the Auth Req retransmission interval

from Auth Wait state."

REFERENCE

"Table 343 and subclause 11.9.19 in IEEE Std 802.16-2004"

DEFVAL      { 10 }

::= { wmanIf2BsPkmV1ConfigEntry 4 }

wmanIf2BsPkmV1ReauthWaitTimeout OBJECT-TYPE

SYNTAX      Integer32 (2 .. 30)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the Auth Req retransmission interval

from Reauth Wait state."

REFERENCE

"Table 343 and subclause 11.9.19 in IEEE Std 802.16-2004"

DEFVAL      { 10 }

::= { wmanIf2BsPkmV1ConfigEntry 5 }

wmanIf2BsPkmV1AuthGraceTime OBJECT-TYPE

SYNTAX      Integer32 (300 .. 3024000)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The value of this object is the grace time for an

authorization key. A SS is expected to start trying to get

a new authorization key beginning AuthGraceTime seconds

before the authorization key actually expires."

REFERENCE

"Table 343 and subclause 11.9.19 in IEEE Std 802.16-2004"

DEFVAL      { 600 }

::= { wmanIf2BsPkmV1ConfigEntry 6 }

wmanIf2BsPkmV1OpWaitTimeout OBJECT-TYPE

SYNTAX      Integer32 (1 .. 10)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the Key Req retransmission interval

from Op Wait state."

REFERENCE

"Table 343 and subclause 11.9.19 in IEEE Std 802.16-2004"

DEFVAL      { 1 }

::= { wmanIf2BsPkmV1ConfigEntry 7 }

wmanIf2BsPkmV1RekeyWaitTimeout OBJECT-TYPE

SYNTAX      Integer32 (1 .. 10)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the Key Req retransmission interval

from Rekey Wait state."

REFERENCE

"Table 343 and subclause 11.9.19 in IEEE Std 802.16-2004"

DEFVAL      { 1 }

::= { wmanIf2BsPkmV1ConfigEntry 8 }

wmanIf2BsPkmV1TekGraceTime OBJECT-TYPE

SYNTAX      Integer32 (300 .. 3024000)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The value of this object is the grace time for the TEK in

seconds. The SS is expected to start trying to acquire a

new TEK beginning TEK GraceTime seconds before the

expiration of the most recent TEK."

REFERENCE

"Table 343 and subclause 11.9.19 in IEEE Std 802.16-2004"

DEFVAL      { 3600 }

::= { wmanIf2BsPkmV1ConfigEntry 9 }

wmanIf2BsPkmV1AuthRejectWaitTimeout OBJECT-TYPE

SYNTAX      Integer32 (10 .. 600)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the Delay before resending Auth Request

after receiving Auth Reject."

REFERENCE

"Table 343 and subclause 11.9.19 in IEEE Std 802.16-2004"

DEFVAL      { 60 }

::= { wmanIf2BsPkmV1ConfigEntry 10 }

wmanIf2BsPkmV1CheckCertValidityPeriods OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Setting this object to TRUE causes all certificates

received thereafter to have their validity periods (and

their chain's validity periods) checked against the current

time of day. A FALSE setting will cause all certificates

received Thereafter to not have their validity periods

(nor their chain's validity periods) checked against the

current time of day."

::= { wmanIf2BsPkmV1ConfigEntry 11 }

--

-- Table wmanIf2BsSsPkmV1AuthorizationTable

--

wmanIf2BsSsPkmV1AuthorizationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsPkmV1AuthorizationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains information related to SS's

authorization process."

REFERENCE

"Table 28 and 37 in IEEE Std 802.16-2004"

::= { wmanIf2BsPkmV1Objects 2 }

wmanIf2BsSsPkmV1AuthorizationEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsPkmV1AuthorizationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry contains objects that define the SS

authorization attributes for each SS associated with each

BS sector."

INDEX     { ifIndex, wmanIf2BsSsPkmV1AuthMacAddress }

::= { wmanIf2BsSsPkmV1AuthorizationTable  1 }

WmanIf2BsSsPkmV1AuthorizationEntry ::= SEQUENCE {

wmanIf2BsSsPkmV1AuthMacAddress          MacAddress,

wmanIf2BsSsPkmV1CaCertificate           OCTET STRING,

wmanIf2BsSsPkmV1SsCertificate           OCTET STRING,

wmanIf2BsSsPkmV1PrimarySaId             INTEGER,

wmanIf2BsSsPkmV1AuthKeySequenceNumber   Integer32,

wmanIf2BsSsPkmV1AuthKeyLifetime         Integer32,

wmanIf2BsSsPkmV1AuthRejectError         WmanIf2PkmErrorCode,

wmanIf2BsSsPkmV1AuthInvalidError        WmanIf2PkmErrorCode,

wmanIf2BsSsPkmV1AkN-1ExpireTime         DateAndTime,

wmanIf2BsSsPkmV1AkNExpireTime           DateAndTime,

wmanIf2BsSsPkmV1CertificateStatus       WmanIf2CertificateStat,

wmanIf2BsSsPkmV1AuthReset               INTEGER}

wmanIf2BsSsPkmV1AuthMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The value of this object is the physical address of the SS

to which the authorization association applies."

::= { wmanIf2BsSsPkmV1AuthorizationEntry 1 }

wmanIf2BsSsPkmV1CaCertificate OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SS sends the CA-Certificate in the Auth Info message. It

contains an X.509 CA certificate for the manufacturer of

the SS. The SS's X.509 user certificate shall have been

issued by the CA identified by the X.509 CA certificate."

REFERENCE

"Table 37 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1AuthorizationEntry 2 }

wmanIf2BsSsPkmV1SsCertificate OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SS sends the SS-Certificate in the Auth Request message.

It contains an X.509 SS certificate issued by the SS's

manufacturer. The SS's X.509 certificate is a public-key

certificate which binds the SS's identifying information

to its RSA public key in a verifiable manner. The X.509

certificate is digitally signed by the SS's manufacturer,

and that signature can be verified by a BS that knows

the manufacturer's public key. The manufacturer's public

key is placed in an X.509 certification authority (CA)

certificate, which in turn is signed by a higher level CA."

REFERENCE

"Table 28 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1AuthorizationEntry 3 }

wmanIf2BsSsPkmV1PrimarySaId OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SS's primary SAID equal to the Basic CID."

REFERENCE

"Subclause 6.3.2.3.9.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1AuthorizationEntry 4 }

wmanIf2BsSsPkmV1AuthKeySequenceNumber OBJECT-TYPE

SYNTAX      Integer32 (0 .. 15)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the most recent authorization key

sequence number in the Auth Reply message for an SS."

REFERENCE

"Table 29 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1AuthorizationEntry 5 }

wmanIf2BsSsPkmV1AuthKeyLifetime OBJECT-TYPE

SYNTAX      Integer32 (86400..6048000)

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the lifetime of an authorization

key (AK) the BS assigns to a SS."

REFERENCE

"Table 343 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1AuthorizationEntry 6 }

wmanIf2BsSsPkmV1AuthRejectError OBJECT-TYPE

SYNTAX      WmanIf2PkmErrorCode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The Error Code in most recent Authorization Reject message

transmitted to the SS.

The valid codes are:

0 - no failure

1 - unauthorized SS

2 - unauthorized SAID

6..11 - permanent authorization failure"

REFERENCE

"Table 371, Subclause 11.9.10, in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1AuthorizationEntry 7 }

wmanIf2BsSsPkmV1AuthInvalidError OBJECT-TYPE

SYNTAX      WmanIf2PkmErrorCode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The Error Code in most recent Authorization Invalid message

transmitted to the SS.

The valid codes are:

0 - no failure

1 - unauthorized SS

3 - unsolicited

4 - invalid key sequence

5 - key request authentication failure"

REFERENCE

"Table 371, Subclause 11.9.10, in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1AuthorizationEntry 8 }

wmanIf2BsSsPkmV1AkN-1ExpireTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the time when AK(N-1) expires.

wmanIf2BsSsPkmV1AkN-1ExpireTime =

Auth Reply[AK(N-1)] arrival time +  AK(N-1) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV1AkN-1ExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV1AuthorizationEntry 9 }

wmanIf2BsSsPkmV1AkNExpireTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the time when AK(N) expires.

wmanIf2BsSsPkmV1AkNExpireTime =

Auth Reply[AK(N)] arrival time +  AK(N) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV1AkNExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV1AuthorizationEntry 10 }

wmanIf2BsSsPkmV1CertificateStatus OBJECT-TYPE

SYNTAX      WmanIf2CertificateStat

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicate the reason why a SS's certificate is deemed valid

or invalid."

::= { wmanIf2BsSsPkmV1AuthorizationEntry 11 }

wmanIf2BsSsPkmV1AuthReset OBJECT-TYPE

SYNTAX      INTEGER {noResetRequested(1),

invalidateAuth(2),

sendAuthInvalid(3),

invalidateTeks(4)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Setting this object to:

1 - no reset

2 - causes the BS to invalidate the current SS

authorization key(s), but not to transmit an

Authorization Invalid message nor to invalidate

unicast TEKs.

3 - causes the BS to invalidate the current SS

authorization key(s), and to transmit an

Authorization Invalid message to the SS, but not

to invalidate unicast TEKs.

4 - causes the BS to invalidate the current SS

authorization key(s), to transmit an Authorization

Invalid message to the SS, and to invalidate all

unicast TEKs associated with this SS authorization.

Reading this object returns the most-recently-set value

of this object, or returns noResetRequested(1) if the

object has not been set since the last BS reboot."

::= { wmanIf2BsSsPkmV1AuthorizationEntry 12 }

--

-- Table wmanIf2BsSsPkmV1TekTable

--

wmanIf2BsSsPkmV1TekTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsPkmV1TekEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the TEK attributes that are associated

with each SAID."

::= { wmanIf2BsPkmV1Objects 3 }

wmanIf2BsSsPkmV1TekEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsPkmV1TekEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsSsPkmV1SaidIndex }

::= { wmanIf2BsSsPkmV1TekTable  1 }

WmanIf2BsSsPkmV1TekEntry ::= SEQUENCE {

wmanIf2BsSsPkmV1SaidIndex               INTEGER,

wmanIf2BsSsPkmV1SaType                  WmanIf2SaType,

wmanIf2BsSsPkmV1TekDataEncryptAlgorithm WmanIf2DataEncryptAlgId,

wmanIf2BsSsPkmV1TekDataAuthentAlgorithm WmanIf2DataAuthAlgId,

wmanIf2BsSsPkmV1TekEncryptAlgorithm     WmanIf2TekEncryptAlgId,

wmanIf2BsSsPkmV1TekN-1SequenceNumber    Integer32,

wmanIf2BsSsPkmV1TekN-1Lifetime          Integer32,

wmanIf2BsSsPkmV1TekNSequenceNumber      Integer32,

wmanIf2BsSsPkmV1TekNLifetime            Integer32,

wmanIf2BsSsPkmV1KeyRejectError          WmanIf2PkmErrorCode,

wmanIf2BsSsPkmV1TekInvalidError         WmanIf2PkmErrorCode,

wmanIf2BsSsPkmV1TekN-1ExpireTime        DateAndTime,

wmanIf2BsSsPkmV1TekNExpireTime          DateAndTime,

wmanIf2BsSsPkmV1TekReset                TruthValue}

wmanIf2BsSsPkmV1SaidIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"SAID index to the wmanIf2BsSsPkmV1TekTable."

::= { wmanIf2BsSsPkmV1TekEntry 1 }

wmanIf2BsSsPkmV1SaType OBJECT-TYPE

SYNTAX      WmanIf2SaType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SA Type attribute that is included in the Auth Reply

message."

::= { wmanIf2BsSsPkmV1TekEntry 2 }

wmanIf2BsSsPkmV1TekDataEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The data encryption algorithm attribute that is included

in the Auth Reply message."

REFERENCE

"Table 375, IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 3 }

wmanIf2BsSsPkmV1TekDataAuthentAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataAuthAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The data authentication algorithm attribute that is

included in the Auth Reply message."

REFERENCE

"Table 376, IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 4 }

wmanIf2BsSsPkmV1TekEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2TekEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The TEK key encryption algorithm attribute that is

included in the Auth Reply message."

REFERENCE

"Table 377, IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 5 }

wmanIf2BsSsPkmV1TekN-1SequenceNumber OBJECT-TYPE

SYNTAX      Integer32 (0 .. 3)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"At all times the BS maintains two sets of active

generations of keying material per SAID. One set

corresponds to the 'N-1' generation of keying material,

the second set corresponds to the 'N' generation of

keying material. The N generation has a key sequence

number one greater than (modulo 4) that of the N-1

generation. This object provides the older TEK sequence

number in the Key Reply message for an SS."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 6 }

wmanIf2BsSsPkmV1TekN-1Lifetime OBJECT-TYPE

SYNTAX      Integer32 (1800 .. 604800)

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the N-1 TEK Remaining Lifetime."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 7 }

wmanIf2BsSsPkmV1TekNSequenceNumber OBJECT-TYPE

SYNTAX      Integer32 (0 .. 3)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the N TEK sequence number in the

Key Reply message for an SS."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 8 }

wmanIf2BsSsPkmV1TekNLifetime OBJECT-TYPE

SYNTAX      Integer32 (1800 .. 604800)

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the N TEK Remaining Lifetime."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 9 }

wmanIf2BsSsPkmV1KeyRejectError OBJECT-TYPE

SYNTAX      WmanIf2PkmErrorCode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The Error Code in the most recent Key Reject message sent

in response to a Key Request for this SAID.

The valid error codes are:

0 - no failure

2 - unauthorized SAID"

REFERENCE

"Table 373; IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 10 }

wmanIf2BsSsPkmV1TekInvalidError OBJECT-TYPE

SYNTAX      WmanIf2PkmErrorCode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The Error Code in the most recent TEK Invalid message sent

in association with this SAID.

The valid error codes are:

0 - no failure

4 - invalid key sequence"

REFERENCE

"Table 373; IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV1TekEntry 11 }

wmanIf2BsSsPkmV1TekN-1ExpireTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the time when TEK(N-1) expires.

wmanIf2BsSsPkmV1TekN-1ExpireTime =

Key Reply[TEK(N-1)] arrival time +  TEK(N-1) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV1TekN-1ExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV1TekEntry 12 }

wmanIf2BsSsPkmV1TekNExpireTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the time when TEK(N) expires.

wmanIf2BsSsPkmV1TekNExpireTime =

Key Reply[TEK(N)] arrival time +  TEK(N) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV1TekNExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV1TekEntry 13 }

wmanIf2BsSsPkmV1TekReset OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Setting this object to TRUE causes the BS to invalidate

the current active TEK(s) (plural due to key transition

periods), and to generate a new TEK for the associated

SAID; the BS MAY also generate an unsolicited TEK Invalid

message, to optimize the TEK synchronization between the BS

and the SS. Reading this object always returns FALSE."

::= { wmanIf2BsSsPkmV1TekEntry 14 }

wmanIf2BsPkmV2Objects OBJECT IDENTIFIER ::= { wmanIf2BsSm 4 }

--

-- Table wmanIf2BsPkmV2ConfigTable

--

wmanIf2BsPkmV2ConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsPkmV2ConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the configuration of the PKM

attributes that are needed to PKM operation."

REFERENCE

"Table 343 in IEEE Std 802.16-2004 and 802.16e-2005"

::= { wmanIf2BsPkmV2Objects  1 }

wmanIf2BsPkmV2ConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2BsPkmV2ConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry contains objects that define the PKM attributes

of each BS."

INDEX     { ifIndex }

::= { wmanIf2BsPkmV2ConfigTable  1 }

WmanIf2BsPkmV2ConfigEntry ::= SEQUENCE {

wmanIf2BsPkmPmkPrehandshakeLifetime     Integer32,

wmanIf2BsPkmPmkLifetime                 Integer32,

wmanIf2BsSaChallengeTimeout             Integer32,

wmanIf2BsMaxSaTekChallenge              Integer32,

wmanIf2BsSaTekTimeout                   Integer32,

wmanIf2BsMaxSaTekRequest                Integer32}

wmanIf2BsPkmPmkPrehandshakeLifetime OBJECT-TYPE

SYNTAX      Integer32 (5 .. 900)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the PMK or PAK prehandshake lifetime."

REFERENCE

"Table 343 in IEEE Std 802.16e-2005"

DEFVAL         { 10 }

::= { wmanIf2BsPkmV2ConfigEntry 1 }

wmanIf2BsPkmPmkLifetime OBJECT-TYPE

SYNTAX      Integer32 (60 .. 86400)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines PMK lifetime, if MSK lifetime is

unspecified (i.e., by AAA server)."

REFERENCE

"Table 343 in IEEE Std 802.16e-2005"

DEFVAL         { 3600 }

::= { wmanIf2BsPkmV2ConfigEntry 2 }

wmanIf2BsSaChallengeTimeout OBJECT-TYPE

SYNTAX      Integer32 (500 .. 2000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the timeout value for SA-TEKChallenge

retransmission."

REFERENCE

"Table 343 in IEEE Std 802.16e-2005"

DEFVAL         { 1000 }

::= { wmanIf2BsPkmV2ConfigEntry 3 }

wmanIf2BsMaxSaTekChallenge OBJECT-TYPE

SYNTAX      Integer32 (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the maximum number of SA-TEK-Challenge

transmissions."

REFERENCE

"Table 343 in IEEE Std 802.16e-2005"

DEFVAL         { 3 }

::= { wmanIf2BsPkmV2ConfigEntry 4 }

wmanIf2BsSaTekTimeout OBJECT-TYPE

SYNTAX      Integer32 (100 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the timeout value for SA-TEKRequest

retransmission."

REFERENCE

"Table 343 in IEEE Std 802.16e-2005"

DEFVAL         { 300 }

::= { wmanIf2BsPkmV2ConfigEntry 5 }

wmanIf2BsMaxSaTekRequest OBJECT-TYPE

SYNTAX      Integer32 (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object defines the maximum number of SA-TEK-Request

retransmission."

REFERENCE

"Table 343 in IEEE Std 802.16e-2005"

DEFVAL         { 3 }

::= { wmanIf2BsPkmV2ConfigEntry 6 }

--

-- Table wmanIf2BsSsPkmV2RsaAuthTable

--

wmanIf2BsSsPkmV2RsaAuthTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsPkmV2RsaAuthEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains information related to PKMV2

RSA based authorization process."

REFERENCE

"Subclause 6.3.2.3.9.11 in IEEE Std 802.16e-2005"

::= { wmanIf2BsPkmV2Objects 2 }

wmanIf2BsSsPkmV2RsaAuthEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsPkmV2RsaAuthEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry contains objects that define the SS

authorization attributes for each SS associated with each

BS sector."

INDEX     { ifIndex, wmanIf2BsSsMacAddress }

::= { wmanIf2BsSsPkmV2RsaAuthTable  1 }

WmanIf2BsSsPkmV2RsaAuthEntry ::= SEQUENCE {

wmanIf2BsSsPkmV2BsCertificate           OCTET STRING,

wmanIf2BsSsPkmV2SsCertificate           OCTET STRING,

wmanIf2BsSsPkmV2SaId                    INTEGER,

wmanIf2BsSsPkmV2SsRandom                OCTET STRING,

wmanIf2BsSsPkmV2BsRandom                OCTET STRING,

wmanIf2BsSsPkmV2AuthKeySequenceNumber   Integer32,

wmanIf2BsSsPkmV2AuthKeyLifetime         Integer32,

wmanIf2BsSsPkmV2AuthResult              INTEGER,

wmanIf2BsSsPkmV2AuthFailure             WmanIf2PkmErrorCode,

wmanIf2BsSsPkmV2AkN-1ExpireTime         DateAndTime,

wmanIf2BsSsPkmV2AkNExpireTime           DateAndTime,

wmanIf2BsSsPkmV2CertificateStatus       WmanIf2CertificateStat}

wmanIf2BsSsPkmV2BsCertificate OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"BS sends the BS-Certificate in the PKMV2 RSA-Reply message

for BS-SS mutual authentication. It is the DER-encoded

ASN.1 X.509 BS Certificate."

REFERENCE

"Subclause 11.9.24 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 1 }

wmanIf2BsSsPkmV2SsCertificate OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SS sends the SS-Certificate in the PKMV2 RSA-Request

message. It contains an X.509 SS certificate issued by the

SS's manufacturer. The SS's X.509 certificate is a

public-key certificate which binds the SS's identifying

information to its RSA public key in a verifiable manner.

The X.509 certificate is digitally signed by the SS's

manufacturer, and that signature can be verified by a BS

that knows the manufacturer's public key.

The manufacturer's public key is placed in an X.509

certification authority (CA) certificate, which in turn

is signed by a higher level CA."

REFERENCE

"Subclause 11.9.12 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 2 }

wmanIf2BsSsPkmV2SaId OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SS's primary SAID equal to the Basic CID. SS sends the SAID

in the PKMV2 RSA-Request message."

REFERENCE

"Subclause 6.3.2.3.9.2 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 3 }

wmanIf2BsSsPkmV2SsRandom OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(8))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This attribute contains a quantity that is pseudo random

number generated from the MS and used as fresh number for

mutual authorization message handshake. SS sends the

SS-Random in the PKMV2 RSA-Request message."

REFERENCE

"Subclause 11.9.21 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 4 }

wmanIf2BsSsPkmV2BsRandom OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(8))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This attribute contains a quantity that is pseudo random

number generated from the BS and used as fresh number for

mutual authorization message handshake.BS sends the

BS-Random in the PKMV2 RSA-Reply message."

REFERENCE

"Subclause 11.9.22 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 5 }

wmanIf2BsSsPkmV2AuthKeySequenceNumber OBJECT-TYPE

SYNTAX      Integer32 (0 .. 15)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the most recent authorization key

sequence number in the PKMV2 RSA-Reply message for an SS."

REFERENCE

"Subclause 11.9.5 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 6 }

wmanIf2BsSsPkmV2AuthKeyLifetime OBJECT-TYPE

SYNTAX      Integer32 (86400..6048000)

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the lifetime of an authorization

key (AK) the BS assigns to a SS. BS sends the key lifetime

in the PKMV2 RSA-Reply message."

REFERENCE

"Subclause 11.9.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 7 }

wmanIf2BsSsPkmV2AuthResult OBJECT-TYPE

SYNTAX      INTEGER {success(0),

reject(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This attribute contains the result code of the RSA-based

authorization. SS sends the result code in PKMV2

RSA-Acknowledgement message."

REFERENCE

"Subclause 11.9.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 8 }

wmanIf2BsSsPkmV2AuthFailure OBJECT-TYPE

SYNTAX      WmanIf2PkmErrorCode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"BS returs PKMV2 RSA-Rejects message if an authorization

failure is detected.

Failure type umknownManufactur(4)- ssBsIncompatibleSc(9)are

considered permanent authorization failure, since any

attempts of reauthorization would continue to result in

Authorization Rejects. Details about the cause of a

Permanent Authorization Failure may be reported to the SS

in an optional Display-String attribute that may accompany

the Error-Code attribute in Authorization Reject messages.

Note that the BS may log the Display-String attribute and

Authorization failures in wmanIfDevMib, and generate a trap

to an SNMP manager."

REFERENCE

"Subclause 11.9.10 in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV2RsaAuthEntry 9 }

wmanIf2BsSsPkmV2AkN-1ExpireTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the time when AK(N-1) expires.

wmanIf2BsSsPkmV2AkN-1ExpireTime =

RSA-Reply[AK(N-1)] arrival time +  AK(N-1) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV2AkN-1ExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV2RsaAuthEntry 10 }

wmanIf2BsSsPkmV2AkNExpireTime OBJECT-TYPE

SYNTAX      DateAndTime

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the time when AK(N) expires.

wmanIf2BsSsPkmV2AkNExpireTime =

RSA-Reply[AK(N)] arrival time +  AK(N) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV2AkNExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV2RsaAuthEntry 11 }

wmanIf2BsSsPkmV2CertificateStatus OBJECT-TYPE

SYNTAX      WmanIf2CertificateStat

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicate the reason why a SS's certificate is deemed valid

or invalid."

::= { wmanIf2BsSsPkmV2RsaAuthEntry 12 }

--

-- Table wmanIf2BsSsPkmV2TekTable

--

wmanIf2BsSsPkmV2TekTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsPkmV2TekEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the TEK attributes that are associated

with each SAID."

::= { wmanIf2BsPkmV2Objects 3 }

wmanIf2BsSsPkmV2TekEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsPkmV2TekEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex,

wmanIf2BsSsMacAddress,

wmanIf2BsSsPkmV2SaidIndex }

::= { wmanIf2BsSsPkmV2TekTable  1 }

WmanIf2BsSsPkmV2TekEntry ::= SEQUENCE {

wmanIf2BsSsPkmV2SaidIndex               INTEGER,

wmanIf2BsSsPkmV2SaType                  WmanIf2SaType,

wmanIf2BsSsPkmV2SaServiceType           WmanIf2SaServiceType,

wmanIf2BsSsPkmV2TekDataEncryptAlgorithm WmanIf2DataEncryptAlgId,

wmanIf2BsSsPkmV2TekDataAuthentAlgorithm WmanIf2DataAuthAlgId,

wmanIf2BsSsPkmV2TekEncryptAlgorithm     WmanIf2TekEncryptAlgId,

wmanIf2BsSsPkmV2TekN-1SequenceNumber    Integer32,

wmanIf2BsSsPkmV2TekN-1Lifetime          Integer32,

wmanIf2BsSsPkmV2TekNSequenceNumber      Integer32,

wmanIf2BsSsPkmV2TekNLifetime            Integer32,

wmanIf2BsSsPkmV2TekInvalidError         WmanIf2PkmErrorCode,

wmanIf2BsSsPkmV2TekN-1ExpireTime        DateAndTime,

wmanIf2BsSsPkmV2TekNExpireTime          DateAndTime}

wmanIf2BsSsPkmV2SaidIndex OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"SAID index to the wmanIf2BsSsPkmV2TekTable."

::= { wmanIf2BsSsPkmV2TekEntry 1 }

wmanIf2BsSsPkmV2SaType OBJECT-TYPE

SYNTAX      WmanIf2SaType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SA Type attribute that is included in the PKMv2

SA-TEK-response message."

REFERENCE

"Table 383, IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 2 }

wmanIf2BsSsPkmV2SaServiceType OBJECT-TYPE

SYNTAX      WmanIf2SaServiceType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"SA Type attribute that is included in the PKMv2

SA-TEK-response message."

REFERENCE

"Table 383, IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 3 }

wmanIf2BsSsPkmV2TekDataEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The data encryption algorithm attribute that is included

in the PKMv2 SA-TEK-response message."

REFERENCE

"Table 383, IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 4 }

wmanIf2BsSsPkmV2TekDataAuthentAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2DataAuthAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The data authentication algorithm attribute that is

included in the PKMv2 SA-TEK-response message."

REFERENCE

"Table 383, IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 5 }

wmanIf2BsSsPkmV2TekEncryptAlgorithm OBJECT-TYPE

SYNTAX      WmanIf2TekEncryptAlgId

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The TEK key encryption algorithm attribute that is

included in the PKMv2 SA-TEK-response message."

REFERENCE

"Table 383, IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 6 }

wmanIf2BsSsPkmV2TekN-1SequenceNumber OBJECT-TYPE

SYNTAX      Integer32 (0 .. 3)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"At all times the BS maintains two sets of active

generations of keying material per SAID. One set

corresponds to the 'N-1' generation of keying material,

the second set corresponds to the 'N' generation of

keying material. The N generation has a key sequence

number one greater than (modulo 4) that of the N-1

generation. This object provides the older TEK sequence

number in the Key Reply message for an SS."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 7 }

wmanIf2BsSsPkmV2TekN-1Lifetime OBJECT-TYPE

SYNTAX      Integer32 (1800 .. 604800)

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the N-1 TEK Remaining Lifetime."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 8 }

wmanIf2BsSsPkmV2TekNSequenceNumber OBJECT-TYPE

SYNTAX      Integer32 (0 .. 3)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the N TEK sequence number in the

Key Reply message for an SS."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 9 }

wmanIf2BsSsPkmV2TekNLifetime OBJECT-TYPE

SYNTAX      Integer32 (1800 .. 604800)

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides the N TEK Remaining Lifetime."

REFERENCE

"Subclause 11.9.8 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 10 }

wmanIf2BsSsPkmV2TekInvalidError OBJECT-TYPE

SYNTAX      WmanIf2PkmErrorCode

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"BS returns the PKMv2 TEK-Invalid message if the BS

determines that the MS encrypted an UL PDU with an invalid

TEK.

Note that the BS may log the Display-String attribute and

PKMv2 TEK-Invalid error in wmanIfDevMib."

REFERENCE

"Subclause 11.9.10 in IEEE Std 802.16-2004, and

Subclause 6.3.2.3.9.25 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV2TekEntry 11 }

wmanIf2BsSsPkmV2TekN-1ExpireTime OBJECT-TYPE

SYNTAX DateAndTime

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object is the time when TEK(N-1) expires.

wmanIf2BsSsPkmV2TekN-1ExpireTime =

Key Reply[TEK(N-1)] arrival time +  TEK(N-1) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV2TekN-1ExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV2TekEntry 12 }

wmanIf2BsSsPkmV2TekNExpireTime OBJECT-TYPE

SYNTAX DateAndTime

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object is the time when TEK(N) expires.

wmanIf2BsSsPkmV2TekNExpireTime =

Key Reply[TEK(N)] arrival time +  TEK(N) lifetime

If this FSM has only one authorization key, then

wmanIf2BsSsPkmV2TekNExpireTime = the activation of FSM."

::= { wmanIf2BsSsPkmV2TekEntry 13 }

--

-- Table wmanIf2BsSsPkmV23wayHandshakeTable

--

wmanIf2BsSsPkmV23wayHandshakeTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2BsSsPkmV23wayHandshakeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains information related to PKMV2 3-way

handshake process."

REFERENCE

"Subclause 7.8.1 in IEEE Std 802.16e-2005"

::= { wmanIf2BsPkmV2Objects 4 }

wmanIf2BsSsPkmV23wayHandshakeEntry OBJECT-TYPE

SYNTAX      WmanIf2BsSsPkmV23wayHandshakeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry contains objects that define the SS 3-way

handshake attributes for each SS associated with each

BS sector."

INDEX     { ifIndex, wmanIf2BsSsMacAddress }

::= { wmanIf2BsSsPkmV23wayHandshakeTable  1 }

WmanIf2BsSsPkmV23wayHandshakeEntry ::= SEQUENCE {

wmanIf2BsSsPkmV2SaTekBsRandom           OCTET STRING,

wmanIf2BsSsPkmV2SaTekAkSequenceNumber   Integer32,

wmanIf2BsSsPkmV2SaTekAkId               OCTET STRING,

wmanIf2BsSsPkmV2KeyLifetime             Integer32,

wmanIf2BsSsPkmV2SaTekMsRandom           OCTET STRING}

wmanIf2BsSsPkmV2SaTekBsRandom OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(8))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This attribute contains a quantity that is a random number

generated from the BS and used as fresh number for 3-way

handshake process. BS sends the BS-Random in the PKMV2

SA-TEK-Challenge message."

REFERENCE

"Table 37h, Subclause 11.9.22 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV23wayHandshakeEntry 1 }

wmanIf2BsSsPkmV2SaTekAkSequenceNumber OBJECT-TYPE

SYNTAX      Integer32 (0 .. 15)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the sequence number of root keys

(PAK and PMK) for the AK. This value is the most

significant 2-bit of PAK sequence number concatenated

with the least significant 2-bit of PMK sequence number.

BS sends the sequence number in the PKMV2 SA-TEK-Challenge

message."

REFERENCE

"Table 37h, 133a in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV23wayHandshakeEntry 2 }

wmanIf2BsSsPkmV2SaTekAkId OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(8))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This objects is used to identify the AK. BS sends the AKID

attribute in the PKMV2 SA-TEK-Challenge message."

REFERENCE

"Table 37h, 133a in IEEE Std 802.16-2004"

::= { wmanIf2BsSsPkmV23wayHandshakeEntry 3 }

wmanIf2BsSsPkmV2KeyLifetime OBJECT-TYPE

SYNTAX      Integer32 (60 .. 86400)

UNITS       "seconds"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the PMK lifetime. BS sends the key

lifetime attribute in the PKMV2 SA-TEK-Challenge message."

REFERENCE

"Table 37h, Subclause 11.9.4 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV23wayHandshakeEntry 4 }

wmanIf2BsSsPkmV2SaTekMsRandom OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(8))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This attribute contains a quantity that is a random number

generated from the MS and used as fresh number for 3-way

handshake process. MS sends the MS-Random in the PKMV2

SA-TEK-Request message."

REFERENCE

"Table 37i, Subclause 11.9.21 in IEEE Std 802.16e-2005"

::= { wmanIf2BsSsPkmV23wayHandshakeEntry 5 }

--

-- Conformance Information

--

wmanIf2MibConformance OBJECT IDENTIFIER ::= {wmanIf2BsMib 2}

wmanIf2BsMibGroups      OBJECT IDENTIFIER ::= {wmanIf2MibConformance 1}

wmanIf2BsMibCompliances OBJECT IDENTIFIER ::= {wmanIf2MibConformance 2}

-- compliance statements

wmanIf2BsMibCompliance MODULE-COMPLIANCE

STATUS      current

DESCRIPTION

"The compliance statement for devices that implement

Wireless MAN interfaces as defined in IEEE Std 802.16-2004."

MODULE  -- wmanIf2BsMib

GROUP wmanIf2MibBsGroup      -- conditionally mandatory group

DESCRIPTION

"This group is mandatory for Base Station."

GROUP wmanIf2MibBsAasGroup   -- optional group

DESCRIPTION

"This group is mandatory for Base Station."

GROUP wmanIf2MibBsOfdmGroup  -- conditionally mandatory group

DESCRIPTION

"This group is mandatory for Base Station

implementaing the OFDM PHY."

GROUP wmanIf2MibBsOfdmaGroup -- conditionally mandatory group

DESCRIPTION

"This group is mandatory for Base Station

implementaing the OFDMA PHY."

GROUP wmanIf2MibBsNotificationGroup -- unconditionally

-- mandatory groups

DESCRIPTION

"This group is mandatory for Base Station."

::= { wmanIf2BsMibCompliances 1 }

wmanIf2MibBsGroup     OBJECT-GROUP

OBJECTS {-- Capability negotiation

wmanIf2BsSsBasicCid,

wmanIf2BsSsPrimaryCid,

wmanIf2BsSsSecondaryCid,

wmanIf2BsSsManagementSupport,

wmanIf2BsSsIpManagementMode,

wmanIf2Bs2ndMgmtDlQoSProfileIndex,

wmanIf2Bs2ndMgmtUlQoSProfileIndex,

wmanIf2BsAutoSfidEnabled,

wmanIf2BsAutoSfidRangeMin,

wmanIf2BsAutoSfidRangeMax,

wmanIf2BsResetSector,

wmanIf2BsSs2ndMgmtArqEnable,

wmanIf2BsSs2ndMgmtArqWindowSize,

wmanIf2BsSs2ndMgmtArqDnLinkTxDelay,

wmanIf2BsSs2ndMgmtArqUpLinkTxDelay,

wmanIf2BsSs2ndMgmtArqDnLinkRxDelay,

wmanIf2BsSs2ndMgmtArqUpLinkRxDelay,

wmanIf2BsSs2ndMgmtArqBlockLifetime,

wmanIf2BsSs2ndMgmtArqSyncLossTimeout,

wmanIf2BsSs2ndMgmtArqDeliverInOrder,

wmanIf2BsSs2ndMgmtArqRxPurgeTimeout,

wmanIf2BsSs2ndMgmtArqBlockSize,

wmanIf2BsSsVendorIdEncoding,

wmanIf2BsSsAasBroadcastPermission,

wmanIf2BsSsMaxTxPowerBpsk,

wmanIf2BsSsMaxTxPowerQpsk,

wmanIf2BsSsMaxTxPower16Qam,

wmanIf2BsSsMaxTxPower64Qam,

-- Configuration parameters

wmanIf2BsSsMacVersion,

wmanIf2BsDcdInterval,

wmanIf2BsUcdInterval,

wmanIf2BsUcdTransition,

wmanIf2BsDcdTransition,

wmanIf2BsInitialRangingInterval,

wmanIf2BsSsULMapProcTime,

wmanIf2BsSsRangRespProcTime,

wmanIf2BsT9Timeout,

wmanIf2BsT13Timeout,

wmanIf2BsT15Timeout,

wmanIf2BsT17Timeout,

wmanIf2BsT27IdleTimer,

wmanIf2BsT27ActiveTimer,

-- Performance monitoring

wmanIf2BsHistogramIndex,

wmanIf2BsChannelNumber,

wmanIf2BsStartFrame,

wmanIf2BsDuration,

wmanIf2BsBasicReport,

wmanIf2BsMeanCinrReport,

wmanIf2BsMeanRssiReport,

wmanIf2BsStdDeviationCinrReport,

wmanIf2BsStdDeviationRssiReport,

-- Capability negotiation

wmanIf2BsSsActionsResetSs,

wmanIf2BsSsActionsAbortSs,

wmanIf2BsSsActionsOverrideDnFreq,

wmanIf2BsSsActionsOverrideChannelId,

wmanIf2BsSsActionsDeReRegSs,

wmanIf2BsSsActionsDeReRegSsCode,

wmanIf2BsSsActionsRowStatus,

-- Privacy sublayer

wmanIf2BsPkmV1AkLifetime,

wmanIf2BsPkmV1TekLifetime,

wmanIf2BsPkmV1SelfSigManufCertTrust,

wmanIf2BsPkmV1CheckCertValidityPeriods,

-- Notification

wmanIf2BsTrapControlRegister,

wmanIf2BsStatusTrapControlRegister,

wmanIf2BsRssiLowThreshold,

wmanIf2BsRssiHighThreshold,

wmanIf2BsSsNotificationMacAddr,

wmanIf2BsSsStatusValue,

wmanIf2BsSsStatusInfo,

wmanIf2BsDynamicServiceType,

wmanIf2BsDynamicServiceFailReason,

wmanIf2BsSsRssiStatus,

wmanIf2BsSsRssiStatusInfo,

wmanIf2BsSsRegisterStatus}

STATUS       current

DESCRIPTION

"This group contains objects for BS, and are

independent of PHY."

::= { wmanIf2BsMibGroups 1 }

wmanIf2MibBsAasGroup     OBJECT-GROUP

OBJECTS {-- AAS Configuration parameters

wmanIf2BsAasChanFbckReqFreq,

wmanIf2BsAasBeamSelectFreq,

wmanIf2BsAasChanFbckReqResolution,

wmanIf2BsAasBeamReqResolution,

wmanIf2BsAasNumOptDiversityZones}

STATUS       current

DESCRIPTION

"This group contains objects for AAS in BS."

::= { wmanIf2BsMibGroups 2 }

wmanIf2MibBsOfdmGroup     OBJECT-GROUP

OBJECTS {wmanIf2BsOfdmBwReqOppSize,

wmanIf2BsOfdmRangReqOppSize,

wmanIf2BsOfdmNumSubChReqRegionFull,

wmanIf2BsOfdmNumSymbolsReqRegionFull,

wmanIf2BsOfdmSubChFocusCtCode,

wmanIf2BsOfdmFrameDurationCode,

wmanIf2BsOfdmUcdFecCodeType,

wmanIf2BsOfdmFocusCtPowerBoost,

wmanIf2BsOfdmUcdTcsEnable,

wmanIf2BsOfdmUcdBurstProfileRowStatus,

wmanIf2BsOfdmDownlinkFrequency,

wmanIf2BsOfdmDcdFecCodeType,

wmanIf2BsOfdmDiucMandatoryExitThresh,

wmanIf2BsOfdmDiucMinEntryThresh,

wmanIf2BsOfdmTcsEnable,

wmanIf2BsOfdmDcdBurstProfileRowStatus,

wmanIf2BsOfdmMinReqRegionFullTxOpp,

wmanIf2BsOfdmMinFocusedCtTxOpp,

wmanIf2BsOfdmMaxRoundTripDelay,

wmanIf2BsOfdmRangeAbortTimingThold,

wmanIf2BsOfdmRangeAbortPowerThold ,

wmanIf2BsOfdmRangeAbortFreqThold,

wmanIf2BsOfdmDnlkRateId,

wmanIf2BsSsOfdmReqCapFftSizes,

wmanIf2BsSsOfdmReqCapSsDemodulator,

wmanIf2BsSsOfdmReqCapSsModulator,

wmanIf2BsSsOfdmReqCapFocusedCtSupport,

wmanIf2BsSsOfdmReqCapTcSublayerSupport,

wmanIf2BsSsOfdmRspCapFftSizes,

wmanIf2BsSsOfdmRspCapSsDemodulator,

wmanIf2BsSsOfdmRspCapSsModulator,

wmanIf2BsSsOfdmRspCapFocusedCtSupport,

wmanIf2BsSsOfdmRspCapTcSublayerSupport,

wmanIf2BsOfdmCapFftSizes,

wmanIf2BsOfdmCapSsDemodulator,

wmanIf2BsOfdmCapSsModulator,

wmanIf2BsOfdmCapFocusedCtSupport,

wmanIf2BsOfdmCapTcSublayerSupport,

wmanIf2BsOfdmCapCfgFftSizes,

wmanIf2BsOfdmCapCfgSsDemodulator,

wmanIf2BsOfdmCapCfgSsModulator,

wmanIf2BsOfdmCapCfgFocusedCtSupport,

wmanIf2BsOfdmCapCfgTcSublayerSupport}

STATUS       current

DESCRIPTION

"This group contains objects for BS and OFDM PHY."

::= { wmanIf2BsMibGroups 3 }

wmanIf2MibBsOfdmaGroup     OBJECT-GROUP

OBJECTS {wmanIf2BsOfdmaStartOfRngCodes,

wmanIf2BsOfdmaPermutationBase,

wmanIf2BsOfdmaULAllocSubchBitmap,

wmanIf2BsOfdmaOptPermULAllocSubchBitmap,

wmanIf2BsOfdmaBandAMCAllocThreshold,

wmanIf2BsOfdmaBandAMCReleaseThreshold,

wmanIf2BsOfdmaBandAMCAllocTimer,

wmanIf2BsOfdmaBandAMCReleaseTimer,

wmanIf2BsOfdmaBandStatRepMAXPeriod,

wmanIf2BsOfdmaBandAMCRetryTimer,

wmanIf2BsOfdmaFrameDurationCode,

wmanIf2BsOfdmaUcdFecCodeType,

wmanIf2BsOfdmaRangingDataRatio,

wmanIf2BsOfdmaUcdBurstProfileRowStatus,

wmanIf2BsOfdmaDownlinkFrequency,

wmanIf2BsOfdmaDcdFecCodeType,

wmanIf2BsOfdmaDcdBurstProfileRowStatus}

STATUS       current

DESCRIPTION

"This group contains objects for BS and OFDMA PHY."

::= { wmanIf2BsMibGroups 4 }

wmanIf2MibBsNotificationGroup     NOTIFICATION-GROUP

NOTIFICATIONS {wmanIf2BsSsStatusNotificationTrap,

wmanIf2BsSsRssiStatusChangeTrap,

wmanIf2BsSsPkmFailTrap}

STATUS       current

DESCRIPTION

"This group contains event notifications for BS."

::= { wmanIf2BsMibGroups 5 }

END