| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | wmanIf2sBsMib |  |
| Date Submitted | 2007-11-11 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes wmanIf2sBsMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# wmanIf2sBsMib

Joey Chou

Intel

# Introduction

This document proposes wmanIf2sBsMib.

# Proposed ASN.1 Notations

WMAN-IF2S-BS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF

ifIndex

FROM IF-MIB;

wmanIf2sBsMib MODULE-IDENTITY

LAST-UPDATED    "200711090000Z" -- November 09, 2007

ORGANIZATION    "IEEE 802.16"

CONTACT-INFO

"WG E-mail: stds-802-16@ieee.org

WG Chair:  Roger B. Marks

Postal:    NextWave Broadband, Inc.

E-mail:    r.b.marks@ieee.org

TG Chair:  Phillip Barber

Postal:    Huawei Technologies Co., Ltd

E-mail:    pbarber@futurewei.com

Editor:    Joey Chou

Postal:    Intel Corporation

5000 W. Chandler Blvd,

Chandler, AZ 85227, USA

E-mail:    joey.chou@intel.com"

DESCRIPTION

"This material is from IEEE Std 802.16i

Copyright (c) 2007 IEEE.

This MIB Module enables the management station to directly

provision the service flows at the Base Station. This MIB

module is primarily used to support fixed broadband

wireless networks.

BS allowing direct SNMP MIB based service flows

provisioning shall implement wmanIf2sMib. The choice of

mode(s) of service flows provisioning is implementation

specific."

REVISION        "200711090000Z"

DESCRIPTION

"Change to wmanIf2sBsMib."

REVISION        "200710020000Z"

DESCRIPTION

"The first revision of WMAN-IF2S-MIB module that supports

direct service flow provisioning."

::= { iso std(0) iso8802(8802) wman(16) 4 }

-- Textual Conventions

WmanIf2sSfState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the state of a service flow."

SYNTAX      INTEGER {authorized(1),

admitted(2),

active(3)}

WmanIf2sCsType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the types of convergence sublayer."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {reserved(0),

packetIpV4(1),

packetIpV6(2),

packet802dot3Ethernet(3),

packet802dot1QVlan(4),

packetIpV4Over802dot3(5),

packetIpV6Over802dot3(6),

packetIpV4Over802dot1Q(7),

packetIpV6Over802dot1Q(8),

atm(9),

packet802dot3EthernetRohcHc(10),

packet802dot3EthernetEcrtpHc(11),

packetIp2RohcHc(12),

packetIp2EcrtpHc(13)}

WmanIf2sServClassName ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the type of service class name."

SYNTAX      OCTET STRING (SIZE(2..128))

WmanIf2sSchedulingType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The scheduling service provided by a SC for an upstream

service flow. If the parameter is omitted from an upstream

QOS Parameter Set, this object takes the value of

bestEffort (2). This parameter must be reported as

undefined (1) for downstream QOS Parameter Sets."

SYNTAX      INTEGER {undefined(1),

bestEffort(2),

nonRealTimePollingService(3),

realTimePollingService(4),

extRealTimePollingService(5),

unsolicitedGrantService(6)}

WmanIf2sPhsRuleVerify ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression. If PHSV is

enabled, the sender shall compare the bytes in the packet

header with the bytes in the PHSF that are to be

suppressed as indicated by the PHSM."

REFERENCE

"Subclause 11.13.19.3.7.5 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {phsVerifyEnable(0),

phsVerifyDisable(1)}

WmanIf2sClassifierMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"A bit of of this object is set to 1 if the parameter

indicated by the comment was present in the classifier

encoding, and 0 otherwise.

Note: that BITS are encoded most significant bit first,

so that if e.g. bits 6 and 7 are set, this object is

encoded as the octet string '030000'H."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16e-2005"

SYNTAX      BITS {priority(0),

ipTos(1),

ipProtocol(2),

ipMaskedSrcAddr(3),

ipMaskedDestAddr(4),

srcPort(5),

destPort(6),

destMacAddr(7),

srcMacAddr(8),

ethernetProtocol(9),

userPriority(10),

vlanId(11),

ipv6FlowLabel(12),

actionRule(13)}

WmanIf2sIpv6FlowLabel ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field. As the flow label field has a

length of 20 bits, the first 4 bits of the most significant

byte shall be set to 0x0 and disregarded."

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2sActionRule ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule

Bit #0: 0 = none.

1 = Discard packet"

REFERENCE

"Subclause 11.13.19.3.4.17 in IEEE Std 802.16e-2005"

SYNTAX      BITS {discardPacket(0)}

WmanIf2sCidType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of CID."

SYNTAX      INTEGER (0 .. 65535)

--

-- BS object group – containing tables and objects to be implemented in

-- the Base station

wmanIf2sBsProvisionedSfTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sBsProvisionedSfEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains service flow profiles provisioned by

NMS. The service flow should be created with SS(s)

following instruction given by wmanIf2sBsSfState object.

1. The QoS parameters of the service flow are provisioned

in wmanIf2sBsServiceClassTable and referenced by

wmanIf2sBsServiceClassIndex.

2. The classifier rules of the service flow are provisioned

in wmanIf2sBsClassifierRuleTable, where they refer to SF

via wmanIf2sBsSfId.

The MAC addresses of SSs the service flow is created with

are provisioned in wmanIf2sBsSsProvisionedForSfTable, where

they refer to SF via wmanIf2sBsSfId."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2sBsMib 1 }

wmanIf2sBsProvisionedSfEntry OBJECT-TYPE

SYNTAX      WmanIf2sBsProvisionedSfEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each service flow

provisioned by NMS."

INDEX     { ifIndex,

wmanIf2sBsSsProvMacAddress,

wmanIf2sBsSfId }

::= { wmanIf2sBsProvisionedSfTable 1 }

WmanIf2sBsProvisionedSfEntry ::= SEQUENCE {

wmanIf2sBsSsProvMacAddress              MacAddress,

wmanIf2sBsSfId                          Unsigned32,

wmanIf2sBsSfDirection                   INTEGER,

wmanIf2sBsServiceClassIndex             INTEGER,

wmanIf2sBsSfState                       WmanIf2sSfState,

wmanIf2sBsSfProvisionedTime             TimeStamp,

wmanIf2sBsSfCsSpecification             WmanIf2sCsType,

wmanIf2sBsProvisionedSfRowStatus        RowStatus}

wmanIf2sBsSsProvMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The MAC address of the SS, the service flow is created

with."

::= { wmanIf2sBsProvisionedSfEntry 1 }

wmanIf2sBsSfId OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A 32 bit quantity that uniquely identifies a service flow

to both the subscriber station and base station (BS)."

::= { wmanIf2sBsProvisionedSfEntry 2 }

wmanIf2sBsSfDirection OBJECT-TYPE

SYNTAX      INTEGER {downstream(1),

upstream(2)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"An attribute indicating the service flow is downstream or

upstream."

::= { wmanIf2sBsProvisionedSfEntry 3 }

wmanIf2sBsServiceClassIndex OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The index in wmanIf2sBsServiceClassTable describing the

service class or QoS parameters for such service flow.

If no associated entry in wmanIf2sBsServiceClassTable

exists, this object returns a value of zero."

::= { wmanIf2sBsProvisionedSfEntry 4 }

wmanIf2sBsSfState OBJECT-TYPE

SYNTAX      WmanIf2sSfState

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"wmanIf2sBsSfState determines the requested state of a

service flow.

- authorized state: A service flow is provisioned but

not resource is reserved yet

- admitted state: service flow has resources reserved.

- active state: has resources committed by the BS (e.g., is

actively sending maps containing unsolicited grants for a

UGS-based service flow),"

REFERENCE

"Subclause 6.3.14.6, in IEEE Std 802.16-2004"

::= { wmanIf2sBsProvisionedSfEntry 5 }

wmanIf2sBsSfProvisionedTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Indicates the date and time when the service flow is

provisioned."

::= { wmanIf2sBsProvisionedSfEntry 6 }

wmanIf2sBsSfCsSpecification OBJECT-TYPE

SYNTAX      WmanIf2sCsType

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This parameter specifies the convergence sublayer

encapsulation mode."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16-2004"

::= { wmanIf2sBsProvisionedSfEntry 7 }

wmanIf2sBsProvisionedSfRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or delete

an existing row in this table.

If the implementator of this MIB has choosen not to

implement 'dynamic assignment' of profiles, this object is

not useful and should return noSuchName upon SNMP request."

::= { wmanIf2sBsProvisionedSfEntry 8 }

wmanIf2sBsServiceClassTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sBsServiceClassEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Each entry of the table contains corresponding service flow

characteristic attributes (e.g. QoS parameter set). The

value of wmanIf2sBsQoSProfileIndex is obtained from

wmanIf2sBsServiceClassIndex in wmanIf2sBsProvisionedSfTable"

REFERENCE

"Subclause 6.3.14.4 in IEEE Std 802.16-2004"

::= { wmanIf2sBsMib 2 }

wmanIf2sBsServiceClassEntry OBJECT-TYPE

SYNTAX      WmanIf2sBsServiceClassEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each service class"

INDEX     { ifIndex, wmanIf2sBsQoSProfileIndex }

::= { wmanIf2sBsServiceClassTable 1 }

WmanIf2sBsServiceClassEntry ::= SEQUENCE {

wmanIf2sBsQoSProfileIndex               INTEGER,

wmanIf2sBsQosServiceClassName           WmanIf2sServClassName,

wmanIf2sBsQoSTrafficPriority            INTEGER,

wmanIf2sBsQoSMaxSustainedRate           Unsigned32,

wmanIf2sBsQoSMaxTrafficBurst            Unsigned32,

wmanIf2sBsQoSMinReservedRate            Unsigned32,

wmanIf2sBsQoSToleratedJitter            Unsigned32,

wmanIf2sBsQoSMaxLatency                 Unsigned32,

wmanIf2sBsQoSFixedVsVariableSduInd      INTEGER,

wmanIf2sBsQoSSduSize                    Unsigned32,

wmanIf2sBsQosScSchedulingType           WmanIf2sSchedulingType,

wmanIf2sBsQosScArqEnable                TruthValue,

wmanIf2sBsQosScArqWindowSize            INTEGER,

wmanIf2sBsQosScArqBlockLifetime         INTEGER,

wmanIf2sBsQosScArqSyncLossTimeout       INTEGER,

wmanIf2sBsQosScArqDeliverInOrder        TruthValue,

wmanIf2sBsQosScArqRxPurgeTimeout        INTEGER,

wmanIf2sBsQosScArqBlockSize             INTEGER,

wmanIf2sBsQosSCMinRsvdTolerableRate     Unsigned32,

wmanIf2sBsQoSReqTxPolicy                BITS,

wmanIf2sBsQoSServiceClassRowStatus      RowStatus}

wmanIf2sBsQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry

in the wmanIf2sBsServiceClassTable"

::= { wmanIf2sBsServiceClassEntry 1 }

wmanIf2sBsQosServiceClassName  OBJECT-TYPE

SYNTAX      WmanIf2sServClassName

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Refers to the Service Class Name"

REFERENCE

"Subclause 11.13.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 2 }

wmanIf2sBsQoSTrafficPriority OBJECT-TYPE

SYNTAX      INTEGER (0..7)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this parameter specifies the priority

assigned to a service flow. For uplink service flows,

the BS should use this parameter when determining

precedence in request service and grant generation,

and the SS shall preferentially select contention

Request opportunities for Priority Request CIDs

based on this priority. Higher numbers indicate higher

priority"

REFERENCE

"Subclause 11.13.5 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 3 }

wmanIf2sBsQoSMaxSustainedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This parameter defines the peak information rate of the

service. The rate is expressed in bits per second and

pertains to the SDUs at the input to the system."

REFERENCE

"Subclause 11.13.6 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 4 }

wmanIf2sBsQoSMaxTrafficBurst OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This parameter defines the maximum burst size that must be

accommodated for the service."

REFERENCE

"Subclause 11.13.7 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 5 }

wmanIf2sBsQoSMinReservedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This parameter specifies the minimum rate reserved for this

service flow."

REFERENCE

"Subclause 11.13.8 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 6 }

wmanIf2sBsQoSToleratedJitter OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This parameter defines the Maximum delay variation (jitter)

for the connection."

REFERENCE

"Subclause 11.13.13 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 7 }

wmanIf2sBsQoSMaxLatency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this parameter specifies the maximum latency

between the reception of a packet by the BS or SS on its

network interface and the forwarding of the packet to its

RF Interface."

REFERENCE

"Subclause 11.13.14 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 8 }

wmanIf2sBsQoSFixedVsVariableSduInd OBJECT-TYPE

SYNTAX      INTEGER {variableLength(0),

fixedLength(1)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this parameter specifies whether the SDUs on

the service flow are variable-length (0) or fixed-length

(1). The parameter is used only if packing is on for the

service flow. The default value is 0, i.e.,variable-length

SDUs."

REFERENCE

"Subclause 11.13.15 in IEEE Std 802.16-2004"

DEFVAL      { variableLength }

::= { wmanIf2sBsServiceClassEntry 9 }

wmanIf2sBsQoSSduSize OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this parameter specifies the length of the

SDU for a fixed-length SDU service flow. This parameter

is used only if packing is on and the service flow is

indicated as carrying fixed-length SDUs. The default

value is 49 bytes, i.e., VC-switched ATM cells with PHS.

The parameter is relevant for both ATM and Packet

Convergence Sublayers."

REFERENCE

"Subclause 11.13.16 in IEEE Std 802.16-2004"

DEFVAL      { 49 }

::= { wmanIf2sBsServiceClassEntry 10 }

wmanIf2sBsQosScSchedulingType OBJECT-TYPE

SYNTAX      WmanIf2sSchedulingType

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Specifies the upstream scheduling service used for upstream

service flow. If the referenced parameter is not present in

the corresponding 802.16 QOS Parameter Set of an upstream

service flow, the default value of this object is

bestEffort(2)."

REFERENCE

"Subclause 11.13.11 in IEEE Std 802.16-2004"

DEFVAL	    {bestEffort}

::= { wmanIf2sBsServiceClassEntry 11 }

wmanIf2sBsQosScArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the connection."

REFERENCE

"Subclause 11.13.18 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 12 }

wmanIf2sBsQosScArqWindowSize	 OBJECT-TYPE

SYNTAX      INTEGER (1 .. 1024)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged

fragments at any time."

REFERENCE

"Subclause 11.13.18 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 13 }

wmanIf2sBsQosScArqBlockLifetime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be managed

by the transmitter ARQ machine, once initial transmission

of the fragment has occured. If transmission or

retransmission of the fragment is not acknowledged by the

receiver before the time limit is reached, the fragment is

discarded. A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18 in IEEE Std 802.16-2004"

DEFVAL		{0}

::= { wmanIf2sBsServiceClassEntry 14 }

wmanIf2sBsQosScArqSyncLossTimeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535 )

UNITS       "10 us"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss of

synchronization of the sender and receiver state machines.

A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18 in IEEE Std 802.16-2004"

DEFVAL	    {0}

::= { wmanIf2sBsServiceClassEntry 15 }

wmanIf2sBsQosScArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered by the

receiving MAC to its client application in the order in

which data was handed off to the originating MAC."

REFERENCE

"Subclause 11.13.18 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 16 }

wmanIf2sBsQosScArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

UNITS       "10 us"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means Infinite."

REFERENCE

"Subclause 11.13.18 in IEEE Std 802.16-2004"

DEFVAL	    {0}

::= { wmanIf2sBsServiceClassEntry 17 }

wmanIf2sBsQosScArqBlockSize OBJECT-TYPE

SYNTAX      INTEGER (1..2040)

UNITS       "byte"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this parameter specifies the size of an ARQ

block. This parameter shall be established by negotiation

during the connection creation dialog."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 18 }

wmanIf2sBsQosSCMinRsvdTolerableRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"Minimum Tolerable Traffic Rate = R (bits/sec) with time

base T(sec) means the following. Let S denote additional

demand accumulated at the MAC SAP of the transmitter during

an arbitrary time interval of the length T. Then the amount

of data forwarded at the receiver to CS (in bits) during

this interval should be not less than min {S, R * T}."

REFERENCE

"Subclause 11.13.9 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 19 }

wmanIf2sBsQoSReqTxPolicy OBJECT-TYPE

SYNTAX      BITS {noBroadcastBwReq(0),

reserved1(1),

noPiggybackReq(2),

noFragmentData(3),

noPHS(4),

noSduPacking(5),

noCrc(6),

reserved2(7)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this parameter provides the capability to

specify certain attributes for the associated service

flow. An attribute is enabled by setting the corresponding

bit position to 1."

REFERENCE   "Subclause 11.13.12 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceClassEntry 20 }

wmanIf2sBsQoSServiceClassRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or delete

an existing row in this table.

If the implementator of this MIB has choosen not to

implement 'dynamic assignment' of profiles, this object is

not useful and should return noSuchName upon SNMP request."

::= { wmanIf2sBsServiceClassEntry 21 }

wmanIf2sBsServiceFlowTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sBsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains Service Flow managed objects that

are common in BS and SS."

::= { wmanIf2sBsMib 3 }

wmanIf2sBsServiceFlowEntry OBJECT-TYPE

SYNTAX      WmanIf2sBsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each created service

flow for a given MacAddress."

INDEX     { ifIndex,

wmanIf2sBsSfMacAddress,

wmanIf2sBsSfId }

::= { wmanIf2sBsServiceFlowTable 1 }

WmanIf2sBsServiceFlowEntry::= SEQUENCE {

wmanIf2sBsSfMacAddress                  MacAddress,

wmanIf2sBsSfCid                         WmanIf2sCidType,

wmanIf2sBsServiceFlowDirection          INTEGER,

wmanIf2sBsServiceFlowState              WmanIf2sSfState,

wmanIf2sBsTrafficPriority               INTEGER,

wmanIf2sBsMaxSustainedRate              Unsigned32,

wmanIf2sBsMaxTrafficBurst               Unsigned32,

wmanIf2sBsMinReservedRate               Unsigned32,

wmanIf2sBsToleratedJitter               Unsigned32,

wmanIf2sBsMaxLatency                    Unsigned32,

wmanIf2sBsFixedVsVariableSduInd         INTEGER,

wmanIf2sBsSduSize                       Unsigned32,

wmanIf2sBsSfSchedulingType              WmanIf2sSchedulingType,

wmanIf2sBsArqEnable                     TruthValue,

wmanIf2sBsArqWindowSize                 INTEGER,

wmanIf2sBsArqBlockLifetime              INTEGER,

wmanIf2sBsArqSyncLossTimeout            INTEGER,

wmanIf2sBsArqDeliverInOrder             TruthValue,

wmanIf2sBsArqRxPurgeTimeout             INTEGER,

wmanIf2sBsArqBlockSize                  INTEGER,

wmanIf2sBsMinRsvdTolerableRate          Unsigned32,

wmanIf2sBsReqTxPolicy                   BITS,

wmanIf2sBsCsSpecification               WmanIf2sCsType,

wmanIf2sBsTargetSaid                    INTEGER}

wmanIf2sBsSfMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"When this table is implemented on the basestation, this

object contains the SS Mac address, the reported service

flow was created for. On the SS, the value returned is

the SS's own Mac address."

::= { wmanIf2sBsServiceFlowEntry 1 }

wmanIf2sBsSfCid OBJECT-TYPE

SYNTAX      WmanIf2sCidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 16 bit channel identifier to identify the connection

being created by DSA."

::= { wmanIf2sBsServiceFlowEntry 2 }

wmanIf2sBsServiceFlowDirection OBJECT-TYPE

SYNTAX      INTEGER {downstream(1),

upstream(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An attribute indicating the service flow is downstream or

upstream."

::= { wmanIf2sBsServiceFlowEntry 3 }

wmanIf2sBsServiceFlowState OBJECT-TYPE

SYNTAX      WmanIf2sSfState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2sBsSfState indicates the service flow state:

Authorized (1), Admitted (2), and Active (3) service

flow state."

REFERENCE

"Subclause 6.3.14.6, in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 4 }

wmanIf2sBsTrafficPriority OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the priority

assigned to a service flow. For uplink service flows,

the BS should use this parameter when determining

precedence in request service and grant generation,

and the SS shall preferentially select contention

Request opportunities for Priority Request CIDs

based on this priority"

REFERENCE

"Subclause 11.13.5 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 5 }

wmanIf2sBsMaxSustainedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the peak information rate

of the service. The rate is expressed in bits per

second and pertains to the SDUs at the input to

the system."

REFERENCE

"Subclause 11.13.6 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 6 }

wmanIf2sBsMaxTrafficBurst OBJECT-TYPE

SYNTAX      Unsigned32

UNITS	    "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the maximum burst size that

must be accommodated for the service."

REFERENCE

"Subclause 11.13.7 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 7 }

wmanIf2sBsMinReservedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the minimum rate reserved

for this service flow."

REFERENCE

"Subclause 11.13.8 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 8 }

wmanIf2sBsToleratedJitter OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the Maximum delay

variation (jitter) for the connection."

REFERENCE

"Subclause 11.13.13 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 9 }

wmanIf2sBsMaxLatency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the maximum

latency between the reception of a packet by the BS

or SS on its network interface and the forwarding

of the packet to its RF Interface."

REFERENCE

"Subclause 11.13.14 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 10 }

wmanIf2sBsFixedVsVariableSduInd OBJECT-TYPE

SYNTAX      INTEGER {variableLength(0),

fixedLength(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies whether the SDUs

on the service flow are variable-length (0) or

fixed-length (1). The parameter is used only if

packing is on for the service flow. The default value

is 0, i.e.,variable-length SDUs."

REFERENCE

"Subclause 11.13.15 in IEEE Std 802.16-2004"

DEFVAL      { variableLength }

::= { wmanIf2sBsServiceFlowEntry 11 }

wmanIf2sBsSduSize OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the length of the

SDU for a fixed-length SDU service flow. This parameter

is used only if packing is on and the service flow is

indicated as carrying fixed-length SDUs. The default

value is 49 bytes, i.e., VC-switched ATM cells with PHS.

The parameter is relevant for both ATM and Packet

Convergence Sublayers."

REFERENCE

"Subclause 11.13.16 in IEEE Std 802.16-2004"

DEFVAL      { 49 }

::= { wmanIf2sBsServiceFlowEntry 12 }

wmanIf2sBsSfSchedulingType OBJECT-TYPE

SYNTAX      WmanIf2sSchedulingType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the upstream scheduling service used for

upstream service flow. If the referenced parameter

is not present in the corresponding 802.16 QOS

Parameter Set of an upstream service flow, the

default value of this object is bestEffort(2)."

REFERENCE

"Subclause 11.13.11 in IEEE Std 802.16-2004"

DEFVAL         { bestEffort }

::= { wmanIf2sBsServiceFlowEntry 13 }

wmanIf2sBsArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the connection."

::= { wmanIf2sBsServiceFlowEntry 14 }

wmanIf2sBsArqWindowSize	 OBJECT-TYPE

SYNTAX	    INTEGER (1..1024)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged

fragments at any time."

::= { wmanIf2sBsServiceFlowEntry 15 }

wmanIf2sBsArqBlockLifetime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be

managed by the transmitter ARQ machine, once

initial transmission of the fragment has occured.

If transmission or retransmission of the fragment

is not acknowledged by the receiver before the

time limit is reached, the fragment is discarded.

A value of 0 means Infinite."

::= { wmanIf2sBsServiceFlowEntry 16 }

wmanIf2sBsArqSyncLossTimeout OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535 )

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss

of synchronization of the sender and receiver

state machines. A value of 0 means Infinite."

::= { wmanIf2sBsServiceFlowEntry 17 }

wmanIf2sBsArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered

by the receiving MAC to its client application

in the order in which data was handed off to the

originating MAC."

::= { wmanIf2sBsServiceFlowEntry 18 }

wmanIf2sBsArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means

Infinite."

::= { wmanIf2sBsServiceFlowEntry 19 }

wmanIf2sBsArqBlockSize OBJECT-TYPE

SYNTAX	    INTEGER (1..2040)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value of this parameter specifies the size of an

ARQ block. This parameter shall be established by

negotiation during the connection creation dialog."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 20 }

wmanIf2sBsMinRsvdTolerableRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Minimum Tolerable Traffic Rate = R (bits/sec) with

time base T(sec) means the following. Let S denote

additional demand accumulated at the MAC SAP of the

transmitter during an arbitrary time interval of the

length T. Then the amount of data forwarded at the

receiver to CS (in bits) during this interval should

be not less than min {S, R * T}."

REFERENCE

"Subclause 11.13.9 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 21 }

wmanIf2sBsReqTxPolicy OBJECT-TYPE

SYNTAX      BITS {noBroadcastBwReq(0),

reserved1(1),

noPiggybackReq(2),

noFragmentData(3),

noPHS(4),

noSduPacking(5),

noCrc(6),

reserved2(7)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter provides the capability to

specify certain attributes for the associated service

flow. An attribute is enabled by setting the

corresponding bit position to 1."

REFERENCE

"Subclause 11.13.12 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 22 }

wmanIf2sBsCsSpecification OBJECT-TYPE

SYNTAX      WmanIf2sCsType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the convergence sublayer

encapsulation mode."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 23 }

wmanIf2sBsTargetSaid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The target SAID parameter indicates the SAID onto

which the service flow being set up shall be mapped."

REFERENCE

"Subclause 11.13.17 in IEEE Std 802.16-2004"

::= { wmanIf2sBsServiceFlowEntry 24 }

wmanIf2sBsClassifierRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sBsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains packet classifier rules associated

with service flows."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16-2004"

::= { wmanIf2sBsMib 4 }

wmanIf2sBsClassifierRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2sBsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each packet classifier

rule."

INDEX     { ifIndex,

wmanIf2sBsSfId,

wmanIf2sBsClassifierRuleIndex }

::= { wmanIf2sBsClassifierRuleTable 1 }

WmanIf2sBsClassifierRuleEntry::= SEQUENCE {

wmanIf2sBsClassifierRuleIndex           Unsigned32,

wmanIf2sBsClassifierRulePriority        INTEGER,

wmanIf2sBsClassifierRuleIpTosLow        INTEGER,

wmanIf2sBsClassifierRuleIpTosHigh       INTEGER,

wmanIf2sBsClassifierRuleIpTosMask       INTEGER,

wmanIf2sBsClassifierRuleIpProtocol      Integer32,

wmanIf2sBsClassifierRuleIpSrcAddr       InetAddress,

wmanIf2sBsClassifierRuleIpSrcMask       InetAddress,

wmanIf2sBsClassifierRuleIpDestAddr      InetAddress,

wmanIf2sBsClassifierRuleIpDestMask      InetAddress,

wmanIf2sBsClassifierRuleSrcPortStart    Integer32,

wmanIf2sBsClassifierRuleSrcPortEnd      Integer32,

wmanIf2sBsClassifierRuleDestPortStart   Integer32,

wmanIf2sBsClassifierRuleDestPortEnd     Integer32,

wmanIf2sBsClassifierRuleDestMacAddr     MacAddress,

wmanIf2sBsClassifierRuleDestMacMask     MacAddress,

wmanIf2sBsClassifierRuleSrcMacAddr      MacAddress,

wmanIf2sBsClassifierRuleSrcMacMask      MacAddress,

wmanIf2sBsClassifierRuleEnetProtType    INTEGER,

wmanIf2sBsClassifierRuleEnetProtocol    Integer32,

wmanIf2sBsClassifierRuleUserPriLow      Integer32,

wmanIf2sBsClassifierRuleUserPriHigh     Integer32,

wmanIf2sBsClassifierRuleVlanId          Integer32,

wmanIf2sBsClassifierRulePhsSize         Integer32,

wmanIf2sBsClassifierRulePhsMask         OCTET STRING,

wmanIf2sBsClassifierRulePhsVerify       WmanIf2sPhsRuleVerify,

wmanIf2sBsClassifierRuleIpv6FlowLabel   WmanIf2sIpv6FlowLabel,

wmanIf2sBsClassifierRuleActionRule      WmanIf2sActionRule,

wmanIf2sBsClassifierRuleBitMap          WmanIf2sClassifierMap,

wmanIf2sBsClassifierRuleRowStatus       RowStatus}

wmanIf2sBsClassifierRuleIndex  OBJECT-TYPE

SYNTAX      Unsigned32 (1..4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index is assigned to a classifier in BS classifiers

table"

::= { wmanIf2sBsClassifierRuleEntry 1 }

wmanIf2sBsClassifierRulePriority OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value specifies the priority for the Classifier, which

is used for determining the order of the Classifier. A

higher value indicates higher priority. Classifiers may

have priorities in the range 0..255."

REFERENCE

"Subclause 11.13.19.3.4.1 in IEEE Std 802.16-2004"

DEFVAL      { 0 }

::= { wmanIf2sBsClassifierRuleEntry 2 }

wmanIf2sBsClassifierRuleIpTosLow OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The low value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 3 }

wmanIf2sBsClassifierRuleIpTosHigh OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The 8-bit high value of a range of TOS byte values.

If the referenced parameter is not present in a classifier,

this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 4 }

wmanIf2sBsClassifierRuleIpTosMask OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this object specifies the matching parameter

for the IP type of service/DSCP [IETF RFC 2474] byte mask.

An IP packet with IP type of service (ToS) byte value

ip-tos matches this parameter if tos-low less than or

equal (ip-tos AND tos-mask) less than or equal tos-high."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 5 }

wmanIf2sBsClassifierRuleIpProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object indicates the value of the IP Protocol field

required for IP packets to match this rule. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 6 }

wmanIf2sBsClassifierRuleIpSrcAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Source Address

required for packets to match this rule. An IP packet

matches the rule when the packet ip source address bitwise

ANDed with the wmanIf2sBsClassifierRuleIpSrcMask value

equals the wmanIf2sBsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier,

this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 7 }

wmanIf2sBsClassifierRuleIpSrcMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP Source

Address that are compared to match this rule. An IP packet

matches the rule when the packet source address bitwise

ANDed with the

wmanIf2sBsClassifierRuleIpSrcMask value equals the

wmanIf2sBsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier,

this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 8 }

wmanIf2sBsClassifierRuleIpDestAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Destination

Address required for packets to match this rule. An IP

packet matches the rule when the packet IP destination

address bitwise ANDed with the

wmanIf2sBsClassifierRuleIpDestMask value equals the

wmanIf2sBsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a

classifier, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 9 }

wmanIf2sBsClassifierRuleIpDestMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP

Destination Address that are compared to match this rule.

An IP packet matches the rule when the packet destination

address bitwise ANDed with the

wmanIf2sBsClassifierRuleIpDestMask value equals the

wmanIf2sBsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 10 }

wmanIf2sBsClassifierRuleSrcPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 11 }

wmanIf2sBsClassifierRuleSrcPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a classifier,

this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 12 }

wmanIf2sBsClassifierRuleDestPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 13 }

wmanIf2sBsClassifierRuleDestPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present in a

classifier, this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 14 }

wmanIf2sBsClassifierRuleDestMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sBsClassifierRuleDestMacMask equals the value of

wmanIf2sBsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 15 }

wmanIf2sBsClassifierRuleDestMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sBsClassifierRuleDestMacMask equals the value of

wmanIf2sBsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 16 }

wmanIf2sBsClassifierRuleSrcMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"An Ethernet packet matches this entry when its source

MAC address  bitwise ANDed with

wmanIf2sBsClassifierRuleSrcMacMask equals the value

of wmanIf2sBsClassifierRuleSrcMacAddr. If the

referenced parameter is not present in a classifier,

this object reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 17 }

wmanIf2sBsClassifierRuleSrcMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its source

MAC address bitwise ANDed with

wmanIf2sBsClassifierRuleSrcMacMask equals the value of

wmanIf2sBsClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 18 }

wmanIf2sBsClassifierRuleEnetProtType OBJECT-TYPE

SYNTAX      INTEGER {none(0),

ethertype(1),

dsap(2)}

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object indicates the format of the layer 3 protocol

id in the Ethernet packet. A value of none(0) means that

the rule does not use the layer 3 protocol type as a

matching criteria. A value of ethertype(1) means that the

rule applies only to frames which contains an EtherType

value. Ethertype values are contained in packets using

the Dec-Intel-Xerox (DIX) encapsulation or the RFC1042

Sub-Network Access Protocol (SNAP) encapsulation formats.

A value of dsap(2) means that the rule applies only to

frames using the IEEE802.3 encapsulation format with a

Destination Service Access Point (DSAP) other than 0xAA

(which is reserved for SNAP). If the Ethernet frame

contains an 802.1P/Q Tag header (i.e. EtherType 0x8100),

this object applies to the embedded EtherType field within

the 802.1P/Q header. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 19 }

wmanIf2sBsClassifierRuleEnetProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"If wmanIf2sBsClassifierRuleEnetProtocolType is none(0),

this object is ignored when considering whether a packet

matches the current rule.

If wmanIf2sBsClassifierRuleEnetProtocolType is ethertype(1),

this object gives the 16-bit value of the EtherType that

the packet must match in order to match the rule.

If wmanIf2sBsClassifierRuleEnetProtocolType is dsap(2), the

lower 8 bits of this object's value must match the DSAP

byte of the packet in order to match the rule.

If the Ethernet frame contains an 802.1P/Q Tag header

(i.e. EtherType 0x8100), this object applies to the

embedded EtherType field within the 802.1P/Q header.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 20 }

wmanIf2sBsClassifierRuleUserPriLow OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority field

within the range of wmanIf2sBsClassifierRuleUserPriLow and

wmanIf2sBsClassifierRuleUserPriHigh in order to match this

rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 21 }

wmanIf2sBsClassifierRuleUserPriHigh OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority

field within the range of wmanIf2sBsClassifierRuleUserPriLow

and wmanIf2sBsClassifierRuleUserPriHigh in order to match

this rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 7."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 22 }

wmanIf2sBsClassifierRuleVlanId OBJECT-TYPE

SYNTAX      Integer32 (0..4095)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header.

If this object's value is nonzero, tagged packets must

have a VLAN Identifier that matches the value in order

to match the rule.

Only the least significant 12 bits of this object's

value are valid.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.12 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 23 }

wmanIf2sBsClassifierRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to configure the PHS rule for this

classifer. The value of this field - PHSS is the total

number of bytes in the header to be suppressed and then

restored in a service flow that uses PHS. If the value of

this field is 0 bytes then PHS is disabled for this

classifer. If flag phsMask in wmanIf2sBsClassifierRuleBitMap

is set to 0 and flag phsSize in

wmanIf2sBsClassifierRuleBitMap is set to 0, then BS can still

create PHS rules using its own custom mask (i.e. the rule

is not configured by NMS)."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2sBsClassifierRuleEntry 25 }

wmanIf2sBsClassifierRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to configure the PHS rule for this

classifer. It is encoded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2sBsClassifierRulePhsSize/8). BS should use this

value to create a new PHS rule index (PHSI) and field

(PHSF) as defined in the standard. If flag phsMask in

wmanIf2sBsClassifierRuleBitMap is set to 0 and flag phsSize

in wmanIf2sBsClassifierRuleBitMap is set to 0, then BS can

still create PHS rules using its own custom mask (i.e. the

rule is not configured by NMS)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 26 }

wmanIf2sBsClassifierRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2sPhsRuleVerify

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2sBsClassifierRuleEntry 27 }

wmanIf2sBsClassifierRuleIpv6FlowLabel OBJECT-TYPE

SYNTAX      WmanIf2sIpv6FlowLabel

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2sBsClassifierRuleEntry 28 }

wmanIf2sBsClassifierRuleActionRule OBJECT-TYPE

SYNTAX      WmanIf2sActionRule

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule. If this classification action rule

exists, its action shall be applied on the packets that

match this classifier rule."

::= { wmanIf2sBsClassifierRuleEntry 29 }

wmanIf2sBsClassifierRuleBitMap OBJECT-TYPE

SYNTAX      WmanIf2sClassifierMap

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2sBsClassifierRuleEntry 30 }

wmanIf2sBsClassifierRuleRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or

delete an existing row in this table.

If the implementator of this MIB has choosen not

to implement 'dynamic assignment' of profiles, this

object is not useful and should return noSuchName

upon SNMP request."

::= { wmanIf2sBsClassifierRuleEntry 31 }

wmanIf2sBsPhsRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sBsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When a classifier is deleted, any associated

PHS rule shall also be deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsMib 5 }

wmanIf2sBsPhsRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2sBsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data."

INDEX     { ifIndex, wmanIf2sBsSfId,

wmanIf2sBsPhsRulePhsIndex }

::= { wmanIf2sBsPhsRuleTable 1 }

WmanIf2sBsPhsRuleEntry::= SEQUENCE {

wmanIf2sBsPhsRulePhsIndex               INTEGER,

wmanIf2sBsPhsRulePhsField               OCTET STRING,

wmanIf2sBsPhsRulePhsMask                OCTET STRING,

wmanIf2sBsPhsRulePhsSize                Integer32,

wmanIf2sBsPhsRulePhsVerify              WmanIf2sPhsRuleVerify}

wmanIf2sBsPhsRulePhsIndex OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2sBsPhsRuleEntry 1 }

wmanIf2sBsPhsRulePhsField OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2sBsPhsRuleEntry 2 }

wmanIf2sBsPhsRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2sBsPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsPhsRuleEntry 3 }

wmanIf2sBsPhsRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2sBsPhsRuleEntry 4 }

wmanIf2sBsPhsRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2sPhsRuleVerify

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2sBsPhsRuleEntry 5 }

END