| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | wmanIf2mSsMib |  |
| Date Submitted | 2007-11-11 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes wmanIf2mSsMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# wmanIf2mSsMib

Joey Chou

Intel

# Introduction

This document proposes wmanIf2mSsMib.

# Proposed ASN.1 Notations

WMAN-IF2M-SS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF

ifIndex

FROM IF-MIB;

wmanIf2mSsMib MODULE-IDENTITY

LAST-UPDATED    "200711090000Z" -- November 09, 2007

ORGANIZATION    "IEEE 802.16"

CONTACT-INFO

"WG E-mail: stds-802-16@ieee.org

WG Chair:  Roger B. Marks

Postal:    NextWave Broadband, Inc.

E-mail:    r.b.marks@ieee.org

TG Chair:  Phillip Barber

Postal:    Huawei Technologies Co., Ltd

E-mail:    pbarber@futurewei.com

Editor:    Joey Chou

Postal:    Intel Corporation

5000 W. Chandler Blvd,

Chandler, AZ 85227, USA

E-mail:    joey.chou@intel.com"

DESCRIPTION

"This material is from IEEE Std 802.16i

Copyright (c) 2007 IEEE.

This MIB Module defines managed objects for Subscriber

Station and Base Station based on IEEE Std 802.16-2004

and its amendment IEEE Std 802.16e-2005.

The MIB contains managed objects that are specific

to mobile Broadband Wireless Networks."

REVISION        "200711090000Z"

DESCRIPTION

"Change to wmanIf2sSsMib"

REVISION        "200710020000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

Malaga meeting"

REVISION        "200705220000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

Portland meeting"

REVISION        "200703280000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

Orlando meeting"

REVISION        "200702100000Z"

DESCRIPTION

"Includes changes as per comment resolutions agreed at the

London meeting"

REVISION        "200610160000Z"

DESCRIPTION

"The 1st revision of WMAN-IF2M-MIB module."

::= { iso std(0) iso8802(8802) wman(16) 5 }

wmanIf2mSsCm OBJECT IDENTIFIER ::= { wmanIf2mSsMib 1 }

wmanIf2mSsFm OBJECT IDENTIFIER ::= { wmanIf2mSsMib 2 }

wmanIf2mSsPm OBJECT IDENTIFIER ::= { wmanIf2mSsMib 3 }

wmanIf2mSsSm OBJECT IDENTIFIER ::= { wmanIf2mSsMib 4 }

wmanIf2mSsAm OBJECT IDENTIFIER ::= { wmanIf2mSsMib 5 }

--

-- Textual Conventions

--

WmanIf2mOfdmaMobility ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates whether or not the MS supports

mobility hand-over, Slee-pmode, and Idle-mode. A bit value

of 0 indicates 'not supported' while 1 indicates it is

supported."

REFERENCE

"Subclause 11.7.14.1 in IEEE Std 802.16e-2005"

SYNTAX      BITS {handoverSupport(0),

sleepModeSupport(1),

idleModeSupport(2)}

WmanIf2mHandoverType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates what type(s) of Handover the BS and the MS

supports.

bit#0: if set to 1, MDHO/FBSS HO not supported, the BS

shall ignore all other bits.

bit#1: if set to 1, FBSS/MDHO DL RF Combining is supported

with monitoring MAPs from active BSs

bit#2: if set to 1, MDHO DL soft Combining is supported

with monitoring single MAP from anchor BS

bit#3: if set to 1, MDHO DL soft combining is supported

with monitoring MAPs from active BSs

bit#3: if set to 1, MDHO UL Multiple transmission is

supported"

REFERENCE

"Subclause 11.7.8.12 in IEEE Std 802.16e-2005"

SYNTAX      BITS {mdhcFbssHoNotSpported(0),

mdhcFbssDlMapsFromActiveBss(1),

mdhcDlMapFromAnchorBs(2),

mdhcDlMapsFromActiveBss(3),

mdhcUlMultipleTx(4)}

WmanIf2mCidType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of CID."

SYNTAX      INTEGER (0 .. 65535)

WmanIf2mPsClassId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Indicates the index to Power Saving Classes. The ID shall

be unique within the group of Power Saving Classes

associated with the MS. This ID may be used in further

MOB_SLP-REQ/RSP messages for activation / deactivation of

Power Saving Class."

REFERENCE

"Subclause 6.3.2.3.44 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER (0..63)

WmanIf2mPsClassType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The types of power saving classes."

REFERENCE

"Table 374a in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {powerSavingClassTypeI(1),

powerSavingClassTypeII(2),

powerSavingClassTypeIII(3)}

WmanIf2mPsClassCidDir ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The direction of power saving class's CIDs.

0b00 = Unspecified. Each CID has its own direction assign

in its connection creation. Can be DL, UL, or both

(in the case of management connections).

0b01 = Downlink direction only.

0b10 = Uplink direction only."

REFERENCE

"Subclause 6.3.2.3.44 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {unspecified(0),

downlink(1),

uplink(2)}

WmanIf2mPowerSavingMode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Power saving class mode active or not active."

REFERENCE

"Subclause 6.3.2.3.44 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {psNotActive(0),

psActive(1)}

WmanIf2mSkipOptBitMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"If set to 1, its corresponding field will be omitted."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      BITS {omitOperatorId(0),

omitNeighborBsId(1),

omitHoProcOptimization(2),

omitQosRelatedField(3)}

WmanIf2mNbrBsId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The least significant 24 bits of the Base Station ID

parameter in the DL-MAP message broadcast by the Neighbor

BS. The BSID is a 6 byte number and follows the encoding

rules of MacAddress textual convention, i.e. as if it were

transmitted least-significant bit first. The value should

be displayed with 2 parts clearly separated by a colon.

Example 001DFF:00003A - 00003A is the Base Station ID. "

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mNbrOperatorId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The most significant 24 bits of the Base Station ID

parameter in the DL-MAP message broadcast by the Neighbor

BS. The BSID is a 6 byte number and follows the encoding

rules of MacAddress textual convention, i.e. as if it were

transmitted least-significant bit first. The value should

be displayed with 2 parts clearly separated by a colon.

Example 001DFF:00003A - 001DFF is the Operator ID. "

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mPhyProfileId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"For systems using OFDM or OFDMA, the definition of the PHY

Profile ID is shown as follows:

bit#0: If set to 1, BS (or FA) is co-located with the

serving BS

bit#1: If set to 1, the BS has the same number of FAs and

frequencies as the BS broadcasting the NBR-ADV

bit#2: 0b00 = Unsynchronized

bit#3: 0b01 = Time synchronization

0b10 = Time and Frequency synchronization

If time synchronization is indicated for the OFDMA

PHY, then the downlink frames transmitted by the

serving BS and the Neighbor BS shall be synchronized

to a level of at least 1/8 cyclic prefix length. If

frequency synchronization is indicated for the OFDMA

PHY, then the BS reference clocks shall be

synchronized to a level that yields RF center

frequency offset of no more than 1% of the OFDMA

carrier spacing of the Neighbor BS.

bit#4: If set to 1, the BS EIRP follows the PHY Profile ID

bit#5: 0b0- The DCD/UCD settings of this neighbor BS are

the same as those of the serving BS unless the

TLV information specifies.

0b1- The DCD/UCD settings of this neighbor BS are

the same as those of the preceding neighbor BS

unless the TLV information specifies.

bit#6: If set to 1, the FA Index follows the PHY Profile ID.

In addition, if the FA Indicator is followed, the DL

center frequency shall be omitted in the DCD/UCD

difference TLV information.

bit#7: The Trigger Reference Indicator is related to the

Neighbor BS trigger metric TLV information of this

neighbor BS.

0b0- The trigger settings of this neighbor BS are

the same as those provided by the serving BS

(via DCD).If the TLV information is present, it

overrides values inherited from preceding

neighbor BS.

0b1- The trigger settings of this neighbor BS are

the same as those of the preceding neighbor BS."

REFERENCE

"Table 109g in IEEE Std 802.16e-2005"

SYNTAX      BITS {colocatedFaInd(0),

faConfigInd(1),

timeFreqSyncInd1(2),

timeFreqSyncInd2(3),

bsEirpInd(4),

dcdUcdRefInd(5),

faIndexInd(6),

triggRefInd(7)}

WmanIf2mHoProcOptm ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"For each Bit location, a value of '0' indicates the

associated reentry management messages shall be required,

a value of '1' indicates the reentry management message may

be omitted. Regardless of the HO Process Optimization TLV

settings, the target BS may send unsolicited SBC-RSP and/ or

REG-RSP management messages

bit#0: Omit SBC-REQ/RSP management messages during re-entry

processing

bit#1: Omit PKM Authentication phase except TEK phase during

current re-entry processing

bit#2: Omit PKM TEK creation phase during reentry processing

bit#3: Omit REG-REQ/RSP management during current re-entry

processing

bit#4: Omit Network Address Acquisition management messages

during current reentry processing

bit#5: Omit Time of Day Acquisition management messages

during current reentry processing

bit#6: Omit TFTP management messages during current re-entry

processing

bit#7: Full service and operational state transfer or

sharing between serving BS and target BS (ARQ, timers

, counters, MAC state machines, etc...)"

REFERENCE

"Table 109f in IEEE Std 802.16e-2005"

SYNTAX      BITS {omitSbcReq(0),

omitPkmAuth(1),

omitPkmTek(2),

omitRegReq(3),

omitNtwkAddrAcq(4),

omitTimeOfDay(5),

omitTftp(6),

fullService(7)}

WmanIf2mSchedulingSupp ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Bitmap to indicate if BS supports a particular scheduling

service. 1 indicates support, 0 indicates not support:

bit#0: Unsolicited Grant Service (UGS)

bit#1: Real-time Polling Service (rtPS)

bit#2: Non-real-time Polling Service (nrtPS)

bit#3: Best Effort

bit#4: Extended real-time Polling Service (ertPS)

If the value of bit 0 through bit 4 is 0b00000, it indicates

no information on service available."

REFERENCE

"Table 109f in IEEE Std 802.16e-2005"

SYNTAX      BITS {ugs(0),

rtPs(1),

nrtPs(2),

be(3),

ertPs(4)}

WmanIf2mMacVersion ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Version number of IEEE 802.16."

SYNTAX      INTEGER {ieee802Dot16Of2001(1),

ieee802Dot16cOf2002(2),

ieee802Dot16aOf2003(3),

ieee802Dot16Of2004(4),

ieee802Dot16e(5),

tbd(6)}

WmanIf2mHarqAckDelay ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"HARQ ACK delay for UL and DL bursts

1 = one frame offset

2 = two frames offset

3 = three frames offset"

REFERENCE

"Table 353 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {oneframeoffset(1),

twoframesoffset(2),

threeframesoffset(3)}

WmanIf2mPowerSaveType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"For MS supporting sleep mode, this parameter defines the

capability of the MS supporting different power save class

types in sleep mode.

A bit 0 - 'not supported'

1 - 'supported'"

REFERENCE

"Subclause 11.8.5 in IEEE Std 802.16e-2005"

SYNTAX      BITS {psClassTypeI(0),

psClassTypeII(1),

psClassTypeIII(2)}

WmanIf2mHoTrigMatrix ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates trigger metrics that MS or BS supports.

A bit 0 - 'not supported'

1 - 'supported'"

REFERENCE

"Subclause 11.8.7 in IEEE Std 802.16e-2005"

SYNTAX      BITS {bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRoundTripDelay(3)}

WmanIf2mAssociationTyp ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"This field indicates the association level supported by the

MS or the BS. If a bit is set to '1', then MS or BS

indicates support at the respective association type and

level. The MS may associate according to arrangements by

the BS at levels up to and including the one for which the

MS has indicated support."

REFERENCE

"Subclause 11.8.8 in IEEE Std 802.16e-2005"

SYNTAX      BITS {scanWoAssociation(0),

scanOrAssocWoCoordination(1),

assocWithCoordination(2),

ntwkAssistAssociation(3),

directAssociation(4)}

WmanIf2mOfdmaFftSize ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"FFT size for OFDMA PHY

0b000 = 2048

0b001 = 1024

0b010 = 512

0b100 = 128"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {fft2048(0),

fft1024(1),

fft512(2),

reserved(3),

fft128(4)}

WmanIf2mOfdmaCp ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Cycle prefix for OFDMA PHY

0b00 = 1/4

0b01 = 1/8

0b10 = 1/16

0b11 = 1/32"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {oneForth(0),

oneEighth(1),

oneSixteenth(2),

oneThirtySecond(3)}

WmanIf2mOfdmaFrame ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Frame duration for OFDMA PHY

0b0000 = 2.0 ms

0b0001 = 2.5 ms

0b0010 = 4 ms

0b0011 = 5 ms

0b0100 = 8 ms

0b0101 = 10 ms

0b0110 = 12.5 ms

0b0111 = 20 ms"

REFERENCE

"Subclause 11.18.1, Table 384b in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {twoMs(0),

twoPointFiveMs(1),

fourMs(2),

fiveMs(3),

eightMs(4),

tenMs(5),

twelvePointFiveMs(6),

twentyMs(7)}

WmanIf2mPagingAction ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Paging action instruction to MS

0b00 = No Action Required

0b01 = Perform Ranging to establish location and

acknowledge message

0b10 = Enter Network"

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noAction(0),

performRanging(1),

enterNetwork(2)}

WmanIf2mSsMacAddrHash ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"24 bit SS MAC address hash that is obtained by computing a

CRC24 on the MS 48-bit MAC address."

REFERENCE

"Subclause 6.3.2.3.56, Table 109p in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mSfState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the state of a service flow.

'inactive' - A service flow is inactive, when the MS owns

this service flow has handoff to another BS.

'provisioned' - A service flow is provisioned, but not

yet activated.

'admitted' - This maps to the 1st phase of the two-phase

activation model that the bandwidth a service

flow is reserved. But, no traffic can be

sent on this service flow yet.

'active' - This maps to the 2nd phase of the two-phase

activation model that bandwidth is granted,

(e.g., is actively sending UL maps containing

unsolicited grants for a UGS service flow)."

REFERENCE

"Subclause 6.3.14.6 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {inactive(0),

provisioned(1),

admitted(2),

active(3)}

WmanIf2mGlobalSrvClass ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Global Service Class Name contains 8 information fields

that map to predefined QoS attributes as shown in

subclause 6.3.14.4.1.

bit#0:     Uplink/Downlink indicator

0 - uplink

1 - downlink

bit#1-6:   Maximum sustained traffic rate in bps that is

defined in Table 124b

bit#7:     0 - no traffic indication

1 - traffic indication

bit#8-13:  Maximum traffic burst defines the maximum burst

size that must be accommodated for the service.

bit#14-19: Minimum reserved traffic rate parameter

specifies the minimum rate, in bits per second,

reserved for this service flow.

bit#20-25: Maximum latency specifies the maximum interval

between the reception of a packet at CS of BS

or SS and the arrival of the packet to the peer

device.

bit#26:    SDU indicator specifies whether the SDUs on the

service flow are fixed-length or variable-length.

0 - variable length

1 - fixed length

bit#27:    Paging indicator of an MS preference for the

reception of paging advisory messages during

idle mode. When set, it indicates that the BS

may present paging advisory messages or other

indicative messages to the MS when data SDUs

bound for the MS are present while the MS is in

Idle Mode.

0 - no paging generation

1 - paging generation"

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

SYNTAX      BITS {ulDlIndicator(0),

maxSustainedRate0(1),

maxSustainedRate1(2),

maxSustainedRate2(3),

maxSustainedRate3(4),

maxSustainedRate4(5),

maxSustainedRate5(6),

trafficIndication(7),

maxTrafficBurst0(8),

maxTrafficBurst1(9),

maxTrafficBurst2(10),

maxTrafficBurst3(11),

maxTrafficBurst4(12),

maxTrafficBurst5(13),

minReservedRate0(14),

minReservedRate1(15),

minReservedRate2(16),

minReservedRate3(17),

minReservedRate4(18),

minReservedRate5(19),

maxLatency0(20),

maxLatency1(21),

maxLatency2(22),

maxLatency3(23),

maxLatency4(24),

maxLatency5(25),

sduIndicator(26),

pagingGeneration(27),

reserved0(28),

reserved1(29),

reserved2(30),

reserved3(31)}

WmanIf2mSfDirection ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The direction of a service flow"

SYNTAX      INTEGER {downstream(1),

upstream(2)}

WmanIf2mReqTxPolicy ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Specify certain attributes for the associated service

flow. An attribute is enabled by setting the

corresponding bit position to 1.

bit#0: Service flow shall not use broadcast bandwidth

request opportunities. (Uplink only)

bit#1: reserved

bit#2: The service flow shall not piggyback requests with

data. (Uplink only)

bit#3: The service flow shall not fragment data.

bit#4: The service flow shall not suppress payload headers

(CS parameter)

bit#5: The service flow shall not pack multiple SDUs (or

fragments) into single MAC PDUs.

bit#6: The service flow shall not include CRC in the MAC

PDU."

REFERENCE

"Subclause 11.13.12 in IEEE Std 802.16-2004"

SYNTAX      BITS {noBroadcastBwReq(0),

reserved1(1),

noPiggybackReq(2),

noFragmentData(3),

noPHS(4),

noSduPacking(5),

noCrc(6),

reserved2(7)}

WmanIf2mPhsRuleVerify ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression. If PHSV is

enabled, the sender shall compare the bytes in the packet

header with the bytes in the PHSF that are to be

suppressed as indicated by the PHSM."

REFERENCE

"Subclause 11.13.19.3.7.5 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {phsVerifyEnable(0),

phsVerifyDisable(1)}

WmanIf2mSchedulingType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The scheduling service provided by a SC for an

upstream service flow. If the parameter is omitted

from an upstream QOS Parameter Set, this object takes

the value of bestEffort (2). This parameter must be

reported as undefined (1) for downstream QOS Parameter

Sets."

SYNTAX      INTEGER {undefined(1),

bestEffort(2),

nonRealTimePollingService(3),

realTimePollingService(4),

extRealTimePollingService(5),

unsolicitedGrantService(6)}

WmanIf2mCsSpecification ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the types of convergence sublayer."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {reserved(0),

packetIpV4(1),

packetIpV6(2),

packet802dot3Ethernet(3),

packet802dot1QVlan(4),

packetIpV4Over802dot3(5),

packetIpV6Over802dot3(6),

packetIpV4Over802dot1Q(7),

packetIpV6Over802dot1Q(8),

atm(9),

packet802dot3EthernetRohcHc(10),

packet802dot3EthernetEcrtpHc(11),

packetIp2RohcHc(12),

packetIp2EcrtpHc(13)}

WmanIf2mIpv6FlowLabel ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field. As the flow label field has a

length of 20 bits, the first 4 bits of the most

significant byte shall be set to 0x0 and disregarded."

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2mClassifierBitMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"A bit of of this object is set to 1 if the parameter

indicated by the comment was present in the classifier

encoding, and 0 otherwise.

Note: that BITS are encoded most significant bit first,

so that if e.g. bits 6 and 7 are set, this object is

encoded as the octet string '030000'H."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16-2004"

SYNTAX      BITS {priority(0),

ipTos(1),

ipProtocol(2),

ipMaskedSrcAddr(3),

ipMaskedDestAddr(4),

srcPort(5),

destPort(6),

destMacAddr(7),

srcMacAddr(8),

ethernetProtocol(9),

userPriority(10),

vlanId(11),

ipv6FlowLabel(12)}

-- XXX

WmanIf2mReportMode ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Action code for an MS's report of CINR measurement:

0b00: The MS measures channel quality of the Available BSs

without reporting.

0b01: The MS reports the result of the measurement to

Serving BS periodically. The period of reporting is

different from that of scanning.

0b10: The MS reports the result of the measurement to

Serving BS after each measurement."

REFERENCE

"Subclause 6.3.2.3.49 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {noReport(0),

periodicReport(1),

eventTriggeredReport(2)}

-- XXX

WmanIf2mReportMatric ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Bitmap indicator of trigger metrics that the serving BS

requests the MS to report. Serving BS shall indicate only

the trigger metrics agreed during SBC-REQ/RSP negotiation.

Each bit indicates whether reports will be initiated by

trigger based on the corresponding metric:

Bit 0:    BS CINR mean

Bit 1:    BS RSSI mean

Bit 2:    Relative delay

Bit 3:    BS RTD; this metric shall be only measured on

serving BS/anchor BS"

REFERENCE

"Subclause 6.3.2.3.48 in IEEE Std 802.16-2004"

SYNTAX      BITS {bsCinrMean(0),

bsRssiMean(1),

relativeDelay(2),

bsRtd(3)}

-- XXX

WmanIf2mFullBsId ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The 48 bit field sent in the DL-MAP message to identifies

the BS."

REFERENCE

"Subclause 6.3.2.3.47 in IEEE Std 802.16e-2005"

SYNTAX      OCTET STRING (SIZE(6))

-- XXX

WmanIf2mScanType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of scanning or association used by the MS and

coordinated by the Serving BS:

0b000: Scanning without Association Scanning

0b001: Scanning with Association level 0: association

without coordination

0b010: Scanning with Association level 1: association

with coordination

0b011: Scanning with Association level 2: network assisted

association"

REFERENCE

"Subclause 6.3.2.3.49 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {scanWoAssociation(0),

scanWithAssociation0(1),

scanWithAssociation1(2),

scanWithAssociation2(3)}

-- XXX

WmanIf2mSsSsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"xxxReq: Indicates that SS reports its capabilities in

RNG-REQ, SBC-REQ, or REG-REQ messages.

xxxRsp: Indicates that BS confirms SS capabilities in

RNG-RSP, SBC-RSP, or REG-RSP messages."

SYNTAX INTEGER {xxxReq (1),

xxxRsp (2)}

WmanIf2mSsCapType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"inheritedBsCap: Indicates BS' basic HW and SW capabilities.

configedBsCap: Indicates BS' capabilities that has been

configured."

SYNTAX      INTEGER {inheritedBsCap (1),

configedBsCap (2)}

--

-- wmanIf2mSsConfigurationTable contains global parameters for SS

--

wmanIf2mSsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains one row for the SS system parameters."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2mSsCm 1 }

wmanIf2mSsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2mSsConfigurationTable 1 }

WmanIf2mSsConfigurationEntry ::= SEQUENCE {

wmanIf2mSsMinSleepInterval              INTEGER,

wmanIf2mSsMaxSleepInterval              INTEGER,

wmanIf2mSsListeningInterval             INTEGER,

wmanIf2mSsNrbBsIndexValidityTime        INTEGER,

wmanIf2mSsAscAgingTimer                 INTEGER,

wmanIf2mSsServingBsidAgingTimer         INTEGER,

wmanIf2mSsT42Timer                      INTEGER,

wmanIf2mSsFastTrackingRspProcTime       INTEGER,

wmanIf2mSsModeSelectFeedbackProcTime    INTEGER,

wmanIf2mSsIdleModeTimer                 Unsigned32,

wmanIf2mSsT43Timer                      INTEGER,

wmanIf2mSsT44Timer                      INTEGER,

wmanIf2mSsT45Timer                      INTEGER,

wmanIf2mSsDregReqRetryCount             INTEGER,

wmanIf2mSsHoProcOptimizeMsTimerRetry    INTEGER,

wmanIf2mSsPagingInterval                INTEGER,

wmanIf2mSsMaxDirScanTime                INTEGER}

wmanIf2mSsMinSleepInterval OBJECT-TYPE

SYNTAX      INTEGER (2 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum sleeping time allowed to MS."

::= { wmanIf2mSsConfigurationEntry 1 }

wmanIf2mSsMaxSleepInterval OBJECT-TYPE

SYNTAX      INTEGER (2 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum sleeping time allowed to MS."

::= { wmanIf2mSsConfigurationEntry 2 }

wmanIf2mSsListeningInterval OBJECT-TYPE

SYNTAX      INTEGER (1 .. 64)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time duration during which the MS, after waking up and

synchronizing with the DL transmissions, can demodulate

downlink transmissions and decide whether to stay awake or

go back to sleep."

::= { wmanIf2mSsConfigurationEntry 3 }

wmanIf2mSsNrbBsIndexValidityTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 5)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time duration during which the MS can use the neighbor BS

list in MOB_NBR-ADV message for the compression of neighbor

BSIDs."

::= { wmanIf2mSsConfigurationEntry 4 }

wmanIf2mSsAscAgingTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time for aging of MS associations"

::= { wmanIf2mSsConfigurationEntry 5 }

wmanIf2mSsServingBsidAgingTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 5000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time for aging of serving BS association. Timer

recycles on successful serving BS DL-MAP read."

::= { wmanIf2mSsConfigurationEntry 6 }

wmanIf2mSsT42Timer OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MOB_HO-IND timeout when sent with HO_IND_type = 0b10."

::= { wmanIf2mSsConfigurationEntry 7 }

wmanIf2mSsFastTrackingRspProcTime OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an MS following receipt of a UL-MAP Fast

tracking indication response before it is expected to apply

the corrections instructed by the BS.

Default value = One DL subframe duration"

::= { wmanIf2mSsConfigurationEntry 8 }

wmanIf2mSsModeSelectFeedbackProcTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "microseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time allowed between the end of the burst carrying the

Mode Selection Feedback subheader and the start of the UL

subframe carrying the Mode Selection Feedback response.

Minimum value = 1 frame duration for TDD

1/2 Frame duration for FDD"

::= { wmanIf2mSsConfigurationEntry 9 }

wmanIf2mSsIdleModeTimer OBJECT-TYPE

SYNTAX      Unsigned32 (128 .. 65536)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MS timed interval to conduct Location Update. Set timer to

MS Idle Mode Timeout capabilities setting. Timer recycles

on successful Idle Mode Location Update."

DEFVAL      { 4096 }

::= { wmanIf2mSsConfigurationEntry 10 }

wmanIf2mSsT43Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for MOB_SLP-RSP."

::= { wmanIf2mSsConfigurationEntry 11 }

wmanIf2mSsT44Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for MOB_SCN-RSP."

::= { wmanIf2mSsConfigurationEntry 12 }

wmanIf2mSsT45Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 500)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for DREGCMD."

DEFVAL      { 250 }

::= { wmanIf2mSsConfigurationEntry 13 }

wmanIf2mSsDregReqRetryCount OBJECT-TYPE

SYNTAX      INTEGER (3 .. 16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on DREG Request Message."

DEFVAL      { 3 }

::= { wmanIf2mSsConfigurationEntry 14 }

wmanIf2mSsHoProcOptimizeMsTimerRetry OBJECT-TYPE

SYNTAX      INTEGER (3 .. 100)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of SBC-REQ and/or REG-REQ retries while waiting

for unsolicited SBC-RSP and/or REG-RSP as part of MS

network re-entry and as indicated by HO Process

Optimization message element of RNGRSP."

::= { wmanIf2mSsConfigurationEntry 15 }

wmanIf2mSsPagingInterval OBJECT-TYPE

SYNTAX      INTEGER (8 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time duration of Paging Interval of the BS."

DEFVAL      { 64 }

::= { wmanIf2mSsConfigurationEntry 16 }

wmanIf2mSsMaxDirScanTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum scanning time of neighbor BSs by MS before

reporting any results."

::= { wmanIf2mSsConfigurationEntry 17 }

wmanIf2mSsServiceFlowTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains the service flow database. When an SS

first registers at the BS, the BS should download the

SS' service flow profile (e.g. QoS parameter set and

classification rules) from the home AAA server."

REFERENCE

"Subclause 6.3.14 in IEEE Std 802.16e-2005"

::= { wmanIf2mSsCm 2 }

wmanIf2mSsServiceFlowEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each service flow. It

supports both unicast and multicast service flows:

Unicast - a SS (wmanIf2mSsMacAddress) may contain

multiple service flows (wmanIf2mSsSfId)

Multicast - a service flow (wmanIf2mSsSfId) may be

multicast to multiple SS

(wmanIf2mSsSsMacAddress)"

INDEX     { ifIndex,

wmanIf2mSsMacAddress,

wmanIf2mSsSfId }

::= { wmanIf2mSsServiceFlowTable 1 }

WmanIf2mSsServiceFlowEntry ::= SEQUENCE {

wmanIf2mSsMacAddress                    MacAddress,

wmanIf2mSsSfId                          Unsigned32,

wmanIf2mSsServiceFlowDirection          WmanIf2mSfDirection,

wmanIf2mSsProvisionedGlobalServiceClass WmanIf2mGlobalSrvClass,

wmanIf2mSsAdmittedGlobalServiceClass    WmanIf2mGlobalSrvClass,

wmanIf2mSsActiveGlobalServiceClass      WmanIf2mGlobalSrvClass,

wmanIf2mSsProvisionedQoSProfileIndex    INTEGER,

wmanIf2mSsAdmittedQoSProfileIndex       INTEGER,

wmanIf2mSsActiveQoSProfileIndex         INTEGER,

wmanIf2mSsClassifierRuleIndex           Unsigned32,

wmanIf2mSsPhsRuleIndex                  INTEGER,

wmanIf2mSsArqAttributeIndex             INTEGER,

wmanIf2mSsServiceFlowState              WmanIf2mSfState,

wmanIf2mSsCid                           INTEGER,

wmanIf2mSsSfCsSpecification             WmanIf2mCsSpecification,

wmanIf2mSsSfMinTolerableTrafficRate     Unsigned32,

wmanIf2mSsSfReqTxPolicy                 WmanIf2mReqTxPolicy,

wmanIf2mSsSfTargetSaid                  INTEGER,

wmanIf2mSsSfEstablishTime               TimeStamp,

wmanIf2mSsSfTerminateTime               TimeStamp}

wmanIf2mSsMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The MAC address of the SS that the service flow is

associated with."

::= { wmanIf2mSsServiceFlowEntry 1 }

wmanIf2mSsSfId OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A 32 bit quantity that uniquely identifies a service flow."

::= { wmanIf2mSsServiceFlowEntry 2 }

wmanIf2mSsServiceFlowDirection OBJECT-TYPE

SYNTAX      WmanIf2mSfDirection

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An attribute indicating the direction of a service flow."

::= { wmanIf2mSsServiceFlowEntry 3 }

wmanIf2mSsProvisionedGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2mGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the ProvisionedQoSParamSet for this

service flow. When '0' is returned from reading this object

, it means either no global service class is defined, or

its Qos profile may be defined in

wmanIf2mSsProvisionedQoSProfileIndex."

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

::= { wmanIf2mSsServiceFlowEntry 4 }

wmanIf2mSsAdmittedGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2mGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the AdmittededQoSParamSet for this

service flow. When '0' is returned from reading this object

, it means either no global service class is defined, or

its Qos profile may be defined in

wmanIf2mSsAdmittedQoSProfileIndex. AdmittededQoSParamSet is

a subset of ProvisionedQoSParamSet."

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

::= { wmanIf2mSsServiceFlowEntry 5 }

wmanIf2mSsActiveGlobalServiceClass OBJECT-TYPE

SYNTAX      WmanIf2mGlobalSrvClass

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object defines the ActiveQoSParamSet for this service

flow. When '0' is returned from reading this object, it

means either no global service class is defined, or its Qos

profile may be defined in wmanIf2mSsActiveQoSProfileIndex.

ActiveQoSParamSet is a subset of AdmittededQoSParamSet."

REFERENCE

"Subclause 6.3.14.4.1 Table 124a in IEEE Std 802.16e-2005"

::= { wmanIf2mSsServiceFlowEntry 6 }

wmanIf2mSsProvisionedQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mSsQoSProfileTable

that defines the ProvisionedQoSParamSet of a service flow.

If WmanIf2mSfState = 'provisioned', then

ProvisionedQoSParamSet is the QoS profile for this service

flow. When '0' is returned from reading this object, it

means the QoS profile either is not defined, or is defined

in wmanIf2mSsProvisionedQoSProfileIndex."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 7 }

wmanIf2mSsAdmittedQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mSsQoSProfileTable

that defines the AdmittedQoSParamSet of a service flow. If

WmanIf2mSfState = 'admitted', then AdmittedQoSParamSet is

the QoS profile for this service flow. When '0' is returned

from reading this object, it means the QoS profile either

is not defined, or is defined in

wmanIf2mSsAdmittedQoSProfileIndex. AdmittededQoSParamSet is

a subset of ProvisionedQoSParamSet."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 8 }

wmanIf2mSsActiveQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mSsQoSProfileTable

that defines the ActiveQoSParamSet of a service flow. If

WmanIf2mSfState = 'active', then ActiveQoSParamSet is the

QoS profile for this service flow. When '0' is returned

from reading this object, it means the QoS profile either

is not defined, or is defined in

wmanIf2mSsActiveQoSProfileIndex. ActiveQoSParamSet is a

subset of AdmittedQoSParamSet."

REFERENCE

"Subclause 6.3.13 and 6.3.14 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 9 }

wmanIf2mSsClassifierRuleIndex OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 4294967295)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in

wmanIf2mSsClassifierRuleTable that defines the

classification rules for a service flow. When '0' is

returned from reading this object, it means the

classification rules are not defined for this service

flow."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 10 }

wmanIf2mSsPhsRuleIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in wmanIf2mSsPhsRuleTable

that defines the packet suppression rules for a service

flow. When '0' is returned from reading this object, it

means the PHS rules are not defined for this service

flow."

REFERENCE

"Subclause 11.13.19.3.5 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 11 }

wmanIf2mSsArqAttributeIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This index points to an entry in

wmanIf2mSsArqAttributeTable that defines the ARQ

attributes for a service flow. When '0' is returned from

reading this object, it means the ARQ attributes are not

defined for this service flow."

REFERENCE

"Subclause 11.13.19.3.5 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 12 }

wmanIf2mSsServiceFlowState OBJECT-TYPE

SYNTAX      WmanIf2mSfState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2SsServiceFlowState determines the state of a service

flow."

REFERENCE

"Subclause 6.3.14.6, in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 13 }

wmanIf2mSsCid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 16 bit channel identifier points to the connection being

created by DSA for this service flow. When '0' is returned

from reading this object, it means no CID has been assigned

to this service flow yet."

::= { wmanIf2mSsServiceFlowEntry 14 }

wmanIf2mSsSfCsSpecification OBJECT-TYPE

SYNTAX      WmanIf2mCsSpecification

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the convergence sublayer

encapsulation mode."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 15 }

wmanIf2mSsSfMinTolerableTrafficRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Minimum Tolerable Traffic Rate = R (bits/sec) with time

base T(sec) means the following. Let S denote additional

demand accumulated at the MAC SAP of the transmitter

during an arbitrary time interval of the length T. Then the

amount of data forwarded at the receiver to CS (in bits)

during this interval should be not less than min {S, R * T}

."

REFERENCE

"Subclause 11.13.9 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 16 }

wmanIf2mSsSfReqTxPolicy OBJECT-TYPE

SYNTAX      WmanIf2mReqTxPolicy

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter provides the capability to

specify certain attributes for the associated service flow.

An attribute is enabled by setting the corresponding bit

position to 1."

REFERENCE

"Subclause 11.13.12 in IEEE Std 802.16-2004"

::= { wmanIf2mSsServiceFlowEntry 17 }

wmanIf2mSsSfTargetSaid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The target SAID parameter indicates the SAID onto which the

service flow being set up shall be mapped."

REFERENCE

"Subclause 11.13.17 in IEEE Std 802.16e-2005"

::= { wmanIf2mSsServiceFlowEntry 18 }

wmanIf2mSsSfEstablishTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the service flow is

established that means wmanIf2mSsServiceFlowState is

either in 'provisioned', 'admitted', or 'active' state."

::= { wmanIf2mSsServiceFlowEntry 19 }

wmanIf2mSsSfTerminateTime OBJECT-TYPE

SYNTAX      TimeStamp

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the date and time when the service flow is

terminated that means wmanIf2mSsServiceFlowState is

in 'inactive' state."

::= { wmanIf2mSsServiceFlowEntry 20 }

wmanIf2mSsClassifierRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains packet classifier rules associated

with service flows."

::= { wmanIf2mSsCm 3 }

wmanIf2mSsClassifierRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each packet classifier rule

."

INDEX     { ifIndex, wmanIf2mSsClassifierRuleId }

::= { wmanIf2mSsClassifierRuleTable 1 }

WmanIf2mSsClassifierRuleEntry::= SEQUENCE {

wmanIf2mSsClassifierRuleId              Unsigned32,

wmanIf2mSsClassifierRulePriority        INTEGER,

wmanIf2mSsClassifierRuleIpTosLow        INTEGER,

wmanIf2mSsClassifierRuleIpTosHigh       INTEGER,

wmanIf2mSsClassifierRuleIpTosMask       INTEGER,

wmanIf2mSsClassifierRuleIpProtocol      Integer32,

wmanIf2mSsClassifierRuleIpSrcAddr       InetAddress,

wmanIf2mSsClassifierRuleIpSrcMask       InetAddress,

wmanIf2mSsClassifierRuleIpDestAddr      InetAddress,

wmanIf2mSsClassifierRuleIpDestMask      InetAddress,

wmanIf2mSsClassifierRuleSrcPortStart    Integer32,

wmanIf2mSsClassifierRuleSrcPortEnd      Integer32,

wmanIf2mSsClassifierRuleDestPortStart   Integer32,

wmanIf2mSsClassifierRuleDestPortEnd     Integer32,

wmanIf2mSsClassifierRuleDestMacAddr     MacAddress,

wmanIf2mSsClassifierRuleDestMacMask     MacAddress,

wmanIf2mSsClassifierRuleSrcMacAddr      MacAddress,

wmanIf2mSsClassifierRuleSrcMacMask      MacAddress,

wmanIf2mSsClassifierRuleEnetType        INTEGER,

wmanIf2mSsClassifierRuleEnetProtocol    Integer32,

wmanIf2mSsClassifierRuleUserPriLow      Integer32,

wmanIf2mSsClassifierRuleUserPriHigh     Integer32,

wmanIf2mSsClassifierRuleVlanId          Integer32,

wmanIf2mSsClassifierRuleIpv6FlowLabel   WmanIf2mIpv6FlowLabel,

wmanIf2mSsClassifierRulePkts            Counter64,

wmanIf2mSsClassifierRuleBitMap          WmanIf2mClassifierBitMap

}

wmanIf2mSsClassifierRuleId  OBJECT-TYPE

SYNTAX      Unsigned32 (1..4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index is assigned to each classifier in the classifiers

table"

REFERENCE

"Subclause 11.13.19.3.4.14 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 1 }

wmanIf2mSsClassifierRulePriority OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value specifies the order of evaluation of the

classifiers. The higher the value the higher the priority.

The value of 0 is used as default in provisioned service

flows classifiers. The default value of 64 is used for

dynamic service flow classifiers. If the referenced

parameter is not present in a classifier, this object

reports the default value as defined above"

REFERENCE

"Subclause 11.13.19.3.4.1 in IEEE Std 802.16-2004"

DEFVAL      { 0 }

::= { wmanIf2mSsClassifierRuleEntry 2 }

wmanIf2mSsClassifierRuleIpTosLow OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The low value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 3 }

wmanIf2mSsClassifierRuleIpTosHigh OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The 8-bit high value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 4 }

wmanIf2mSsClassifierRuleIpTosMask OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The mask value is bitwise ANDed with TOS byte in an IP

packet and this value is used for the range checking of

TosLow and TosHigh. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 5 }

wmanIf2mSsClassifierRuleIpProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the value of the IP Protocol field

required for IP packets to match this rule. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 6 }

wmanIf2mSsClassifierRuleIpSrcAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Source Address

required for packets to match this rule. An IP packet

matches the rule when the packet ip source address bitwise

ANDed with the wmanIf2mSsClassifierRuleIpSrcMask value

equals the wmanIf2mSsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 7 }

wmanIf2mSsClassifierRuleIpSrcMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP Source

Address that are compared to match this rule. An IP packet

matches the rule when the packet source address bitwise

ANDed with the

wmanIf2mSsClassifierRuleIpSrcMask value equals the

wmanIf2mSsClassifierRuleIpSrcAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 8 }

wmanIf2mSsClassifierRuleIpDestAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Destination

Address required for packets to match this rule. An IP

packet matches the rule when the packet IP destination

address bitwise ANDed with the

wmanIf2mSsClassifierRuleIpDestMask value equals the

wmanIf2mSsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a

classifier, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 9 }

wmanIf2mSsClassifierRuleIpDestMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP

Destination Address that are compared to match this rule.

An IP packet matches the rule when the packet destination

address bitwise ANDed with the

wmanIf2mSsClassifierRuleIpDestMask value equals the

wmanIf2mSsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 10 }

wmanIf2mSsClassifierRuleSrcPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP source port numbers to which a packet is compared

. This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 11 }

wmanIf2mSsClassifierRuleSrcPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a classifier,

this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 12 }

wmanIf2mSsClassifierRuleDestPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 13 }

wmanIf2mSsClassifierRuleDestPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of

65535."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 14 }

wmanIf2mSsClassifierRuleDestMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2mSsClassifierRuleDestMacMask equals the value of

wmanIf2mSsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 15 }

wmanIf2mSsClassifierRuleDestMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2mSsClassifierRuleDestMacMask equals the value of

wmanIf2mSsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 16 }

wmanIf2mSsClassifierRuleSrcMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches this entry when its source MAC

address  bitwise ANDed with

wmanIf2mSsClassifierRuleSrcMacMask equals the value of

wmanIf2mSsClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 17 }

wmanIf2mSsClassifierRuleSrcMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2mSsClassifierRuleSrcMacMask equals the value of

wmanIf2mSsClassifierRuleSrcMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 18 }

wmanIf2mSsClassifierRuleEnetType OBJECT-TYPE

SYNTAX      INTEGER {none(0),

ethertype(1),

dsap(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the format of the layer 3 protocol

id in the Ethernet packet. A value of none(0) means that

the rule does not use the layer 3 protocol type as a

matching criteria. A value of ethertype(1) means that the

rule applies only to frames which contains an EtherType

value. Ethertype values are contained in packets using

the Dec-Intel-Xerox (DIX) encapsulation or the RFC1042

Sub-Network Access Protocol (SNAP) encapsulation formats.

A value of dsap(2) means that the rule applies only to

frames using the IEEE802.3 encapsulation format with a

Destination Service Access Point (DSAP) other than 0xAA

(which is reserved for SNAP). If the Ethernet frame

contains an 802.1P/Q Tag header (i.e. EtherType 0x8100),

this object applies to the embedded EtherType field within

the 802.1P/Q header. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 19 }

wmanIf2mSsClassifierRuleEnetProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If wmanIf2mSsClassifierRuleEnetType is none(0), this

object is ignored when considering whether a packet matches

the current rule. If wmanIf2mSsClassifierRuleEnetType is

ethertype(1), this object gives the 16-bit value of the

EtherType that the packet must match in order to match the

rule. If wmanIf2mSsClassifierRuleEnetType is dsap(2), the

lower 8 bits of this object's value must match the DSAP

byte of the packet in order to match the rule.

If the Ethernet frame contains an 802.1P/Q Tag header

(i.e. EtherType 0x8100), this object applies to the

embedded EtherType field within the 802.1P/Q header.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 20 }

wmanIf2mSsClassifierRuleUserPriLow OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority field

within the range of wmanIf2mSsClassifierRulePriLow and

wmanIf2mSsClassifierRulePriHigh in order to match this

rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 21 }

wmanIf2mSsClassifierRuleUserPriHigh OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority

field within the range of wmanIf2mSsClassifierRulePriLow

and wmanIf2mSsClassifierRulePriHigh in order to match

this rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 7."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 22 }

wmanIf2mSsClassifierRuleVlanId OBJECT-TYPE

SYNTAX      Integer32 (0..4095)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header.

If this object's value is nonzero, tagged packets must

have a VLAN Identifier that matches the value in order

to match the rule.

Only the least significant 12 bits of this object's

value are valid.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.12 in IEEE Std 802.16-2004"

::= { wmanIf2mSsClassifierRuleEntry 23 }

wmanIf2mSsClassifierRuleIpv6FlowLabel OBJECT-TYPE

SYNTAX      WmanIf2mIpv6FlowLabel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2mSsClassifierRuleEntry 24 }

wmanIf2mSsClassifierRulePkts OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of packets that have

been classified using this entry."

::= { wmanIf2mSsClassifierRuleEntry 25 }

wmanIf2mSsClassifierRuleBitMap OBJECT-TYPE

SYNTAX      WmanIf2mClassifierBitMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2mSsClassifierRuleEntry 26 }

wmanIf2mSsPhsRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When a classifier is deleted, any associated

PHS rule shall also be deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2mSsCm 4 }

wmanIf2mSsPhsRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data."

INDEX     { ifIndex, wmanIf2mSsPhsRuleId }

::= { wmanIf2mSsPhsRuleTable 1 }

WmanIf2mSsPhsRuleEntry::= SEQUENCE {

wmanIf2mSsPhsRuleId                     INTEGER,

wmanIf2mSsPhsRulePhsField               OCTET STRING,

wmanIf2mSsPhsRulePhsMask                OCTET STRING,

wmanIf2mSsPhsRulePhsSize                Integer32,

wmanIf2mSsPhsRulePhsVerify              WmanIf2mPhsRuleVerify}

wmanIf2mSsPhsRuleId OBJECT-TYPE

SYNTAX      INTEGER (1 .. 255)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2mSsPhsRuleEntry 1 }

wmanIf2mSsPhsRulePhsField OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2mSsPhsRuleEntry 2 }

wmanIf2mSsPhsRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2mSsPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2mSsPhsRuleEntry 3 }

wmanIf2mSsPhsRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2mSsPhsRuleEntry 4 }

wmanIf2mSsPhsRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2mPhsRuleVerify

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2mSsPhsRuleEntry 5 }

wmanIf2mSsQoSProfileTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsQoSProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains QoS profiles that are associated with

service flows or CIDs via the wmanIf2mSsQoSProfileIndex.

The following table shows the required parameters for

different UL grant scheduling type.

0   - not required

1   - required

0-1 - optional

QoS Parameters                BE  ertPS  UGS  rtPS  nrtPS

---------------------------------------------------------

Traffic priority              0-1  0-1    0    0-1   0-1

Max sustained traffic rate    0-1  0-1    1    0-1   0-1

Min reserved traffic rate      0    1    0-1    1     1

Minimum traffic burst          0   0-1    0    0-1   0-1

Tolerated jitter               0   0-1    1     0     0

Maximum latency                0    1     1     1     0

Unsolicited Grant Interval     0    1     1     0     0

SDU size                       0    0    0-1    0     0

Unsolicited Polling Interval   0    0     0     1     0"

REFERENCE

"Subclause 6.3.14.4 in IEEE Std 802.16-2004"

::= { wmanIf2mSsCm 5 }

wmanIf2mSsQoSProfileEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsQoSProfileEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each QoS parameter Set."

INDEX     { ifIndex, wmanIf2mSsQoSProfileIndex }

::= { wmanIf2mSsQoSProfileTable 1 }

WmanIf2mSsQoSProfileEntry ::= SEQUENCE {

wmanIf2mSsQoSProfileIndex               INTEGER,

wmanIf2mSsQosServiceClassName           OCTET STRING,

wmanIf2mSsQosUlGrantScheduleType        WmanIf2mSchedulingType,

wmanIf2mSsQosTrafficPriority            INTEGER,

wmanIf2mSsQosMaximumSustainedRate       Unsigned32,

wmanIf2mSsQosMinimumReservedRate        Unsigned32,

wmanIf2mSsQosMaximumTrafficBurst        Unsigned32,

wmanIf2mSsQosToleratedJitter            Unsigned32,

wmanIf2mSsQosMaxLatency                 Unsigned32,

wmanIf2mSsQosUnsolicitedGrantInterval   Unsigned32,

wmanIf2mSsQosSduSize                    Unsigned32,

wmanIf2mSsQosUnsolicitedPollInterval    Unsigned32}

wmanIf2mSsQoSProfileIndex OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry in the

wmanIf2mSsQoSProfileTable"

::= { wmanIf2mSsQoSProfileEntry 1 }

wmanIf2mSsQosServiceClassName  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(2..128))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object is the Null-terminated string of ASCII

characters. It refers to a predefined BS service

configuration to be used for a service flow."

REFERENCE

"Subclause 11.13.3 in IEEE Std 802.16-2004"

::= { wmanIf2mSsQoSProfileEntry 2 }

wmanIf2mSsQosUlGrantScheduleType OBJECT-TYPE

SYNTAX      WmanIf2mSchedulingType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the Uplink grant scheduling type

that shall be enabled for the associated uplink service

flow upstream service flow. If the parameter is not

present in the corresponding 802.16 QOS Parameter Set of

an upstream service flow, the default value is assumed."

REFERENCE

"Subclause 11.13.11 in IEEE Std 802.16e-2004"

DEFVAL	    { bestEffort }

::= { wmanIf2mSsQoSProfileEntry 3 }

wmanIf2mSsQosTrafficPriority OBJECT-TYPE

SYNTAX      INTEGER (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the priority assigned

to a service flow. For uplink service flows, the BS should

use this parameter when determining precedence in request

service and grant generation, Higher numbers indicate

higher priority"

REFERENCE

"Subclause 11.13.5 in IEEE Std 802.16e-2005"

::= { wmanIf2mSsQoSProfileEntry 4 }

wmanIf2mSsQosMaximumSustainedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the peak information rate of the

service. The rate is expressed in bits per second and

pertains to the SDUs at the input to the Convergence

Sublayer."

REFERENCE

"Subclause 11.13.6 in IEEE Std 802.16e-2005"

::= { wmanIf2mSsQoSProfileEntry 5 }

wmanIf2mSsQosMinimumReservedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "bps"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the minimum rate reserved for this

service flow. It specifies the minimum amount of data to be

transported on behalf of the service flow when averaged

over time."

REFERENCE

"Subclause 11.13.8 in IEEE Std 802.16e-2004"

::= { wmanIf2mSsQoSProfileEntry 6 }

wmanIf2mSsQosMaximumTrafficBurst OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the maximum burst size that must be

accommodated for the service. It defines the maximum

continuous burst the system should accommodate for the

service assuming the service is not currently using any of

its available resources."

REFERENCE

"Subclause 11.13.7 in IEEE Std 802.16-2004"

::= { wmanIf2mSsQoSProfileEntry 7 }

wmanIf2mSsQosToleratedJitter OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the Maximum delay variation (jitter)

for the connection."

REFERENCE

"Subclause 11.13.13 in IEEE Std 802.16-2004"

::= { wmanIf2mSsQoSProfileEntry 8 }

wmanIf2mSsQosMaxLatency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the maximum latency between the

ingress of a packet to the Convergence Sublayer and the

forwarding of the SDU to its Air Interface."

REFERENCE

"Subclause 11.13.14 in IEEE Std 802.16-2004"

::= { wmanIf2mSsQoSProfileEntry 9 }

wmanIf2mSsQosUnsolicitedGrantInterval OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the nominal interval between

successive data grant opportunities for a service flow."

REFERENCE

"Subclause 11.13.20 in IEEE Std 802.16e-2004"

::= { wmanIf2mSsQoSProfileEntry 10 }

wmanIf2mSsQosSduSize OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the length of the SDU for a

fixed-length SDU service flow. It is used only if packing

is on and the service flow is indicated as carrying

fixed-length SDUs. If this object is omitted in the QoS

parameter set, it should return 0 that means the

variable-length service flow."

REFERENCE

"Subclause 11.13.16 in IEEE Std 802.16-2004"

::= { wmanIf2mSsQoSProfileEntry 11 }

wmanIf2mSsQosUnsolicitedPollInterval OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the maximal nominal interval between

successive polling grants opportunities for this Service

Flow."

REFERENCE

"Subclause 11.13.21 in IEEE Std 802.16e-2004"

::= { wmanIf2mSsQoSProfileEntry 12 }

wmanIf2mSsArqAttributeTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsArqAttributeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains ARQ parameters that are associated

with the Service Flows."

::= { wmanIf2mSsCm 6 }

wmanIf2mSsArqAttributeEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsArqAttributeEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each created service flow

for a given MacAddress."

INDEX     { ifIndex, wmanIf2mSsArqIndex  }

::= { wmanIf2mSsArqAttributeTable 1 }

WmanIf2mSsArqAttributeEntry::= SEQUENCE {

wmanIf2mSsArqIndex                      INTEGER,

wmanIf2mSsArqEnable                     TruthValue,

wmanIf2mSsArqWindowSize                 INTEGER,

wmanIf2mSsArqBlockLifetime              INTEGER,

wmanIf2mSsArqSyncLossTimeout            INTEGER,

wmanIf2mSsArqDeliverInOrder             TruthValue,

wmanIf2mSsArqRxPurgeTimeout             INTEGER,

wmanIf2mSsArqBlockSize                  INTEGER,

wmanIf2mSsArqAckProcessingTime          INTEGER}

wmanIf2mSsArqIndex OBJECT-TYPE

SYNTAX      INTEGER ( 1 .. 65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The index value which uniquely identifies an entry in the

in the wmanIf2mSsArqAttributeTable."

::= { wmanIf2mSsArqAttributeEntry 1 }

wmanIf2mSsArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the connection."

::= { wmanIf2mSsArqAttributeEntry 2 }

wmanIf2mSsArqWindowSize	 OBJECT-TYPE

SYNTAX	    INTEGER (1..1024)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged fragments

at any time."

::= { wmanIf2mSsArqAttributeEntry 3 }

wmanIf2mSsArqBlockLifetime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be managed

by the transmitter ARQ machine, once initial transmission

of the fragment has occured. If transmission or

retransmission of the fragment is not acknowledged by the

receiver before the time limit is reached, the fragment is

discarded. A value of 0 means Infinite."

::= { wmanIf2mSsArqAttributeEntry 4 }

wmanIf2mSsArqSyncLossTimeout OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535 )

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss of

synchronization of the sender and receiver state machines.

A value of 0 means Infinite."

::= { wmanIf2mSsArqAttributeEntry 5 }

wmanIf2mSsArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered by the

receiving MAC to its client application in the order in

which data was handed off to the originating MAC."

::= { wmanIf2mSsArqAttributeEntry 6 }

wmanIf2mSsArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means

Infinite."

::= { wmanIf2mSsArqAttributeEntry 7 }

wmanIf2mSsArqBlockSize OBJECT-TYPE

SYNTAX	    INTEGER (1..2040)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value of this parameter specifies the size of an ARQ

block. This parameter shall be established by negotiation

during the connection creation dialog."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2mSsArqAttributeEntry 8 }

wmanIf2mSsArqAckProcessingTime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 255)

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter indicates the number of ms required by the

ARQ receiver to process the received ARQ blocks and provide

a valid ACK or NAK."

REFERENCE

"Subclause 11.13.18.9 in IEEE Std 802.16e-2005"

::= { wmanIf2mSsArqAttributeEntry 9 }

END