| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | wmanIf2SsMib |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes wmanIf2mSsMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# wmanIf2SsMib

Joey Chou

Intel

# Introduction

This document proposes wmanIf2SsMib.

# Proposed ASN.1 Notations

WMAN-IF2-SS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF

ifIndex

FROM IF-MIB;

wmanIf2SsMib MODULE-IDENTITY

LAST-UPDATED    "200711130000Z" -- November 13, 2007

ORGANIZATION    "IEEE 802.16"

CONTACT-INFO

"WG E-mail: stds-802-16@ieee.org

WG Chair:  Roger B. Marks

Postal:    NextWave Broadband, Inc.

E-mail:    r.b.marks@ieee.org

TG Chair:  Phillip Barber

Postal:    Huawei Technologies Co., Ltd

E-mail:    pbarber@futurewei.com

Editor:    Joey Chou

Postal:    Intel Corporation

5000 W. Chandler Blvd,

Chandler, AZ 85227, USA

E-mail:    joey.chou@intel.com"

DESCRIPTION

"This material is from IEEE Std 802.16i

Copyright (c) 2007 IEEE.

This MIB Module defines managed objects for Subscriber

Station based on IEEE Std 802.16-2004 and its amendment

IEEE Std 802.16e-2005.

All objects with wmanIf2m prefix are designed for the

mobile Broadband Wireless Networks. Others are designed for

fixed Broadband Wireless Networks."

REVISION        "200711130000Z"

DESCRIPTION

"The first revision of WMAN-IF2-SS-MIB module"

::= { iso std(0) iso8802(8802) wman(16) 5 }

--

-- Textual Conventions

--

--

-- wmanIf2sSsConfigurationTable contains global parameters for SS

--

wmanIf2sSsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains one row for the SS system

parameters."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2SsMib 1 }

wmanIf2sSsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2sSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2sSsConfigurationTable 1 }

WmanIf2sSsConfigurationEntry ::= SEQUENCE {

wmanIf2sSsLostDLMapInterval             INTEGER,

wmanIf2sSsLostULMapInterval             INTEGER,

wmanIf2sSsContentionRangRetries         INTEGER,

wmanIf2sSsRequestRetries                INTEGER,

wmanIf2sSsRegRequestRetries             INTEGER,

wmanIf2sSsTftpBackoffStart              INTEGER,

wmanIf2sSsTftpBackoffEnd                INTEGER,

wmanIf2sSsTftpRequestRetries            INTEGER,

wmanIf2sSsTftpDownloadRetries           INTEGER,

wmanIf2sSsTftpWait                      INTEGER,

wmanIf2sSsToDRetries                    INTEGER,

wmanIf2sSsToDRetryPeriod                INTEGER,

wmanIf2sSsT1Timeout                     INTEGER,

wmanIf2sSsT2Timeout                     INTEGER,

wmanIf2sSsT3Timeout                     INTEGER,

wmanIf2sSsT4Timeout                     INTEGER,

wmanIf2sSsT6Timeout                     INTEGER,

wmanIf2sSsT12Timeout                    INTEGER,

wmanIf2sSsT14Timeout                    INTEGER,

wmanIf2sSsT18Timeout                    INTEGER,

wmanIf2sSsT19Timeout                    INTEGER,

wmanIf2sSsT20Timeout                    INTEGER,

wmanIf2sSsT21Timeout                    INTEGER,

wmanIf2sSsSBCRequestRetries             INTEGER,

wmanIf2sSsTftpCpltRetries               INTEGER,

wmanIf2sSsT26Timeout                    INTEGER,

wmanIf2sSsDLManagProcTime               INTEGER,

wmanIf2sSsPowerControlIeProcTime        INTEGER,

wmanIf2sSsT28Timeout                    INTEGER,

wmanIf2sSsT29Timeout                    INTEGER,

wmanIf2sSsT30Timeout                    INTEGER,

wmanIf2sSsSaChallengeTimer              INTEGER,

wmanIf2sSsSaChallengeMaxResends         INTEGER,

wmanIf2sSsSaTekTimer                    INTEGER,

wmanIf2sSsSaTekReqMaxResends            INTEGER,

wmanIf2sSsUlMapProcTime                 Unsigned32,

wmanIf2sSsRangRespProcTime              Unsigned32,

wmanIf2sSsInvitedRangRetries            INTEGER,

wmanIf2sSsDSxReqRetries                 Unsigned32,

wmanIf2sSsDSxRespRetries                Unsigned32,

wmanIf2sSsT7Timeout                     INTEGER,

wmanIf2sSsT8Timeout                     INTEGER,

wmanIf2sSsT10Timeout                    INTEGER,

wmanIf2sSsT22Timeout                    INTEGER}

wmanIf2sSsLostDLMapInterval OBJECT-TYPE

SYNTAX      INTEGER (0..600)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time since last received DL-MAP message before downlink

synchronization is considered lost in ms."

::= { wmanIf2sSsConfigurationEntry 1 }

wmanIf2sSsLostULMapInterval OBJECT-TYPE

SYNTAX      INTEGER (0..600)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time since last received UL-MAP message before uplink

synchronization is considered lost in ms."

::= { wmanIf2sSsConfigurationEntry 2 }

wmanIf2sSsContentionRangRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on contention Ranging Requests."

::= { wmanIf2sSsConfigurationEntry 3 }

wmanIf2sSsRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on bandwidth allocation requests."

::= { wmanIf2sSsConfigurationEntry 4 }

wmanIf2sSsRegRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on registration requests."

::= { wmanIf2sSsConfigurationEntry 5 }

wmanIf2sSsTftpBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial value for TFTP backoff in second."

::= { wmanIf2sSsConfigurationEntry 6 }

wmanIf2sSsTftpBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Last value for TFTP backoff in second."

::= { wmanIf2sSsConfigurationEntry 7 }

wmanIf2sSsTftpRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on TFTP request."

::= { wmanIf2sSsConfigurationEntry 8 }

wmanIf2sSsTftpDownloadRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on entire TFTP downloads."

::= { wmanIf2sSsConfigurationEntry 9 }

wmanIf2sSsTftpWait OBJECT-TYPE

SYNTAX      INTEGER (2..65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration between two consecutive Transfer

operational parameters (TFTP) retries in min."

::= { wmanIf2sSsConfigurationEntry 10 }

wmanIf2sSsToDRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Retries to establisg the Time of Day."

::= { wmanIf2sSsConfigurationEntry 11 }

wmanIf2sSsToDRetryPeriod OBJECT-TYPE

SYNTAX      INTEGER (5..65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The retry period to re-establisg the Time of Day, as

describe in the network entry procedure."

::= { wmanIf2sSsConfigurationEntry 12 }

wmanIf2sSsT1Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..50000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DCD timeout in ms."

::= { wmanIf2sSsConfigurationEntry 13 }

wmanIf2sSsT2Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for broadcast ranging timeout in ms."

::= { wmanIf2sSsConfigurationEntry 14 }

wmanIf2sSsT3Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Ranging Response reception timeout following the

transmission of a Ranging Request in ms."

DEFVAL	    {50}

::= { wmanIf2sSsConfigurationEntry 15 }

wmanIf2sSsT4Timeout OBJECT-TYPE

SYNTAX      INTEGER (1 .. 35)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for ranging opportunity or data grant. If the

pending until complete field was used earlier by this SS,

then the value of that field shall be added to this

interval in second."

::= { wmanIf2sSsConfigurationEntry 16 }

wmanIf2sSsT6Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..3000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for registration response in ms."

::= { wmanIf2sSsConfigurationEntry 17 }

wmanIf2sSsT12Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..50000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for UCD descriptor in ms."

::= { wmanIf2sSsConfigurationEntry 18 }

wmanIf2sSsT14Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSX-RVD Timeout in ms."

::= { wmanIf2sSsConfigurationEntry 19 }

wmanIf2sSsT18Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"wait for SBC-RSP timeout in ms."

::= { wmanIf2sSsConfigurationEntry 21 }

wmanIf2sSsT19Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..1048575)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time DL-channel remains unusable in ms."

::= { wmanIf2sSsConfigurationEntry 22 }

wmanIf2sSsT20Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time SS searches for preambles on a given channel in ms."

::= { wmanIf2sSsConfigurationEntry 23 }

wmanIf2sSsT21Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 11000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time SS searches for DL-MAP on a given channel in ms."

::= { wmanIf2sSsConfigurationEntry 24 }

wmanIf2sSsSBCRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (3..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on SBC Request."

::= { wmanIf2sSsConfigurationEntry 25 }

wmanIf2sSsTftpCpltRetries OBJECT-TYPE

SYNTAX      INTEGER (3..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on TFTP-CPLT."

::= { wmanIf2sSsConfigurationEntry 26 }

wmanIf2sSsT26Timeout OBJECT-TYPE

SYNTAX      INTEGER (10..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for TFTP-RSP in ms."

::= { wmanIf2sSsConfigurationEntry 27 }

wmanIf2sSsDLManagProcTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Max. time between reception of Fast Power Control

management message and compliance to its instructions

by SS in us."

::= { wmanIf2sSsConfigurationEntry 28 }

wmanIf2sSsPowerControlIeProcTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an SS following receipt of a UL-MAP

including a power control IE before it is expected to

apply the corrections instructed by the BS."

::= { wmanIf2sSsConfigurationEntry 29 }

wmanIf2sSsT28Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 60000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"DBPC-REQ re-try timer for requesting less robust burst

profile after rejection by the BS"

DEFVAL      { 1000 }

::= { wmanIf2sSsConfigurationEntry 30 }

wmanIf2sSsT29Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 30000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"RNG-REQ/DBPC-REQ re-try timer for requesting more robust

burst profile after rejecting by the BS"

DEFVAL      { 1000 }

::= { wmanIf2sSsConfigurationEntry 31 }

wmanIf2sSsT30Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"DBPC-RSP reception timeout following the transmission of

a DBPC-REQ."

DEFVAL      { 200 }

::= { wmanIf2sSsConfigurationEntry 32 }

wmanIf2sSsSaChallengeTimer OBJECT-TYPE

SYNTAX      INTEGER (500 .. 2000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Challenge."

DEFVAL      { 1000 }

::= { wmanIf2sSsConfigurationEntry 33 }

wmanIf2sSsSaChallengeMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Challenge."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 34 }

wmanIf2sSsSaTekTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Request."

DEFVAL      { 300 }

::= { wmanIf2sSsConfigurationEntry 35 }

wmanIf2sSsSaTekReqMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Request."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 36 }

wmanIf2sSsUlMapProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (200 .. 4294967295)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time provided between arrival of the last bit of a UL-MAP

at an SS and effectiveness of that map in us. For OFDMA

mode, the time shall be counted starting from the end of

the burst carrying the UL-MAP.

Minimum value: SC = 200us

OFDM = 1ms

OFDMA = frame duration"

::= { wmanIf2sSsConfigurationEntry 37 }

wmanIf2sSsRangRespProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an SS following receipt of a RNG-RSP

before it is expected to apply the corrections instructed

by the BS Minimum value."

::= { wmanIf2sSsConfigurationEntry 38 }

wmanIf2sSsInvitedRangRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on inviting Ranging Requests."

::= { wmanIf2sSsConfigurationEntry 39 }

wmanIf2sSsDSxReqRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Requests."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 40 }

wmanIf2sSsDSxRespRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Responses."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 41 }

wmanIf2sSsT7Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Response Timeout in ms."

::= { wmanIf2sSsConfigurationEntry 42 }

wmanIf2sSsT8Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 300)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Acknowledge Timeout in ms."

::= { wmanIf2sSsConfigurationEntry 43 }

wmanIf2sSsT10Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 3000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for Transaction End timeout in ms."

::= { wmanIf2sSsConfigurationEntry 44 }

wmanIf2sSsT22Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 500)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for ARQ Reset in ms."

::= { wmanIf2sSsConfigurationEntry 45 }

--

-- wmanIf2mSsConfigurationTable contains global parameters for SS

--

wmanIf2mSsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains one row for the SS system parameters."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2SsMib 2 }

wmanIf2mSsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2mSsConfigurationTable 1 }

WmanIf2mSsConfigurationEntry ::= SEQUENCE {

wmanIf2mSsMinSleepInterval              INTEGER,

wmanIf2mSsMaxSleepInterval              INTEGER,

wmanIf2mSsListeningInterval             INTEGER,

wmanIf2mSsNrbBsIndexValidityTime        INTEGER,

wmanIf2mSsAscAgingTimer                 INTEGER,

wmanIf2mSsServingBsidAgingTimer         INTEGER,

wmanIf2mSsT42Timer                      INTEGER,

wmanIf2mSsFastTrackingRspProcTime       INTEGER,

wmanIf2mSsModeSelectFeedbackProcTime    INTEGER,

wmanIf2mSsIdleModeTimer                 Unsigned32,

wmanIf2mSsT43Timer                      INTEGER,

wmanIf2mSsT44Timer                      INTEGER,

wmanIf2mSsT45Timer                      INTEGER,

wmanIf2mSsDregReqRetryCount             INTEGER,

wmanIf2mSsHoProcOptimizeMsTimerRetry    INTEGER,

wmanIf2mSsPagingInterval                INTEGER,

wmanIf2mSsMaxDirScanTime                INTEGER}

wmanIf2mSsMinSleepInterval OBJECT-TYPE

SYNTAX      INTEGER (2 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum sleeping time allowed to MS."

::= { wmanIf2mSsConfigurationEntry 1 }

wmanIf2mSsMaxSleepInterval OBJECT-TYPE

SYNTAX      INTEGER (2 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum sleeping time allowed to MS."

::= { wmanIf2mSsConfigurationEntry 2 }

wmanIf2mSsListeningInterval OBJECT-TYPE

SYNTAX      INTEGER (1 .. 64)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time duration during which the MS, after waking up and

synchronizing with the DL transmissions, can demodulate

downlink transmissions and decide whether to stay awake or

go back to sleep."

::= { wmanIf2mSsConfigurationEntry 3 }

wmanIf2mSsNrbBsIndexValidityTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 5)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time duration during which the MS can use the neighbor BS

list in MOB_NBR-ADV message for the compression of neighbor

BSIDs."

::= { wmanIf2mSsConfigurationEntry 4 }

wmanIf2mSsAscAgingTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time for aging of MS associations"

::= { wmanIf2mSsConfigurationEntry 5 }

wmanIf2mSsServingBsidAgingTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 5000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time for aging of serving BS association. Timer

recycles on successful serving BS DL-MAP read."

::= { wmanIf2mSsConfigurationEntry 6 }

wmanIf2mSsT42Timer OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MOB_HO-IND timeout when sent with HO_IND_type = 0b10."

::= { wmanIf2mSsConfigurationEntry 7 }

wmanIf2mSsFastTrackingRspProcTime OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an MS following receipt of a UL-MAP Fast

tracking indication response before it is expected to apply

the corrections instructed by the BS.

Default value = One DL subframe duration"

::= { wmanIf2mSsConfigurationEntry 8 }

wmanIf2mSsModeSelectFeedbackProcTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "microseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time allowed between the end of the burst carrying the

Mode Selection Feedback subheader and the start of the UL

subframe carrying the Mode Selection Feedback response.

Minimum value = 1 frame duration for TDD

1/2 Frame duration for FDD"

::= { wmanIf2mSsConfigurationEntry 9 }

wmanIf2mSsIdleModeTimer OBJECT-TYPE

SYNTAX      Unsigned32 (128 .. 65536)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MS timed interval to conduct Location Update. Set timer to

MS Idle Mode Timeout capabilities setting. Timer recycles

on successful Idle Mode Location Update."

DEFVAL      { 4096 }

::= { wmanIf2mSsConfigurationEntry 10 }

wmanIf2mSsT43Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for MOB_SLP-RSP."

::= { wmanIf2mSsConfigurationEntry 11 }

wmanIf2mSsT44Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for MOB_SCN-RSP."

::= { wmanIf2mSsConfigurationEntry 12 }

wmanIf2mSsT45Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 500)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for DREGCMD."

DEFVAL      { 250 }

::= { wmanIf2mSsConfigurationEntry 13 }

wmanIf2mSsDregReqRetryCount OBJECT-TYPE

SYNTAX      INTEGER (3 .. 16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on DREG Request Message."

DEFVAL      { 3 }

::= { wmanIf2mSsConfigurationEntry 14 }

wmanIf2mSsHoProcOptimizeMsTimerRetry OBJECT-TYPE

SYNTAX      INTEGER (3 .. 100)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of SBC-REQ and/or REG-REQ retries while waiting

for unsolicited SBC-RSP and/or REG-RSP as part of MS

network re-entry and as indicated by HO Process

Optimization message element of RNGRSP."

::= { wmanIf2mSsConfigurationEntry 15 }

wmanIf2mSsPagingInterval OBJECT-TYPE

SYNTAX      INTEGER (8 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time duration of Paging Interval of the BS."

DEFVAL      { 64 }

::= { wmanIf2mSsConfigurationEntry 16 }

wmanIf2mSsMaxDirScanTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum scanning time of neighbor BSs by MS before

reporting any results."

::= { wmanIf2mSsConfigurationEntry 17 }

--

-- Subscriber station wmanIf2SsMib contains the SS SNMP Trap objects

--

wmanIf2SsTrapControl OBJECT IDENTIFIER ::= { wmanIf2SsMib 3 }

wmanIf2SsTrapDefinitions OBJECT IDENTIFIER ::= { wmanIf2SsMib 4 }

-- This object groups all NOTIFICATION-TYPE objects for SS.

-- It is defined following RFC2758 sections 8.5 and 8.6

-- for the compatibility with SNMPv1.

wmanIf2SsTrapPrefix OBJECT IDENTIFIER ::= {wmanIf2SsTrapDefinitions 0}

wmanIf2SsTrapControlRegister    OBJECT-TYPE

SYNTAX      BITS {wmanIf2SsDhcpSuccess(0),

wmanIf2SsRssiStatusChange(1),

wmanIf2SsPkmSilentState(2)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The object is used to enable Subscriber Station traps.

From left to right, the set bit indicates the corresponding

Subscriber Station trap is enabled."

::= { wmanIf2SsTrapControl  1 }

wmanIf2SsThresholdConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2SsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains threshold objects that can be set to

detect the threshold crossing events."

::= { wmanIf2SsTrapControl 2 }

wmanIf2SsThresholdConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2SsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS."

INDEX     { ifIndex }

::= { wmanIf2SsThresholdConfigTable 1 }

WmanIf2SsThresholdConfigEntry ::= SEQUENCE {

wmanIf2SsRssiLowThreshold               Integer32,

wmanIf2SsRssiHighThreshold              Integer32}

wmanIf2SsRssiLowThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Low RSSI threshold for generating the RSSI alarm trap."

::= { wmanIf2SsThresholdConfigEntry 1 }

wmanIf2SsRssiHighThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"High RSSI threshold for generating a trap to indicate

the RSSI is restored."

::= { wmanIf2SsThresholdConfigEntry 2 }

wmanIf2SsDhcpSuccessTrap    NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2SsMacAddress}

STATUS      current

DESCRIPTION

"An event to report a successful Handshake to establish IP

connectivity."

::= { wmanIf2SsTrapPrefix 1 }

wmanIf2SsRssiStatusChangeTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2SsMacAddress,

wmanIf2SsRssiStatus,

wmanIf2SsRssiStatusInfo}

STATUS      current

DESCRIPTION

"An event to report that the downlink RSSI is below

wmanIf2SsRssiLowThreshold, or above

wmanIf2SsRssiHighThreshold after restore."

::= { wmanIf2SsTrapPrefix 2 }

wmanIf2SsPkmSilentStateTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2SsMacAddress,

wmanIf2SsSilentStateInfo}

STATUS      current

DESCRIPTION

"An event to report that SS PKM has entered into the

siltent state."

::= { wmanIf2SsTrapPrefix 3 }

wmanIf2SsNotificationObjectsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2SsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains SS notification objects that have been

reported by the trap."

::= { wmanIf2SsTrapDefinitions 1 }

wmanIf2SsNotificationObjectsEntry OBJECT-TYPE

SYNTAX      WmanIf2SsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has

generated traps."

INDEX     { ifIndex }

::= { wmanIf2SsNotificationObjectsTable 1 }

WmanIf2SsNotificationObjectsEntry ::= SEQUENCE {

wmanIf2SsMacAddress                     MacAddress,

wmanIf2SsRssiStatus                     INTEGER,

wmanIf2SsRssiStatusInfo                 OCTET STRING,

wmanIf2SsSilentStateInfo                OCTET STRING}

wmanIf2SsMacAddress  OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The MAC address of the SS generating the trap."

::= { wmanIf2SsNotificationObjectsEntry 1 }

wmanIf2SsRssiStatus  OBJECT-TYPE

SYNTAX      INTEGER {ssRssiAlarm(1),

ssRssiNoAlarm(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A RSSI alarm is generated if the RSSI is lower than

wmanIf2SsRssiLowThreshold, or above

wmanIf2SsRssiHighThreshold after alarm is restored."

::= { wmanIf2SsNotificationObjectsEntry 2 }

wmanIf2SsRssiStatusInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides additional information about RSSI

alarm. It is implementation specific."

::= { wmanIf2SsNotificationObjectsEntry 3 }

wmanIf2SsSilentStateInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides additional information about PKM

silent State. It is implementation specific."

::= { wmanIf2SsNotificationObjectsEntry 4 }

END