| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | wmanIf2SsMib |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes wmanIf2mSsMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# wmanIf2SsMib

Joey Chou

Intel

# Introduction

This document proposes wmanIf2SsMib.

# Proposed ASN.1 Notations

WMAN-IF2-SS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF

ifIndex

FROM IF-MIB;

wmanIf2SsMib MODULE-IDENTITY

LAST-UPDATED    "200711130000Z" -- November 13, 2007

ORGANIZATION    "IEEE 802.16"

CONTACT-INFO

"WG E-mail: stds-802-16@ieee.org

WG Chair:  Roger B. Marks

Postal:    NextWave Broadband, Inc.

E-mail:    r.b.marks@ieee.org

TG Chair:  Phillip Barber

Postal:    Huawei Technologies Co., Ltd

E-mail:    pbarber@huawei.com

Editor:    Joey Chou

Postal:    Intel Corporation

5000 W. Chandler Blvd,

Chandler, AZ 85227, USA

E-mail:    joey.chou@intel.com"

DESCRIPTION

"This material is from IEEE Std 802.16i

Copyright (c) 2007 IEEE.

This MIB Module defines managed objects for Subscriber

Station based on IEEE Std 802.16

.

All objects with wmanIf2m prefix are designed for the

mobile Broadband Wireless Networks. Others are designed for

fixed Broadband Wireless Networks."

REVISION        "200711130000Z"

DESCRIPTION

"The first revision of WMAN-IF2-SS-MIB module"

::= { iso std(0) iso8802(8802) wman(16) 5 }

--

-- Textual Conventions

--

--

-- wmanIf2SsConfigurationTable contains global parameters for SS

--

wmanIf2SsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2SsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains one row for the SS system

parameters."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2SsMib 1 }

wmanIf2SsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2SsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2SsConfigurationTable 1 }

WmanIf2SsConfigurationEntry ::= SEQUENCE {

wmanIf2SsLostDLMapInterval             INTEGER,

wmanIf2SsLostULMapInterval             INTEGER,

wmanIf2SsContentionRangRetries         INTEGER,

wmanIf2SsRequestRetries                INTEGER,

wmanIf2SsRegRequestRetries             INTEGER,

wmanIf2SsTftpBackoffStart              INTEGER,

wmanIf2SsTftpBackoffEnd                INTEGER,

wmanIf2SsTftpRequestRetries            INTEGER,

wmanIf2SsTftpDownloadRetries           INTEGER,

wmanIf2SsTftpWait                      INTEGER,

wmanIf2SsToDRetries                    INTEGER,

wmanIf2SsToDRetryPeriod                INTEGER,

wmanIf2SsT1Timeout                     INTEGER,

wmanIf2SsT2Timeout                     INTEGER,

wmanIf2SsT3Timeout                     INTEGER,

wmanIf2SsT4Timeout                     INTEGER,

wmanIf2SsT6Timeout                     INTEGER,

wmanIf2SsT12Timeout                    INTEGER,

wmanIf2SsT14Timeout                    INTEGER,

wmanIf2SsT18Timeout                    INTEGER,

wmanIf2SsT19Timeout                    INTEGER,

wmanIf2SsT20Timeout                    INTEGER,

wmanIf2SsT21Timeout                    INTEGER,

wmanIf2SsSBCRequestRetries             INTEGER,

wmanIf2SsTftpCpltRetries               INTEGER,

wmanIf2SsT26Timeout                    INTEGER,

wmanIf2SsDLManagProcTime               INTEGER,

wmanIf2SsPowerControlIeProcTime        INTEGER,

wmanIf2SsT28Timeout                    INTEGER,

wmanIf2SsT29Timeout                    INTEGER,

wmanIf2SsT30Timeout                    INTEGER,

wmanIf2SsSaChallengeTimer              INTEGER,

wmanIf2SsSaChallengeMaxResends         INTEGER,

wmanIf2SsSaTekTimer                    INTEGER,

wmanIf2SsSaTekReqMaxResends            INTEGER,

wmanIf2SsUlMapProcTime                 Unsigned32,

wmanIf2SsRangRespProcTime              Unsigned32,

wmanIf2SsInvitedRangRetries            INTEGER,

wmanIf2SsDSxReqRetries                 Unsigned32,

wmanIf2SsDSxRespRetries                Unsigned32,

wmanIf2SsT7Timeout                     INTEGER,

wmanIf2SsT8Timeout                     INTEGER,

wmanIf2SsT10Timeout                    INTEGER,

wmanIf2SsT22Timeout                    INTEGER}

wmanIf2SsLostDLMapInterval OBJECT-TYPE

SYNTAX      INTEGER (0..600)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time since last received DL-MAP message before downlink

synchronization is considered lost in ms."

::= { wmanIf2SsConfigurationEntry 1 }

wmanIf2SsLostULMapInterval OBJECT-TYPE

SYNTAX      INTEGER (0..600)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time since last received UL-MAP message before uplink

synchronization is considered lost in ms."

::= { wmanIf2SsConfigurationEntry 2 }

wmanIf2SsContentionRangRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on contention Ranging Requests."

::= { wmanIf2SsConfigurationEntry 3 }

wmanIf2SsRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on bandwidth allocation requests."

::= { wmanIf2SsConfigurationEntry 4 }

wmanIf2SsRegRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on registration requests."

::= { wmanIf2SsConfigurationEntry 5 }

wmanIf2SsTftpBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial value for TFTP backoff in second."

::= { wmanIf2SsConfigurationEntry 6 }

wmanIf2SsTftpBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Last value for TFTP backoff in second."

::= { wmanIf2SsConfigurationEntry 7 }

wmanIf2SsTftpRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on TFTP request."

::= { wmanIf2SsConfigurationEntry 8 }

wmanIf2SsTftpDownloadRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on entire TFTP downloads."

::= { wmanIf2SsConfigurationEntry 9 }

wmanIf2SsTftpWait OBJECT-TYPE

SYNTAX      INTEGER (2..65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration between two consecutive Transfer

operational parameters (TFTP) retries in min."

::= { wmanIf2SsConfigurationEntry 10 }

wmanIf2SsToDRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Retries to establisg the Time of Day."

::= { wmanIf2SsConfigurationEntry 11 }

wmanIf2SsToDRetryPeriod OBJECT-TYPE

SYNTAX      INTEGER (5..65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The retry period to re-establisg the Time of Day, as

describe in the network entry procedure."

::= { wmanIf2SsConfigurationEntry 12 }

wmanIf2SsT1Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..50000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DCD timeout in ms."

::= { wmanIf2SsConfigurationEntry 13 }

wmanIf2SsT2Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for broadcast ranging timeout in ms."

::= { wmanIf2SsConfigurationEntry 14 }

wmanIf2SsT3Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Ranging Response reception timeout following the

transmission of a Ranging Request in ms."

DEFVAL	    {50}

::= { wmanIf2SsConfigurationEntry 15 }

wmanIf2SsT4Timeout OBJECT-TYPE

SYNTAX      INTEGER (1 .. 35)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for ranging opportunity or data grant. If the

pending until complete field was used earlier by this SS,

then the value of that field shall be added to this

interval in second."

::= { wmanIf2SsConfigurationEntry 16 }

wmanIf2SsT6Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..3000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for registration response in ms."

::= { wmanIf2SsConfigurationEntry 17 }

wmanIf2SsT12Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..50000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for UCD descriptor in ms."

::= { wmanIf2SsConfigurationEntry 18 }

wmanIf2SsT14Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSX-RVD Timeout in ms."

::= { wmanIf2SsConfigurationEntry 19 }

wmanIf2SsT18Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"wait for SBC-RSP timeout in ms."

::= { wmanIf2SsConfigurationEntry 21 }

wmanIf2SsT19Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..1048575)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time DL-channel remains unusable in ms."

::= { wmanIf2SsConfigurationEntry 22 }

wmanIf2SsT20Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time SS searches for preambles on a given channel in ms."

::= { wmanIf2SsConfigurationEntry 23 }

wmanIf2SsT21Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 11000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time SS searches for DL-MAP on a given channel in ms."

::= { wmanIf2SsConfigurationEntry 24 }

wmanIf2SsSBCRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (3..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on SBC Request."

::= { wmanIf2SsConfigurationEntry 25 }

wmanIf2SsTftpCpltRetries OBJECT-TYPE

SYNTAX      INTEGER (3..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on TFTP-CPLT."

::= { wmanIf2SsConfigurationEntry 26 }

wmanIf2SsT26Timeout OBJECT-TYPE

SYNTAX      INTEGER (10..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for TFTP-RSP in ms."

::= { wmanIf2SsConfigurationEntry 27 }

wmanIf2SsDLManagProcTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Max. time between reception of Fast Power Control

management message and compliance to its instructions

by SS in us."

::= { wmanIf2SsConfigurationEntry 28 }

wmanIf2SsPowerControlIeProcTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an SS following receipt of a UL-MAP

including a power control IE before it is expected to

apply the corrections instructed by the BS."

::= { wmanIf2SsConfigurationEntry 29 }

wmanIf2SsT28Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 60000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"DBPC-REQ re-try timer for requesting less robust burst

profile after rejection by the BS"

DEFVAL      { 1000 }

::= { wmanIf2SsConfigurationEntry 30 }

wmanIf2SsT29Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 30000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"RNG-REQ/DBPC-REQ re-try timer for requesting more robust

burst profile after rejecting by the BS"

DEFVAL      { 1000 }

::= { wmanIf2SsConfigurationEntry 31 }

wmanIf2SsT30Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"DBPC-RSP reception timeout following the transmission of

a DBPC-REQ."

DEFVAL      { 200 }

::= { wmanIf2SsConfigurationEntry 32 }

wmanIf2SsSaChallengeTimer OBJECT-TYPE

SYNTAX      INTEGER (500 .. 2000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Challenge."

DEFVAL      { 1000 }

::= { wmanIf2SsConfigurationEntry 33 }

wmanIf2SsSaChallengeMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Challenge."

DEFVAL      { 3 }

::= { wmanIf2SsConfigurationEntry 34 }

wmanIf2SsSaTekTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Request."

DEFVAL      { 300 }

::= { wmanIf2SsConfigurationEntry 35 }

wmanIf2SsSaTekReqMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Request."

DEFVAL      { 3 }

::= { wmanIf2SsConfigurationEntry 36 }

wmanIf2SsUlMapProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (200 .. 4294967295)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time provided between arrival of the last bit of a UL-MAP

at an SS and effectiveness of that map in us. For OFDMA

mode, the time shall be counted starting from the end of

the burst carrying the UL-MAP.

Minimum value: SC = 200us

OFDM = 1ms

OFDMA = frame duration"

::= { wmanIf2SsConfigurationEntry 37 }

wmanIf2SsRangRespProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an SS following receipt of a RNG-RSP

before it is expected to apply the corrections instructed

by the BS Minimum value."

::= { wmanIf2SsConfigurationEntry 38 }

wmanIf2SsInvitedRangRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on inviting Ranging Requests."

::= { wmanIf2SsConfigurationEntry 39 }

wmanIf2SsDSxReqRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Requests."

DEFVAL      { 3 }

::= { wmanIf2SsConfigurationEntry 40 }

wmanIf2SsDSxRespRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Responses."

DEFVAL      { 3 }

::= { wmanIf2SsConfigurationEntry 41 }

wmanIf2SsT7Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Response Timeout in ms."

::= { wmanIf2SsConfigurationEntry 42 }

wmanIf2SsT8Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 300)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Acknowledge Timeout in ms."

::= { wmanIf2SsConfigurationEntry 43 }

wmanIf2SsT10Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 3000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for Transaction End timeout in ms."

::= { wmanIf2SsConfigurationEntry 44 }

wmanIf2SsT22Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 500)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for ARQ Reset in ms."

::= { wmanIf2SsConfigurationEntry 45 }

--

-- Subscriber station wmanIf2SsMib contains the SS SNMP Trap objects

--

wmanIf2SsTrapControl OBJECT IDENTIFIER ::= { wmanIf2SsMib 2 }

wmanIf2SsTrapDefinitions OBJECT IDENTIFIER ::= { wmanIf2SsMib 3 }

-- This object groups all NOTIFICATION-TYPE objects for SS.

-- It is defined following RFC2758 sections 8.5 and 8.6

-- for the compatibility with SNMPv1.

wmanIf2SsTrapPrefix OBJECT IDENTIFIER ::= {wmanIf2SsTrapDefinitions 0}

wmanIf2SsTrapControlRegister    OBJECT-TYPE

SYNTAX      BITS {wmanIf2SsDhcpSuccess(0),

wmanIf2SsRssiStatusChange(1),

wmanIf2SsPkmSilentState(2)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The object is used to enable Subscriber Station traps.

From left to right, the set bit indicates the corresponding

Subscriber Station trap is enabled."

::= { wmanIf2SsTrapControl  1 }

wmanIf2SsThresholdConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2SsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains threshold objects that can be set to

detect the threshold crossing events."

::= { wmanIf2SsTrapControl 2 }

wmanIf2SsThresholdConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2SsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS."

INDEX     { ifIndex }

::= { wmanIf2SsThresholdConfigTable 1 }

WmanIf2SsThresholdConfigEntry ::= SEQUENCE {

wmanIf2SsRssiLowThreshold               Integer32,

wmanIf2SsRssiHighThreshold              Integer32}

wmanIf2SsRssiLowThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Low RSSI threshold for generating the RSSI alarm trap."

::= { wmanIf2SsThresholdConfigEntry 1 }

wmanIf2SsRssiHighThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"High RSSI threshold for generating a trap to indicate

the RSSI is restored."

::= { wmanIf2SsThresholdConfigEntry 2 }

wmanIf2SsDhcpSuccessTrap    NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2SsMacAddress}

STATUS      current

DESCRIPTION

"An event to report a successful Handshake to establish IP

connectivity."

::= { wmanIf2SsTrapPrefix 1 }

wmanIf2SsRssiStatusChangeTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2SsMacAddress,

wmanIf2SsRssiStatus,

wmanIf2SsRssiStatusInfo}

STATUS      current

DESCRIPTION

"An event to report that the downlink RSSI is below

wmanIf2SsRssiLowThreshold, or above

wmanIf2SsRssiHighThreshold after restore."

::= { wmanIf2SsTrapPrefix 2 }

wmanIf2SsPkmSilentStateTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2SsMacAddress,

wmanIf2SsilentStateInfo}

STATUS      current

DESCRIPTION

"An event to report that SS PKM has entered into the

siltent state."

::= { wmanIf2SsTrapPrefix 3 }

wmanIf2SsNotificationObjectsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2SsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains SS notification objects that have been

reported by the trap."

::= { wmanIf2SsTrapDefinitions 1 }

wmanIf2SsNotificationObjectsEntry OBJECT-TYPE

SYNTAX      WmanIf2SsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has

generated traps."

INDEX     { ifIndex }

::= { wmanIf2SsNotificationObjectsTable 1 }

WmanIf2SsNotificationObjectsEntry ::= SEQUENCE {

wmanIf2SsMacAddress                     MacAddress,

wmanIf2SsRssiStatus                     INTEGER,

wmanIf2SsRssiStatusInfo                 OCTET STRING,

wmanIf2SsilentStateInfo                OCTET STRING}

wmanIf2SsMacAddress  OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The MAC address of the SS generating the trap."

::= { wmanIf2SsNotificationObjectsEntry 1 }

wmanIf2SsRssiStatus  OBJECT-TYPE

SYNTAX      INTEGER {ssRssiAlarm(1),

ssRssiNoAlarm(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A RSSI alarm is generated if the RSSI is lower than

wmanIf2SsRssiLowThreshold, or above

wmanIf2SsRssiHighThreshold after alarm is restored."

::= { wmanIf2SsNotificationObjectsEntry 2 }

wmanIf2SsRssiStatusInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides additional information about RSSI

alarm. It is implementation specific."

::= { wmanIf2SsNotificationObjectsEntry 3 }

wmanIf2SsilentStateInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides additional information about PKM

silent State. It is implementation specific."

::= { wmanIf2SsNotificationObjectsEntry 4 }

--

-- wmanIf2mSsConfigurationTable contains global parameters for SS

--

wmanIf2mSsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2mSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains one row for the SS system parameters."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2SsMib 4 }

wmanIf2mSsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2mSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2mSsConfigurationTable 1 }

WmanIf2mSsConfigurationEntry ::= SEQUENCE {

wmanIf2mSsMinSleepInterval              INTEGER,

wmanIf2mSsMaxSleepInterval              INTEGER,

wmanIf2mSsListeningInterval             INTEGER,

wmanIf2mSsNrbBsIndexValidityTime        INTEGER,

wmanIf2mSsAscAgingTimer                 INTEGER,

wmanIf2mSsServingBsidAgingTimer         INTEGER,

wmanIf2mSsT42Timer                      INTEGER,

wmanIf2mSsFastTrackingRspProcTime       INTEGER,

wmanIf2mSsModeSelectFeedbackProcTime    INTEGER,

wmanIf2mSsIdleModeTimer                 Unsigned32,

wmanIf2mSsT43Timer                      INTEGER,

wmanIf2mSsT44Timer                      INTEGER,

wmanIf2mSsT45Timer                      INTEGER,

wmanIf2mSsDregReqRetryCount             INTEGER,

wmanIf2mSsHoProcOptimizeMsTimerRetry    INTEGER,

wmanIf2mSsPagingInterval                INTEGER,

wmanIf2mSsMaxDirScanTime                INTEGER}

wmanIf2mSsMinSleepInterval OBJECT-TYPE

SYNTAX      INTEGER (2 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Minimum sleeping time allowed to MS."

::= { wmanIf2mSsConfigurationEntry 1 }

wmanIf2mSsMaxSleepInterval OBJECT-TYPE

SYNTAX      INTEGER (2 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum sleeping time allowed to MS."

::= { wmanIf2mSsConfigurationEntry 2 }

wmanIf2mSsListeningInterval OBJECT-TYPE

SYNTAX      INTEGER (1 .. 64)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time duration during which the MS, after waking up and

synchronizing with the DL transmissions, can demodulate

downlink transmissions and decide whether to stay awake or

go back to sleep."

::= { wmanIf2mSsConfigurationEntry 3 }

wmanIf2mSsNrbBsIndexValidityTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 5)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time duration during which the MS can use the neighbor BS

list in MOB_NBR-ADV message for the compression of neighbor

BSIDs."

::= { wmanIf2mSsConfigurationEntry 4 }

wmanIf2mSsAscAgingTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time for aging of MS associations"

::= { wmanIf2mSsConfigurationEntry 5 }

wmanIf2mSsServingBsidAgingTimer OBJECT-TYPE

SYNTAX      INTEGER (0 .. 5000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Nominal time for aging of serving BS association. Timer

recycles on successful serving BS DL-MAP read."

::= { wmanIf2mSsConfigurationEntry 6 }

wmanIf2mSsT42Timer OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MOB_HO-IND timeout when sent with HO_IND_type = 0b10."

::= { wmanIf2mSsConfigurationEntry 7 }

wmanIf2mSsFastTrackingRspProcTime OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an MS following receipt of a UL-MAP Fast

tracking indication response before it is expected to apply

the corrections instructed by the BS.

Default value = One DL subframe duration"

::= { wmanIf2mSsConfigurationEntry 8 }

wmanIf2mSsModeSelectFeedbackProcTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "microseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The time allowed between the end of the burst carrying the

Mode Selection Feedback subheader and the start of the UL

subframe carrying the Mode Selection Feedback response.

Minimum value = 1 frame duration for TDD

1/2 Frame duration for FDD"

::= { wmanIf2mSsConfigurationEntry 9 }

wmanIf2mSsIdleModeTimer OBJECT-TYPE

SYNTAX      Unsigned32 (128 .. 65536)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"MS timed interval to conduct Location Update. Set timer to

MS Idle Mode Timeout capabilities setting. Timer recycles

on successful Idle Mode Location Update."

DEFVAL      { 4096 }

::= { wmanIf2mSsConfigurationEntry 10 }

wmanIf2mSsT43Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for MOB_SLP-RSP."

::= { wmanIf2mSsConfigurationEntry 11 }

wmanIf2mSsT44Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for MOB_SCN-RSP."

::= { wmanIf2mSsConfigurationEntry 12 }

wmanIf2mSsT45Timer OBJECT-TYPE

SYNTAX      INTEGER (1 .. 500)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time the MS waits for DREGCMD."

DEFVAL      { 250 }

::= { wmanIf2mSsConfigurationEntry 13 }

wmanIf2mSsDregReqRetryCount OBJECT-TYPE

SYNTAX      INTEGER (3 .. 16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on DREG Request Message."

DEFVAL      { 3 }

::= { wmanIf2mSsConfigurationEntry 14 }

wmanIf2mSsHoProcOptimizeMsTimerRetry OBJECT-TYPE

SYNTAX      INTEGER (3 .. 100)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of SBC-REQ and/or REG-REQ retries while waiting

for unsolicited SBC-RSP and/or REG-RSP as part of MS

network re-entry and as indicated by HO Process

Optimization message element of RNGRSP."

::= { wmanIf2mSsConfigurationEntry 15 }

wmanIf2mSsPagingInterval OBJECT-TYPE

SYNTAX      INTEGER (8 .. 1024)

UNITS       "frames"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time duration of Paging Interval of the BS."

DEFVAL      { 64 }

::= { wmanIf2mSsConfigurationEntry 16 }

wmanIf2mSsMaxDirScanTime OBJECT-TYPE

SYNTAX      INTEGER (1 .. 65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum scanning time of neighbor BSs by MS before

reporting any results."

::= { wmanIf2mSsConfigurationEntry 17 }

END