| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | wmanIf2sSsMib |  |
| Date Submitted | 2007-11-11 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes wmanIf2sSsMib. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# wmanIf2sSsMib

Joey Chou

Intel

# Introduction

This document proposes wmanIf2sSsMib.

# Proposed ASN.1 Notations

WMAN-IF2S-SS-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF

ifIndex

FROM IF-MIB;

wmanIf2sSsMib MODULE-IDENTITY

LAST-UPDATED    "200711090000Z" -- November 09, 2007

ORGANIZATION    "IEEE 802.16"

CONTACT-INFO

"WG E-mail: stds-802-16@ieee.org

WG Chair:  Roger B. Marks

Postal:    NextWave Broadband, Inc.

E-mail:    r.b.marks@ieee.org

TG Chair:  Phillip Barber

Postal:    Huawei Technologies Co., Ltd

E-mail:    pbarber@futurewei.com

Editor:    Joey Chou

Postal:    Intel Corporation

5000 W. Chandler Blvd,

Chandler, AZ 85227, USA

E-mail:    joey.chou@intel.com"

DESCRIPTION

"This material is from IEEE Std 802.16i

Copyright (c) 2007 IEEE.

This MIB Module defines managed objects for Subscriber

Station based on IEEE Std 802.16-2004 and its amendment

IEEE Std 802.16e-2005. The MIB contains managed objects

that are specific to fixed Broadband Wireless Networks."

REVISION        "200711090000Z"

DESCRIPTION

"TChange to wmanIf2sSsMib."

REVISION        "200710020000Z"

DESCRIPTION

"The first revision of WMAN-IF2S-MIB module that supports

direct service flow provisioning."

::= { iso std(0) iso8802(8802) wman(16) 6 }

-- Textual Conventions

WmanIf2sSfState ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the state of a service flow."

SYNTAX      INTEGER {authorized(1),

admitted(2),

active(3)}

WmanIf2sCsType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the types of convergence sublayer."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16e-2005"

SYNTAX      INTEGER {reserved(0),

packetIpV4(1),

packetIpV6(2),

packet802dot3Ethernet(3),

packet802dot1QVlan(4),

packetIpV4Over802dot3(5),

packetIpV6Over802dot3(6),

packetIpV4Over802dot1Q(7),

packetIpV6Over802dot1Q(8),

atm(9),

packet802dot3EthernetRohcHc(10),

packet802dot3EthernetEcrtpHc(11),

packetIp2RohcHc(12),

packetIp2EcrtpHc(13)}

WmanIf2sServClassName ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Defines the type of service class name."

SYNTAX      OCTET STRING (SIZE(2..128))

WmanIf2sSchedulingType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The scheduling service provided by a SC for an upstream

service flow. If the parameter is omitted from an upstream

QOS Parameter Set, this object takes the value of

bestEffort (2). This parameter must be reported as

undefined (1) for downstream QOS Parameter Sets."

SYNTAX      INTEGER {undefined(1),

bestEffort(2),

nonRealTimePollingService(3),

realTimePollingService(4),

extRealTimePollingService(5),

unsolicitedGrantService(6)}

WmanIf2sPhsRuleVerify ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression. If PHSV is

enabled, the sender shall compare the bytes in the packet

header with the bytes in the PHSF that are to be

suppressed as indicated by the PHSM."

REFERENCE

"Subclause 11.13.19.3.7.5 in IEEE Std 802.16-2004"

SYNTAX      INTEGER {phsVerifyEnable(0),

phsVerifyDisable(1)}

WmanIf2sClassifierMap ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"A bit of of this object is set to 1 if the parameter

indicated by the comment was present in the classifier

encoding, and 0 otherwise.

Note: that BITS are encoded most significant bit first,

so that if e.g. bits 6 and 7 are set, this object is

encoded as the octet string '030000'H."

REFERENCE

"Subclause 11.13.19.3.4 in IEEE Std 802.16e-2005"

SYNTAX      BITS {priority(0),

ipTos(1),

ipProtocol(2),

ipMaskedSrcAddr(3),

ipMaskedDestAddr(4),

srcPort(5),

destPort(6),

destMacAddr(7),

srcMacAddr(8),

ethernetProtocol(9),

userPriority(10),

vlanId(11),

ipv6FlowLabel(12),

actionRule(13)}

WmanIf2sIpv6FlowLabel ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field. As the flow label field has a

length of 20 bits, the first 4 bits of the most significant

byte shall be set to 0x0 and disregarded."

SYNTAX      OCTET STRING (SIZE(3))

WmanIf2sActionRule ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule

Bit #0: 0 = none.

1 = Discard packet"

REFERENCE

"Subclause 11.13.19.3.4.17 in IEEE Std 802.16e-2005"

SYNTAX      BITS {discardPacket(0)}

WmanIf2sCidType ::= TEXTUAL-CONVENTION

STATUS      current

DESCRIPTION

"Type of CID."

SYNTAX      INTEGER (0 .. 65535)

--

-- wmanIf2sSsConfigurationTable contains global parameters for SS

--

wmanIf2sSsConfigurationTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains one row for the SS system

parameters."

REFERENCE

"Subclause 10.1, Table 342 in IEEE Std 802.16e-2005"

::= { wmanIf2sSsMib 1 }

wmanIf2sSsConfigurationEntry OBJECT-TYPE

SYNTAX      WmanIf2sSsConfigurationEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

""

INDEX     { ifIndex }

::= { wmanIf2sSsConfigurationTable 1 }

WmanIf2sSsConfigurationEntry ::= SEQUENCE {

wmanIf2sSsLostDLMapInterval             INTEGER,

wmanIf2sSsLostULMapInterval             INTEGER,

wmanIf2sSsContentionRangRetries         INTEGER,

wmanIf2sSsRequestRetries                INTEGER,

wmanIf2sSsRegRequestRetries             INTEGER,

wmanIf2sSsTftpBackoffStart              INTEGER,

wmanIf2sSsTftpBackoffEnd                INTEGER,

wmanIf2sSsTftpRequestRetries            INTEGER,

wmanIf2sSsTftpDownloadRetries           INTEGER,

wmanIf2sSsTftpWait                      INTEGER,

wmanIf2sSsToDRetries                    INTEGER,

wmanIf2sSsToDRetryPeriod                INTEGER,

wmanIf2sSsT1Timeout                     INTEGER,

wmanIf2sSsT2Timeout                     INTEGER,

wmanIf2sSsT3Timeout                     INTEGER,

wmanIf2sSsT4Timeout                     INTEGER,

wmanIf2sSsT6Timeout                     INTEGER,

wmanIf2sSsT12Timeout                    INTEGER,

wmanIf2sSsT14Timeout                    INTEGER,

wmanIf2sSsT18Timeout                    INTEGER,

wmanIf2sSsT19Timeout                    INTEGER,

wmanIf2sSsT20Timeout                    INTEGER,

wmanIf2sSsT21Timeout                    INTEGER,

wmanIf2sSsSBCRequestRetries             INTEGER,

wmanIf2sSsTftpCpltRetries               INTEGER,

wmanIf2sSsT26Timeout                    INTEGER,

wmanIf2sSsDLManagProcTime               INTEGER,

wmanIf2sSsPowerControlIeProcTime        INTEGER,

wmanIf2sSsT28Timeout                    INTEGER,

wmanIf2sSsT29Timeout                    INTEGER,

wmanIf2sSsT30Timeout                    INTEGER,

wmanIf2sSsSaChallengeTimer              INTEGER,

wmanIf2sSsSaChallengeMaxResends         INTEGER,

wmanIf2sSsSaTekTimer                    INTEGER,

wmanIf2sSsSaTekReqMaxResends            INTEGER,

wmanIf2sSsUlMapProcTime                 Unsigned32,

wmanIf2sSsRangRespProcTime              Unsigned32,

wmanIf2sSsInvitedRangRetries            INTEGER,

wmanIf2sSsDSxReqRetries                 Unsigned32,

wmanIf2sSsDSxRespRetries                Unsigned32,

wmanIf2sSsT7Timeout                     INTEGER,

wmanIf2sSsT8Timeout                     INTEGER,

wmanIf2sSsT10Timeout                    INTEGER,

wmanIf2sSsT22Timeout                    INTEGER}

wmanIf2sSsLostDLMapInterval OBJECT-TYPE

SYNTAX      INTEGER (0..600)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time since last received DL-MAP message before downlink

synchronization is considered lost in ms."

::= { wmanIf2sSsConfigurationEntry 1 }

wmanIf2sSsLostULMapInterval OBJECT-TYPE

SYNTAX      INTEGER (0..600)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time since last received UL-MAP message before uplink

synchronization is considered lost in ms."

::= { wmanIf2sSsConfigurationEntry 2 }

wmanIf2sSsContentionRangRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on contention Ranging Requests."

::= { wmanIf2sSsConfigurationEntry 3 }

wmanIf2sSsRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on bandwidth allocation requests."

::= { wmanIf2sSsConfigurationEntry 4 }

wmanIf2sSsRegRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on registration requests."

::= { wmanIf2sSsConfigurationEntry 5 }

wmanIf2sSsTftpBackoffStart OBJECT-TYPE

SYNTAX      INTEGER (1..65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Initial value for TFTP backoff in second."

::= { wmanIf2sSsConfigurationEntry 6 }

wmanIf2sSsTftpBackoffEnd OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Last value for TFTP backoff in second."

::= { wmanIf2sSsConfigurationEntry 7 }

wmanIf2sSsTftpRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on TFTP request."

::= { wmanIf2sSsConfigurationEntry 8 }

wmanIf2sSsTftpDownloadRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on entire TFTP downloads."

::= { wmanIf2sSsConfigurationEntry 9 }

wmanIf2sSsTftpWait OBJECT-TYPE

SYNTAX      INTEGER (2..65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The duration between two consecutive Transfer

operational parameters (TFTP) retries in min."

::= { wmanIf2sSsConfigurationEntry 10 }

wmanIf2sSsToDRetries OBJECT-TYPE

SYNTAX      INTEGER (3..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Retries to establisg the Time of Day."

::= { wmanIf2sSsConfigurationEntry 11 }

wmanIf2sSsToDRetryPeriod OBJECT-TYPE

SYNTAX      INTEGER (5..65535)

UNITS       "minutes"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The retry period to re-establisg the Time of Day, as

describe in the network entry procedure."

::= { wmanIf2sSsConfigurationEntry 12 }

wmanIf2sSsT1Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..50000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DCD timeout in ms."

::= { wmanIf2sSsConfigurationEntry 13 }

wmanIf2sSsT2Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..10000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for broadcast ranging timeout in ms."

::= { wmanIf2sSsConfigurationEntry 14 }

wmanIf2sSsT3Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Ranging Response reception timeout following the

transmission of a Ranging Request in ms."

DEFVAL	    {50}

::= { wmanIf2sSsConfigurationEntry 15 }

wmanIf2sSsT4Timeout OBJECT-TYPE

SYNTAX      INTEGER (1 .. 35)

UNITS       "seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for ranging opportunity or data grant. If the

pending until complete field was used earlier by this SS,

then the value of that field shall be added to this

interval in second."

::= { wmanIf2sSsConfigurationEntry 16 }

wmanIf2sSsT6Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..3000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for registration response in ms."

::= { wmanIf2sSsConfigurationEntry 17 }

wmanIf2sSsT12Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..50000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for UCD descriptor in ms."

::= { wmanIf2sSsConfigurationEntry 18 }

wmanIf2sSsT14Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSX-RVD Timeout in ms."

::= { wmanIf2sSsConfigurationEntry 19 }

wmanIf2sSsT18Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"wait for SBC-RSP timeout in ms."

::= { wmanIf2sSsConfigurationEntry 21 }

wmanIf2sSsT19Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..1048575)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time DL-channel remains unusable in ms."

::= { wmanIf2sSsConfigurationEntry 22 }

wmanIf2sSsT20Timeout OBJECT-TYPE

SYNTAX      INTEGER (0..65535)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time SS searches for preambles on a given channel in ms."

::= { wmanIf2sSsConfigurationEntry 23 }

wmanIf2sSsT21Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 11000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time SS searches for DL-MAP on a given channel in ms."

::= { wmanIf2sSsConfigurationEntry 24 }

wmanIf2sSsSBCRequestRetries OBJECT-TYPE

SYNTAX      INTEGER (3..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on SBC Request."

::= { wmanIf2sSsConfigurationEntry 25 }

wmanIf2sSsTftpCpltRetries OBJECT-TYPE

SYNTAX      INTEGER (3..16)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on TFTP-CPLT."

::= { wmanIf2sSsConfigurationEntry 26 }

wmanIf2sSsT26Timeout OBJECT-TYPE

SYNTAX      INTEGER (10..200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for TFTP-RSP in ms."

::= { wmanIf2sSsConfigurationEntry 27 }

wmanIf2sSsDLManagProcTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Max. time between reception of Fast Power Control

management message and compliance to its instructions

by SS in us."

::= { wmanIf2sSsConfigurationEntry 28 }

wmanIf2sSsPowerControlIeProcTime OBJECT-TYPE

SYNTAX      INTEGER (0 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an SS following receipt of a UL-MAP

including a power control IE before it is expected to

apply the corrections instructed by the BS."

::= { wmanIf2sSsConfigurationEntry 29 }

wmanIf2sSsT28Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 60000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"DBPC-REQ re-try timer for requesting less robust burst

profile after rejection by the BS"

DEFVAL      { 1000 }

::= { wmanIf2sSsConfigurationEntry 30 }

wmanIf2sSsT29Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 30000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"RNG-REQ/DBPC-REQ re-try timer for requesting more robust

burst profile after rejecting by the BS"

DEFVAL      { 1000 }

::= { wmanIf2sSsConfigurationEntry 31 }

wmanIf2sSsT30Timeout OBJECT-TYPE

SYNTAX      INTEGER (200 .. 200)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"DBPC-RSP reception timeout following the transmission of

a DBPC-REQ."

DEFVAL      { 200 }

::= { wmanIf2sSsConfigurationEntry 32 }

wmanIf2sSsSaChallengeTimer OBJECT-TYPE

SYNTAX      INTEGER (500 .. 2000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Challenge."

DEFVAL      { 1000 }

::= { wmanIf2sSsConfigurationEntry 33 }

wmanIf2sSsSaChallengeMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Challenge."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 34 }

wmanIf2sSsSaTekTimer OBJECT-TYPE

SYNTAX      INTEGER (100 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time prior to re-send of SATEK-Request."

DEFVAL      { 300 }

::= { wmanIf2sSsConfigurationEntry 35 }

wmanIf2sSsSaTekReqMaxResends OBJECT-TYPE

SYNTAX      INTEGER (1 .. 3)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of transmissions of SA-TEK-Request."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 36 }

wmanIf2sSsUlMapProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (200 .. 4294967295)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time provided between arrival of the last bit of a UL-MAP

at an SS and effectiveness of that map in us. For OFDMA

mode, the time shall be counted starting from the end of

the burst carrying the UL-MAP.

Minimum value: SC = 200us

OFDM = 1ms

OFDMA = frame duration"

::= { wmanIf2sSsConfigurationEntry 37 }

wmanIf2sSsRangRespProcTime OBJECT-TYPE

SYNTAX      Unsigned32 (1 .. 2500)

UNITS       "micro seconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Time allowed for an SS following receipt of a RNG-RSP

before it is expected to apply the corrections instructed

by the BS Minimum value."

::= { wmanIf2sSsConfigurationEntry 38 }

wmanIf2sSsInvitedRangRetries OBJECT-TYPE

SYNTAX      INTEGER (16..65535)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of retries on inviting Ranging Requests."

::= { wmanIf2sSsConfigurationEntry 39 }

wmanIf2sSsDSxReqRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Requests."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 40 }

wmanIf2sSsDSxRespRetries OBJECT-TYPE

SYNTAX      Unsigned32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Number of Timeout Retries on DSA/DSC/DSD Responses."

DEFVAL      { 3 }

::= { wmanIf2sSsConfigurationEntry 41 }

wmanIf2sSsT7Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 1000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Response Timeout in ms."

::= { wmanIf2sSsConfigurationEntry 42 }

wmanIf2sSsT8Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 300)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for DSA/DSC/DSD Acknowledge Timeout in ms."

::= { wmanIf2sSsConfigurationEntry 43 }

wmanIf2sSsT10Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 3000)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for Transaction End timeout in ms."

::= { wmanIf2sSsConfigurationEntry 44 }

wmanIf2sSsT22Timeout OBJECT-TYPE

SYNTAX      INTEGER (0 .. 500)

UNITS       "milliseconds"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Wait for ARQ Reset in ms."

::= { wmanIf2sSsConfigurationEntry 45 }

wmanIf2sSsServiceFlowTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sSsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains Service Flow managed objects that

are common in BS and SS."

::= { wmanIf2sSsMib 2 }

wmanIf2sSsServiceFlowEntry OBJECT-TYPE

SYNTAX      WmanIf2sSsServiceFlowEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each created service

flow for a given MacAddress."

INDEX     { ifIndex,

wmanIf2sSsSfMacAddress,

wmanIf2sSsSfId }

::= { wmanIf2sSsServiceFlowTable 1 }

WmanIf2sSsServiceFlowEntry::= SEQUENCE {

wmanIf2sSsSfMacAddress                  MacAddress,

wmanIf2sSsSfId                          Unsigned32,

wmanIf2sSsSfCid                         WmanIf2sCidType,

wmanIf2sSsSfDirection                   INTEGER,

wmanIf2sSsSfState                       WmanIf2sSfState,

wmanIf2sSsTrafficPriority               INTEGER,

wmanIf2sSsMaxSustainedRate              Unsigned32,

wmanIf2sSsMaxTrafficBurst               Unsigned32,

wmanIf2sSsMinReservedRate               Unsigned32,

wmanIf2sSsToleratedJitter               Unsigned32,

wmanIf2sSsMaxLatency                    Unsigned32,

wmanIf2sSsFixedVsVariableSduInd         INTEGER,

wmanIf2sSsSduSize                       Unsigned32,

wmanIf2sSsSfSchedulingType              WmanIf2sSchedulingType,

wmanIf2sSsArqEnable                     TruthValue,

wmanIf2sSsArqWindowSize                 INTEGER,

wmanIf2sSsArqBlockLifetime              INTEGER,

wmanIf2sSsArqSyncLossTimeout            INTEGER,

wmanIf2sSsArqDeliverInOrder             TruthValue,

wmanIf2sSsArqRxPurgeTimeout             INTEGER,

wmanIf2sSsArqBlockSize                  INTEGER,

wmanIf2sSsMinRsvdTolerableRate          Unsigned32,

wmanIf2sSsReqTxPolicy                   BITS,

wmanIf2sSsSfCsSpecification             WmanIf2sCsType,

wmanIf2sSsTargetSaid                    INTEGER}

wmanIf2sSsSfMacAddress OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"When this table is implemented on the basestation, this

object contains the SS Mac address, the reported service

flow was created for. On the SS, the value returned is

the SS's own Mac address."

::= { wmanIf2sSsServiceFlowEntry 1 }

wmanIf2sSsSfId OBJECT-TYPE

SYNTAX      Unsigned32 ( 1 .. 4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A 32 bit quantity that uniquely identifies a service flow

to both the subscriber station and base station (BS)."

::= { wmanIf2sSsServiceFlowEntry 2 }

wmanIf2sSsSfCid OBJECT-TYPE

SYNTAX      WmanIf2sCidType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A 16 bit channel identifier to identify the connection

being created by DSA."

::= { wmanIf2sSsServiceFlowEntry 3 }

wmanIf2sSsSfDirection OBJECT-TYPE

SYNTAX      INTEGER {downstream(1),

upstream(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An attribute indicating the service flow is downstream or

upstream."

::= { wmanIf2sSsServiceFlowEntry 4 }

wmanIf2sSsSfState OBJECT-TYPE

SYNTAX      WmanIf2sSfState

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"wmanIf2sSsSfState indicates the service flow state:

Authorized (1), Admitted (2), and Active (3) service

flow state."

REFERENCE

"Subclause 6.3.14.6, in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 5 }

wmanIf2sSsTrafficPriority OBJECT-TYPE

SYNTAX      INTEGER (0 .. 7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the priority

assigned to a service flow. For uplink service flows,

the BS should use this parameter when determining

precedence in request service and grant generation,

and the SS shall preferentially select contention

Request opportunities for Priority Request CIDs

based on this priority"

REFERENCE

"Subclause 11.13.5 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 6 }

wmanIf2sSsMaxSustainedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the peak information rate

of the service. The rate is expressed in bits per

second and pertains to the SDUs at the input to

the system."

REFERENCE

"Subclause 11.13.6 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 7 }

wmanIf2sSsMaxTrafficBurst OBJECT-TYPE

SYNTAX      Unsigned32

UNITS	    "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the maximum burst size that

must be accommodated for the service."

REFERENCE

"Subclause 11.13.7 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 8 }

wmanIf2sSsMinReservedRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the minimum rate reserved

for this service flow."

REFERENCE

"Subclause 11.13.8 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 9 }

wmanIf2sSsToleratedJitter OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter defines the Maximum delay

variation (jitter) for the connection."

REFERENCE

"Subclause 11.13.13 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 10 }

wmanIf2sSsMaxLatency OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "millisecond"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the maximum

latency between the reception of a packet by the BS

or SS on its network interface and the forwarding

of the packet to its RF Interface."

REFERENCE

"Subclause 11.13.14 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 11 }

wmanIf2sSsFixedVsVariableSduInd OBJECT-TYPE

SYNTAX      INTEGER {variableLength(0),

fixedLength(1)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies whether the SDUs

on the service flow are variable-length (0) or

fixed-length (1). The parameter is used only if

packing is on for the service flow. The default value

is 0, i.e.,variable-length SDUs."

REFERENCE

"Subclause 11.13.15 in IEEE Std 802.16-2004"

DEFVAL      { variableLength }

::= { wmanIf2sSsServiceFlowEntry 12 }

wmanIf2sSsSduSize OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter specifies the length of the

SDU for a fixed-length SDU service flow. This parameter

is used only if packing is on and the service flow is

indicated as carrying fixed-length SDUs. The default

value is 49 bytes, i.e., VC-switched ATM cells with PHS.

The parameter is relevant for both ATM and Packet

Convergence Sublayers."

REFERENCE

"Subclause 11.13.16 in IEEE Std 802.16-2004"

DEFVAL      { 49 }

::= { wmanIf2sSsServiceFlowEntry 13 }

wmanIf2sSsSfSchedulingType OBJECT-TYPE

SYNTAX      WmanIf2sSchedulingType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specifies the upstream scheduling service used for

upstream service flow. If the referenced parameter

is not present in the corresponding 802.16 QOS

Parameter Set of an upstream service flow, the

default value of this object is bestEffort(2)."

REFERENCE

"Subclause 11.13.11 in IEEE Std 802.16-2004"

DEFVAL         { bestEffort }

::= { wmanIf2sSsServiceFlowEntry 14 }

wmanIf2sSsArqEnable OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"True(1) ARQ enabling is requested for the connection."

::= { wmanIf2sSsServiceFlowEntry 15 }

wmanIf2sSsArqWindowSize	 OBJECT-TYPE

SYNTAX	    INTEGER (1..1024)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the maximum number of unacknowledged

fragments at any time."

::= { wmanIf2sSsServiceFlowEntry 16 }

wmanIf2sSsArqBlockLifetime OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum time interval an ARQ fragment will be

managed by the transmitter ARQ machine, once

initial transmission of the fragment has occured.

If transmission or retransmission of the fragment

is not acknowledged by the receiver before the

time limit is reached, the fragment is discarded.

A value of 0 means Infinite."

::= { wmanIf2sSsServiceFlowEntry 17 }

wmanIf2sSsArqSyncLossTimeout OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535 )

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The maximum interval before declaring a loss

of synchronization of the sender and receiver

state machines. A value of 0 means Infinite."

::= { wmanIf2sSsServiceFlowEntry 18 }

wmanIf2sSsArqDeliverInOrder  OBJECT-TYPE

SYNTAX      TruthValue

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates whether or not data is to be delivered

by the receiving MAC to its client application

in the order in which data was handed off to the

originating MAC."

::= { wmanIf2sSsServiceFlowEntry 19 }

wmanIf2sSsArqRxPurgeTimeout  OBJECT-TYPE

SYNTAX	    INTEGER (0 .. 65535)

UNITS	    "10 us"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the time interval the ARQ window is advanced

after a fragment is received. A value of 0 means

Infinite."

::= { wmanIf2sSsServiceFlowEntry 20 }

wmanIf2sSsArqBlockSize OBJECT-TYPE

SYNTAX	    INTEGER (1..2040)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value of this parameter specifies the size of an

ARQ block. This parameter shall be established by

negotiation during the connection creation dialog."

REFERENCE

"Subclause 11.13.18.8 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 21 }

wmanIf2sSsMinRsvdTolerableRate OBJECT-TYPE

SYNTAX      Unsigned32

UNITS       "b/s"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Minimum Tolerable Traffic Rate = R (bits/sec) with

time base T(sec) means the following. Let S denote

additional demand accumulated at the MAC SAP of the

transmitter during an arbitrary time interval of the

length T. Then the amount of data forwarded at the

receiver to CS (in bits) during this interval should

be not less than min {S, R * T}."

REFERENCE

"Subclause 11.13.9 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 22 }

wmanIf2sSsReqTxPolicy OBJECT-TYPE

SYNTAX      BITS {noBroadcastBwReq(0),

reserved1(1),

noPiggybackReq(2),

noFragmentData(3),

noPHS(4),

noSduPacking(5),

noCrc(6),

reserved2(7)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this parameter provides the capability to

specify certain attributes for the associated service

flow. An attribute is enabled by setting the

corresponding bit position to 1."

REFERENCE

"Subclause 11.13.12 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 23 }

wmanIf2sSsSfCsSpecification OBJECT-TYPE

SYNTAX      WmanIf2sCsType

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This parameter specifies the convergence sublayer

encapsulation mode."

REFERENCE

"Subclause 11.13.19.1 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 24 }

wmanIf2sSsTargetSaid OBJECT-TYPE

SYNTAX      INTEGER (0 .. 65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The target SAID parameter indicates the SAID onto

which the service flow being set up shall be mapped."

REFERENCE

"Subclause 11.13.17 in IEEE Std 802.16-2004"

::= { wmanIf2sSsServiceFlowEntry 25 }

wmanIf2sSsClassifierRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sSsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains packet classifier rules associated

with service flows."

::= { wmanIf2sSsMib 3 }

wmanIf2sSsClassifierRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2sSsClassifierRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each packet classifier

rule."

INDEX     { ifIndex,

wmanIf2sSsSfId,

wmanIf2sSsClassifierRuleIndex }

::= { wmanIf2sSsClassifierRuleTable 1 }

WmanIf2sSsClassifierRuleEntry::= SEQUENCE {

wmanIf2sSsClassifierRuleIndex           Unsigned32,

wmanIf2sSsClassifierRulePriority        INTEGER,

wmanIf2sSsClassifierRuleIpTosLow        INTEGER,

wmanIf2sSsClassifierRuleIpTosHigh       INTEGER,

wmanIf2sSsClassifierRuleIpTosMask       INTEGER,

wmanIf2sSsClassifierRuleIpProtocol      Integer32,

wmanIf2sSsClassifierRuleIpSourceAddr    InetAddress,

wmanIf2sSsClassifierRuleIpSourceMask    InetAddress,

wmanIf2sSsClassifierRuleIpDestAddr      InetAddress,

wmanIf2sSsClassifierRuleIpDestMask      InetAddress,

wmanIf2sSsClassifierRuleSourcePortStart Integer32,

wmanIf2sSsClassifierRuleSourcePortEnd   Integer32,

wmanIf2sSsClassifierRuleDestPortStart   Integer32,

wmanIf2sSsClassifierRuleDestPortEnd     Integer32,

wmanIf2sSsClassifierRuleDestMacAddr     MacAddress,

wmanIf2sSsClassifierRuleDestMacMask     MacAddress,

wmanIf2sSsClassifierRuleSourceMacAddr   MacAddress,

wmanIf2sSsClassifierRuleSourceMacMask   MacAddress,

wmanIf2sSsClassifierRuleEnetProtocolTyp INTEGER,

wmanIf2sSsClassifierRuleEnetProtocol    Integer32,

wmanIf2sSsClassifierRuleUserPriLow      Integer32,

wmanIf2sSsClassifierRuleUserPriHigh     Integer32,

wmanIf2sSsClassifierRuleVlanId          Integer32,

wmanIf2sSsClassifierRulePkts            Counter64,

wmanIf2sSsClassifierRuleIpv6FlowLabel   WmanIf2sIpv6FlowLabel,

wmanIf2sSsClassifierRuleActionRule      WmanIf2sActionRule,

wmanIf2sSsClassifierRuleBitMap          WmanIf2sClassifierMap}

wmanIf2sSsClassifierRuleIndex  OBJECT-TYPE

SYNTAX      Unsigned32 (1..4294967295)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index is assigned to each classifier in the classifiers

table"

REFERENCE

"Subclause 11.13.19.3.4.14 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 1 }

wmanIf2sSsClassifierRulePriority OBJECT-TYPE

SYNTAX      INTEGER (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value specifies the order of evaluation of the

classifiers. The higher the value the higher the

priority. The value of 0 is used as default in

provisioned service flows classifiers. The default

value of 64 is used for dynamic service flow classifiers.

If the referenced parameter is not present in a classifier

, this object reports the default value as defined above"

REFERENCE

"Subclause 11.13.19.3.4.1 in IEEE Std 802.16-2004"

DEFVAL      { 0 }

::= { wmanIf2sSsClassifierRuleEntry 2 }

wmanIf2sSsClassifierRuleIpTosLow OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The low value of a range of TOS byte values. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 3 }

wmanIf2sSsClassifierRuleIpTosHigh OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The 8-bit high value of a range of TOS byte values.

If the referenced parameter is not present in a classifier

, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 4 }

wmanIf2sSsClassifierRuleIpTosMask OBJECT-TYPE

SYNTAX      INTEGER (0 .. 255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The mask value is bitwise ANDed with TOS byte in an IP

packet and this value is used for the range checking of

TosLow and TosHigh. If the referenced parameter is not

present in a classifier, this object reports the value

of 0."

REFERENCE

"Subclause 11.13.19.3.4.2 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 5 }

wmanIf2sSsClassifierRuleIpProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the value of the IP Protocol field

required for IP packets to match this rule. If the

referenced parameter is not present in a classifier, this

object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.3 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 6 }

wmanIf2sSsClassifierRuleIpSourceAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Source Address

required for packets to match this rule. An IP packet

matches the rule when the packet ip source address bitwise

ANDed with the wmanIf2sSsClassifierRuleIpSourceMask value

equals the wmanIf2sSsClassifierRuleIpSourceAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 7 }

wmanIf2sSsClassifierRuleIpSourceMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP Source

Address that are compared to match this rule. An IP packet

matches the rule when the packet source address bitwise

ANDed with the

wmanIf2sSsClassifierRuleIpSourceMask value equals the

wmanIf2sSsClassifierRuleIpSourceAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.4 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 8 }

wmanIf2sSsClassifierRuleIpDestAddr OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the value of the IP Destination

Address required for packets to match this rule. An IP

packet matches the rule when the packet IP destination

address bitwise ANDed with the

wmanIf2sSsClassifierRuleIpDestMask value equals the

wmanIf2sSsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a

classifier, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 9 }

wmanIf2sSsClassifierRuleIpDestMask OBJECT-TYPE

SYNTAX      InetAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies which bits of a packet's IP

Destination Address that are compared to match this rule.

An IP packet matches the rule when the packet destination

address bitwise ANDed with the

wmanIf2sSsClassifierRuleIpDestMask value equals the

wmanIf2sSsClassifierRuleIpDestAddr value.

If the referenced parameter is not present in a classifier

, this object reports the value of 0.0.0.0."

REFERENCE

"Subclause 11.13.19.3.4.5 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 10 }

wmanIf2sSsClassifierRuleSourcePortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP source port numbers to which a packet is compared

. This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a

classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 11 }

wmanIf2sSsClassifierRuleSourcePortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP source port numbers to which a packet is compared.

This object is irrelevant for non-TCP/UDP IP packets.

If the referenced parameter is not present in a classifier,

this object reports the value of 65535."

REFERENCE

"Subclause 11.13.19.3.4.6 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 12 }

wmanIf2sSsClassifierRuleDestPortStart OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the low end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of 0."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 13 }

wmanIf2sSsClassifierRuleDestPortEnd OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object specifies the high end inclusive range of

TCP/UDP destination port numbers to which a packet is

compared. If the referenced parameter is not present

in a classifier, this object reports the value of

65535."

REFERENCE

"Subclause 11.13.19.3.4.7 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 14 }

wmanIf2sSsClassifierRuleDestMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sSsClassifierRuleDestMacMask equals the value of

wmanIf2sSsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 15 }

wmanIf2sSsClassifierRuleDestMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sSsClassifierRuleDestMacMask equals the value of

wmanIf2sSsClassifierRuleDestMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.8 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 16 }

wmanIf2sSsClassifierRuleSourceMacAddr OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches this entry when its source

MAC address  bitwise ANDed with

wmanIf2sSsClassifierRuleSourceMacMask equals the value

of wmanIf2sSsClassifierRuleSourceMacAddr. If the

referenced parameter is not present in a classifier,

this object reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 17 }

wmanIf2sSsClassifierRuleSourceMacMask OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"An Ethernet packet matches an entry when its destination

MAC address bitwise ANDed with

wmanIf2sSsClassifierRuleSourceMacMask equals the value of

wmanIf2sSsClassifierRuleSourceMacAddr. If the referenced

parameter is not present in a classifier, this object

reports the value of '000000000000'H."

REFERENCE

"Subclause 11.13.19.3.4.9 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 18 }

wmanIf2sSsClassifierRuleEnetProtocolTyp OBJECT-TYPE

SYNTAX      INTEGER {none(0),

ethertype(1),

dsap(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates the format of the layer 3 protocol

id in the Ethernet packet. A value of none(0) means that

the rule does not use the layer 3 protocol type as a

matching criteria. A value of ethertype(1) means that the

rule applies only to frames which contains an EtherType

value. Ethertype values are contained in packets using

the Dec-Intel-Xerox (DIX) encapsulation or the RFC1042

Sub-Network Access Protocol (SNAP) encapsulation formats.

A value of dsap(2) means that the rule applies only to

frames using the IEEE802.3 encapsulation format with a

Destination Service Access Point (DSAP) other than 0xAA

(which is reserved for SNAP). If the Ethernet frame

contains an 802.1P/Q Tag header (i.e. EtherType 0x8100),

this object applies to the embedded EtherType field within

the 802.1P/Q header. If the referenced parameter is not

present in a classifier, this object reports the value of

0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 19 }

wmanIf2sSsClassifierRuleEnetProtocol OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"If wmanIf2sSsClassifierRuleEnetProtocolTyp is none(0),

this object is ignored when considering whether a packet

matches the current rule.

If wmanIf2sSsClassifierRuleEnetProtocolTyp is ethertype(1)

,this object gives the 16-bit value of the EtherType that

the packet must match in order to match the rule.

If wmanIf2sSsClassifierRuleEnetProtocolTyp is dsap(2), the

lower 8 bits of this object's value must match the DSAP

byte of the packet in order to match the rule.

If the Ethernet frame contains an 802.1P/Q Tag header

(i.e. EtherType 0x8100), this object applies to the

embedded EtherType field within the 802.1P/Q header.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.10 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 20 }

wmanIf2sSsClassifierRuleUserPriLow OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority field

within the range of wmanIf2sSsClassifierRulePriLow and

wmanIf2sSsClassifierRulePriHigh in order to match this

rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 21 }

wmanIf2sSsClassifierRuleUserPriHigh OBJECT-TYPE

SYNTAX      Integer32 (0..7)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header (indicated with EtherType 0x8100).

Such frames include a 16-bit Tag that contains a 3 bit

Priority field and a 12 bit VLAN number.

Tagged Ethernet packets must have a 3-bit Priority

field within the range of wmanIf2sSsClassifierRulePriLow

and wmanIf2sSsClassifierRulePriHigh in order to match

this rule.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 7."

REFERENCE

"Subclause 11.13.19.3.4.11 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 22 }

wmanIf2sSsClassifierRuleVlanId OBJECT-TYPE

SYNTAX      Integer32 (0..4095)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object applies only to Ethernet frames using the

802.1P/Q tag header.

If this object's value is nonzero, tagged packets must

have a VLAN Identifier that matches the value in order

to match the rule.

Only the least significant 12 bits of this object's

value are valid.

If the referenced parameter is not present in the

classifier, the value of this object is reported as 0."

REFERENCE

"Subclause 11.13.19.3.4.12 in IEEE Std 802.16-2004"

::= { wmanIf2sSsClassifierRuleEntry 23 }

wmanIf2sSsClassifierRulePkts OBJECT-TYPE

SYNTAX      Counter64

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object counts the number of packets that have

been classified using this entry."

::= { wmanIf2sSsClassifierRuleEntry 25 }

wmanIf2sSsClassifierRuleIpv6FlowLabel OBJECT-TYPE

SYNTAX      WmanIf2sIpv6FlowLabel

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2sSsClassifierRuleEntry 26 }

wmanIf2sSsClassifierRuleActionRule OBJECT-TYPE

SYNTAX      WmanIf2sActionRule

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule. If this classification action rule

exists, its action shall be applied on the packets that

match this classifier rule."

::= { wmanIf2sSsClassifierRuleEntry 27 }

wmanIf2sSsClassifierRuleBitMap OBJECT-TYPE

SYNTAX      WmanIf2sClassifierMap

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2sSsClassifierRuleEntry 28 }

wmanIf2sSsPhsRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sSsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When a classifier is deleted, any associated

PHS rule shall also be deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2sSsMib 4 }

wmanIf2sSsPhsRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2sSsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data."

INDEX     { ifIndex, wmanIf2sSsSfId,

wmanIf2sSsPhsRuleId }

::= { wmanIf2sSsPhsRuleTable 1 }

WmanIf2sSsPhsRuleEntry::= SEQUENCE {

wmanIf2sSsPhsRuleId                     INTEGER,

wmanIf2sSsPhsRulePhsField               OCTET STRING,

wmanIf2sSsPhsRulePhsMask                OCTET STRING,

wmanIf2sSsPhsRulePhsSize                Integer32,

wmanIf2sSsPhsRulePhsVerify              WmanIf2sPhsRuleVerify}

wmanIf2sSsPhsRuleId OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2sSsPhsRuleEntry 1 }

wmanIf2sSsPhsRulePhsField OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2sSsPhsRuleEntry 2 }

wmanIf2sSsPhsRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2sSsPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2sSsPhsRuleEntry 3 }

wmanIf2sSsPhsRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2sSsPhsRuleEntry 4 }

wmanIf2sSsPhsRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2sPhsRuleVerify

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2sSsPhsRuleEntry 5 }

--

-- Subscriber station wmanIf2sSsMib contains the SS SNMP Trap objects

--

wmanIf2sSsTrapControl OBJECT IDENTIFIER ::= { wmanIf2sSsMib 5 }

wmanIf2sSsTrapDefinitions OBJECT IDENTIFIER ::= { wmanIf2sSsMib 6 }

-- This object groups all NOTIFICATION-TYPE objects for SS.

-- It is defined following RFC2758 sections 8.5 and 8.6

-- for the compatibility with SNMPv1.

wmanIf2sSsTrapPrefix OBJECT IDENTIFIER ::= {wmanIf2sSsTrapDefinitions 0}

wmanIf2sSsTrapControlRegister    OBJECT-TYPE

SYNTAX      BITS {wmanIf2sSsDhcpSuccess(0),

wmanIf2sSsRssiStatusChange(1),

wmanIf2sSsPkmSilentState(2)}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The object is used to enable Subscriber Station traps.

From left to right, the set bit indicates the corresponding

Subscriber Station trap is enabled."

::= { wmanIf2sSsTrapControl  1 }

wmanIf2sSsThresholdConfigTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sSsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains threshold objects that can be set to

detect the threshold crossing events."

::= { wmanIf2sSsTrapControl 2 }

wmanIf2sSsThresholdConfigEntry OBJECT-TYPE

SYNTAX      WmanIf2sSsThresholdConfigEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS."

INDEX     { ifIndex }

::= { wmanIf2sSsThresholdConfigTable 1 }

WmanIf2sSsThresholdConfigEntry ::= SEQUENCE {

wmanIf2sSsRssiLowThreshold              Integer32,

wmanIf2sSsRssiHighThreshold             Integer32}

wmanIf2sSsRssiLowThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Low RSSI threshold for generating the RSSI alarm trap."

::= { wmanIf2sSsThresholdConfigEntry 1 }

wmanIf2sSsRssiHighThreshold OBJECT-TYPE

SYNTAX      Integer32

UNITS       "dBm"

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"High RSSI threshold for generating a trap to indicate

the RSSI is restored."

::= { wmanIf2sSsThresholdConfigEntry 2 }

wmanIf2sSsDhcpSuccessTrap    NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2sSsMacAddress}

STATUS      current

DESCRIPTION

"An event to report a successful Handshake to establish IP

connectivity."

::= { wmanIf2sSsTrapPrefix 1 }

wmanIf2sSsRssiStatusChangeTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2sSsMacAddress,

wmanIf2sSsRssiStatus,

wmanIf2sSsRssiStatusInfo}

STATUS      current

DESCRIPTION

"An event to report that the downlink RSSI is below

wmanIf2sSsRssiLowThreshold, or above

wmanIf2sSsRssiHighThreshold after restore."

::= { wmanIf2sSsTrapPrefix 2 }

wmanIf2sSsPkmSilentStateTrap NOTIFICATION-TYPE

OBJECTS     {ifIndex,

wmanIf2sSsMacAddress,

wmanIf2sSsSilentStateInfo}

STATUS      current

DESCRIPTION

"An event to report that SS PKM has entered into the

siltent state."

::= { wmanIf2sSsTrapPrefix 3 }

wmanIf2sSsNotificationObjectsTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sSsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains SS notification objects that have been

reported by the trap."

::= { wmanIf2sSsTrapDefinitions 1 }

wmanIf2sSsNotificationObjectsEntry OBJECT-TYPE

SYNTAX      WmanIf2sSsNotificationObjectsEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each SS that has

generated traps."

INDEX     { ifIndex }

::= { wmanIf2sSsNotificationObjectsTable 1 }

WmanIf2sSsNotificationObjectsEntry ::= SEQUENCE {

wmanIf2sSsMacAddress                    MacAddress,

wmanIf2sSsRssiStatus                    INTEGER,

wmanIf2sSsRssiStatusInfo                OCTET STRING,

wmanIf2sSsSilentStateInfo               OCTET STRING}

wmanIf2sSsMacAddress  OBJECT-TYPE

SYNTAX      MacAddress

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The MAC address of the SS generating the trap."

::= { wmanIf2sSsNotificationObjectsEntry 1 }

wmanIf2sSsRssiStatus  OBJECT-TYPE

SYNTAX      INTEGER {ssRssiAlarm(1),

ssRssiNoAlarm(2)}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A RSSI alarm is generated if the RSSI is lower than

wmanIf2sSsRssiLowThreshold, or above

wmanIf2sSsRssiHighThreshold after alarm is restored."

::= { wmanIf2sSsNotificationObjectsEntry 2 }

wmanIf2sSsRssiStatusInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides additional information about RSSI

alarm. It is implementation specific."

::= { wmanIf2sSsNotificationObjectsEntry 3 }

wmanIf2sSsSilentStateInfo  OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This object provides additional information about PKM

silent State. It is implementation specific."

::= { wmanIf2sSsNotificationObjectsEntry 4 }

END