| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fixes to wmanIf2sBsClassifierRuleTable |  |
| Date Submitted | 2007-11-13 |  |
| Source(s) | Richard van Leeuwen Motorola Joey Chou Intel | E-mail: rvanleeuwen@motorola.com E-mail: joey.chou@intel.com |
| Re: | IEEE 802.16 Letter Ballot Recirculation #25d, on P802.16i/D5, as announced in IEEE 802.16-07/044 |  |
| Abstract | This document proposes fixes to wmanIf2sBsClassifierRuleTable. |  |
| Purpose | Adopt proposed remedy. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fixes to wmanIf2sBsClassifierRuleTable

Richard van Leeuwen, Joey Chou,

Motorola, Intel

# Introduction

This document proposes fixes to wmanIf2sBsClassifierRuleTable.

# Proposed ASN.1 changes

WmanIf2sBsClassifierRuleEntry::= SEQUENCE {

wmanIf2sBsClassifierRuleIndex           Unsigned32,

wmanIf2sBsClassifierRulePriority        INTEGER,

wmanIf2sBsClassifierRuleIpTosLow        INTEGER,

wmanIf2sBsClassifierRuleIpTosHigh       INTEGER,

wmanIf2sBsClassifierRuleIpTosMask       INTEGER,

wmanIf2sBsClassifierRuleIpProtocol      Integer32,

wmanIf2sBsClassifierRuleIpSrcAddr       InetAddress,

wmanIf2sBsClassifierRuleIpSrcMask       InetAddress,

wmanIf2sBsClassifierRuleIpDestAddr      InetAddress,

wmanIf2sBsClassifierRuleIpDestMask      InetAddress,

wmanIf2sBsClassifierRuleSrcPortStart    Integer32,

wmanIf2sBsClassifierRuleSrcPortEnd      Integer32,

wmanIf2sBsClassifierRuleDestPortStart   Integer32,

wmanIf2sBsClassifierRuleDestPortEnd     Integer32,

wmanIf2sBsClassifierRuleDestMacAddr     MacAddress,

wmanIf2sBsClassifierRuleDestMacMask     MacAddress,

wmanIf2sBsClassifierRuleSrcMacAddr      MacAddress,

wmanIf2sBsClassifierRuleSrcMacMask      MacAddress,

wmanIf2sBsClassifierRuleEnetProtType    INTEGER,

wmanIf2sBsClassifierRuleEnetProtocol    Integer32,

wmanIf2sBsClassifierRuleUserPriLow      Integer32,

wmanIf2sBsClassifierRuleUserPriHigh     Integer32,

wmanIf2sBsClassifierRuleVlanId          Integer32,

wmanIf2sBsClassifierRulePhsSize         Integer32,

wmanIf2sBsClassifierRulePhsMask         OCTET STRING,

wmanIf2sBsClassifierRulePhsVerify       WmanIf2sPhsRuleVerify,

wmanIf2sBsClassifierRuleIpv6FlowLabel   WmanIf2sIpv6FlowLabel,

wmanIf2sBsClassifierRuleActionRule      WmanIf2sActionRule,

wmanIf2sBsClassifierRuleBitMap          WmanIf2sClassifierMap,

wmanIf2sBsClassifierRuleAssociatedPhsi  INTEGER,

wmanIf2sBsClassifierRuleRowStatus       RowStatus}

wmanIf2sBsClassifierRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to configure the PHS rule for this

classifer. The value of this field - PHSS is the total

number of bytes in the header to be suppressed and then

restored in a service flow that uses PHS. If the value of

this field is 0 bytes then PHS is disabled for this

classifer. If flag phsMask in wmanIf2sBsClassifierRuleBitMap

is set to 0 and flag phsSize in

wmanIf2sBsClassifierRuleBitMap is set to 0, then BS can still

create PHS rules using its own custom mask (i.e. the rule

is not configured by NMS)."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2sBsClassifierRuleEntry 25 }

wmanIf2sBsClassifierRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to configure the PHS rule for this

classifer. It is encoded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2sBsClassifierRulePhsSize/8). BS should use this

value to create a new PHS rule index (PHSI) and field

(PHSF) as defined in the standard. If flag phsMask in

wmanIf2sBsClassifierRuleBitMap is set to 0 and flag phsSize

in wmanIf2sBsClassifierRuleBitMap is set to 0, then BS can

still create PHS rules using its own custom mask (i.e. the

rule is not configured by NMS)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 26 }

wmanIf2sBsClassifierRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2sPhsRuleVerify

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2sBsClassifierRuleEntry 27 }

wmanIf2sBsClassifierRuleAssociatedPhsi OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The Associated PHSI has a value between 1 and 255, which

shall mirror the PHSI value of a PHS rule. Packets matching

the Packet Classification Rule containing the Associated

PHSI parameter shall undergo PHS according to the

corresponding PHS rule. A value '0' indicates that no PHS

rule is associated with this classifier rule."

REFERENCE

"Subclause 11.13.19.3.4.13 in IEEE Std 802.16-2004"

::= { wmanIf2sBsClassifierRuleEntry 24 }

wmanIf2sBsClassifierRuleIpv6FlowLabel OBJECT-TYPE

SYNTAX      WmanIf2sIpv6FlowLabel

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this field specifies the matching values for

the IPv6 Flow label field."

::= { wmanIf2sBsClassifierRuleEntry 285 }

wmanIf2sBsClassifierRuleActionRule OBJECT-TYPE

SYNTAX      WmanIf2sActionRule

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"The value of this field specifies an action associated with

the classifier rule. If this classification action rule

exists, its action shall be applied on the packets that

match this classifier rule."

::= { wmanIf2sBsClassifierRuleEntry 296 }

wmanIf2sBsClassifierRuleBitMap OBJECT-TYPE

SYNTAX      WmanIf2sClassifierMap

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object indicates which parameter encodings were

actually present in the entry. A bit set to '1' indicates

the corresponding classifier encoding is present, and '0'

means otherwise"

::= { wmanIf2sBsClassifierRuleEntry 3027 }

wmanIf2sBsClassifierRuleRowStatus OBJECT-TYPE

SYNTAX      RowStatus

MAX-ACCESS  read-create

STATUS      current

DESCRIPTION

"This object is used to create a new row or modify or

delete an existing row in this table.

If the implementator of this MIB has choosen not

to implement 'dynamic assignment' of profiles, this

object is not useful and should return noSuchName

upon SNMP request."

::= { wmanIf2sBsClassifierRuleEntry 28 }

wmanIf2sBsPhsRuleTable OBJECT-TYPE

SYNTAX      SEQUENCE OF WmanIf2sBsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table contains PHS rule dictionary entries. Each

entry contains the data of the header to be suppressed

along with its identification - PHSI. The classifier

uniquely maps packets to its associated PHS Rule. The

receiving entity uses the CID and the PHSI to restore the

PHSF. Once a PHSF has been assigned to a PHSI, it shall

not be changed. To change the value of a PHSF on a

service flow, a new PHS rule shall be defined, the old

rule is removed from the service flow, and the new rule

is added. When a classifier is deleted, any associated

PHS rule shall also be deleted."

REFERENCE

"Subclause 5.2.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsMib 5 }

wmanIf2sBsPhsRuleEntry OBJECT-TYPE

SYNTAX      WmanIf2sBsPhsRuleEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"This table provides one row for each PHS rule created

dynamically by the BS and SS on a given service flow. The

PHS rule is defined by the pair (PHSS, PHSM) for each

distinct header data."

INDEX     { ifIndex, wmanIf2sBsSfId,

wmanIf2sBsPhsRulePhsIndex }

::= { wmanIf2sBsPhsRuleTable 1 }

WmanIf2sBsPhsRuleEntry::= SEQUENCE {

wmanIf2sBsPhsRulePhsIndex               INTEGER,

wmanIf2sBsPhsRulePhsField               OCTET STRING,

wmanIf2sBsPhsRulePhsMask                OCTET STRING,

wmanIf2sBsPhsRulePhsSize                Integer32,

wmanIf2sBsPhsRulePhsVerify              WmanIf2sPhsRuleVerify}

wmanIf2sBsPhsRulePhsIndex OBJECT-TYPE

SYNTAX      INTEGER (1..255)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The PHSI (PHS Index) has a value between 1 and 255, which

uniquely references the suppressed byte string. The index

is unique per service flow. The uplink and downlink PHSI

values are independent of each other."

REFERENCE

"Subclause 11.13.19.3.7.1 in IEEE Std 802.16-2004"

::= { wmanIf2sBsPhsRuleEntry 1 }

wmanIf2sBsPhsRulePhsField OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSF (PHS Field) is a string of bytes containing the

header information to be suppressed by the sending CS and

reconstructed by the receiving CS. The most significant

byte of the string corresponds to the first byte of the

CS-SDU."

REFERENCE

"Subclause 11.13.19.3.7.2 in IEEE Std 802.16-2004"

::= { wmanIf2sBsPhsRuleEntry 2 }

wmanIf2sBsPhsRulePhsMask OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..65535))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The PHSM An 8-bit mask that indicates which bytes in the

PHS Field (PHSF) to suppress and which bytes to not

suppress. The PHSM allows fields, such as sequence numbers

or checksums (which vary in value), to be excluded from

suppression with the constant bytes around them suppressed.

It is encodded as follows:

bit 0:

0 = don't suppress the 1st byte of the suppression field

1 = suppress first byte of the suppression field

bit 1:

0 = don't suppress the 2nd byte of the suppression field

1 = suppress second byte of the suppression field

bit x:

0 = don't suppress the (x+1) byte of the suppression

field

1 = suppress (x+1) byte of the suppression field

where the length of the octet string is ceiling

(wmanIf2sBsPhsRulePhsSize/8)."

REFERENCE

"Subclause 11.13.19.3.7.3 in IEEE Std 802.16-2004"

::= { wmanIf2sBsPhsRuleEntry 3 }

wmanIf2sBsPhsRulePhsSize OBJECT-TYPE

SYNTAX      Integer32 (0..255)

UNITS       "byte"

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field - PHSS is the total number of bytes

in the header to be suppressed and then restored in a

service flow that uses PHS."

REFERENCE

"Subclause 11.13.19.3.7.4 in IEEE Std 802.16-2004"

DEFVAL      {0}

::= { wmanIf2sBsPhsRuleEntry 4 }

wmanIf2sBsPhsRulePhsVerify OBJECT-TYPE

SYNTAX      WmanIf2sPhsRuleVerify

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of this field indicates to the sending entity

whether or not the packet header contents are to be

verified prior to performing suppression."

DEFVAL      { phsVerifyEnable }

::= { wmanIf2sBsPhsRuleEntry 5 }