| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | M2M System Architecture |  |
| Date Submitted | 2010-04-05 |  |
| Source(s) | Hyunjeong Kang, Simon Kong, Rakesh Taori, Jungje Son, Hokyu Choi, Sudhir Ramakrishna, Ying Li, Anil Agiwal, Anshuman Nigam, Lei Zhou, Xufeng Zheng Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Call for Contributions for Machine to Machine Communication Study Report (IEEE802.16-10/0024) |  |
| Abstract | The contribution proposes high level system architecture for machine to machine communication. |  |
| Purpose | To discuss a high level system architecture for machine to machine communication for the 802.16 project planning committee |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

M2M System Architecture

- Introduction

This contribution presents a high level system architecture where M2M devices communicate with M2M server through IEEE 802.16 network. The IEEE 802.16 M2M system architecture consists of IEEE 802.16 entities with M2M functionality and M2M server that are applied to the existing network reference model in [1].

Reference

[1] WMF-T32-001-R015V01, WiMAX Forum Network Architecture – Stage 2 – Release 1.5, Nov. 2009

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Add the following texts under subclause 2 in IEEE 802.16-10/0024:]

2 M2M System Architecture Considerations

Figure 1 illustrates a high level system architecture, identifying the network elements enabling M2M communication and consisting of the following entities: IEEE 802.16 M2M device, IEEE 802.16 base station (BS), Access Service Network (ASN), Connectivity Service Network (CSN) and M2M server. The architecture can be considered with the case where the M2M server resides in CSN of Mobile Network Operator. Depending on the deployment scenario, however, the M2M server may be outside CSN. The M2M application runs on the IEEE 802.16 M2M device and the M2M server.

Figure 1 Network Elements Supporting M2M

The IEEE 802.16 M2M device is an IEEE 802.16 MS running as M2M application. The IEEE 802.16 M2M device may be a gateway that can interoperate and interconnect to other devices such as Bluetooth, Zigbee, RFID, WiFi or PLC, etc.

The ASN in figure 1 provides radio access to IEEE 802.16 M2M devices.

The CSN in figure 1 provides user plane connectivity services to IEEE 802.16 M2M devices.

The M2M server is an entity that communicates to one or more IEEE 802.16 M2M devices. The M2M server also has an interface which can be accessed by an M2M user.

-------------------------------  Text End  ---------------------------------------------------