| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Requirements for Machine to Machine Communication |  |
| Date Submitted | 2010-04-05 |  |
| Source(s) | Hyunjeong Kang, Simon Kong, Rakesh Taori, Jungje Son, Hokyu Choi, Sudhir Ramakrishna, Ying Li, Anil Agiwal, Anshuman Nigam, Lei Zhou, Xufeng Zheng Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Call for Contributions for Machine to Machine Communication Study Report (IEEE802.16-10/0024) |  |
| Abstract | The contribution proposes functional requirements for machine to machine communication. |  |
| Purpose | To discuss the requirements for machine to machine communication for the 802.16 project planning committee |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Requirements for Machine to Machine Communication

- Introduction

This contribution proposes functional requirements for IEEE 802.16 machine to machine communication in [1]. It is proposed to describe the details on the defined requirement such as extremely lower power, access priority and security. In addition, it is proposed to insert new functional requirements for backward compatibility, addressing, group management, small data transmission, low mobility and time controlled operation to be considered in M2M communication.

Reference

[1] IEEE 802.16-10/0024, Machine to Machine (M2M) Communication Study Report (Draft), Mar. 2010

- Text change

-------------------------------  Text Start  ---------------------------------------------------

[Add the following texts under subclause 3.1 in IEEE 802.16-10/0024:]

3.1 Extremely Low Power

The M2M specification should support extra low power consumption for M2M devices whose battery is infrequently recharged or replaced. The system should be able to support power saving operation with extremely long term. The M2M application can be surveillance or tracking

[Add the following texts under subclause 3.3 in IEEE 802.16-10/0024:]

3.3 Access Priority

The M2M specification should support access prioritization for M2M devices. The system should be able to support the M2M device with a higher access priority to transmit a message in the event of theft alarm, an emergency situation or other needs for immediate attention. The M2M application for higher priority access can be the healthcare. The system should be able to support the M2M device with a lower access priority to transmit time tolerant data. The M2M application for lower priority access can be smart meter.

[Change the title of subclause 3.7 and add the following texts under the subclause 3.7 in IEEE 802.16-10/0024:]

3.7 Enhanced Security

The M2M specification should support appropriate level of authentication for the M2M device or M2M gateway. The M2M specification should support verification and validation of the exchanged data.

[Insert the following new subclauses under subclause 3 in IEEE 802.16-10/0024:]

3.8 Backward compatibility

The M2M specification dealing with air-interface shall be backward compatible to IEEE 802.16m specification. The ASN and CSN specifications shall be backward compatible to the existing WiMAX NWG specification.

3.9 Addressing

The M2M system should be able to address large number of devices individually or address a group of devices.

3.10 Group Management

The M2M specification should support group management for wide distribution of M2M devices.

3.11 Small Data Transmission

The M2M specification should support transmission of small amount of data while minimizing the signaling overhead. The M2M application can be smart meter.

3.12 Low Mobility

The M2M specification should support the M2M devices with low mobility that do not move, move infrequently or move only within a certain region. The system should be able to minimize or reduce mobility management such as location update or handover. The M2M application can be smart meter, surveillance camera, or vending machine.

3.13 Time Controlled Operation

The M2M specification should support the time controlled operation, where an M2M device sends or receives a data only at certain period of time. The system should avoid the unnecessary signaling outside these pre-defined periods. The M2M application can be smart meter.

-------------------------------  Text End  ---------------------------------------------------