| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Huawei Comments on Proposed Standards Impacts for the M2M Study Report Draft |  |
| Date Submitted | 2010-04-26 |  |
| Source(s) | Vibhor Julka Huawei Technologies Co.,Ltd. | E-mail: vjulka@huawei.com |
| Re: | none |  |
| Abstract | Huawei Comments to C802.16ppc-10/0017r1 on Proposed Standards Impacts for the M2M Study Report Draft |  |
| Purpose | For discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on Proposed Standards Impacts for the M2M Study Report Draft

# I.  Introduction

# This contribution presents some initial comments to 802.16ppc-10/0017r1 “Proposed Standards Impacts for the M2M Study Report Draft”

# II. Proposed Edits to M2M Study Report

Table of Contents

I.  Introduction	2

This contribution proposes standards impacts for inclusion into the M2M Study Report.	2

II. Proposed Edits to M2M Study Report	2

5	802.16 Standards Impact	3

5.1	Extremely Low Power Consumption	3

5.2	High Reliability	3

5.3	Enhanced Access Priority	3

5.4	Mass Device Transmission	3

5.5	Group Management	3

5.6	Enhanced Security	4

5.7	Small Data Transmission	4

5.8	Low/No Mobility	4

5.9	Time Controlled Operation	4

5.10	Time Tolerant Operation	4

5.11	One-way Data Traffic	4

5.12	Extremely Long Range Access	4

5.13	Extremely Low Latency	4

5.14	Infrequent Traffic	4

# 802.16 Standards Impact

## Extremely Low Power Consumption

Many 802.16 protocols may need to be updated to enable extremely low power consumption.  The most obvious is sleep/idle mode.  When frequent or ad-hoc reporting is not required, M2M devices can enter into sleep mode for extremely long periods of time.  Thus, an extremely long sleep cycle should be enabled.

The link adaptation protocol can also be updated to support extremely low power communication.  Since most M2M traffic carries a very small payload with no delay constraint, a very low modulation and coding scheme can be used to save power on the transmission.

Another protocol that saves significant amounts of power is device collaboration.  This protocol enables M2M devices with good/excellent BS links to aid in transmission of data from M2M devices with poor BS links.  Studies show that collaboration saves considerable amounts of power across the network.

## High Reliability

High reliability implies that device-BS connectivity is always good.  In order to enable consistently good connectivity will need robust modulation/coding schemes (MCS), redundant/alternate paths, device collaboration, and interference mitigation techniques.

There are 802.16 MCSs that are very robust.  However, reliability may be further improved by adding more robust schemes. Redundant/alternate path generation is an obvious way of improving reliability.  This is an area that GRIDMAN is looking at.

Device collaboration can significantly improve connectivity to the base station.  All M2M devices must have adequate link quality to initially associate and receive control signaling from the base station.  However, the benefit of device collaboration is that if there are moments of poor link quality when the device can not communicate with the base station successfully, its collaborator can do so on its behalf.

Finally, interference mitigation can go a long way towards improving link quality.  Thus, in order to guarantee good device-to-BS connectivity, advanced interference mitigation protocols may be necessary.

## Enhanced Access Priority

Access priority implies that when the device needs to communicate with the base station, it is given priority in doing so.  This is clearly a function of the bandwidth request protocol.  Thus, changes to this protocol may be required to prioritize M2M traffic.  Updates to the ARQ/HARQ protocols may be required for the same reason.   Changes to the frame structure may also facilitate prioritization of M2M traffic.

## Mass Device Transmission

Mass device transmission may require changes to the frame structure, link adaptation, the bandwidth request protocol, and HARQ/ARQ procedure.  In addition, due to potential changes in addressing, the network entry procedure may need to be updated.

## Group Management

Group management implies that the network can address and handle groups of M2M devices as a unit.  This may require changes to the network entry protocol, bandwidth allocation protocol, and service flow and connection management protocols.

## Enhanced Security

Enhanced security may require updates to the network entry procedure.

## Small Data Transmission

The burst management protocol needs to be updated to support very small bursts.  In addition, changes to the frame structure may be required.

## Low/No Mobility

The low/no mobility feature impacts the mobility management protocol.  In particular, it implies that the signaling related to handover preparation and execution is no longer necessary.  Thus, an option to turn this feature off is required.

This feature may also enable optimizations to the frame structure.

This feature may also impact the use of idle mode, since idle mode is designed for mobile users with no data traffic.  Note that stationarity does not impact the idle mode protocol itself, only whether or not it is used.

## Time Controlled Operation

Time controlled traffic implies the absence of “ad-hoc” packet transmission (to or from the M2M device).  This implies that contention-based protocols are not required, which may also impact the frame structure.

## Time Tolerant Operation

Time tolerant operation implies that latency is not an issue for traffic.  This may allow updates to the bandwidth request and allocation protocols in order to optimize power and channel resource usage.

## One-way Data Traffic

One-way data traffic implies that data travels in one direction only.  This may impact the bandwidth request and allocation protocols as well as the network entry and addressing procedures.

## Extremely Long Range Access

To enable very long range access extremely low modulation schemes are required as well as changes to the symbol structure.

## Extremely Low Latency

This feature implies extremely low latency on M2M data transmissions.  This may require changes to the frame structure, ARQ/HARQ, and control signaling.

## Infrequent Traffic

Infrequent traffic implies that M2M transmissions are infrequent with large amounts of time between transmissions.  This implies that M2M devices can sleep for potentially very long periods of time.  As a result, we may need to enable longer sleep cycles in the sleep mode protocol.