| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments on M2M Study Report Draft |  |
| Date Submitted | 2010-05-13 |  |
| Source(s) | Vibhor Julka Huawei Technologies Co.,Ltd. | E-mail: vjulka@huawei.com |
| Re: | none |  |
| Abstract | Comments on M2M Study Report Draft |  |
| Purpose | For discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments on M2M Study Report Draft

# I.  Introduction

# This contribution presents comments to “M2M Study Report Draft” 802.16ppc-10_002r7” to modify the feature definition(s) associated with handling of extremely number of M2M devices.

# II. Proposal 1

## Handling Transmission Attempts from Extremely Large Number of Devices

This feature deals with the handling of simultaneous or near simultaneous transmission attempts to the access network’s base station from an extremely large number of M2M devices. This feature may be required for many use cases such as Secured Access & Surveillance, Tracking, Tracing, & Recovery, Public Safety, Healthcare, Remote Maintenance & Control, and Metering.

……

## Handling Transmission Attempts from  Extremely Large Number of Devices

Support for this feature may require changes to the network entry/re-entry and bandwidth request protocols, link adaptation, HARQ/ARQ, and/or the frame structure. Changes to control signaling may also be required.

# III. Proposal 2

## Handling of Extremely Large Number of Devices

This feature deals with the handling of an extremely large number of M2M devices. It addresses issues such as the need to address large numbers of devices individually, as well as the handling of simultaneous or near simultaneous transmission attempts to the access network’s base station from an extremely large number of M2M devices. This feature may be required for many use cases such as Secured Access & Surveillance, Tracking, Tracing, & Recovery, Public Safety, Healthcare, Remote Maintenance & Control, and Metering.

……..

## Handling of Extremely Large Number of Devices

Support for this feature may require changes to the network entry/re-entry and bandwidth request protocols, link adaptation, HARQ/ARQ, and/or the frame structure. Changes to control signaling may also be required. In addition, addressing extremely large number of devices may require extending the addressing space or updating the addressing scheme