| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Requirements for 802.16 M2M communication |  |
| Date Submitted | 2010-07-12 |  |
| Source(s) | Hyunjeong Kang, Simon Kong, Rakesh Taori, Jungje Son, Hokyu Choi, Sudhir Ramakrishna, Ying Li, Anil Agiwal, Anshuman Nigam, Lei Zhou, Hai Wang, Xufeng Zheng Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Call for Contributions for 802.16 Machine to Machine Communication |  |
| Abstract | The contribution proposes requirements for 802.16 machine to machine communication. |  |
| Purpose | To discuss the requirements for 802.16 machine to machine communication for the 802.16 project planning committee |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Requirements for 802.16 Machine to Machine Communication

- Introduction

This contribution proposes functional requirements for IEEE 802.16 machine to machine communication based on M2M Study report [1] and the scope in 16p drafting PAR [2]. The proposed requirements are to support extremely lower power, small burst transmission support, large number of devices support and security in 802.16 M2M communication.

Reference

[1] IEEE 802.16ppc-10/0002r7, Machine to Machine (M2M) Communication Study Report, May. 2010

[2] IEEE 802.16ppc-10/0003r6, Machine to Machine (M2M) Communication PAR Form and Five Criteria, May. 2010

- Text change

-------------------------------  Text Start  ---------------------------------------------------

1. Low power consumption support

1.1 The 16p specification should support a mechanism of low power consumption for M2M devices whose battery is infrequently recharged or replaced.

1.2 The 16p system should be able to support power saving operation with extremely long term.

2. Large number of device support

2.1 The 16p system should be able to address large number of devices individually or address a group of devices.

2.2 The 16p specification should support group management for wide distribution of M2M devices.

2.3 The 16p specification should support accesses for large number of M2M devices.

2.4 The 16p specification should support large number of M2M devices in connected state as well as in idle state.

3. Small Data Transmission support

3.1 The 16p specification should support the transmission of the size of small data while minimizing the signaling overhead.

4. Security support

4.1 The 16p specification should support the M2M device authentication.

4.2 The 16p specification should support verification and validation of the exchanged data for M2M service.

-------------------------------  Text End  ---------------------------------------------------