| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Power Outage Alarm for Smart Metering Applications |  |
| Date Submitted | 2010-09-09 |  |
| Source(s) | R. Golla, W. Bell, M. Lin, GridNet N. Himayat, S. Talwar, K. Johnsson, Intel Corp. | E-mail: rgolla@gridnet.com nageen.himayat@intel.com |
| Re: | Power outage alarm for smart metering applications |  |
| Abstract |  |  |
| Purpose | For review and adoption into the IEEE 802.16p Systems Requirements Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Background

Reference [1] captured high level IEEE 802.16p requirements mandated by Smart Grid applications. This contribution highlights additional use cases not captured by earlier contributions as well as suggests additional requirements for inclusion in the System Requirement Document [3].

Power Outage Alarms

It is critical for Utilities to minimize power outages on the grid, as these can result in fairly significant financial penalties. Hence timely reporting of power outages and subsequent restoration is a very important smart metering feature, which allows the Utilities to assess net duration of power outages. Typically, delays in reporting restoration of power are tolerated as the smart meter has the ability to store the time of power restoration and report it at a later stage. However, reporting of power outages is especially challenging as electric meters typically report this alarm even without battery support. Here, smart meter devices rely on a charge stored within a capacitor, which allows them to report “one last alarm” on the loss of power before they shut down. Typical time scales involved in reporting this alarm are on the order of 150-250 milliseconds.

Given that power outage may affect potentially thousands of meters in a given sector, the simultaneous reporting of the power outage alarm by very large number of meters can overwhelm the system. Additionally the large number of devices involved implies that most smart meters are likely to remain in idle state due to limitations on the number of connections and address space supported by the existing standard.  Hence a power outage event can result in simultaneous random access attempts by thousands of devices, which means that all power outage alarms cannot be reliably received within the required latency of milliseconds. There is clearly a need to find efficient mechanisms to address the reporting of a power outage event. One method is to provide a low latency means to temporarily increase the number of random access opportunities available in the system to address the expected surge in network access attempts. The network can also assist in determining the number of meters in outage by initiating a global query upon reception of even a few outage events. Here efficient group based polling mechanisms must be supported to query thousands of devices within the required latency.

Additionally, current smart meter implementations report the power outage alarm by using the “power down location update” provided by the 802.16m standard [2].  However, the original intent of this message was to indicate the manual power down of a device by a human. Given the distinct processing that is required upon reception of a power outage event compared to voluntary power down event, the standard must provide means to distinguish the reporting of this alarm event from the existing power down location update mechanism.

Software Updates & Load Control

The network may also want to send system wide software updates as well as load control signaling to large number of smart metering devices, which will require efficient communication to support a reasonable latency.  Here too, given the limitations on address space and simultaneous network connections, efficient group-based broadcast and multi-cast mechanisms may be required to avoid large delays involved in paging and communicating with a large number of idle devices.

# Requirements for IEEE 802.16p

Based on the use cases discussed in the previous section, we propose that the IEEE 802.16p system should address the following additional requirements.

- Ability to distinguish power outage event from power down events for certain M2M device types.

- Ability to handle simultaneous alarm events from large number of devices.

- Ability to poll a large number of devices in a time and overhead efficient manner.

# Text Proposal

Add the following functional requirements to Section 6.2 of [3] as shown.

---------------------------Begin Text Proposal-------------------------------

## 6.2 Large Numbers of Devices

6.2.x The 802.16p system shall efficiently handle simultaneous alarm events from large number of devices.

6.2.y The system shall have the ability to distinguish a power outage alarm from a voluntary power down event.

6.2.z The system shall have the ability to poll a large number of devices in a time and overhead efficient manner.

-----------------------------------------End Text Proposal-----------------------

# References

- N. Himayat et. al, “Smart grid requirements for IEEE 802.16 M2M networks,” IEEE C802.16ppc-10_0042r2, July 2010.

- IEEE P802.16m/D7

- IEEE 80216ppc-10_0011, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” Initial working document, July, 2010.