| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Suggested Enhancements on Requirements for Low Power Consumption |  |
| Date Submitted | 2010-09-09 |  |
| Source(s) | Kerstin Johnsson, Nageen Himayat, Shilpa Talwar Intel Corp. | E-mail: kerstin.johnsson@intel.com |
| Re: | Requirements for Low Power Consumption |  |
| Abstract | Response to call for contributions on M2M SRD |  |
| Purpose | For review and adoption into the IEEE 802.16p Systems Requirements Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Background

Many M2M use cases require low power consumption. However, the extent of the power restriction varies by use case. Fortunately, for the most “power restrictive” use cases, performance requirements are conversely light. Thus, we can (and should) take advantage of lighter requirements to allow the device to sleep/idle longer and/or transmit less control signaling.

For example, if we know the M2M device is stationary, we can reduce or eliminate mobility control signaling.  If we know the traffic is very delay-tolerant, we can postpone transmission until channel quality is better or to simply maximize the idle/sleep interval. If we know the traffic is completely cyclical, we can create a permanent schedule where sleep/idle interval is maximized, and all “maintenance” is run at the same time as the cyclical traffic.

These are only a few examples of how power can be conserved by taking advantage of “lighter” performance requirements. We want to be sure to identify these opportunities and how they can be taken advantage of in the “Low Power Consumption” requirements section.

# Text Proposal

Modify the functional requirements described in Section 6.1 as shown.

--------------------------  Begin Text Proposal   -------------------------------

## Low Power Consumption

### The 802.16p specification system should shall support mechanisms for low power consumption in M2M devices.

### The 802.16p system should shall support power saving over long time intervals.

### The protocol should support reduced mobility signaling for stationary M2M devices to conserve power The 802.16p system shall provide optimized operation for M2M devices with no/low mobility, to conserve power.

- The 802.16p system shall support a simplified mobility protocol for M2M devices

### The M2M devices that have batteries, which are infrequently recharged or replaced should be supported The 802.16p system shall support optimized operation for time controlled, time-tolerant and infrequent traffic to conserve power.

- The 802.16p system shall support efficient radio resource management for time-controlled and time-tolerant traffic.

### The amendment should support signaling for time controlled, time-tolerant and in-frequent traffic to conserve power

-----------------------------  End Text Proposal   ----------------------------------