| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on Hierarchical Networks Simulation Methodology |  |
| Date Submitted | 2010-09-13 |  |
| Source(s) | Shu-ping Yeh, Shilpa Talwar, Nageen Himayat, Kerstin Johnsson Intel Corporation | E-mail: shu-ping.yeh@intel.com, shilpa.talwar@intel.com |
| Re: | Call for contributions on IEEE 80216ppc-10_0008, “Study Report on Hierarchical Networks.” |  |
| Abstract | This contribution proposes to include the specified simulation methodology for hierarchical networks as an Appendix in the study report on Hierarchical Networks (IEEE 80216ppc-10_0008). |  |
| Purpose | Review and adopt |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Text proposal on Simulation Methodology for Hierarchical Networks

(for inclusion in the study report on Hierarchical Networks)

Shu-ping Yeh, Shilpa Talwar, Nageen Himayat, Kerstin Johnsson

Intel Corporation

Introduction

This contribution proposes that the study report on hierarchical networks, [1], include an Appendix on the simulation methodology required to evaluate the performance of multi-tier networks. In [2], we proposed a simulation methodology for multi-tier networks focusing on femto-cell overlay networks as an example. Here we outline a specific text proposal for inclusion in the study report.

The proposed methodology builds on the IEEE 802.16m evaluation methodology for single tier networks specified in [3]. To recap, the following factors are considered as guidelines in developing the proposed simulation methodology:

- Simple but representative multi-tier (hierarchical) deployment model

The deployment pattern used for femtocell access point (FAP) deployment is environment dependent, e.g., a city or a suburban area can have very different FAP deployment patterns. More specific modeling can give more accurate result but is less extensible to different environment settings.

- Comprehensive channel model

Existing single-tier network simulation methodologies are insufficient to model inter-tier interference. In addition, channel conditions are very different for outdoor and indoor environment. Antenna characteristics and power levels difference for Macro-BS and FAP should also be considered.

- Realistic user distribution

User locations can significantly bias the performance results. Proper indoor versus outdoor user ratio should be selected.

- Practical performance metrics

The performance metrics should demonstrate load balancing between macrocells and femtocells. New metrics such as areal capacity should be considered.

- Reasonable system level simulation (SLS) complexity

SLS complexity grows as the number of FAPs increases. It is important to manage the simulation complexity for the extremely dense urban scenario.

The proposed methodology has the following advantages.

- A simple and unified deployment model for different FAP density situations.

- Fixed outdoor and indoor user ratio to better capture the traffic off-loading to femtocell network.

- Use of different channel models for MBS to indoor users and FAP to outdoor/neighbor users.

References

    - “Study Report on Hierarchical Networks (Working Document),” IEEE 80216ppc-10_0008, July 2010.

    - “Multi-tier Simulation Methodology” IEEE C802.16ppc-10/0039r2.

    - IEEE 802.16m Evaluation Methodology Document (EMD), January 2009

Text Proposal

Insert the following text as an Appendix in [1].

--- ----------------------------Begin Text Proposal---------------------------------------

# A.1 Hierarchical Network Simulation Methodology

This annex will address the simulation methodology for hierarchical network simulations. This simulation methodology addresses the performance gain and interference issues critical to hierarchical network performance evaluation. Multi-tier femto-cell networks are used as a representative case to illustrate the methodology. Note that two possible usage scenarios may be considered for overlay devices serving as access points in multi-tier networks: open access and closed subscriber group (CSG). The open access devices are available to all subscribers. They are usually public infrastructure based, like picocell base stations (BS) and relay stations, and the deployment is planned by the operators. On the contrary, only a restricted group of users have permission to access a CSG device. This is the typical scenario for femtocells since Femto access points (FAP) are usually privately owned and deployed by users.

# A.1.1 Multi-tier Femtocell Overlay Networks

This subsection describes the simulation methodology for femtocell overlay networks, considering both CSG and OSG (open access) types of FAPs. The methodology covers general settings for spectrum usage, traffic model, scheduling, and FAP device parameters. We also describe a deployment model to locate FAP and subscriber stations within the macro-cellular network. In addition, channel models, interference modeling and performance metrics are all addressed in this document.

# A.1.1.1 General Simulation Settings

The simulation methodology for hierarchical networks is built on top of existing 802.16m evaluation methodology document [1]. Parameters specified in [1] should be adopted in simulations for femtocell overlay networks. If not stated explicitly, the simulation setting from [1] will be directly inherited. Additional parameter settings are described in the following.

FAP spectrum usage:

Table 1 FAP spectrum usage

| Scenario | Description |
| --- | --- |
| Co-channel operation | FAPs share the same carrier as MBSs (macro base stations). |
| Separate channel operation | FAPs transmit at different carrier as MBSs. |

The operating band for FAPs can be either the same as MBSs or using a separate spectrum. The co-channel operating case where FAPs share the same spectrum with the MBSs, is the case of most interest as it targets the desired goal of full spectrum reuse between tiers. Additionally, it also fully captures interference issue present in multi-tier deployments.

Traffic Model:

For simplicity, only full buffer traffic is considered at this stage. More advanced traffic patterns may be considered in the future.

Scheduling

Simple round-robin scheduling may be used for initial performance evaluation. More realistic scheduling schemes, like proportional-fair scheduling, may also be considered.

MBS, Subscriber Station and FAP parameters:

For MBSs and subscriber stations, the parameters from [1] are adopted. For FAPs, the parameters are summarized as follows.

Table 2 FAP settings

| Parameters | Value |
| --- | --- |
| Antenna Gain | 0dB |
| Antenna Height | 2 meters + floor height |
| Antenna Pattern | A(θ) = 0 (omni-directional) |
| Maximum Transmit Power Level | -10, 0, 10, 20 dBm |

Macrocell Deployment:

The macrocell deployment model is the same as in [1]. A hexagonal grid with 19 cell sites, each with 3 sectors (Total 57 sectors), is considered. Additional 6 clusters wrap around can be added. The deployment is show in Figure 1.

Figure 1 Macrocells deployment

Two more scenarios, in addition to the baseline settings in [1], should be evaluated.

The small cell scenario:

    - Cell radius = 500 meters (Site to site distance = 866 meters)

    - BS TX Power = 36dBm

The large cell scenario:

    - Cell radius = 1500 meters (Site to site distance = 2598 meters)

    - BS TX Power = 46dBm

# A.1.1.2 FAP and Subscribers Deployment Model

The FAP deployment is illustrated in Figure 2. Single floor circular houses with 10 meters radius are assumed. There is one FAP in each house located at the center of the house. The house locations are determined as follows. First a square grid with 20m minimum separation is formed and then house locations are randomly select from this grid. A fully populated grid has around 538 houses per sector (2500 FAPs/km2).

The FAP density is a programmable parameter. Two representative deployment densities may be considered: dense deployment with about 100 FAPs per sector (~465 FAPs/km2) and sparse deployment with about 10 FAPs per sector (~46 FAPs/km2).

Figure 2. Illustration of FAP deployment

For the subscribers, indoor and outdoor users are separately deployed. Indoor user locations are uniformly distributed within the houses they are in. The probabilities that there are 1, 2, 3 and 4 users per house are 80%, 12%, 6% and 2%, respectively. The outdoor users are uniformly distributed over the area outside of houses, i.e., 10 meters away from all FAPs. The ratio of the number of indoor users to outdoor users is programmable. Typically, there can be equal number of indoor and outdoor users or 70% subscribers being indoors.

For simplicity, all FAPs are assumed to be CSG (or OSG). Only users within the same house as the FAP are assumed to have access permission to the CSG FAP. CSG user will choose between all MBSs and its FAP and pick the serving base station based on association rules such as picking the one with maximum received power at SS. Users not in CSG can only associate with MBSs or OSG FAPs and will choose the serving base station based on the association rule used in the network. The cell association scenario is demonstrated in Figure 3.

Statistics are only collected from SSs associated with MBSs and FAPs locating inside the center cell. However, to take into account the shadowing effect and to better capture the interference behavior, both FAPs and SSs are deployed inside a hexagon with radius equals five times of the macrocell radius.

Figure 3 CSG cell association

# A.1.1.3 Channel Model

- Path loss Models and Shadowing Models

The path loss, shadow fading (SF) and penetration loss under different cases are summarized in Table 3. A combination of ITU channel models [2] and Winner models [2] is used for static channel modeling.

Table 3 Channel Models

|  | Path Loss | SF | Penetration |
| --- | --- | --- | --- |
| Macro-BS to outdoor SS | (>500m) ITU vehicular: 40(1-4×10-3hb)log10(R[km]) + 21log10(f[MHz]) + 80 – 18log10(hb) (≤500m) ITU pedestrian: 40log10(R[km]) + 30log10(f[MHz]) + 49 | 10 dB | 0 |
| Macro-BS to indoor SS | (>500m) ITU vehicular (≤500m) ITU pedestrian | 12 dB | Mean 12dB, Std 8dB |
| Femto-AP to indoor SS | Winner A1 NLOS (through wall): PLfree_space = 46.4 + 20log10(R[m]) + 20log10(f[GHz]/5) | 6 dB | One light wall (3dB) every 3 meters |
| Femto-AP to outdoor SS | Winner A2 NLOS: max( PLfree_space, PLB1) If d<dBP, PLB1 = 41 + 22.7log10(d[m]*) + 20log10(f[GHz]/5) If d≥dBP, PLB1 = 41 + 22.7log10(d[m]*) + 40log10(d[m]/dBP) + 20log10(f[GHz]/5) | 7 dB | PLtw=(14+15(1-cosθ)2 ); PLin = 0.5din; |
| Femto-AP to neighbor SS | Same as above | 7 dB | Above + 12dB wall loss |

*dBP = 4h’BSh’MSfc/c, where fc is the center frequency in Hz, c = 3.0×108 m/s is the propagation velocity in free space, and h’BS and h’MS are computed as h’BS = hBS – 1[m] and h’MS = hMS – 1[m], where hBS and hMS are the actual antenna heights and the effective environment height in urban environments is assumed to be equal to 1 meter.

Fast fading models in [2] and [2] can also be used to model the fast fading effects in femtocell overlay networks.

# A.1.1.4 Interference Modeling

The interference modeling for dynamic simulation generally follows [1]. The following describes the procedure, and the differences are highlighted in bold.

- Determine the path loss, BS/FAP antenna gain, and shadowing from all interfering sectors and FAPs to MS.

- Rank the interfering sectors and FAPs in order of received power (based on pathloss, BS/FAP antenna gain, and shadowing).

- Model the channels of the strongest (strong I ) interferers as the signal path. (account for the pathloss, BS antenna gain, shadowing, and fast fading variations.) The value of strong I is set to 8 for MBSs and 18 for FAPs.

- Model the remaining sectors as spatially white Gaussian noise processes whose variances are based on a spectrally flat Rayleigh fading process. The power of the Rayleigh fading process includes the effects of pathloss, BS antenna gain, and shadowing. The fading processes for all links between MS and BS are assumed to be independent, and the Doppler rate is determined by the speed of the mobile. At any instant in time, the total received interference power is the summation of the receive power from of all weak interferers. Hence, the interference power is varying in time during a simulation drop.

# A.1.1.5 Performance Metrics

The following metrics should be considered for multi-tier network performance evaluation.

  - 5% outdoor throughput

  - 5% indoor throughput

  - 50% outdoor throughput

  - 50% indoor throughput

  - Overall areal throughput

  - Percentage of subscribers associated with FAPs.

An example of simulation results is shown in the following table:

Table 4 Example Simulation Results (~ 10 FAPs per sector, 48% SS with FAPs)

| FFR, w/ FFZ | FFZ ratio | 5% Outdoor Rate (Mbps) | 5% Indoor Rate (Mbps) | 50% Outdoor Rate (Mbps) | 50% Indoor Rate (Mbps) | Center Cell Throughput |
| --- | --- | --- | --- | --- | --- | --- |
| Macro Only |  | 0.0753 | 0.0537 | 0.4164 | 0.4556 | 44.4224 |
| FAP Tx Pwr = 10dBm | 4/8 | 0.1319 | 1.5578 | 0.4182 | 13.7848 | 809.7991 |
| Macro Only |  | 0.0762 | 0.0544 | 0.4176 | 0.4539 | 44.3857 |
| FAP Tx Pwr = 10dBm | 5/8 | 0.1716 | 1.2557 | 0.4558 | 10.3334 | 613.4495 |
| Macro Only |  | 0.0762 | 0.0534 | 0.4199 | 0.4545 | 44.3714 |
| FAP Tx Pwr = 10dBm | 6/8 | 0.1842 | 1.0322 | 0.4464 | 7.1107 | 417.1955 |
| Macro Only |  | 0.0756 | 0.0539 | 0.4190 | 0.4510 | 44.4351 |
| FAP Tx Pwr = 10dBm | 7/8 | 0.1352 | 0.6857 | 0.4501 | 3.5303 | 220.1630 |

# A.1.1.6 References

- IEEE 802.16m Evaluation Methodology Document (EMD), January 2009

- “Guidelines for evaluation of radio transmission technologies for IMT-2000”, Rec. ITU-R M.1225

- IST-WINNER II Deliverable D1.1.1 V1.1, WINNER II interim channel models, September 2007.

-------------------------------End Text Proposal---------------------------------------