| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | [DRAFT] Response to ITU-R WP5A on “Mobile Wireless access systems providing telecommunications for a large number of ubiquitous sensors and/or actuators scattered over wide areas in the land mobile service” |  |
| Date Submitted | 2010-09-14 |  |
| Source(s) | Shilpa Talwar, Kerstin Johnsson, Nageen Himayat, Jose Puthenkulam Intel Corporation | Email: shilpa.talwar@intel.com |
| Re: | Response to ITU-R Liaison Group |  |
| Abstract | The objective of this contribution is to inform ITU-R WP5A of 802.16p project on “Enhancements to Support Machine to Machine Applications” |  |
| Purpose | For approval by PPC working group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Introduction

IEEE 802.16 project planning committee (PPC) has reviewed the document from ITU-R Working Party 5A on “Mobile Wireless access systems providing telecommunications for a large number of ubiquitous sensors and/or actuators scattered over wide areas in the land mobile service” [1].  The IEEE PPC would like to inform the ITU-WP5A about our related project on enhancements to the IEEE 802.16 air interface to support machine-to-machine (M2M) communications [2].  Developing M2M capability in broadband wireless access networks will accelerate implementation of the “Internet of things” vision.

We started the M2M project in PPC as a result of an IEEE tutorial in March 2010 on the challenges and potential technology opportunities in future 802.16 networks [3].  The tutorial identified M2M services and applications as a key area for development and as an opportunity to enable operators to create new revenue streams. This initiated a study group in March with the objective of identifying M2M usages and applications, M2M features that are common across multiple applications, M2M system architecture, and potential optimizations to the IEEE 802.16 air interface [4]. While the study group wrote a comprehensive report on the scope and standards implications for a broad range of M2M applications, it was recommended that the IEEE 802.16 working group consider a phased approach to address M2M requirements. Specifically, near term requirements would be addressed as part of the initial project scope to enable basic M2M capability to quickly enable the market, and longer term advanced requirements would be addressed in a follow-on project.

Based on recommendations from the M2M study group, a project scope was drafted which was submitted to the 802 Executive Committee for approval in July. The new project, IEEE 802.16p, was authorized by the Executive Committee and work has begun on drafting system requirements [5].

- 802.16p Project

The objective of this project is to enable a range of machine-to-machine applications that are automated rather than human-controlled, which require WWAN communication in licensed bands. These applications have network access requirements that are significantly different from those used to support standard cellular connections. Some of the M2M applications of interest include secured access and surveillance, tracking, tracing and recovery, public safety, vehicular telematics, healthcare monitoring of bio-sensors, remote maintenance and control, smart metering, automated services on consumer devices, and retail digital signage. The current IEEE 802.16 standard does not address the unique requirements posed by these usages, such as very low power consumption, large numbers of devices, short burst transmissions, robust device integrity, high reliability, staged access priority, time tolerant operation, time controlled operation, low/no mobility, extremely low latency, and others.

The 802.16p project will specify enhancements to the IEEE 802.16 medium access control (MAC) and minimal modifications to the IEEE 802.16 Physical Layer (PHY), which is based on orthogonal frequency division multiple access (OFDMA). Minimal PHY changes are required to avoid hardware changes to base station/device equipment that may otherwise prevent quick entry into the M2M market. In this first stage of M2M optimizations, the following features will be supported: low power consumption at the device, support of extremely large numbers of devices, efficient small burst transmissions, and improved device authentication. While these features are not all-encompassing to the M2M applications space, they are common to several key applications, such as smart metering, remote maintenance and control, etc.

Figure 1 shows the basic system architecture that is assumed for IEEE 802.16 based M2M communications. In this figure, the IEEE 802.16 M2M device is an IEEE 802.16 mobile station with M2M functionality. The M2M server is an entity that communicates to one or more IEEE 802.16 M2M devices. The M2M server also has an interface which can be accessed by an M2M service consumer. The M2M service consumer is a user of M2M services (e.g. a utility company). The M2M server may reside within or outside the Connectivity Service Network (CSN) and can provide M2M specific services for one or more M2M devices. An M2M application runs on the M2M device and the M2M server.

Figure 1.  High-level M2M service system architecture

The M2M service system architecture in IEEE 802.16 supports two types of M2M communication: a) communication between one or more M2M devices and an M2M server, and b) point-to-multipoint communication between M2M devices and the BS. An IEEE 802.16 M2M device can act as an aggregation point for non IEEE 802.16 M2M devices. The non M2M devices typically comprise one or more sensors and a low power, low cost connectivity solution such as IEEE 802.11, IEEE 802.15, etc. The aggregation functionality will be critical to M2M deployments since not all devices will be able to afford cellular connectivity, and the aggregation device will provide an efficient and cost-effective means of supporting a large number of localized devices/sensors.

- Discussion

In general, IEEE 802.16’s PPC is aligned with ITU-R WP5A on the wireless system features needed to enable machine-to-machine usages [1].  There are, however, some items for which we have comments, and some that require further clarification:

- Long-term connectivity of the network interface: It is not clear whether longevity requirement is 5 to 10 years or several decades?

- Communication range: The text calls for space diversity techniques to improve coverage.  In [3], several architectures and enabling technologies are discussed that can improve coverage, such as femtocell networks, multi-RAT networks, relays, coordinated MIMO, and others. Are these within scope?

- Accommodation capability: In our view, a video camera for surveillance applications may be regarded as a sensor. In this case, the data generated may be large, and can potentially challenge system capacity available. In our view the system should be optimized for small data bursts but also accommodate large data transmissions.

- Suppression of interference: In addition to the techniques mentioned in [1] for interference mitigation, smart resource scheduling (time/frequency resources) and/or MIMO techniques can also be extremely effective, as discussed in [3].

- Security: Since the M2M device may be deployed remotely over long time periods, the network should provide support for the device to report if there is tampering or damage to the device.

The PPC would like to initiate liaison with ITU-R WP5A, and exchange information on status and progress between the two groups, given our common goals.

Our comments are included in attached documents.

- References

[1] R07-WP5A-100510-TD-0201/0202, “Mobile wireless access systems for communications to a large number of ubiquitous sensors and/or actuators scattered over wide areas in the land mobile service  (Question ITU-R 250/5)”

[2] IEEE 802.16ppc-10/0003r9, Machine to Machine (M2M) Communication PAR Form and Five Criteria, May. 2010

[3] IEEE C80216-10_0016r1, “Future 802.16 network: Challenges and Possibilities,” IEEE 802 Call for Interest Tutorial, March, 2010.

[4] IEEE C80216-10_0002r7, “Machine to Machine (M2M) Communication Study Report,” IEEE 802.16 Contribution, May, 2010.

[5] IEEE C80216-10_0011r1, “IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD),” IEEE 802.16 Contribution, September, 2010.