| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed texts for protocol structure of hierarchical network project |  |
| Date Submitted | 2011-03-11 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Heejeong Cho LG Electronics | E-mail: eunjong.lee@lge.com, youngsoo.yuk@lge.com, heejeong.cho@lge.com |
| Re: |  |  |
| Abstract | This contribution proposes inclusion of protocol structure for Hierarchical Network Project |  |
| Purpose | For discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed texts for protocol structure of hierarchical network project

Eunjong Lee, Youngsoo Yuk and Heejeong Cho

- LG Electronics Inc.

# Introduction

The HN study report is to clarify the scope and the goal of the HN project and protocol structure is useful to clarify the scope of the HN project. So, we propose to include protocol structure in the HN study report. Our proposed structure is based on the protocol structure described in SDD (System Description Document) of IEEE802.16m [1]. In addition, the network architecture of the single-RAT system is not needed for the study report and the figure 7 and 8 need to be updated.

So, this contribution proposes to modify the HN study report as follows.

- To re-organize the section 3

- Change the section title from Network Architecture as System Architecture

- Remove “single-RAT Network Architecture” section

- Insert “Protocol Structure” section

  - New figure (from SDD of IEEE802.16m) for protocol structure is included.

  - New functional blocks from IEEE802.16m are proposed, and some descriptions are provided.

- To update the figure 7 and 8

- Replace “?” by “multi-RAT interface” (which is used in IEEE802.16m SDD) in the figure 7 and 8

- Remove Converged Gateway in the figure 8

# References

[1] 80216m-09_0034r4, “IEEE 802.16m System Description Document (SDD),” 11, Dec. 2010.

# Text proposal for the HN study report

--------------------------------------------- Start of the Proposal Text  -----------------------------------

# Hierarchical Network System Network Architecture

## Protocol Structure Single-RAT Network Architecture

Figure x shows the IEEE 802.16 HN Protocol Structure.

Figure x: IEEE 802.16 HN Protocol Structure

### Single-RAT Protocol Structure Protocol Architecture

### Multi-RAT Protocol Structure

In order to support Multi-RAT hierarchical network operation, following functional blocks are included over existing IEEE 802.16m protocol.

- Multi-RAT Discovery & Selection Management

- Multi-RAT Service Flow Management

- Multi-RAT Connection Management

- Multi-RAT Handover Management

- Multi-RAT Coordination

The Multi-RAT Discovery & Selection Management block manages scanning of neighbor multi-RAT networks (APs) or multi-RAT devices, and includes function selecting a specific AP or cooperative device. The Multi-RAT Discovery & Selection Management block also supports function providing the information on neighbor Multi-RAT devices/networks.

The Multi-RAT Service Flow Management block manages service flow related operations. The operations include QoS-aware mapping of service flows to different Multi-RAT connections and IP flow management to different Multi-RAT carriers, etc.

The Multi-RAT Connection Management block is in charge of establishment and release for the Multi-RAT connection(s) between AP and device or between devices. The Multi-RAT Connection Management block also handles connection loss between devices.

The Multi-RAT Handover Management block supports functions related to performing Multi-RAT handover and Inter-AP handover.

The Multi-RAT Coordination block is in charge of configuring transmission parameters of APs, AP registration/updating and Inter-AP coordination.

In order to support multi-RAT hierarchical network operation, the functions in the following blocks may be enhanced.

- Control Signaling

- Network Entry Management

- Multi-Radio Coexistence

- Security Management

- Data forwarding

Figure 7 shows the example of protocol architecture of multi-RAT devices

Figure 1: Example of Protocol architecture of Multi-RAT devices

## Multi-RAT Network Architecture

### 3.2.1 System Architecture

Figure 2: Network System architecture of Multi-RAT system devices

--------------------------------------------- End of the Proposal Text 2 -----------------------------------