| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed texts for protocol structure of hierarchical network project |  |
| Date Submitted | 2011-03-16 |  |
| Source(s) | Eunjong Lee, Youngsoo Yuk and Heejeong Cho LG Electronics Peretz Feder, Dan Gal ALU | E-mail: eunjong.lee@lge.com, youngsoo.yuk@lge.com, heejeong.cho@lge.com |
| Re: |  |  |
| Abstract | This contribution proposes inclusion of protocol structure for Hierarchical Network Project |  |
| Purpose | For discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed texts for protocol structure of hierarchical network project

Eunjong Lee, Youngsoo Yuk and Heejeong Cho

- LG Electronics Inc.

# Introduction

The HN study report is to clarify the scope and the goal of the HN project and protocol structure is useful to clarify the scope of the HN project. So, we propose to include protocol structure in the HN study report. Our proposed structure is based on the protocol structure described in SDD (System Description Document) of IEEE802.16m [1]. In addition, the network architecture of the single-RAT system is not needed for the study report and the figure 7 and 8 need to be updated.

So, this contribution proposes to modify the HN study report as follows.

- To re-organize the section 3

- Change the section title from Network Architecture as System Architecture

- Remove “single-RAT Network Architecture” section

- Insert “Protocol Structure” section

  - New figure (from SDD of IEEE802.16m) for protocol structure is included.

  - New functional blocks from IEEE802.16m are proposed, and some descriptions are provided.

- To update the figure 7 and 8

- Replace “?” by “L2_5” (which is used in IEEE802.16m SDD) in the figure 7 and 8

- Remove Converged Gateway in the figure 8

# References

[1] 80216m-09_0034r4, “IEEE 802.16m System Description Document (SDD),” 11, Dec. 2010.

# Text proposal for the HN study report

--------------------------------------------- Start of the Proposal Text  -----------------------------------

# Hierarchical Network System Network Architecture

## Protocol and Functional Structure Single-RAT Network Architecture

Figure x shows the IEEE 802.16 HN functional entities within the Protocol Structure.

Figure x: IEEE 802.16 HN Functional decomposition within the Protocol Structure

### Single-RAT Protocol and Functional Structure Protocol Architecture

### Multi-RAT Protocol and Functional Structure

In order to support Multi-RAT hierarchical network operation, the following functional blocks are included within the IEEE 802.16m protocol.

- Multi-RAT Discovery & Selection Management

- Multi-RAT Service Flow Management

- Multi-RAT Connection Management

- Multi-RAT Handover Management

- Multi-RAT Coordination

The Multi-RAT Discovery & Selection Management entity helps the network discovery process managing the scanning procedures of neighboring multi-RAT networks (APs) or multi-RAT devices. It may include query and response to/from an information server containing data related to pertinent accessible networks and/or a function providing pertinent information on neighboring Multi-RAT devices/networks. The Multi-RAT Discovery & Selection Management entity also supports a function that selects a specific AP or cooperative device.

The Multi-RAT Service Flow Management entity manages service flow related operations. The entity is QoS-aware and enables mapping of service flows to the various Multi-RAT connections and IP flow management to different Multi-RAT carriers, etc.

The Multi-RAT Connection Management entity is enabling the establishment and release of the Multi-RAT connection(s) between AP and device or between peer devices. The Multi-RAT Connection Management entity also handles connection loss.

The Multi-RAT Handover Management entity supports functions related to performing Multi-RAT handovers and Inter-AP handovers.

The Multi-RAT Coordination entity can enable the configuring of transmission parameters of APs, enables AP registration/updating and Inter-AP coordination.

In order to support multi-RAT hierarchical network operation, the functions in the following entities may be enhanced.

- Control Signaling

- Network Entry Management

- Multi-Radio Coexistence

- Security Management

- Data forwarding

Figure 7 shows the example of protocol architecture of multi-RAT devices

Figure 7: Example of Protocol architecture of Multi-RAT devices

## Multi-RAT Network Architecture

### 3.2.1 System Architecture

Figure 8: Network System architecture of Multi-RAT system devices

--------------------------------------------- End of the Proposal Text 2 -----------------------------------