| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal for interference management of cooperative small base-stations in hierarchical networks |  |
| Date Submitted | 2011-11-05 |  |
| Source(s) | Yong-Up Jang, Seungbae Kim, Ju Yong Lee, Dong-Ho Cho KAIST IT Convergence Jaeweon Cho Samsung Electronics | E-mail: yujang@itc.kaist.ac.kr, sbkim@itc.kaist.ac.kr, jylee@itc.kaist.ac.kr, dhcho@kaist.ac.kr jaeweon.cho@samsung.com |
| Re: | Call for contributions for the 802.16 PPC |  |
| Abstract | It proposes interference management of cooperative small base-stations in Hierarchical Networks Study Report. |  |
| Purpose | For discussion |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal for interference management of cooperative small base-stations in hierarchical networks

Yong-Up Jang, Seungbae Kim, Ju Yong Lee, Dong-Ho Cho

KAIST IT Convergence

Jaeweon Cho

Samsung Electronics

# Introduction

This contribution suggests interference management for cooperative small base-stations in C802.16ppc-11/0004r1, where wireless back-haul is adopted to easily deploy the cooperative small base-stations including micro, pico, and femto BSs. The proposed text is described in the usage model section and Figure 2 is modified accordingly.

# Proposed text change

Adopt the following remedies in IEEE 802.16ppc-11/0004r1.

---------------------------------- The start of text -----------------------------------------

[Remedy #1: Replace Figure 2, and modify the texts as indicated.]

# 2  Usage Models

## 2.1  Single Radio Access Technology

### 2.1.1  Multi-tier Networks

Multi-tier networks refer to a hierarchical or overlay deployments of cells which may have increasingly base stations of smaller sizes: macro base station, micro base station, pico base station, femto base station, and relay base station (Error: Reference source not found). The hierarchy shown in Error: Reference source not found is not strict and is mainly illustrative of the increasingly smaller cell sizes that may be included as part of a multi-tier deployment.  For example, in a given deployment, a 2 tier hierarchical relationship may exist between a Macro and both pico and/or a femto cells, with the pico and femto cells comprising the same tier in the hierarchy. Typical deployment would consist of the tiers operating on the same radio access technology (RAT). Femto and relay are included as a part of the IEEE 802.16m specification [Error: Reference source not found]. Coordination techniques between devices and between cells across the multiple tiers and among cooperating small BSs are important aspects of multi-tier network design to achieve cell capacity enhancements and interference mitigation techniques across tiers are also critical to achieving the user throughput enhancements. However, it is not fully enabled in IEEE 802.16m because multiple types of base stations (including Macro, Pico, Femto, Relay) will be deployed in multi-tier networks and this will cause more complicated interference among tiers. Therefore enhanced interference mitigation and coordination techniques are needed with the enhanced channel measurement, downlink signaling, and uplink feedback schemes, etc. In particular, multi-tier deployments affect the areal capacity (bps/Hz/square meters) of the system due to the deployment of significantly more cells in a given area. The highest tier macro cells are still needed to provide broader coverage and seamless mobility.

Figure 1: An example of Single RAT Hierarchical (Multi-tier) Architecture Framework

### 2.1.1.1  Multi-Tier Deployment Scenarios

As indicated, small base stations may be used in a hierarchical network deployment. The type and location of these base stations will play a significant role in determining the cost and performance of multi-tier deployments. For example, indoor femto cell deployments can utilize the existing back-haul thereby significantly lowering the cost of such deployments. With outdoor pico-cellular deployments, the operator will need to provide back-haul capability and manage more critical spectrum reuse challenges. Especially, the wireless back-haul can be used to reduce CAPEX/OPEX between macro BS and small BSs, and to ease the deployment of small BSs. As mentioned in section 1 and shown in Figure 2, a group of small BSs cooperates to increase the system coverage and capacity. To efficiently form and manage a cluster of cooperative small BSs, the interference management among cooperative small BSs is of high importance. Other deployment models cover indoor enterprise or outdoor campus deployments that may impose different manageability and reliability requirements. Multi-tier deployments across this range of scenarios are not fully addressed by the IEEE 802.16m standard.

…

#### Advanced Interference Management for Multiple Carrier Deployment

Control/data channel interference shall be mitigated to enable multiple carrier multi-tier deployment, where the cells are deployed on multiple available carriers. Large cells and small cells can be deployed over a distinct set of carriers to avoid interference across the tier of large cells and the tier of small cells. Large and small cells can also be deployed over the same set of carriers or having carriers overlapped, where resource (carrier, power, time, space, etc.) allocation can be made, to avoid or mitigate the interference across tiers. Enhanced interference management schemes like joint scheduling and coordinated beamforming can be used for managing interference among cooperating small BSs.

Carrier Aggregation (CA) shall be supported with the considerations interference mitigation or avoidance across tiers. This will account for the overlaying macrocell, overlaid pico and femto cells (including CSG cells). Multi-carrier system may work together with SON algorithms.

…

---------------------------------- The end of text -----------------------------------------