| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16p Machine to Machine (M2M) System Requirements Document (SRD) (initial working document revised) |  |
| Date Submitted | 2010--09-10 |  |
| Source(s) | Jin Lee (M2M System Requirements Editor) Jose Puthenkulam (Project Planning Committee Chair) | Email: jin1.lee@lge.com E-mail: jose.p.puthenkulam@intel.com |
| Re: |  |  |
| Abstract | Initial IEEE 802.16p Machine to Machine (M2M) System Requirements Document |  |
| Purpose | Initial M2M System Requirements working document to enable capturing the requirements for the prospective IEEE 802.16p Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Machine to Machine (M2M) System Requirements Document

1	Overview	3

2	References	3

3	Definitions	3

4	Abbreviations and Acronyms	3

5	General Requirements	3

5.1	Compatibility with other 802.16 equipment	3

5.2	Complexity	3

5.3	M2M Services	4

5.4	802.16p M2M Reference System Architecture	4

6	Functional Requirements	4

6.1	Low Power Consumption	5

6.2	Large Numbers of Devices	5

6.3	Small Burst Transmissions	6

6.4	Security Support	6

# Overview

The 802.16p amendment shall be developed in accordance with the Machine to Machine (M2M) communication project authorization request (PAR) form and five criteria the approved version of IEEE 802.16ppc-10/0003r7 [1].

This document represents the initial high-level system requirements for the 802.16p amendment.

# References

[1] IEEE 802.16ppc-10/0003r7: Machine to Machine (M2M) Communication PAR Form and Five Criteria (Not approved yet).

[2] IEEE Std 802.16-2009: IEEE Standard for Local and metropolitan area networks – Part 16: Air Interface for Fixed Broadband Wireless Access Systems, May 2009.

[3] IEEE P802.16m: IEEE Standard for Local and metropolitan area networks – Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems

[4] IEEE 802.16ppc-10/0002r7: Machine to Machine (M2M) Communication Study Report.

[5]	IEEE P802.16m System Requirements Document (SRD)

# Definitions

| Terms | Description |
| --- | --- |
| M2M device | Embedded device with wireless WAN connectivity |
| Machine-to-Machine (M2M) | This is information exchange between devices (subscriber station) through a base station, or between a device and a server in the core network through a Base Station that may be carried out without any human interaction. |

# Abbreviations and Acronyms

| M2M | Machine to Machine |
| --- | --- |

# General Requirements

## Compatibility with other 802.16 equipment

IEEE 802.16p BS provides continuing support for WirelessMAN-Advanced Air Interface and WirelessMAN-OFDMA R1 Reference System equipment.

## Complexity

IEEE 802.16p provides IEEE Std 802.16 medium access control (MAC) enhancements and minimal OFDMA PHY modifications to provide functionalities for efficient Machine to Machine communication.

## M2M Services

IEEE 802.16p shall support services required for M2M applications (e.g. smart grid, applications on consumer devices, vehicular tracking and healthcare). Detailed M2M services are specified in IEEE 802.16ppc-10/0002r7 Machine to Machine (M2M) Communication Study Report.

## 802.16p M2M Reference System Architecture

- 802.16p M2M service reference system architecture

# Functional Requirements

## Low Power Consumption

### The 802.16p  system shall support mechanisms for low power consumption in M2M devices.

### The 802.16p system  shall support power saving over long time intervals.

### The 802.16p system shall provide optimized operation for M2M devices with no/low mobility to conserve power.

### The 802.16p system shall support optimized operation for time controlled, time tolerant and in-frequent traffic to conserve power.

## Large Numbers of Devices

The 802.16p  system  shall support very large numbers of devices.

### The 802.16p system shall be able to identify large number of devices individually or by group.

### The 802,16p system  shall support efficient group management for large numbers of M2M devices.

### The 802.16p system shall support optimized signaling for resource allocation for large numbers of M2M devices individually or by group.

### The 802.16p system shall support efficient and reliable control and data delivery to and from groups of devices.

### The 802.16p system shall support efficient mechanisms for network access from large numbers of M2M devices.

### [The 802.16p system shall support network access from large number of M2M devices, without incurring excessive delays. The 802.16p shall provide [sufficient capacity][capacity enough] to support random access by large number of devices.]

### The 802.16p system shall provide mechanisms for allowing access with different levels of priority for M2M devices across different classes of M2M services.

### The 802.16p  system shall support large numbers of M2M devices in connected state as well as in idle state.

### The 802.16p system shall support various QoS levels for M2M services.

## Small Burst Transmissions

The 802.16p system shall support very small burst transmissions.

### The 802.16p system shall support efficient transmission of small burst sizes. The 802.16p system shall minimized protocol overhead to support small burst transmission.

## Security Support

### [The 802.16p system shall support improved the M2M device authentication.] [802.16p system shall support an M2M device authentication which includes a device validity check in cooperation between the device and the network.]

### The 802.16p system shall support efficient security for small burst transmissions.