| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Demodulation and Forwarding method in Relay Station |  |
| Date Submitted | 05-Jul-2007 |  |
| Source(s) | Su Chang Chae, Young-il Kim, Hyunjae Kim ETRI Kyu Ha Lee, Changyun Kim, Yong Wook Lee Samsung Thales Aik Chindapol Siemens Adrian Boariu Nokia | schae@etri.re.kr kyuha.lee@samsung.com Aik.Chindalpol@siemens.com Adrian.Boariu@nokia.com |
| Re: | Call for Technical Proposals regarding IEEE project P802.16j |  |
| Abstract | Propose demodulation and forwarding method within a single frame in RS and change the technical tables. |  |
| Purpose | Adoption of the proposed text and tables |  |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate text contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures (Version 1.0) <http://ieee802.org/16/ipr/patents/policy.html>, including the statement “IEEE standards may include the known use of patent(s), including patent applications, if there is technical justification in the opinion of the standards-developing committee and provided the IEEE receives assurance from the patent holder that it will license applicants under reasonable terms and conditions for the purpose of implementing the standard.” Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:r.b.marks@ieee.org> as early as possible, in written or electronic form, of any patents (granted or under application) that may cover technology that is under consideration by or has been approved by IEEE 802.16. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |  |

Demodulation and Forwarding Method in Relay Station

Su Chang Chae, Young-il Kim, Hyunjae Kim, Kyu Ha Lee*, Changyun Kim*, Yong Wook Lee*, Aik Chindapol**, Adrian Boariu***

ETRI, Samsung Thales*, Siemens**, Nokia***

# 1. Introduction

We propose the demodulation and forwarding method in RS. The demodulation and forwarding method means that RS performs only demodulation without channel decoding for received signals and then modulation only without channel encoding procedure while forwarding the data in modem of RS.

We are expected to use this demodulation and forwarding method assuming simple RS optionally. If we may use this forwarding method, we can also change modulation order of forwarded data by RS. This forwarding method can change only modulation order without changing of channel coding rates. Therefore, all of the modulation types of BS-to-RS should include a variety of channel coding rates of RS-to-MS.

Finally, we would like to change the technical tables of UCD and DCD burst profile encoding tables and CTC channel coding per modulation tables.

# 2. Suggested Remedy

## 2.1 Demodulation and Forward

There have two methods to forward data in RS, which are decoding and forwarding method and demodulation and forwarding method. We may apply decoding and forwarding method to the full functional RS in the figure1, demodulation and forwarding method to simple RS in the figure2. In the figure2, the bit selection can be inserted to support IR-HARQ. And also, we may apply decoding and forwarding method to channel status such as coverage extension and demodulation and forwarding method to channel status such as coverage hole in the figure3.

We would like to consider only demodulation and forwarding method in RS. Demodulation and forwarding method have main procedural blocks of FFT, QAM de-mapping, QAM mapping and IFFT without channel decoding and encoding procedures. But, we can change the modulation order in QAM mapping block prior to regeneration to forward data. For example, received data of having a 16QAM and 1/2 code rate can be changed to QPSK, 1/2 without changing code rate of 1/2.

Figure1. Decoding and Forwarding method in full functional RS

Figure2.Demodulation and Forwarding method in simple RS

## 2.2 Usage Scenario

Figure3 shows you usage scenario considering MS’s position.

There are three kinds of MS which are in the good channel, coverage hole and coverage extension area. MS in the good channel can be directly communicated with MMR-BS. MS in the coverage hole can be applied to demodulation and forwarding method in RS. And coverage extension can be applied to decoding and forwarding method in RS.

For example, in the case of using the demodulation and forwarding in RS for downlink, you can see that dark blue colored line indicates a BS-to-RS link and light blue colored line indicates a RS-to-MS link. It has not change code rate and encoding packet size, but modulation order of 64 QAM has changed by QPSK which can be forwarded to MS. Then, all of the MS should have channel decoding procedure.

Figure3. Usage Scenario for Demodulation and Forwarding Method

## 2.3 Operation Scenario

The figure4 depicts that burst#1 at the BS-to-RS region can be forwarded to burst#2 at the RS-to-MS region and burst#3 at the BS-to-RS region can be forwarded to burst#4 at the RS-to-MS region.

If burst#2’s MCS should have QPSK, 1/2 code rate, burst#1’s MCS may have 16QAM or 64QAM without changing of code rates. And also, if legacy MS should have MCS of 64QAM, 5/6 code rate at the RS-to-MS, burst#3’s MCS may have 16QAM, 5/6. However, the legacy specifications of MS have not defined 5/6 code rate for 16QAM modulation type.

Finally, we may define all of the modulation types and channel coding rate for BS-to-RS to include a variety of modulation type and channel coding rates specifying for the legacy MS.

Figure4. Burst allocation for Demodulation and Forwarding

## 2.4 Simulation Results

Figure5 shows you simulation results of additional MCS for down link. We think that the simulation result of uplink is similar to downlink. Our simulation environments have permutation type of PUSC, two hop, antenna type of 1x1, RS relaying method of demodulation and forward and channel status of good channel(using the high SNR of 30dB) for relay link and bad channel(using lower SNR of 6dB~20dB than that of BS-to-RS link) for access link, respectively.

In the figure5, the blue color has the simulation result in case of using the 64-QAM, code rate of 2/3 for BS-to-RS link and QPSK, code rate of 2/3 for RS-to-MS link. The red color has simulation result in case of using the QPSK, code rate of 2/3 for BS-to-RS link and 64-QAM, code rate of 2/3 for RS-to-MS link.

Figure5. Simulation Results of additional MCS

# 3. Proposed Text Change

[Insert the following this entry at TLV Encoding wirelessMAN-OFDMA]

## 11.3.1.1 Uplink burst profile encodings

[Change Table 357 in 11.3.1.1 “Uplink burst profile encodings”, as shown:]

This following Table 357 should be applied to the MR BS and RS for relay uplinks as a FEC code type and modulation type and its values.

Table 357-UCD burst profile encodings - WirelessMAN-OFDMA

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| FEC Code Type and modulation type | 150 | 1 | 0=QPSK(CC) 1/2 1=QPSK(CC)3/4 2=16-QAM(CC)1/2 3=16-QAM(CC)3/4 4=64-QAM(CC)1/2 5=64-QAM(CC)2/3 6=64-QAM(CC)3/4 7=QPSK(BTC)1/2 8=QPSK(BTC)3/4 9=16-QAM(BTC)3/5 10=16-QAM(BTC)4/5 11=64-QAM(BTC)5/8 12=64-QAM(BTC)4/5 13=QPSK(CTC)1/2 14=QPSK(CTC)3/4 15=16-QAM(CTC)1/2 16=16-QAM(CTC)3/4 17=64-QAM(CTC)1/2 18=64-QAM(CTC)2/3 19=64-QAM(CTC)3/4 20=64-QAM(CTC)5/6 21=QPSK(ZT CC) 1/2 22=QPSK(ZT CC)3/4 23=16-QAM(ZT CC)1/2 24=16-QAM(CC)3/4 25=64-QAM(ZT CC)1/2 26=64-QAM(ZT CC)2/3 27=64-QAM(ZT CC)3/4 28=QPSK(LDPC)1/2 29= QPSK(LDPC)2/3 A code 30=QPSK(LDPC)3/4 A code 31=16-QAM(LDPC)1/2 32=16-QAM(LDPC)2/3 A code 33=16-QAM((LDPC)3/4 A code 34=64-QAM(LDPC)1/2 35=64-QAM(LDPC)2/3 A code 36=64QAM(LDPC)3/4 A code 37=QPSK(LDPC)2/3 B code 38= QPSK(LDPC)3/4 B code 39=16-QAM((LDPC)2/3 B code 40=16-QAM((LDPC)3/4 B code 41=64-QAM(LDPC)2/3 B code 42=64QAM(LDPC)3/4 B code 43=QPSK(LDPC)5/6 44=16-QAM(LDPC)5/6 45=64-QAM(LDPC)5/6 46=QPSK(CTC)2/3 47=QPSK(CTC)5/6 48=16-QAM(CTC)2/3 49=16-QAM(CTC)5/6 50..255=Reserved |

UCD burst profile values 46 - 49 shall not be applied to MS.

[Insert the following this entry at TLV Encoding wirelessMAN-OFDMA]

## 11.4.2 Downlink burst profile encodings

[Change Table 363 in 11.4.2 “Downlink burst profile encodings”, as shown:]

This following Table 363 should be applied to the MR BS and RS for relay downlinks as a FEC code type and modulation type and its values.

Table 363-DCD burst profile encodings - WirelessMAN-OFDMA

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| FEC Code Type and modulation type | 150 | 1 | 0=QPSK(CC) 1/2 1=QPSK(CC)3/4 2=16-QAM(CC)1/2 3=16-QAM(CC)3/4 4=64-QAM(CC)1/2 5=64-QAM(CC)2/3 6=64-QAM(CC)3/4 7=QPSK(BTC)1/2 8=QPSK(BTC)3/4 9=16-QAM(BTC)3/5 10=16-QAM(BTC)4/5 11=64-QAM(BTC)5/8 12=64-QAM(BTC)4/5 13=QPSK(CTC)1/2 14=QPSK(CTC)3/4 15=16-QAM(CTC)1/2 16=16-QAM(CTC)3/4 17=64-QAM(CTC)1/2 18=64-QAM(CTC)2/3 19=64-QAM(CTC)3/4 20=64-QAM(CTC)5/6 21=QPSK(ZT CC) 1/2 22=QPSK(ZT CC)3/4 23=16-QAM(ZT CC)1/2 24=16-QAM(CC)3/4 25=64-QAM(ZT CC)1/2 26=64-QAM(ZT CC)2/3 27=64-QAM(ZT CC)3/4 28=QPSK(LDPC)1/2 29= QPSK(LDPC)2/3 A code 30=QPSK(LDPC)3/4 A code 31=16-QAM(LDPC)1/2 32=16-QAM(LDPC)2/3 A code 33=16-QAM((LDPC)3/4 A code 34=64-QAM(LDPC)1/2 35=64-QAM(LDPC)2/3 A code 36=64QAM(LDPC)3/4 A code 37=QPSK(LDPC)2/3 B code 38= QPSK(LDPC)3/4 B code 39=16-QAM((LDPC)2/3 B code 40=16-QAM((LDPC)3/4 B code 41=64-QAM(LDPC)2/3 B code 42=64QAM(LDPC)3/4 B code 43=QPSK(LDPC)5/6 44=16-QAM(LDPC)5/6 45=64-QAM(LDPC)5/6 46=QPSK(CTC)2/3 47=QPSK(CTC)5/6 48=16-QAM(CTC)2/3 49=16-QAM(CTC)5/6 50..255=Reserved |

DCD burst profile values 46 - 49 shall not be applied to MS.

[Insert the following this entry at PHY wirelessMAN-OFDMA]

## 8.4.9.2.3.1 CTC encoder

[Insert Table 325a and 326a in 8.4.9.2.3.1 “CTC encoder”, as shown:]

Table 325a specifies the concatenation of sub-channels for different allocation and modulations of MR-BS and RS. It shall not be applied to MS.

Table 325a Encoding sub-channel concatenation for different rates in CTC

| Modulation and rate | j |
| --- | --- |
| QPSK 1/2 | 10 |
| QPSK 2/3 | 7 |
| QPSK 3/4 | 6 |
| QPSK 5/6 | 6 |
| 16QAM 1/2 | 5 |
| 16QAM 2/3 | 3 |
| 16QAM 3/4 | 3 |
| 16QAM 5/6 | 3 |
| 64QAM 1/2 | 3 |
| 64QAM 2/3 | 2 |
| 64QAM 3/4 | 2 |
| 64QAM 5/6 | 2 |

This following Table 326a should be applied the CTC encoder as a modulation orders, data block size, code rate and its parameters of the MR BS and RS for relay links. Table 326a shall not be used with MS's communication.

Table 326a CTC Channel coding per modulation

| Modulation | Data block size (bytes) | Encoded data block size (bytes) | Code rate | N | P0 | P1 | P2 | P3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 6 | 12 | 1/2 | 24 | 5 | 0 | 0 | 0 |
| QPSK | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| QPSK | 18 | 36 | 1/2 | 72 | 11 | 6 | 0 | 6 |
| QPSK | 24 | 48 | 1/2 | 96 | 7 | 48 | 24 | 72 |
| QPSK | 30 | 60 | 1/2 | 120 | 13 | 60 | 0 | 60 |
| QPSK | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| QPSK | 48 | 96 | 1/2 | 192 | 11 | 96 | 48 | 144 |
| QPSK | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| QPSK | 24 | 36 | 2/3 | 96 | 7 | 48 | 24 | 72 |
| QPSK | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| QPSK | 30 | 36 | 5/6 | 120 | 13 | 60 | 0 | 60 |
| QPSK | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |
| 16-QAM | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| 16-QAM | 18 | 36 | 1/2 | 72 | 11 | 6 | 0 | 6 |
| QPSK | 60 | 120 | 1/2 | 240 | 13 | 120 | 60 | 180 |
| 16-QAM | 30 | 60 | 1/2 | 120 | 13 | 60 | 0 | 60 |
| QPSK | 9 | 12 | 3/4 | 36 | 11 | 18 | 0 | 18 |
| QPSK | 18 | 24 | 3/4 | 72 | 11 | 6 | 0 | 6 |
| 16-QAM | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| QPSK | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| QPSK | 36 | 48 | 3/4 | 144 | 17 | 74 | 72 | 2 |
| 16-QAM | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| QPSK | 45 | 60 | 3/4 | 180 | 11 | 90 | 0 | 90 |
| 16-QAM | 45 | 60 | 3/4 | 180 | 11 | 90 | 0 | 90 |
| QPSK | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| 16-QAM | 24 | 36 | 2/3 | 96 | 7 | 48 | 24 | 72 |
| 16-QAM | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| 16-QAM | 30 | 36 | 5/6 | 120 | 13 | 60 | 0 | 60 |
| 16-QAM | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |
| 64-QAM | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| QAM16 | 12 | 24 | 1/2 | 48 | 13 | 24 | 0 | 24 |
| 64-QAM | 24 | 48 | 1/2 | 96 | 7 | 48 | 24 | 72 |
| 64-QAM | 30 | 60 | 1/2 | 120 | 13 | 60 | 0 | 60 |
| 64-QAM | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| 64-QAM | 48 | 96 | 1/2 | 192 | 11 | 96 | 48 | 144 |
| 64-QAM | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| 64-QAM | 60 | 120 | 1/2 | 240 | 13 | 120 | 60 | 180 |
| 64-QAM | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| 64-QAM | 36 | 48 | 3/4 | 144 | 17 | 74 | 72 | 2 |
| QAM16 | 24 | 48 | 1/2 | 96 | 7 | 48 | 24 | 72 |
| QAM16 | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| QAM16 | 48 | 96 | 1/2 | 192 | 11 | 96 | 48 | 144 |
| QAM16 | 60 | 120 | 1/2 | 240 | 13 | 120 | 60 | 180 |
| QAM16 | 18 | 24 | 3/4 | 72 | 11 | 6 | 0 | 6 |
| QAM16 | 36 | 48 | 3/4 | 144 | 17 | 74 | 72 | 2 |
| QAM16 | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| QAM64 | 18 | 36 | 1/2 | 72 | 11 | 6 | 0 | 6 |
| QAM64 | 36 | 72 | 1/2 | 144 | 17 | 74 | 72 | 2 |
| QAM64 | 54 | 108 | 1/2 | 216 | 13 | 108 | 0 | 108 |
| QAM64 | 24 | 36 | 2/3 | 96 | 7 | 48 | 24 | 72 |
| QAM64 | 48 | 72 | 2/3 | 192 | 11 | 96 | 48 | 144 |
| QAM64 | 27 | 36 | 3/4 | 108 | 11 | 54 | 56 | 2 |
| QAM64 | 54 | 72 | 3/4 | 216 | 13 | 108 | 0 | 108 |
| QAM64 | 30 | 36 | 5/6 | 120 | 13 | 60 | 0 | 60 |
| QAM64 | 60 | 72 | 5/6 | 240 | 13 | 120 | 60 | 180 |

# References

C802.16j-07_251r1, “Demodulation and Forwarding method in Relay Station”