| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Multi-frame structure consistent to 802.16e for MR Networks |  |
| Date Submitted | 2007-07-6 |  |
| Source(s) | D.H. Ahn, Junhong Hui, Young-il Kim, C.I.Yeh, ETRI Kyu Ha Lee, Samsung Thales Chung-wook Suh, Securepia Matty Levanda, WiNetworks Jun Bae Ahn Solid Technologies, Inc. Anxin Li, Daqing Gu DoCoMo Beijing Labs | Voice: 82-42-860-6496 dhahn@etri.re.kr junhonghui@etri.re.kr kyuha.lee@samsung.com cwsuh@securepia.com Mattyl@winetworks.com jbahn@st.co.kr liax @docomolabs-beijing.com.cn gu@docomolabs-beijing.com.cn |
| Re: | This is a response to the call for technical contributions IEEE 802.16j-07/019 |  |
| Abstract | This contribution proposes a multiple frame operation consistent with 802.16e legacy frame structure to support multi-hop between MR-BS and MS. |  |
| Purpose | Text proposal for 802.16j Baseline Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Multi-frame structure consistent to 802.16e for MR Networks

D.H. Ahn, Junhong Hui,, Kyu Ha Lee, Chung-wook Suh, etc.

ETRI, Samsung Thales, Securepia, Winetworks,

Solid technologies,DoCoMo Beijing Labs

# Introduction

Based on centralized scheduling and the legacy system frame structure for BS and MS, this contribution proposes a multiple frame structure for consistency with the 802.16e legacy frame structure to support multi-hop relay operation which avoids inter RS and inter MR-BS and RS interference. In our scheme, MR-BS ignores all the intermittent RS and only does resource allocation for every MS in the network in the same way as the legacy BS does. MR-BS broadcasts the topology and frame control information to each RS. After RS receives the signal from MR-BS or superordinate RS in the current frame, it only simply relays the data bursts which belongs to its subordinate RS or MS and ignores the other data bursts.

# 2. Characteristics

2.1 Advantages

o complete backward compatible frame structure to 802.16e legacy system.

o no PHY revision for the legacy BS.

o without RS, multi-frame length is equal to 1. It is exactly the legacy system.

o no interference between MR-BS and RS, or between RSs anywhere in the cell.

o applicable to dynamic topology

o to allow large hop counts and enhance the flexible network deployment capability

o very simple RS deployment and less overhead.

  - Comparison to existing frame structures

| Item | Transparent FS | Non Transparent FS | Multi-frame FS |
| --- | --- | --- | --- |
| 1. Coverage extension | One-hop(preamble, MAP) | Multi-hop | Multi-hop |
| 2. Inter-RS-cell interference | NA | High | None |
| 3. HO burden between RSs | None | Exist | None |
| 4. Performance ( dependent on item 2 and 3) | In BS coverage: High Outer BS coverage: - | In BS coverage: same as 16e Outer BS coverage: medium | In BS coverage: same as 16e Outer BS coverage: medium |
| 5. UpLink delay(i.e 2-hop MS) | Not applicable | 5 frames | 5 frames |
| 6. Revision part of MR-BS | BS scheduler | PHY,MAC & Upper Layer | BS scheduler |
| 7. RS implementation feature | 1 RF modem and gate logic | Bulky system | 1 RF modem and gate logic |
| 8. Usage model best fit | - | Fixed infrastructure RS Temporary RS | Fixed infrastructure RS In-Building RS Temporary RS Mobile RS |
| 9. Client owned RS issue | Low cost | High cost Negative impact on Network stability when frequent on-off and mal-functioning | Low cost Robust when various problems in client owned RS occur |
| 10. Topology flexibility | - | Lower flexibility Need careful RS cell planning | High flexibility b/w m-frames Fixed within multi-frame Easy to deployment |
| 11. Frame operation | Receive/transmit relay signal in one frame | Alternatively receive and transmit between frames | Alternatively receive and transmit between frames |
| 12. MS mobility | High in BS coverage | Frequent hand-off | Well Supported in MR-BS cell |
| 13. Tx/Rx switching times in one frame at RS | 4 | 3 | 1 |
| 14. AMC in RS | Not defined | Supported | Fixed by MR-BS or flexible within a multi-frame |
| 15. Scheduling | Centralized scheduling only | Centralized/distributed scheduling | Centralized scheduling only |

# 3. Proposal for Multi-frame operation

(1) Multi-frame concept

- A Multi-frame consists of L subsequent frames.

A legacy frame can be a multi-frame with multi-frame length of 1. Multi-frame structure is actually an extended version of the legacy system frame structure when it is also used for relay function.

(2) Dynamic Multi-frame considering MR topology and traffic load

The length of Multi-frame(LMF) is determined before the Multi-frame start time, in consideration of topology and traffic load. One method may be used to determine LMF as follows:

LMF = the length of Multi-frame = max (MHR+1, 2 * MHM + 1)

MHR : the maximum hop counts of active RSs

MHM : the maximum hop counts of MSs who have at least one UL traffic at start of Multi-frame

Example:

if no RS in a cell, LMF = max (0+1, 2-1) = 1

else if there exist 1-hop RSs and 2-hop MSs {

if at least an MS has UL data to send, LMF = max (1+1, 2*2+1) = 5

else no MS has UL data to send, LMF = max (1+1, 0+1) = 2

}

(3) Use of identical frame control information to consist of one transparent RS group as an example for interference mitigation

- use frame control information containing Preamble, FCH and MAPs (or including DCD and UCD)

- apply identical frame control information to all of the frames or at the same time within the Multi-frame except the frame number.

- an RS shall update the frame number in the received frame control information by incrementing and reassembling for relaying.

- RS ignores the bursts which are not related to the RS’s subordinates.

(4) RS should transmit the preamble at the beginning of a frame while not receive preamble at the same time during MS-mode.

In order to keep the MS backward compatible, RS should transmit the preamble signal during MS mode.

Preamble shall be transmitted synchronously with the MR-BS while RS shall not receive preamble in DL-

subframe from its superordinate at the same time.

As a simple example for implementing such a transmission, we can use an extra antenna to transmit deterministic preamble with a trigger signal from a timing controller and stops to transmit when preamble transmission ends.

(5) Sample topology

(fig. 1) A sample for showing network topology and frame transmissions of hop links

during kth frame transmission at MR-BS

In figure 1, k is an arbitrary frame sequence number of a Multi-frame with the range of 1 ≤ k ≤ LMF(length of Multi-frame). In addition, before every Multi-frame is transmitted into the air, k shall be renewed and LMF shall be re-calculated.

An RS shall transmit received frame to subordinates according to the control message of the multi-frame.

(6) Burst transmission and reception state diagram for the sample topology

(fig. 2) Burst transmission and reception state diagram at nodes of the sample topology

(7) Based on such a multi-frame structure consistent with the 802.16e legacy system in MMR network, new MAC management messages are proposed for relay frame control.

MR-BS shall transmit the relay request message RLY-CMD to the relay group for relay control using the DL burst. An RS shall transmit received frame to its subordinate RS or MS according to the RLY-CMD and it shall also forward this control message to its subordinate RS.

An RS may transmit a relay report message RLY-RPT using UL burst allocated to the RS if needed to its super ordinate.

Type       Message name     Message description

xx0        RLY-CMD        Relay Request              Basic

xx1        RLY-RPT         Relay Report               Basic

# 4. Text Proposal

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

[Insert new subclause 8.4.4.7.5]

8.4.4.7.5 Multi-frame structure consistent to 802.16e

A Multi-frame consists of certain number of subsequent frames. An example of the multi-frame structure is shown in Figure <xxx>.

Figure <xxx>  Multi-frame structure

[Insert new subclause 8.4.4.7.5.1]

8.4.4.7.5.1 MR-BS multi-frame structure

For the TDD mode, MR-BS uses the legacy 802.16e frame as a unit frame to construct a multi-frame by grouping these units with a repeating pattern according to the network topology. In every multi-frame for MR-BS, each unit frame is the same as in legacy 802.16e Section 8.4.4.2. Within a multi-frame, all the unit frames may use the preamble/FCH/ MAP/UCD/DCD as required by the network planning.

[Insert new subclause 8.4.4.7.5.2]

8.4.4.7.5.2 Relay frame structure

For the TDD mode, Relay has two mode of operation, MS-mode and BS-mode.

During MS-mode, each RS functions as a legacy MS frame operation.

During BS-mode, each RS transmits the unit frame with the same structure as described in legacy 802.16e Section 8.4.4.2, but ignores the bursts which are not related to its subordinate RS or MS.

Relay changes its operation mode between MS-mode and BS-mode.

Within a multi-frame, RS works in MS-mode at first. Only after receiving the correct frame control information (FCH/MAP/UCD/DCD), RS shall change to BS-mode in next frame and then return to MS mode again.

RS should transmit the preamble at the beginning of a frame while not receive preamble at the same time during MS-mode. The other part of MS mode is the same as legacy MS frame structure.

An example for RS unit frame structure in MS-mode is shown in Figure <yyy>.

Figure <yyy> RS unit frame structure in MS-mode

An example for RS unit frame structure in BS-mode is shown in Figure <zzz>.

Figure <zzz> RS unit frame structure in BS-mode

[Append following text into subsection of 6.3.2.3]

6.3.2.3.xx0 Relay command message

In order to control RS’s correct operation, MR-BS shall transmit the RLY-CMD message to the relay group using DL burst in every unit frame of the corresponding multi-frame. An RS shall transmit received frame to its subordinate RS or MS according to the RLY-CMD and it shall also forward this control message to its subordinate RS. RLY-CMD message should include: frame number, start frame number, end frame number, number of RSs to be received the command body and command body according to the control function. The message shall be transmitted on the basic CID or broadcast CID.

MR-BS shall generate RLY-CMD message including parameters shown in Table xx.

Table xx --- RLY-CMD message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| RLY-CMD_message_format() { |  | To multicast id of relay group |
| Management Message Type = xx0 | 8 |  |
| Multi-frame Identification |  |  |
| Start frame number | 8 | The least significant 8 bits |
| End frame number | 8 | The least significant 8 bits |
| N_Relays | 8 | The number of relay stations to receive a command body |
| For (i=0; i< N_Relays; i++) { |  |  |
| CID | 16 | Relay CID |
| Length of command body | 8 |  |
| Command Body | variable | Command dedicated to specific RS |
| Padding | v | Number of bits required to align to byte length. Shall be set to zero. |
| } |  |  |
| } |  |  |

An MR-BS generates RLY-CMDs including all of the following parameters, as shown in Table xx:

Start frame number

Start frame number of current Multi-frame. The value is the least significant 8 bits of the start frame

End frame number

End frame number of current Multi-frame. The value is the least significant 8 bits of the end frame

Command Body

This parameter is reserved for future use of higher layer. This may contain routing information for specific RS or RS’s operation due to MS’s association, topology update information, etc.

[Append following text into subsection of 6.3.2.3]

6.3.2.3.xx1 Relay report message

An RS may transmit a RLY-RPT message using UL burst allocated to the RS. An RS shall generate RLY-RPT message including parameters shown in Table yy.

The message shall be transmitted on the basic CID or broadcast CID.

Table yy --- RLY-RPT message format

| Syntax | Size | Notes |
| --- | --- | --- |
| RLY-RPT_message_format() { |  | From RS via UL unicast |
| Management Message Type = xx1 | 8 |  |
| Length of report body | 4 | Length of the slot |
| Report Body | variable |  |
| Padding | v | Number of bits required to align to byte length. Shall be set to zero. |
| } |  |  |

Report Body

This parameter is reserved for future use. It may contain the ranging information from MSs and/or from neighbors or other functions to be extended.