| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Transmission using Access RS station CID |  |
| Date Submitted | 2007-07-05 |  |
| Source(s) | Ranga Reddy US Army - CERDEC, USA D. J. Shyy MITRE, USA Arnaud Tonnerre THALES COMMUNICATIONS, FRANCE Djamal-Eddine Meddour FRANCE TELECOM, FRANCE Hang Zhang, Peiying Zhu, Mo-Han Fong, Wen Tong, David Steer, Gamini Senarath, Derek Yu, Mark Naden, G.Q. Wang Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 Jeffrey Z. Tao, Koon Hoo Teo, Jinyun Zhang Mitsubishi Electric Research Lab 201 Broadway Cambridge, MA 02139 USA Toshiyuki Kuze Mitsubishi Electric Corp 5-1-1 Ofuna Kamakura, Kanagawa 2478501, Japan Aik Chindapol, Jimmy Chui , Hui Zeng Siemens Corporate Research Princeton, NJ, 08540, USA Teck Hu Siemens Networks Boca Raton, FL 33431, USA Yuan-Ying Hsu Telcordia Applied Research Center Taiwan Co., Taipei, Taiwan Tzu-Ming Lin, Fang-Ching Ren, Chie Ming Chou, I-Kang Fu ITRI/ NCTU Taiwan 195,Sec. 4, Chung Hsing Rd. Chutung, Hsinchu, Taiwan 310, R.O.C. Torsten Fahldieck Alcatel-Lucent R&I Holderaeckerstr.35, Stuttgart, Germany Erwu Liu, Dongyao Wang, Gang Shen, Kaibin Zhang, Jimin Liu, Shan Jin Alcatel Lucent, R&I Shanghai, No.388, Ningqiao Road, Shanghai, P.R.C. | Voice: +1 732-532-0085 Email: Ranga.Reddy@us.army.mil Voice: +1 703 983 6515 Email: djshyy@mitre.org Voice: +33 1 46 13 2850 Email: arnaud.tonnerre@fr.thalesgroup.com Email: djamal.meddour@orange-ft.com Voice: +1 613 7631315 Email: WenTong@nortel.com Email: pyzhu@nortel.com Voice: 617-621-{7557,7527} Email: {tao, tea, jzhang}@merl.com Voice: +81-467-41-2885 Email:Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp Voice: +1 609 734 3364 Email: aik.chindapol@siemens.com Voice: +886-2-37895177#4558 Email: yyhsu@tarc-tw.research.telcordia.com Voice: +886-3-5914616 Email: IKFu@itri.org.tw Voice: +4971182132163 Email: torsten.fahldieck@alcatel-lucent.de Voice: 86-21-50551240-8194 Email: {Erwu.liu,Dongyao.Wang,Gang.A.Shen,Kaibin.Zhang, Jimin.Liu,Shan.Jin}@alcatel-sbell.com.cn |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | Provide a method for streamlining MPDU transmission and reducing overhead |  |
| Purpose | To amend the text of baseline document for Sections 6.3.1.3, 6.3.3.8, and 6.3.14 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Transmission using Access RS station CID

Ranga Reddy
US Army - CERDEC, USA

D. J. Shyy
MITRE, USA

Arnaud Tonnerre

THALES COMMUNICATIONS, FRANCE

Djamal-Eddine Meddour

FRANCE TELECOM, FRANCE

Hang Zhang, Peiying Zhu,Mo-Han Fong,Wen Tong,David Steer, Gamini Senarath,Derek Yu,Mark Naden,G.Q. Wang

Nortel

Jeffrey Z. Tao, Koon Hoo Teo, Jinyun Zhang

Mitsubishi Electric Research Lab

Toshiyuki Kuze

Mitsubishi Electric Corp

Aik Chindapol, Jimmy Chui, Hui Zeng
Siemens Corporate Research

Teck Hu
Siemens Networks

Yuan-Ying Hsu 
Telcordia Applied Research Center Taiwan Co.,

Tzu-Ming Lin, Fang-Ching Ren, Chie Ming Chou, I-Kang Fu 
 ITRI/ NCTU

Torsten Fahldieck
Alcatel-Lucent R&I

# Erwu Liu, Dongyao Wang, Gang Shen, Kaibin Zhang, Jimin Liu, Shan Jin
Alcatel Lucent, R&I

## Purpose

In this contribution, we propose the destination/source RS CID based routing scheme. For implementing this scheme, the following concepts are defined:

- Each access RS needs to be assigned only three connections (see Figure 1)

  - Basic connection and primary connections (defined in 16e) carrying MAC management message of an access

  - Forwarding transport connection for relaying all MS related traffic and messages of mobiles attached to this RS

- MS MPDUs of MSs associated with an access RS are relayed on forwarding transport connection between MR-BS and this access RS. The MS MPDUs with the same QoS class can be encapsulated into an R-MAC PDU and the QoS information is included in the R-MAC header in QoS info field.

- QoS info includes the QoS class of a carried R-MAC PDU and the transmission deadline (frame number).

- The QoS info is inserted into the R-MAC header by the sender which is MR-BS for DL and an access RS for UL

- The intermediate RSs don’t need to know any QoS profiles and routing information of MSs that are not directly attached to it and only simply relay traffic based on QoS class and deadline information provided by the sender.

Figure 1. Connections between a RS and MR-BS.

The data forwarding procedure is illustrated in Figure 2.

Figure 2. Data forwarding example

The main benefits include:

- Signaling overhead reduction compared with other forwarding scheme

  - No signaling overhead for tunnel setup/maintenance

  - No need for MS CID mapping to tunnel at service setup and re-mapping at MS HO/FBSS

- RS process complexity

  - Move significant R-link scheduling burden from intermediate RS to MR-BS for DL case

  - Each access RS only needs to make scheduling decision for MS attached to it for UL traffic

  - Intermediate RSs don’t need to keep lengthy routing table and QoS profiles for each tunnel or  connections of MSs that are served by all subordinate RSs

- Reduce # of required CIDs

  - Only need 1 transport connection is needed to support multiple QoS levels.

The above benefits enable a very simple and low-power-consumption RS.

## Proposed Text Change

##### [Insert the following text at the end of Section 6.3.1.3]

+++++++++++++ start text +++++++++++++++++++++++++++++++++++++++++++++++++

Another type of connection of a RS is called as Forwarding transport connection which is used for carrying MS MPDUs that need to be relayed for Dl and UL. The corresponding connection CID can be expressed as F-CID. One F-CID of a RS can be used for both DL and UL. For DL case, MR-BS shall map all MPDUs of MSs attached to a RS to the forwarding transport connection of this RS. For UL case, an access RS shall map all MPDUs of MSs attached to it to forwarding transport connection of this RS. The F-CID is assigned by a MR-BS through DSA-REQ/RSP message exchange at path setup phase during a RS initial network entry or network re-entry .

+++++++++++++ end text +++++++++++++++++++++++++++++++++++++++++++++++++

[Insert the following subclause after Section 6.3.3.8.2]

+++++++++++++ start text +++++++++++++++++++++++++++++++++++++++++++++++++

6.3.3.8.3 Transmission using access RS forwarding transport connection and source QoS control information

For this type of data forwarding, each access RS needs to be assigned only three connections:

- Basic connection and primary connections carrying MAC management message of an access

- Forwarding transport connection for relaying all MS related traffic and messages. The corresponding CID is expressed as F-CID.

MAC PDUs of MSs associated with an access RS are relayed on the forwarding transport connection between MR-BS and this access RS.

The MS MPDUs with the same QoS class can be encapsulated into a R-MAC PDU and the QoS info  is included in the R-MAC header and/or a sub-header. QoS info includes the QoS class of a carried R-MAC PDU and the transmission deadline (frame number). For DL data forwarding, the MR-BS can include the destination RS F-CID and QoS info in the R-MAC header. For UL, the access RS includes its F-CID and QoS information in the R-MAC header The intermediate RS can schedule the transmission of the MS MPDUs carried in a R-MAC PDU based on QoS information along with the received R-MAC PDU and identify the next hop RS based on F-CID using its routing table.

+++++++++++++ end text +++++++++++++++++++++++++++++++++++++++++++++++++