| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |
| --- | --- |
| Title | Drops during MRS/MS handover |
| Data Submitted | 23-April-2007 |
| Source(s) | Tan Huanxi, Liu Qiaoyan Voice:86-29-88723546 Zhao Lu, Yu Qiuxing Fax:86-29-88723544 Li Geng, Gong Yuanyuan Email: tan.huanxi@zte.com.cn liu.qiaoyanxa@zte.com.cn ZTE Corporation Mary Chion mchion@zteusa.com ZTE USA |
| Re: | This is a response to Call for Technical Proposals regarding IEEE Project P802.16j. |
| Abstract | The document contains technical proposals to handle with drops during MRS or MS handover in IEEE P802.16j network. |
| Purpose | The document is submitted for review by 802.16 Working Group members. |
| Notice | This document has been prepared to assist IEEE 802.16. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |
| Patent Policy and Procedures | The contributor is familiar with the IEEE 802.16 Patent Policy and Procedures <http://ieee802.org/16/ipr/patents/policy.html>, including the statement “IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard. “Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <mailto:chiar@wirelessman.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.16 Working Group. The Chair will disclose this notification via the IEEE 802.16 web site <http://ieee802.org/16/ipr/patents/notices>. |

# Drops during MRS/MS handover

## 1 Introduction

In IEEE 802.16e, a drop is defined as the situation where an MS has stopped communication with its serving BS before the normal handover sequence. Due to the introduction of relay station in the network, the technique to deal with drops during MS handover defined in IEEE 802.16e should be modified to satisfy RS network, especially the mobile relay station (MRS) condition. In addition, procedures should be defined to handle drop during MRS handover.

This contribution proposes modifications to handle drops during non-transparent MRS and MS handover for MR network.

## 2 proposed solution

As discussed above, this contribution describes methods to handle the following types of drop in MR network:

1 MS drop during handover;

2 MRS drop during handover.

For MS drop during handover, MS should perform the same procedure as defined in IEEE802.15-2005 section 6.3.22.2.6. In addition, if MS is attempting to handover to a RS when a drop occurred, the MR-BS controlling the target RS should inform the target RS of the handover failure and remove any MS context from the target RS.

If a drop is detected during MRS handover, the MRS should follow similar procedure as defined for MS in IEEE802.16-2005. However, to minimize impact to the attached MSs, the MRS should always attempt to re-enter the old serving BS first by cancelling HO. This allows fast recovery of MSs’ services when drop occurs during MRS handover. For MRS handover without preamble change, returning to the serving BS enables the MSs to be continuously served by the MRS without interruption. For MRS handover with preamble change, all MSs attached to the MRS will be directed to start handover procedure by MR-BS. In many scenarios, the target station where MS hands over to should be the same MRS with a changed preamble. Hence, a drop of MRS could cause a drop in MS handover as well. If MRS re-enters serving BS by cancelling the HO, the MSs can also perform the HO cancellation procedure by re-entering the MRS as defined in IEEE802.16-2005.

3 Proposed Text Change

[Insert following text at end of section 6.3.22.2.6]

6.3.22.2.6 MS Drops during handover

In MR network, if MS detects a drop when performing network re-entry at the target station, the MS shall perform the same procedure as defined for non-MR network. If the target BS is informed of MS attachment to a different RS or different BS from what is originally targeted due to the drop, the target BS may send MS_INFO-DEL message to make the original target RS discard MS context information. Upon receiving the MS_INFO-DEL message, the RS shall transmit MS_DEL-ACK or ACK header as an acknowledgement and remove the MS context information

[Insert new subclause 6.3.22.4.4 after section 6.3.22.4.3]

6.3.22.4.4 MRS Drops during handover

If MRS detects its drop during network re-entry at target BS, MRS shall attempt resume communication with the serving BS by transmitting MOB_HO_IND with HO_type=0b01 to cancel handover if the resource_retain_time timer has not expired. If the resource_retain_time timer has expired, the MRS may attempt network re-entry with its preferred target BS as through Cell Reselection. If the MRS fails network re-entry with its preferred Target BS, the MRS shall perform initial entry procedure.

When performing network entry, MRS shall perform CDMA ranging with Target BS using codes from HO codes domain.

Upon Target BS sending RNG-RSP with ‘ranging status’ = success, Target BS shall provide CDMA_ALLOC_IE with appropriate UL allocation for RNG-REQ from MRS. MRS shall send RNG-REQ with MAC address and HMAC/CMAC. Target BS may now identify that HO attempt by MRS was not coordinated with Serving BS and may request all relevant MRS context and attached MSs’ context from Serving BS. Using this info Target BS shall now send RNG-RSP with ‘HO process optimization’ bitmap and network re-entry may continue as in the typical, non-drop case.

When serving BS detects a drop, it shall react as if a MOB_HO_IND message has been received with HO_IND_type=0b00.

## Reference

[1] Mobile RS Handover. IEEE C802.16j-07/122r4. Sungkyung Kim, Sungcheol Chang, Chulsik Yoon, Sunggeun Jin. ETRI.

[2] Handover of Mobile Relay Station. IEEE C802.16j-07/147r3. Kaibin Zhang, Gang Shen, Jimin Liu, Shan Jin. Alcatel-Lucent, Rearch &Innovation.