| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Direct Relaying Zone in Transparent RS system |  |
| Date Submitted | 2007-07-19 |  |
| Source(s) | Su Chang Chae, Young-il Kim, Hyunjae Kim,Sunggeun Jin ETRI Kyu Ha Lee, Changyun Kim, Yong Wook Lee Samsung Thales Jai H. Eu, Jcast Networks Korea, Inc Hyung-joon Jeon POSDATA Jun Bae Ahn SOLiD technologies | E-mail:schae@etri.re.kr E-mail:kyuha.lee@samsung.com E-mail:jeu@jcastnet.com E-mail:hjjun@posdata.co.kr jbahn@st.co.kr |
| Re: | This is a response to the call for technical contributions IEEE 802.16j-07/019 |  |
| Abstract | This contribution proposes combined M&F relaying. |  |
| Purpose | To incorporate the proposed text into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy and Procedures | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Direct Relaying Zone in Transparent RS system

# Introduction

There are three relaying method as follows,

- A&F (Amplify and Forward)

- M&F (Modulation and Forward)

- D&F (Decode and Forward)

Since A&F method is relaying data from sender to receiver in time domain (i.e. without FFT, demodulation and FEC decoding), RS can relay data with minimum delay by using A&F method. However Burst Profile for A&F relaying may be dominated by the radio link that has the worst condition between relay link and access link.

M&F method is relaying data without decoding and re-encoding. RS which receives data from sender demodulates and modulates it, and relays regenerated data to receiver. Though some gaps are needed for RS to demodulate and modulate, RS can relay data within one frame by using M&F method.

Decode and forward is the most robust, hence higher modulation schemes can be used, because the Burst Profile on relay link and the Burst Profile on access link can be set individually. However it takes time to decode and re-encode data at RS, RS takes a few frames to relay data from sender to receiver.

This contribution proposes the direct relaying zone on which RS relays data from sender to receiver within one frame by using M&F method. Since RS does not decode data on this zone, MR-BS should notify the zone information to RS and control the transmission on this zone.

# Direct Relaying Zone

In Figure 1, two direct relaying zones are defined in each sub-frame. First direct relaying zone in DL sub-frame is the zone where RS should receive data from MR-BS, and second direct relaying zone in DL sub-frame is the zone where RS should forward signal or data to MS under RS. The direct relaying zone in UL sub-frame is also similar.

Figure 1 Direct Relaying Zone

RS receives data from sender on the direct relaying zone in receiving area, and forwards it to receiver on the direct relaying zone in sending area by using M&F method.

## IE for Direct Relaying Zone

MR-BS decides the direct relaying zone and relaying method on this zone which is A&F method or M&F method, and notifies this information about the zone to RS by DL/UL MAP including a value of frame offset. RS starts to relay at frame defined by this value. The value of frame offset is defined considering of the decoding delay of DL/UL MAP at RS. And the other parameters that MR-BS notifies RS about the direct relaying zone are the following and Figure 2 shows the relation these parameters and the frame.

a) Symbol offset for direct relaying zone

The OFDMA symbol offset in which direct relaying zone starts

b) No. OFDMA symbols for RS receiving

The number of OFDMA symbols which RS should receive for direct relaying

c) No. OFDMA symbols of gap

The number of OFDMA symbols between the receiving area and the forwarding area

d) No. OFDMA symbols for RS relaying

The number of OFDMA symbols by which RS should relay signal without decoding and re-encoding

Figure 2 IE of Direct Relaying Zone

RS which receives these parameters relays signal from sender to receiver without decoding and re-encoding on the zone indicated by these parameters.

# Specific text changes

M&F : Modulation and Forward.

[Change Table 277c385 as indicated.]

Table 277c385—Extended-2 DIUC code assignment for DIUC=14

| Extended-2 DIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | MBS_MAP_IE |
| 01 | HO_Anchor_Active_DL_MAP_IE |
| 02 | HO_Active_Anchor_DL_MAP_IE |
| 03 | HO_CID_Translation_MAP_IE |
| 04 | MIMO_in_another_BS_IE |
| 05 | Macro-MIMO_DL_Basic_IE |
| 06 | Skip_IE |
| 07 | HARQ DL MAP IE |
| 08 | HARQ ACK IE |
| 09 | Enhanced DL MAP IE |
| 0A | Closed-loop MIMO DL Enhanced IE |
| 0B | RS-RNG_RSP_ALLOC_IE |
| 0BC | Direct Relaying DL Zone IE |
| 0B-0D | Reserved |
| 0E | AAS_SDMA_DL_IE |
| 0F | Reserved |

Insert a new subclause 8.4.5.3.28

### 8.4.5.3.28 Direct Relaying DL Zone_IE format

This IE is sent transmitted by MS-BS to RS to notify the zone of downlink for relaying within one frame without decoding and re-encoding. If this IE indicates M&F method for relaying, and RS should relay signal symbol data from MR-BS with FFT, demodulation, modulation and IFFT. In order to change the modulation order used in access zone to another modulation order to use in optional transparent zone for downlink in RS, MR-BS shall allocate slots of both access and optional transparent zone so that channel encoder of MR-BS can fill the padding bits(‘1’s) at the end of the data to be encoded in the access zone for downlink.

Table xxx allows transparent RS to use M&F with direct relaying zone for downlink.

Direct Relaying DL Zone_IE denotes the direct relay zone for downlink sub-frame. It is used for RS to relay symbol data within a single frame. For the purpose, RS shall relay symbol data without the processing of channel decode/encode through direct relay zone for downlink.

When RS relays symbol data within a single downlink subframe according to Direct Relaying DL_Zone_IE, it may change modulation order used in access zone into another modulation order in transparent relay zone as described in 8.4.4.7.1.2.

The number of slots allocated for bursts in access zone can possibly mismatch that in transparent relay zone. For this reason, MR-BS may allocate additional slot(s) for each burst in access zone so as to provide exact modulation order change. In this case, padding bits (‘1’) are appended to the end of aggregate of MPDUs prepared for channel encoding in each burst.

Table xxx allows transparent RS to use M&F with direct relaying zone for downlink.

Table xxx - Direct Relaying DL Zone IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Direct Relaying DL_Zone IE() { |  |  |
| Extended-2 DIUC= | 4 bits | 0x0C |
| Length | 8 bits | 0x03 |
| Relaying Method Type | 1bit | 0:D&F, 1:M&F |
| Symbol offset for direct relaying zone | 6 bits |  |
| No. OFDMA symbols for RS receiving | 46 bits |  |
| No. OFDMA symbols of gap | 46 bits |  |
| No. OFDMA symbols for RS relaying | 46 bits |  |
| Frame offset | 2bits |  |

Symbol offset for direct relaying zone

The OFDMA symbol offset in which direct relaying zone starts

No. OFDMA symbols for RS receiving

The number of OFDMA symbols which RS should receive for relaying without decoding and encoding

No. OFDMA symbols of gap

The number of OFDMA symbols between the receiving area and the forwarding area

No. OFDMA symbols for RS relaying

The number of OFDMA symbols by which RS should relay signal without decoding and encoding

[Change Table 290a 421 as indicated.]

Table 290a421—Extended UIUC Code Assignment for UIUC=15

| Extended UIUC (hexadecimal) | Usage |
| --- | --- |
| 00 | Power_control_IE |
| 01 | Mini-subchannel_allocation_IE |
| 02 | AAS_UL_IE |
| 03 | CQICH_Alloc_IE |
| 04 | UL Zone IE |
| 05 | PHYMOD_UL_IE |
| 06 | MIMO_UL_Basic_IE |
| 07 | UL-MAP_Fast_Tracking_IE |
| 08 | UL_PUSC_Burst_Allocation_in_Other_Segment_IE |
| 09 | Fast_Ranging_IE |
| 0A | UL Allocation Start IE |
| 0B | RS-RNG_RSP_Allocation_IE |
| 0C | UL_Burst_Receive_IE |
| 0D | Direct Relaying UL Zone IE |
| 0DE … 0F | Reserved |

Insert a new subclause 8.4.5.4.29

### 8.4.5.4.29 Direct Relaying Zone UL_IE format

This IE is senttransmitted by MS-BS to RS to notify the zone of uplink for relaying within one frame. If this IE indicates M&F method for relaying, and RS should relay signal symbol data from MR-BS with FFT, demodulation, modulation and IFFT. In order to change the modulation order used in access zozne to another modulation order to use in relay zone for uplink in RS, MR-BS shall allocate slots of both access and relay zone so that channel encoder of MS can fill the padding bits(‘1’s) at the end of the data to be encoded in the access zone for uplink.

Table yyy allows transparent RS to use M&F with direct relaying zone for uplink.

Direct Relaying UL Zone_IE denotes the direct relay zone for uplink sub-frame. It is used for RS to relay symbol data within a single frame. For the purpose, RS shall relay symbol data without the processing of channel decode/encode through direct relay zone for uplink. The remaining procedure for the uplink direct relaying according to Direct Relaying Zone_UL_IE is manipulated in the same manner as described

in 8.4.5.4.29. In case of uplink, MS shall append padding bits (‘1’) to the end of aggregate of MPDUs prepared for channel encoding in each burst.

Table yyy allows transparent RS to use M&F with direct relaying zone for uplink.

Table yyy - Direct Relaying UL Zone IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Direct Relaying UL Zone IE() { |  |  |
| Extended UIUC= | 4 bits | 0x0B |
| Length | 8 bits | 0x03 |
| Relaying Method Type | 1bit | 0:D&F, 1:M&F |
| Symbol offset for direct relaying zone | 6 bits |  |
| No. OFDMA symbols for RS receiving | 46 bits |  |
| No. OFDMA symbols of gap | 46 bits |  |
| No. OFDMA symbols for RS relaying | 46 bits |  |
| Frame offset | 2bits |  |

Symbol offset for direct relaying zone

The OFDMA symbol offset in which direct relaying zone starts

No. OFDMA symbols for RS receiving

The number of OFDMA symbols which RS should receive for relaying without decoding and encoding

No. OFDMA symbols of gap

The number of OFDMA symbols between the receiving area and the forwarding area

No. OFDMA symbols for RS relaying

The number of OFDMA symbols by which RS should relay signal without decoding and encoding

# References

- C802 16j-06_127, “A Proposal for combined A&F and D&F relaying”

- C802 16j-06_250, “Hybrid Relay Structure with a Single Frame”

- C802 16j-06_26r4,”Baseline Documents for Draft Standard for Local and Metropolitan Area Networks”