| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MDHO Operation in Multihop Relay Networks |  |
| Date Submitted | 2026-07-29 |  |
| Source(s) | Israfil Bahceci, Senarath Gamini, Hang Zhang, Peiying Zhu, Wen Tong, Mo Han Fong, Derek Yu, David Steer, Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | Voice: +613-763-1315 E-mail: wentong@nortel.com E-mail: pyzhu@nortel.com |
| Re: | IEEE P802.16j/D1: IEEE 802.16j working group letter ballot #28 |  |
| Abstract | Text changes for MDHO operation in multihop relay networks are proposed. |  |
| Purpose | To incorporate the proposed text into the P802.16j/D1 Baseline Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# MDHO operation in Multihop Relay Networks

Israfil Bahceci, Senarath Gamini, Hang Zhang, Peiying Zhu, Wen Tong, Mo Han Fong, Derek Yu, David Steer, Mark Naden

- Introduction

Macro diversity handover (MDHO) and fast base station switching (FBSS) are two optional modes of soft handover operation in 802.16e. These two modes provide diversity gain, and reduce the disturbance due to frequent hard handovers. For a multihop relay network, we can also benefit from MDHO operation by also including the RS into the diversity set.

First, the macro diversity gain from multiple RSs can improve the performance significantly. Second, when relays are in operation, since relay coverage areas is much smaller than the BS coverage, there may be many handovers among them compared to a 802.16e system. There may be a large number of ping pong situations where mobiles moving from one relay area to the other can return very often. In order to minimize such disturbances, to reduce the impact of these handovers, and to increase the signal quality in the relay boundaries, we propose that MDHO be supported between relay stations and between relay stations and MR-BS(s).

The MDHO procedures defined in the 16e need to be extended for this purpose and the objective of this contribution is to identify those changes necessary and to define the operation procedures for MDHO. In the centralized scheduler case, we only need minor text changes in 16e.

There can be several types of handover situations depending on whether RS, MRBS or 16e BS station is involved and in case of RS involvement, whether they belong to the same MRBS(s) (intra-MRBS) or different MRBS(s) (inter-MRBS) or whether they are transparent, non-transparent or whether RS(s) belong to different RS groups. In this contribution, we focus on inter-MRBS handovers since the standard changes required for other cases could be a subset of the changes indicated for this case.

There is another situation where a RS connects to its parent in MDHO mode. This is helpful for MS MDHO in order to save resources in the uplink when a MS is in MDHO mode. This comes under a separate category of “RS Handover”, although its usage for MS handover is discussed in this contribution. Since the MDHO for an RS follows in a similar way to that for the MS, we do not need further amendments for this case.

- IEEE 802.16e procedures for MDHO and Amendments for MR operation

In this section, we provide some background information for MDHO operation in 16e and propose the required changes to enable it for MR operation.

  - Handover Scenarios for MR Networks with Non-transparent Relays

In MDHO, the standard support is required to enable

updating the diversity set and establishment/disconnection of the parallel data communication paths:

- Diversity set update:

BS and MS keep a diversity set list and an anchor base station for each MS. For this a Temp BSID may be used (diversity member ID). For MDHO, in the downlink, all the members in the diversity set synchronously transmit same data to the MS so that MS does soft combining. For MDHO uplink operation, all the members in the diversity set receives signal and forward to the central node for soft/selection combining. MOB_BSHO_REQ/RSP (from BS to MS), MOB_MSHO_REQ (from MS to BS) messages are used for establishing/updating the diversity sets in MDHO operation.

There are two messages for neighbourhood and topology advertisement:

MR_NBR_INFO: MR-BS sends to RS this message to update RS with information regarding neighbourhood stations.

MOB_NBR_ADV:  MR-BS transmits neighbourhood info for MSs. There are DCD/UCD, preamble index, etc. and some other information in this message (DCD_setting and UCD_setting).

In a MR-BS system, the diversity set may also include RSs in addition to the BSs. In the messages above, it is not mentioned that the neighbourhood stations can include non-transparent RS(s). It is required to update these messages such that they may include parameters for the non-transparent RSs and MRBSs in addition to the BSs. Furthermore, MOB_BSHO_REQ/RSP and MOB_MSHO_REQ messages need to be updated to reflect this.

MRBS and MS keep track of the diversity set for the MSs. There is no special advantage by keeping the diversity set at the RSs. However, each RS in the diversity set should know its own presence in a diversity set when an MS adds that RS to the diversity set. In addition, the anchor station needs to know that it is the anchor station. Therefore, the anchor RS should inform the MRBS explicitly or forward to the MRBS the MS’s request for addition of another RS and again forward back the MRBS’s response. The MRBS can inform the new diversity member (RS) about its addition, by sending a new message.

A removal of an RS from the diversity also can be done in a similar manner. Using 16e procedures the anchor station relays the MS messages to MRBS and MRBS’s messages to the anchor station. The MRBS can then inform the diversity member about its deletion.

- Anchor BS/RS update:

There are two methods defined for this purpose.

(a) HO MAC Management message method: MS inform BS of preferred anchor BS using  MOB_MSHO_REQ. BS sends its recommendation using MOB_BSHO_REQ or MOB_BSHO_RSP. MS can accept/reject it by MOB_HO_IND. When they clash, MS ignores BS’s message or even reject it using MOB_HO_IND message. Final confirmation of the list is sent by MOB_HO_IND (which includes the list) as well. MS can later cancel the set by MOB_HO_IND message.

The RS has to forward these messages to the MR-BS in the centralized operation or responds to them in the decentralized operation. When RS receives MOB_MSHO_REQ or MOB_BSHO_REQ, it can relay the message to the destination node. The MOB_HO_IND message can also be relayed from MS to MRBS by the anchor RS. In case of any update on the diversity set status of the anchor RS, the MRBS sends the RS_Diversity_Set_Update_REQ() message to this RS.

(b) Fast Anchor BS Selection feedback Mechanism:  Fast feedback channel is used for informing the anchor BS on a MS’s anchor Station Selection. BS informs all the MSs L (frame number for minimum wait time) and M (frame number) parameters as a configuration message. After detecting a potential target BS, MS requests an anchor switch indicator to the current anchor BS using the CQICH channel assigned by the current anchor BS. Anchor BS may accept it using the Anchor_BS_switch_IE message which includes the BS (it may also select a different one from the one requested by MS), “time to switch” and the “CQICH channel” information or request cancel indicator. After waiting minimum time of L frames (timer in the MS), the MS could transfer to the new BS. A CQICH is already assigned in the new BS (as indicated above) or MS may request a new channel and continue switching.

If this method is used for updating the anchor station, the RS may send a MOB_MSHO_REQ message to the serving MR-BS, which can inform the new anchor station (if it is a nontransparent RS) via RS_Diversity_Set_Update_REQ() message.

Allocation of CQICH channel: The RS_Diversity_Set_Update_REQ() message may include the CQICH information.

- Control information (FCH/MAP) exchange among diversity set members:

FCH and DL/UL MAP may need to be exchanged among the diversity set members. In centralized scheduling, since all resource allocation information is prepared by the MR-BS, the MAP allocations can be sent to the diversity set members over the backhaul and using the MAP transfer messages described in the standard.

- Share/transfer MAC context between two BSs:

Such MAC context includes all information MS and BS normally exchange during Network Entry, particularly authentication state, so that an MS authenticated/registered with one of BSs from diversity set BSs is automatically authenticated/registered with other BSs from the same diversity set. The context also includes a set of Service Flows and corresponding mapping to connections associated with MS, current authentication, and encryption keys associated with the connections.

Since MRBSs already have all MAC context of MSs that they serve, they can transfer the required portion of the MAC context to the RSs if the that RS is included into the diversity set of the MS. These parameters are included as the MACSetting_TLV  into the RS_Diversity_Set_Update_REQ() message. Alternatively, the DSx-REQ/RSP/ACK messages can also be used to populate this information at the anchor RS.

Furthermore, the MR-BSs can share the capabilities and frame offsets for all RSs taking part in the MDHO operation (any RS(s) in the diversity set and any RSs along path from MR-BS(s) to the RS(s) in the diversity set). Hence, the control information allocation can be synchronized among all diversity set members.

Figure 1 Example message flow for diversity set update

  - MDHO Handover Scenarios for MR Networks with  Transparent Relays and Virtual Relay Groups

Since no changes are required for the standards and current base line document IEEE P802.16j/D1, we omit this case.

  - MDHO for RS Handover

No further changes in the standard are required for this case. The same procedures for MS can be employed for the RS(s).

- Proposed Text Changes

[Modification in 6.3.2.3.47, 6.3.2.3.52, 6.3.2.3.53, 6.3.2.3.54 and 6.3.2.3.55. These messages currently refer to only BSs for diversity sets for MDHO scenarios. They shall be modified to include non-transparent RSs as potential members of a diversity set. At the end of first paragraph of each section, include the following text]

[Insert the following text at the end of the first paragraph in 6.3.2.3.47]

Non-transparent RSs and MRBS(s) may also be included as the neighbor stations in addition to BS(s). Furthermore, this message may also be targeted for a RS.

[Insert the following text at the end of the first paragraphs in 6.3.2.3.52, 6.3.2.3.53, 6.3.2.3.54 and 6.3.2.3.55]

Non-transparent RSs and MRBS(s) may also be included as the diversity set members in addition to BS(s). Furthermore, this message may also be targeted for an RS to enable MHDO for a RS.

[Insert new subclause 6.3.2.3.90]

6.3.2.3.90 RS_Diversity_Set_Update_REQ message

The MRBS may transmit a RS_Diversity_Set_Update_REQ message to a RS when the RS is included into a diversity set of MS or if the status of the RS in a diversity set needs update.

RS_Diversity_Set_Update_REQ is transmitted on RS Basic CID.

| RS_DiversitySet_Update_ _REQ_Message_format() { | Size | Notes |
| --- | --- | --- |
| Management Message Type=XX | 8 bits |  |
| Mode | 4 bits | b0=1: Include RS to the diversity set 0: Remove RS from the diversity set b1=1: Include CID update b2=1: Include CQICH allocation b3=1: Include MAC Context parameters |
| If (b0 of Mode == 1) { |  |  |
| Temp BSID | 3 bits | 0..7: Diversity set member ID |
| Anchor_IND | 1 bit | 0b1: RS is the anchor access station 0b0: RS is a diversity set member |
| } |  |  |
| If (b1 of Mode == 1) { |  |  |
| N_CIDs | 8 bits | Number of CIDs associated with the MS who has this RS as a member of its diversity set |
| for (j=0 ; j<N_CIDs ; j++) { |  |  |
| New CID | 16 bits | New CID to be used after Diversity Set is updated. CIDs for the MS whose data to be forwarded in MDHO mode. The New CIDs shall be set according to the followings: the first CID in the list shall be basic CID; the second CID in the list shall be primary management CID, the third CID in the list shall be Secondary Management CID if secondary management connection is established for the MS at the current serving BS. The remaining CIDs shall be transport CIDs, multicast CIDs are enumerated by the ascending order of corresponding current SFIDs. The MS shall store the CIDs associated with the newly added RS and using the CIDs when the newly added RS becomes the anchor station |
| } |  |  |
| } |  |  |
| If (b2 of Mode ==1 ) { |  | CQICH allocation for the MS included |
| CQICH_ID | Variable | Index to uniquely identify the CQICH resource assigned to the MS after the MS switched to the new anchor station. |
| Feedback channel offset | 6 bits | Index to the fast-feedback channel region of the new Anchor BS marked by UIUC. |
| Period(p) | 2 bits | A CQI feedback is transmitted on the CQICH every 2^p frames. |
| Frame offset | 3 bits | The MS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the MS should start reporting in eight frames. |
| Duration | 3 bits | A CQI feedback is transmitted on the CQI channels indexed by the CQICH_ID for 10×2^d frames. If d == 0b000, the CQI-CH is de-allocated. If d == 0b111, the MS should report until the BS command for the MS to stop. |
| MIMO permutation feedback cycle | 2 bits | 0b00 = No MIMO and permutation mode feedback 0b01 = the MIMO and permutation mode indication shall be transmitted on the CQICH indexed by the CQICH_ID every four CQICH transmission opportunities allocated to the MS in this message. The first indication is sent on the 4th CQICH transmission opportunity allocated to the MS in this message. 0b10 = the MIMO mode and permutation mode indication shall be transmitted on the CQICH indexed by the CQICH_ID every eight CQICH transmission opportunities allocated to the MS in this message. The first indication is sent on the 8th CQICH transmission opportunity allocated to the MS in this message. 0b11 = the MIMO mode and permutation mode indication shall be transmitted on the CQICH indexed by the CQICH_ID every 16 CQICH transmission opportunities allocated to the MS in this message. The first indication is sent on the 16th CQICH transmission opportunity allocated to the MS in this message. |
| } |  |  |
| If (b3 of Mode == 1) { |  |  |
| MAC_setting | variable | TLV encoded info |
| } |  |  |
| Action time | 8 bits | Action time when the anchor station will be updated |
| } |  |  |

MAC setting

MAC_setting is a TLV variable that includes the MAC context that may be required by RS members of the Diversity Set in MDHO operation. The detailed content (SFID, QoS parameters, parameters for HARQ enabled traffic) is TBD.

[Insert new subclause 6.3.2.3.91]

6.3.2.3.91 RS_Diversity_Set_Update_RSP message

The RS may transmit a RS_Diversity_Set_Update_RSP message to a MRBS when the RS receives a RS_Diversity_Set_Update_REQ message.

RS_Diversity_Set_Update_RSP is transmitted on RS Basic CID.

| RS_Diversity_Set_Update_ _RSP_Message_format() { | Size | Notes |
| --- | --- | --- |
| Management Message Type=XX+1 | 8 bits |  |
| ACK IND | 2 bits | 00: RS accepts diversity set membership 01: RS accepts to be an anchor station 10: RS denies diversity set membership 11: Reserved |
| } |  |  |

[Subclause 6.3.22.3]

6.3.22.3 Macro diversity handover and fast BS switching

[Insert the following text at the end of subclause]

The MDHO handover mode may also involve non-transparent RS(s), and the procedures for MDHO operation can also be employed in the presence of RS(s) in the diversity set. RS(s) may be included to diversity set in addition to the MRBS(s) and BS(s). A MR-BS shall inform its subordinate RS(s) via RS Diversity Set Update REQ message when they are included in a diversity set. The RS may acknowledge with RS Diversity Set Update RSP message to the MRBS. The RSs may relay the MOB_BSHO_REQ/RSP, MOB_MSHO_REQ, and MOB_HO_IND messages between the MRBS and RSs/SS whenever they are generated by MSs or MRBSs.