| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Definition of CDMA code range for RS dedicated CDMA codes |  |
| Date Submitted | 2007-07-05 |  |
| Source(s) | Masato Okuda Fujitsu Laboratories LTD. Kamikodanaka 4-1-1, Nakahara-ku Kawasaki, Japan. 211-8588 Shashikant Maheshwari, Yousuf Saifullah, Haihong Zheng Nokia Siemens Networks | Voice: +81-44-754-2811 E-mail: okuda@jp.fujitsu.com E-mail : shashi.maheshwari@nsn.com |
| Re: | IEEE802.16j-07/19, “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | This contribution proposes text to describe a new CDMA ranging code rage for relay stations. |  |
| Purpose | To propose text to describe a new CDMA ranging code rage for relay stations. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Definition of CDMA code range for RS dedicated CDMA codes

Masato Okuda

Shashikant Maheshwari, Yousuf Saifullah, Haihong Zheng

# Introduction

As described in 6.3.10.3.5, RS may be assigned some dedicated ranging codes during initial network entry.

This contribution proposes definition of CDMA code range for those codes.

# Specific Text Changes

Change the fourth paragraph of 8.4.7.3 as indicated:

The number of available codes is 256, numbered 0..255. Each BS uses a subgroup of these codes, where the subgroup is defined by a number S, 0 <= S <= 255. The group of codes will be between S and ((S+O+N+M+L+P+Q) mod 256).

— The first N codes produced are for initial-ranging. Clock the PRBS generator 144 × (S mod 256) times to 144 × ((S + N) mod 256) – 1 times.

— The next M codes produced are for periodic-ranging. Clock the PRBS generator 144 × ((N + S) mod 256) times to 144 × ((N + M + S) mod 256) – 1 times.

— The next L codes produced are for bandwidth-requests. Clock the PRBS generator 144 × ((N + M + S) mod 256) times to 144 × ((N + M + L + S) mod 256) – 1 times.

— The next O codes produced are for handover-ranging. Clock the PRBS generator 144 × ((N + M + L + S) mod 256) times to 144 × ((N + M + L + O + S) mod 256) – 1 times.

— The next P codes produced are for RS initial-ranging. Clock the PRBS generator 144 × ((N + M + L + O + S) mod 256) times to 144 × ((P + N + M + L + O + S) mod 256) – 1 times

— The next Q codes produced are for RS dedicated codes. Clock the PRBS generator 144 × ((P + N + M + L + O + S) mod 256) times to 144 × ((Q + P + N + M + L + O + S) mod 256) – 1 times

Insert the following row into Table 353(.16e)/Table 601(Rev2) in 11.3 UCD management message encoding:

Table 353(.16e)/Table 601(Rev2)—UCD message encodings

| Name | Type (1 byte) | Length | Value (variable-length) |
| --- | --- | --- | --- |
| RS dedicated codes | TBA | 1 | Number of RS dedicated codes. Possible values are 0 – 255. a |

Change the Table 353(.16e)/Table 601(Rev2) in 11.3 UCD management message encoding as indicated:

Table 353(.16e)/Table 601(Rev2)—UCD message encodings

| Name | Type (1 byte) | Length | Value (variable-length) |
| --- | --- | --- | --- |
| Start of ranging codes group | 155 | 1 | Indicates the starting number, S, of the group of codes used for this UL. If not specified, the default value shall be set to zero. All the ranging codes used on this UL will be between S and ((S+O+N+M+L) mod 256) where N is the number of initial ranging codes M is the number of periodic ranging codes L is the number of BR codes O is the number of HO ranging codes P is the number of RS initial ranging codes Q is the number of RS dedicated codes The range of values is 0 ≤ S ≤ 255. |

# References

[1] IEEE 802.16j-07_026r4