| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Common ACKCH Region for Multiple RSs supporting HARQ Traffic |  |
| Date Submitted | 2007-07-05 |  |
| Source(s) | Israfil Bahceci, Senarath Gamini, Peiying Zhu, Wen Tong, Mo Han Fong, Derek Yu, David Steer, Mark Naden Nortel 3500 Carling Avenue Ottawa, Ontario K2H 8E9 | Voice: +613-763-1315 E-mail: wentong@nortel.com Voice: +613-765-8983 E-mail: pyzhu@nortel.com |
| Re: | IEEE 802.16j-07/019: “Call for Technical Comments Regarding IEEE Project 802.16j” |  |
| Abstract | A common ACK/NACK channel is proposed for HARQ assisted by multiple relay stations |  |
| Purpose | To incorporate the proposed text into the P802.16j Baseline Document (IEEE 802.16j-06/026r4) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Common ACKCH Channel Region for Multiple RSs supporting HARQ Traffic

Israfil Bahceci, Senarath Gamini, Peiying Zhu, Wen Tong, Hang Zhang, Mo Han Fong, Derek Yu, David Steer, Mark Naden

Nortel Networks

# Introduction

For a MR network, there are two options for enabling the HARQ traffic:

- Hop-by-hop HARQ: HARQ operation is on a per hop basis so that RSs at each hop are allocated resources for forwarding an HARQ subburst only after they can decode the HARQ burst correctly.

- End-to-end HARQ: Resources are initially allocated end-to-end for potential HARQ subburst forwarding at all hops.

In both options, it is necessary to allocate UL ACK/NACK channels from RSs to the MRBS. It is possible to have multiple RSs involved in both options. In this case, it may be necessary to allocate multiple ACK/NACK channels for each RS involved in the HARQ burst forwarding. However, that may consume significant amount of bandwidth in the UL region. In this contribution, we propose to use a common ACKCH region to transfer encoded ACK/NACK to the MRBS.

# Proposed Method

One ACK channel occupies a half subchannel, e.g., 24 data subcarriers within a supported tile structure. The even half subchannels consist of Tiles 0, 2 and 4, and the odd half subchannels consist of Tiles 1, 3, and 5. Each half subchannel is used for transmitting length-3 codewords over 8-ary alphabets, i.e., a 24-symbol length codeword of QPSK symbols are modulated onto 24 subcarriers in the half subchannel. In the baseline document 802.16-06/026r4, new code words are defined for transmitting a single ACK/NACK for a transparent RS. An RS may transmit C0 or C1 for acknowledging its decoding status.

If there are multiple transparent RSs involved with the same HARQ traffic, all RSs may transmit their decoding statuses over the same ACKCH Region using a set of orthogonal codewords. For N RSs, one needs 2N codewords (N codewords for ACK, N codewords for NAK). Using the orthogonal vectors in Table 298e (or 3101a), we can have at most 8 mutually orthogonal codewords, which can support 4 RSs ACK/NACK channel.

Table 1 provides ACK/NACK encoding for N=4 relay stations.

| Relay # | ACK/NAK | Vector indices per tile Even: Tiles 0, 2, 4 Odd: Tiles 1, 3, 5 | Codwords # |
| --- | --- | --- | --- |
| 0 | 0 (ACK) | 0,0,0 | C0 |
| 0 | 1 (NAK) | 1,1,1 | C1 |
| 1 | 0 (ACK) | 2,2,2 | C2 |
| 1 | 1 (NAK) | 3,3,3 | C3 |
| 2 | 0 (ACK) | 4,4,4 | C4 |
| 2 | 1 (NAK) | 5,5,5 | C5 |
| 3 | 0 (ACK) | 6,6,6 | C6 |
| 3 | 1 (NAK) | 7,7,7 | C7 |

# Proposed Text Changes

+++++++++++++++++++ start text ++++++++++++++++++++++++++++++++++++++++++=

[Insert the following text at the end of Subclause 6.3.17.4.2.2]

If multiple transparent RSs are involved in the HARQ process, they may share a common UL ACK channel region via the orthogonal codes from Table 463b. A common ACKCH region may support upto 4 RSs. Each RS involved in the HARQ process is assigned a 3-bit RS temporary ID for indicating ACK/NAK encodings.

[Insert Table 463b at the end of Subclause 8.4.5.4.13.1]

| RS Temporary ID | ACK/NAK | Vector indices per tile Even: Tiles 0, 2, 4 Odd: Tiles 1, 3, 5 | Codwords # |
| --- | --- | --- | --- |
| 0 | 0 (ACK) | 0,0,0 | C0 |
| 0 | 1 (NAK) | 1,1,1 | C1 |
| 1 | 0 (ACK) | 2,2,2 | C2 |
| 1 | 1 (NAK) | 3,3,3 | C3 |
| 2 | 0 (ACK) | 4,4,4 | C4 |
| 2 | 1 (NAK) | 5,5,5 | C5 |
| 3 | 0 (ACK) | 6,6,6 | C6 |
| 3 | 1 (NAK) | 7,7,7 | C7 |

[Change Table 149a as indicated]

| Syntax | Size | Notes |
| --- | --- | --- |
| Compact DL-MAP_IE(){ |  |  |
| DL-MAP Type = 7 | 3 bits |  |
| DL-MAP subtype | 5 bits |  |
| Temp RS_ID | 4 bits | Temporary ID for multiple transparent RSs assiting the HARQ. These RSs use common ACKCH region to acknowledge the decoding status |
| Reserved | 4 bits | Padding bits |
| N_CID_encoded | 4 bits | Number of CIDs for which RS uses the encoded ACK/NACK |
| N_CID_direct | 4 bits | Number of CIDs for which RS uses the direct feedback |
| For(i=0; i<N_CID_encoded + N_CID_direct; i++){ |  |  |
| RCID_IE(i) | 16 bits | The CIDs of the connections that RS shall monitor in the current frame |
| } |  |  |
| } |  |  |

+++++++++++++++++++++++++++End text +++++++++++++++++++++++++++++++++++